| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0139-v1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on general and misc. HLS topics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 15 proposals on reference picture lists cleanups (the agenda item 3.1.2 in JVET-S0137).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposed items

- Remove the support of the separate colour plane coding from VVCv1, by removing the syntax elements sps_separate_colour_plane_flag and sh_colour_plane_id and the related texts. If such functionality is desired in the future, the SPS and SH extension mechanisms could be used. Another option is to rename these syntax elements to be reserved bits. (S0052)

- Add a constraint: The value of sps_field_seq_flag shall be the same in all SPSs that are referred to by CLVSs in a CVS. (S0056 aspect 1)

- Add a constraint: When sps_field_seq_flag is equal to 1, all fields that compose the same frame shall refer to the same PPS. (S0056 aspect 2)

- Add a constraint: When sps_video_parameter_set_id is equal to 0, parameter sets referred to by all VCL NAL units shall have the same value of nuh_layer_id as the VCL NAL units. (S0075 aspect 1)

- When sps_video_parameter_set_id is equal to 0, the value of TotalNumOlss and NumLayersInOls[ 0 ] are both set equal to equal to 1. (S0075 aspect 2)

- Do the following for setting of PictureOutputFlag after receiving the first slice of a picture and before receiving other slices of the picture: (S0077)

- Add a constraint that when a picture contains a mix of RASL_NUT with NAL unit type(s) other than RADL_NUT or a mixed of RADL_NUT with other NAL unit type(s) other than RASL_NUT, then the following applies:

    - If interlace coding is not used (i.e., sps_field_seq_flag is equal to 0), there shall be at least one non-leading picture between the picture and its associated IRAP picture.

    - Otherwise, if interlace coding is used (i.e., sps_field_seq_flag is equal to 1), there shall be at least two non-leading pictures between the picture and its associated IRAP picture.

- Add a NOTE to clarify the derivation of PictureOutputFlag that when decoder received the first slice of the current picture and the NAL unit type of the slice is either RASL_NUT or RADL_NUT and the value of pps_mixed_nalu_types_in_pic_flag is equal to 1, if decoder has not received any non-leading picture from the last IRAP picture, then decoder can determine that the current picture is a RASL picture.

- Invoke the derivation process for NoOutputOfPriorPicsFlag only once per AU, that is, before the decoding process of the first picture in the CVSS AU but after parsing the slice header of the first slice of the current picture. When NoOutputOfPriorPicsFlag is equal to 1, the process of removing pictures stored in the DPB without outputting them is invoked only once per access unit. (S0078)

- Fix an asserted bug (the derivation of prevTid0Pic that is used in the POC decoding process does not consider the value of ph_non_ref_pic_flag), by adding the following constraint: (S0081)

Let picture picA and picB be two pictures with TemporalId equal to 0, it is a constraint of conformance bitstream that DiffPicOrderCnt( picB, picA ) shall not be greater than MaxPicOrderCntLsb / 2 when all of the following conditions are satisfied:

–	The values of nuh_layer_id of picA and picB are the same.

–	picA and picB are not RADL picture or RASL picture.

–	The value of ph_non_reference_picture_flag of picA and picB are both equal to 0.

–	picB is the first picture with TemporalId equal to 0 that follows picA in output order.

- When the current picture is a RADL picture, allow RASL pictures with pps_mixed_nalu_types_in_pic_flag is equal to 1 in active entries in RefPicList[ 0 ] or RefPicList[ 1 ]. (S0084)

- Do one of the following to the NOTE in the definition of RASL picture: (S0110 aspect 1)

- Remove the sentence "RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures."

- Change the sentence to be: "RASL pictures with mixed_nalu_types_in_pic_flag equal to 0 are not used as reference pictures for the decoding process of non-RASL pictures."

- When aud_irap_or_gdr_au_flag is equal to 1, in the AUD syntax, optionally signal the OLS index of the OLS represented by the CVS (aud_cvs_ols_idx) and the highest temporal sublayer present in the CVS (aud_cvs_htid_plus1), each conditioned by its own presence flag, with the following additional details: (S0087)

- aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0. aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

- aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

- When aud_cvs_htid_plus1 is present, it is used to derive the value of Htid. Otherwise, Htid is set by external means (like in VVC Draft 9).

- When aud_cvs_ols_idx is present, it is used to derive the value of TargetOlsIdx. Otherwise, TargetOlsIdx is set by external means (like in VVC Draft 9).

- The sub-bitstream extraction process is modified so that the highest temporal sublayer and the target OLS given as inputs to the process are used in modifying the AUD NAL units in the output sub-bitstream.

- Add a rule to select among OLSs matching BitstreamToDecode: When TargetOlsIdx is not provided via external means, it is set equal to the OLS with the lowest index that contains all layers of AU 0 of BitstreamToDecode and the highest number of output layers. (S0103 aspect 1)

- Do one of the following to enable setting of PictureOutputFlag equal to 0 for pictures in CVSS AUs (for correct picture output behavior when random accessing from a DRAP AU): (S0104)

- Add sps_pic_in_cvss_au_no_output_flag, which equal to 1 specifies that a picture in a CVSS AU referring to the SPS is not output (regardless of the ph_pic_output_flag value in the PH).

- Add an external-means-determined variable NoOutputBeforeDrapFlag for CVSS AUs. PictureOutputFlag is set equal to 0 for pictures in CVSS AUs with NoOutputBeforeDrapFlag equal to 1.

- In order to enable signalling the picture rate information without requiring signalling a complete HRD model, allow general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag to be both equal to 0, and when that condition is true, only signal the following HRD parameters syntax elements: num_units_in_tick, time_scale, general_same_pic_timing_in_all_ols_flag, fixed_pic_rate_general_flag[ i ], fixed_pic_rate_within_cvs_flag[ i ], and elemental_duration_in_tc_minus1[ i ]. (S0109)

- Add a prediction constraint on the pictures following a recovery point picture in both decoding and output order, as follows: When the current picture follows a recovery point picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that recovery point picture in output order or decoding order of the associated GDR picture in the layer with the nuh_layer_id equal to the nuh_layer_id of the reference picture. (S0114)

- Make the following changes: (S0124)

- Add derivation of TemporalId, ph_non_ref_pic_flag, and parameter sets to the decoding process for generating unavailable reference pictures.

- Remove the constraint that each inter-layer reference picture entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

- Invoke the decoding process for generating unavailable reference pictures for ILRPs or constrain that ILRPs shall not be equal to "no reference picture".

- Move the signalling of WPP enabling flag and the entry point offsets present flag from SPS to PPS, and enable the signalling of only the tile entry points when both tiles and WPP are enabled. (S0128)

# List of proposals

JVET-S0052 AHG9: Removing separate colour plane coding from VVCv1 [Y.-K. Wang, Z. Deng (Bytedance), J. Boyce (Intel)]

This contribution proposes to remove the support of the separate colour plane coding from VVCv1, by removing the syntax elements sps_separate_colour_plane_flag and sh_colour_plane_id and the related texts. If such functionality is desired in the future, the SPS and SH extension mechanisms could be used. Another option is to rename these syntax elements to be reserved bits.

It is asserted that there is no current or a planned profile that supports the separate colour plane coding mode, and currently at least the following aspects in the latest VVC draft text are not fully specified (or broken) when the separate colour plane coding mode is on:

- The specification in clause 6.3.3 of each of the following being a partitioning:

–	The division of each picture into subpictures

–	The division of each picture into tile columns

–	The division of each picture into tile rows

–	The division of each tile into CTUs

–	The division of each picture into slices

–	The division of each slice into CTUs

- The specification of the decoding order of NAL units in the bitstream in clause 7.4.2.4

- All places in the text that involve subpicture index, tile column index, tile row index, tile index, slice index, CTU index or address (need to be clearly specified to be of which colour plane)

The proposed text changes are included in an attachment to this contribution.

JVET-S0056 AHG9: On field coding [B. Choi, S. Wenger, S. Liu (Tencent)]

The following constraints on the flag sps_field_seq_flag in SPS are proposed:

- The value of sps_field_seq_flag shall be the same in all SPSs that are referred to by CLVSs in a CVS, so that all layers in a bitstream have the same field or frame structure.

- When sps_field_seq_flag is equal to 1, all fields that compose the same frame shall refer to the same PPS, so that all fields of the same frame have the same picture size, conformance window and tile partitioning.

JVET-S0075 AHG9: On constraints and inference values when VPS is not present [Hendry, S. Paluri, S. Kim (LGE)]

Aspect 1 of this contribution belongs to this category.

This contribution asserted that the following aspects in the current VVC spec text are not clear when VPS is not present for a CVS (i.e., the value of sps_video_parameter_set_id is equal to 0):

- It is currently constrained that, when VPS is not present in a CVS, the CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id). It is asserted the constraint is not complete. It does not provide constraint on non-VCL NAL units such as parameter sets referred to by the VCL NAL units in the CVS.

- ...

Based on the above assertions, the following changes are proposed:

- When VPS is not present in the CVS, the CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS and the parameter sets they referred to shall have the same value of nuh_layer_id).

- ...

JVET-S0077 AHG9: On derivation of PictureOutputFlag for RASL [Hendry, H. Jang, J. Nam, S. Paluri, S. Kim, J. Lim (LGE)]

It is asserted that in the current specification text, the derivation of PictureOutputFlag for RASL picture is unclear. Because a picture containing mix of RASL_NUT and RADL_NUT NAL units is considered as RASL picture in the decoding process as well, when receiving the first slice of a picture with nal_unit_type that is RASL_NUT or RADL_NUT, decoder cannot determine whether the picture a RASL picture or not. Consequently, decoder cannot set the value of PictureOutputFlag properly since the rest of slices of the picture, which the decoder has not received yet, can be slice with TRAIL_NUT or STSA_NUT.

To solve the above asserted problem, in this contribution, the following is proposed:

- Add a constraint that when a picture contains a mix of RASL_NUT with NAL unit type(s) other than RADL_NUT or a mixed of RADL_NUT with other NAL unit type(s) other than RASL_NUT, then the following applies:

–	If interlace coding is not used (i.e., sps_field_seq_flag is equal to 0), there shall be at least one non-leading picture between the picture and its associated IRAP picture.

–	Otherwise, if interlace coding is used (i.e., sps_field_seq_flag is equal to 1), there shall be at least two non-leading pictures between the picture and its associated IRAP picture.

- Add a NOTE to clarify the derivation of PictureOutputFlag that when decoder received the first slice of the current picture and the NAL unit type of the slice is either RASL_NUT or RADL_NUT and the value of pps_mixed_nalu_types_in_pic_flag is equal to 1, if decoder has not received any non-leading picture from the last IRAP picture, then decoder can determine that the current picture is a RASL picture.

JVET-S0078 AHG9: On derivation of NoOutputOfPriorPicsFlag [Hendry, S. Paluri, S. Kim (LGE)]

It is asserted that in the current VVC specification text, the derivation of NoOutputOfPriorPicsFlag has problem because it is invoked for each picture in the coded video sequence start access unit (CVSS AU) that is not AU 0. This would create a problem when the AU has more than one picture because starting from the second picture, the process would remove the previous picture from the DPB before the previous picture being outputted. This is asserted to be not the correct decoder behaviour.

In this contribution, it is proposed to invoke the derivation process for NoOutputOfPriorPicsFlag only once per AU, that is, before the decoding process of the first picture in the CVSS AU but after parsing the slice header of the first slice of the current picture. When NoOutputOfPriorPicsFlag is equal to 1, the process of removing pictures stored in the DPB without outputting them is invoked only once per access unit.

In the first revision of this contribution, some detail in the proposed spec text are updated to make it more accurate.

JVET-S0081 AHG9: On non-referenced picture and POC derivation [Hendry, H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

In the current VVC spec text, it is asserted that the POC decoding process has a problem related to the non-referenced picture. In the decoding process, PicOrderCntMsb is inferred based on the POC value of the picture that is designated as prevTid0Pic. The picture that is assigned to be prevTid0Pic for the current picture should be the same picture during encoding and decoding in order to derive the POC of the current picture consistently. However in the current draft, the assignment of prevTid0Pic does not take into account the value of ph_non_ref_pic_flag. Currently, the prevTid0Pic can be a picture that may be removed by a system which is possible when the value of ph_non_ref_pic_flag of the picture that is assigned to be prevTid0Pic by encoder is equal to 1.

To solve the asserted problem, in this contribution, the following bitstream conformance constraint is proposed:

Let picture picA and picB be two pictures with TemporalId equal to 0, it is a constraint of conformance bitstream that DiffPicOrderCnt( picB, picA ) shall not be greater than MaxPicOrderCntLsb / 2 when all of the following conditions are satisfied:

–	The values of nuh_layer_id of picA and picB are the same.

–	picA and picB are not RADL picture or RASL picture.

–	The value of ph_non_reference_picture_flag of picA and picB are both equal to 0.

–	picB is the first picture with TemporalId equal to 0 that follows picA in output order.

JVET-S0084 AHG9: On RPL constraint of RADL picture [Hendry (LGE)]

In the current VVC spec text, there is a constraint that disallows an active entry in reference picture list (RPL) of a RADL picture to refer to RASL picture. This constraint is asserted to be not accurate or not complete. A picture with mixed of RASL and RADL slices is also considered as RASL picture. Such RASL picture (i.e., the one that also contains RADL slice(s)) can still be referred to by RADL picture, as long as the reference is to part of the picture that is covered by the RADL slice.

This contribution proposed to update the existing RPL constraint for RADL picture as follows:

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture with pps_mixed_nalu_types_in_pic_flag is equal to 0.

  - A picture that precedes the associated IRAP picture in decoding order.

This contribution is related to the spec ticket #1035.

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 1 of this contribution belongs to this category.

Make either of the following changes:

random access skipped leading (RASL) picture: A coded picture for which there is at least one VCL NAL unit with nal_unit_type equal to RASL_NUT and other VCL NAL units all have nal_unit_type equal to RASL_NUT or RADL_NUT. NOTE – All RASL pictures are leading pictures of an associated CRA picture. When the associated CRA picture has NoOutputBeforeRecoveryFlag equal to 1, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to pictures that are not present in the bitstream. RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures. When sps_field_seq_flag is equal to 0, all RASL pictures, when present, precede, in decoding order, all non-leading pictures of the same associated CRA picture.

The note may also be updated as follows:

NOTE – All RASL pictures are leading pictures of an associated CRA picture. When the associated CRA picture has NoOutputBeforeRecoveryFlag equal to 1, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to pictures that are not present in the bitstream. RASL pictures with mixed_nalu_types_in_pic_flag equal to 0 are not used as reference pictures for the decoding process of non-RASL pictures. When sps_field_seq_flag is equal to 0, all RASL pictures, when present, precede, in decoding order, all non-leading pictures of the same associated CRA picture.

JVET-S0087 AHG9: On target OLS and sublayers for decoding [M. M. Hannuksela (Nokia)]

This contribution proposes signalling of the OLS represented by the CVS and the highest temporal sublayer present in the CVS. The contribution consists of the following pieces:

- The following syntax elements are proposed to be added into the AUD NAL unit, when aud_irap_or_gdr_au_flag is equal to 1:

  - Two gating flags, aud_htid_info_present_flag and aud_ols_info_present_flag

  - aud_cvs_htid_plus1, which is present when aud_htid_info_present_flag is equal to 1

- aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0.

- aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

  - aud_cvs_ols_idx, which is present when aud_ols_info_present_flag is equal to 1

- aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

- When aud_cvs_htid_plus1 is present, it is used to derive the value of Htid. Otherwise, Htid is set by external means (like in VVC Draft 9).

- When aud_cvs_ols_idx is present, it is used to derive the value of TargetOlsIdx. Otherwise, TargetOlsIdx is set by external means (like in VVC Draft 9).

- The sub-bitstream extraction process is modified so that the highest temporal sublayer and the target OLS given as inputs to the process are used in modifying the AUD NAL units in the output sub-bitstream.

JVET-S0103 AHG9: On OLS identification [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 1 of this contribution belongs to this category.

The contribution asserts that it is an issue that the TargetOlsIdx in some cases cannot be detected from the BitstreamToDecode itself and proposes two aspects to overcome the asserted issue.

- Add a rule to select among OLSs matching BitstreamToDecode: When TargetOlsIdx is not provided via external means, it is set equal to the OLS with the lowest index that contains all layers of AU 0 of BitstreamToDecode and the highest number of output layers.

- ...

JVET-S0104 AHG9: On DRAP output [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

This contribution proposes changes to the current specification of VVC to provide what is asserted as missing aspects related to the output of a bitstream providing for DRAP random access. The asserted issue is that the associated IRAP AU of the DRAP AU is output by the decoder.

Two options are proposed to avoid this behavior when accessing a bitstream at a DRAP AU. The first one consists of a flag in the SPS that indicates whether the CVSS AU is output irrespective of the value of ph_pic_output_flag in the picture header. The second option is to introduce external means to omit output of the CVSS AU when random accessing a DRAP AU.

JVET-S0109 AHG9: Signalling of picture rate [B. Heng, W. Wan (Broadcom)]

This contribution proposes to enable signalling picture rate information (num_units_in_tick, time_scale, etc.), without requiring the additional syntax necessary to signal a complete HRD model (bitrate, buffer size, delivery model, etc.).  It is asserted there are applications that can benefit from signaling picture rate without the overhead of also requiring a complete HRD model and also noted that this capability was present in AVC as well as HEVC.

Specifically, this contribution recommends removing the current restriction that general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag shall not be both equal to 0.

Additionally, when general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag are both equal to 0, it is proposed that general HRD parameter syntax that is relevant only when signalling an HRD model can also be skipped.

JVET-S0114 AHG9: On GDR RPL constraint [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes a prediction constraint on the pictures following the recovery point picture in both decoding and output order, as follows:

- When the current picture follows a recovery point picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that recovery point picture in output order or decoding order of the associated GDR picture in the layer with the nuh_layer_id equal to the nuh_layer_id of the reference picture.

JVET-S0124 AHG9: On unavailable reference pictures [V. Seregin, Y. He, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

In VVC, there is a decoding process for generating unavailable reference pictures when reference picture is equal to “no reference picture”. This contribution proposes:

- Add TemporalId, ph_non_reference_picture_flag, and parameter sets to the decoding process for generating unavailable reference pictures.

- Invoke decoding process for generating unavailable reference pictures for inter-layer reference picture or constrain that inter-layer reference picture shall not be equal to “no reference picture”.

- Remove the constraint that each inter-layer reference picture entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

JVET-S0128 AHG12: On entry point signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

In this contribution the following are proposed:

- Move signalling of WPP enabling and presence of entry point offsets from SPS to PPS.

- Enabling signalling of only the tile entry points when both tiles and WPP are enabled.