| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0170 |
| --- | --- |

| Title: | Use of ACT with IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Bug fix |  |  |
| Author(s) or Contact(s): | Steve Keating Kenji Kondo | Email: | steve.keating@sony.com kenji.kondo@sony.com |
| Source: | Sony Europe and Sony Corporation |  |  |

_____________________________

# Abstract

In the current draft of VVC, ACT is not included for IBC blocks.  It is asserted that this omission is a bug and it is proposed that ACT should be included for IBC blocks as is the case in the VTM software, i.e. the text should be aligned to the software.

# Problem statement

In the current draft of VVC, ACT is not included for IBC blocks whereas it is included for other intra and inter blocks.  It is included for IBC blocks in the VTM software.

# Draft text.

The draft text is shown below, relative to JVET-R2001-vA.docx.

8.6	Decoding process for coding units coded in IBC prediction mode

8.6.1	General decoding process for coding units coded in IBC prediction mode

…

The decoding process for coding units coded in IBC prediction mode consists of the following ordered steps:

…

3.	The residual samples of the current coding unit are derived as follows:

–	The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdx set equal to 0 as inputs, and the array resSamplesL as output.

–	When treeType is equal to SINGLE_TREE, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the width nTbW set equal to the chroma coding block width cbWidth / SubWidthC, the height nTbH set equal to the chroma coding block height cbHeight / SubHeightC and the variable cIdx set equal to 1 as inputs, and the array resSamplesCb as output.

–	When treeType is equal to SINGLE_TREE, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the width nTbW set equal to the chroma coding block width cbWidth / SubWidthC, the height nTbH set equal to the chroma coding block height cbHeight / SubHeightC and the variable cIdxset equal to 2 as inputs, and the array resSamplesCr as output.

–	When cu_act_enabled_flag[ xCb ][ yCb ] is equal to 1, the residual modification process for residual blocks using colour space conversion as specified in clause 8.7.4.6 is invoked with the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, the array rY set equal to resSamplesL, the array rCb set equal to resSamplesCb, and the array rCr set equal to resSamplesCr as inputs, and the output are modified versions of the arrays resSamplesL, resSamplesCb and resSamplesCr.

4.	The reconstructed samples of the current coding unit are derived as follows:

The additional text is copied from section 8.5.1 step 6 (General decoding process for coding units coded in inter prediction mode).

# Conclusion

It is proposed that ACT should be included for IBC blocks in the same way it is for inter blocks.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).