| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0173 |
| --- | --- |

| Title: | AHG9/AHG12: Subpicture related cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports seven asserted issues related to subpictures and proposes respective cleanups. The items are as follows.

Item 1) Clarify scope of SLI SEI message to be CVS instead of CLVS

Item 2) Clarify association of subpicture-nested SEI message to either layers or OLSs

Item 3) Include all layers of the OLS in the MinCr related constraint for subpictures

Item 4) Fix alignment constraint of independent subpictures by adding sps_log2_ctu_size_minus5

Item 5) Limit out-of-bounds inference of sli_ref_level_fraction[ i ][ j ]

Item 6) Resolving two editorial notes related to subpictures

Item 7) Fix unnecessary use of SPS syntax in semantics of sli_num_ref_levels_minus1

# Introduction

Item 1) Clarify scope of SLI SEI message to be CVS instead of CLVS

Text fragments in the SLI SEI message semantics limit its scope to a single layer (CLVS), e.g. the following:

When a subpicture level information SEI message is present for any picture of a CLVS, a subpicture level information SEI message shall be present for the first picture of the CLVS. The subpicture level information SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All subpicture level information SEI messages that apply to the same CLVS shall have the same content. A subpicture sequence consists of all subpictures within a CLVS that have the same value of subpicture index.

It is a requirement of bitstream conformance that, when a subpicture level information SEI message is present for a CLVS, the value of sps_subpic_treated_as_pic_flag[ i ] shall be equal to 1 for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive.

These fragments from the O-meeting predate later JVET decisions, for instance, at the Q-meeting to allow subpictures in multi-layer bitstreams and have a subpicture sub-bitstream extraction process that builds on top of the SLI SEI message using multi-layer bitstreams or at the R-meeting to allow a single OLS to entail layers with and without subpictures. While many parts of the specification now exhibit a clear multi-layer (CVS) scope of the SLI SEI message, some text fragments remained in the current draft specification leading to text ambiguities. It is therefore proposed to clean up the related text and clarify that

- an SLI SEI message present in an AU applies to all the pictures present in the AU, and

- only layers with subpictures shall have sps_subpic_treated_as_pic_flag[ i ] equal to 1, and

- CLVS occurrences are replaced with CVS.

The detailed changes are included in the attached draft specification document based on JVET-S0152-v5 alongside the proposed changes of the other items in this document.

Item 2) Clarify association of subpicture-nested SEI messages to either layers or OLSs

The scalable nesting SEI message associates nested SEI messages with subpictures and either layers or OLSs. However, a related value constraint for sn_num_subpics_minus1 in the current text speaks of an associated CLVS and a constraint on the value of sn_subpic_id_len_minus1 is also CLVS specific.

Similar as in the previous item 1, changes to the semantics of the scalable nesting SEI message are proposed to clarify the relation  of the sn_num_subpics_minus1 constraint to either layers or OLSs depending on the value of sn_ols_flag and extend the sn_subpic_id_len_minus1 constraint to the entire CVS as reproduced below.

sn_num_subpics_minus1 plus 1 specifies the number of subpictures to which the scalable-nested SEI messages apply. When sn_ols_flag is equal to 0, the value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS referred to by the pictures of the indicated layer. Otherwise, when sn_ols_flag is equal to 1, the value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS referred to by the pictures of the highest layer in the OLS with sps_num_subpics_minus1 greater than 0.

sn_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sn_subpic_id[ i ]. The value of sn_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of sn_subpic_id_len_minus1 shall be the same for all scalable nesting SEI messages that are present in a CLVS.

Item 3) Include all layers of the OLS in the MinCr related constraint for subpictures

JVET-Q0443 introduced MinCr related constraints to the SLI SEI message semantics to align with the bitstream constraints in A.4.2 as follows.

It is a requirement of bitstream conformance that the bitstream resulting from extracting the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to ref_level_idc[ i ] for i in the range of 0 to sli_num_ref_level_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	The sum of the NumBytesInNalUnit variables for AU 0 corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * ( Max(SubpicSizeY[ j ], fR * MaxLumaSr * OlsRefLevelFraction[ i ][ j ] ÷ 256 ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * OlsRefLevelFraction[ i ][ j ] ) ÷ ( 256 * MinCr ) for the value of SubpicSizeInSamplesY of AU 0, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0, at level sli_ref_level_idc[ i ], and MinCr is derived as indicated in A.4.2.

The green highlights indicate two possible issues.

First, SubpicSizeInSamplesY is not defined and the authors assume SubpicSizeY[ j ] is meant. It is proposed to remove the phrase “for the value of SubpicSizeInSamplesY of AU 0” to align with the original constraint in A.4.2 as the subpicture scope is already expressed earlier in the sentence.

Second, SubpicSizeY[ j ] describes the luma sample size of a single subpicture within one layer with subpictures of the OLS whereas in the related constraint in A.4.2, the variable AuSizeMaxInSamplesY[ 0 ] is used which is defined as follows.

Let numDecPics be the number of pictures in AU n. The variable AuSizeMaxInSamplesY[ n ] is set equal to PicSizeMaxInSamplesY * numDecPics.

When the non-subpicture related MinCr constraints were modified from picture size to max AU size (AuSizeMaxInSamplesY) a similar change was not applied appropriately applied to the subpicture part, probably because it was not clear the interaction between subpictures and layers, which has been already clarified.

Hence, to align with the original constraint, it is proposed to change the SLI SEI message semantics to include the samples of all layers for AU 0 through incorporating OlsRefLevelFraction[ i ][ j ] as follows.

–	The sum of the NumBytesInNalUnit variables for AU 0 corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * ( Max(SubpicSizeY[ j ]AuSizeMaxInSamplesY[ 0 ], fR * MaxLumaSr ) * OlsRefLevelFraction[ i ][ j ] ÷ 256 ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * OlsRefLevelFraction[ i ][ j ] ) ÷ ( 256 * MinCr ), where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0, at level sli_ref_level_idc[ i ], and MinCr and AuSizeMaxInSamplesY[ 0 ] are derived as indicated in A.4.2.

Item 4) Fix alignment constraint of independent subpictures by adding sps_log2_ctu_size_minus5

Currently, the semantics of sps_subpic_treated_as_pic_flag[ i ] contain constraints regarding the position of subpicture boundaries across layers with subpictures in an OLS. The constraints are intended to ensure that subpicture boundaries are aligned across layers. However, as currently specified, the constraints do not incorporate the CTU size of each layer. As sps_subpic_ctu_top_left_x/y[ i ] and sps_subpic_width/height_minus1[ i ] are signalled in in units of CtbSizeY, the constraint fails to fulfill its intended purpose.

Therefore, it is proposed to add a constraint on sps_log2_ctu_size_minus5 to the list of syntax elements that are required to be similar in targetAuSet as follows.

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and sps_log2_ctu_size_minus5 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

Item 5) Limit out-of-bounds inference of sli_ref_level_fraction[ i ][ j ]

sli_ref_level_fraction_minus1[ i ][ j ] is signalled as an u(8), where a value of 255 indicates that the j-th subpicture occupies the complete level limits associated with sli_ref_level_fraction[ i ]. The inference value of sli_ref_level_fraction_minus1[ i ][ j ] depends on the proportion of subpicture size and picture size as highlighted. In addition, the value is scaled by the proportion of the maximum luma picture size of general_level_idc and sli_ref_level_idc[ i ] as follows.

When not present, the value of sli_ref_level_fraction_minus1[ i ][ j ] is inferred to be equal to Ceil( 256 * SubpicSizeY[ j ] ÷ PicSizeInSamplesY * MaxLumaPs( general_level_idc ) ÷ MaxLumaPs( sli_ref_level_idc[ i ] ) − 1.

When the maximum picture size of sli_ref_level_idc[ i ] is smaller than the maximum picture size of general_level_idc, it could happen that the value of sli_ref_level_fraction[ i ] is inferred to be greater than 255 which seems counter-intuitive and exceeds the signaled value range of sli_ref_level_fraction[ i ].

It is therefore proposed to bound the inference rule to a maximum value of 255 as follows.

When not present, the value of sli_ref_level_fraction_minus1[ i ][ j ] is inferred to be equal to Max( 256, Ceil( 256 * SubpicSizeY[ j ] ÷ PicSizeInSamplesY * MaxLumaPs( general_level_idc ) ÷ MaxLumaPs( sli_ref_level_idc[ i ] ) )− 1.

Item 6) Resolving two editorial notes related to subpictures

There are two subpicture related editorial notes in the current draft specification. One is targeted at the minCR related constraints in the SLI SEI message wherein a local variable of the Profile-specific level limit definitions in A.4.2 is used.

[Ed. (RS): fR is a profile-specific variable defined in A.4.2.]

It is proposed to delete the note and clarify the origin of the variable by adding a reference to the A.4.2 as follows.

–	The sum of the NumBytesInNalUnit variables for AU 0 corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * ( Max(SubpicSizeY[ j ], fR * MaxLumaSr * OlsRefLevelFraction[ i ][ j ] ÷ 256 ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * OlsRefLevelFraction[ i ][ j ] ) ÷ ( 256 * MinCr ) for the value of SubpicSizeInSamplesY of AU 0, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0, at level sli_ref_level_idc[ i ], and MinCr and fR areis derived as indicated in A.4.2.

Another editorial note is related to the inference of sli_ref_level_fraction[ i ][ j ] wherein variable PicSizeInSamplesY is used instead of the variable PicSizeMaxInSamplesY.

[Ed. (HD): Shouldn't PicSizeInSamplesY here be PicSizeMaxInSamplesY as specified in clause A.4.2?]

It is proposed delete the note and replace PicSizeInSamplesY with PicSizeMaxInSamplesY and clarify that the picture size of the highest layer with subpictures in the OLS is taken.

Item 7) Fix unnecessary use of SPS syntax in semantics of sli_num_ref_levels_minus1

In order to not use unnecessary SPS syntax elements in the SLI SEI message semantics, replace sps_num_subpics_minus1 with sli_num_subpics_minus1 in the semantics of sli_num_ref_levels_minus1.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)