| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0175 |
| --- | --- |

| Title: | AHG9: Cleanup of picture rate information and HRD operation without timing SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution discusses several issues related to timing information in the bitstream, specifically PT SEI message, frame-field information SEI message and fixed_pic_rate_within_cvs_flag/fixed_pic_rate_general_flag in SPS/VPS.

First, two issues related to fixed_pic_rate_within_cvs_flag/fixed_pic_rate_general_flag are discussed. Currently splicing operations require rewriting of SPS or VPS to modify fixed_pic_rate_general_flag as required which is asserted to be undesirable. In addition, if used for a decoder operation without HRD timing SEI messages further constraints are needed as discussed in the proposal.

Second, the contribution proposes fixes for three asserted issues related to pt_display_elemental_periods_minus1 in the PT SEI message and display_elemental_periods_minus1 in the frame-field information SEI message. It is asserted that the current constraints on the value of pt_display_elemental_periods_minus1 are conflicting for cases where frames are displayed as fields and for multi-layer scenarios with different framerates.

In summary, the following asserted issues are addressed by this contribution

- Inconsistent use of fixed_pic_rate_within_cvs_flag and fixed_pic_rate_general_flag in 7.4.6.2 and D.9.6.

- fixed_pic_rate_general_flag may require to rewrite SPS and VPS at splicing points.

- Conflicting constraints on the values of pt_display_elemental_periods_minus1

- elementalOutputPeriods always set to 1 when PT SEI messages are not present in the bitstream but information conveyed by external means

- elementalOutputPeriods set incorrectly when output layer sets in OLS have different framerate.

- Decoder operation without HRD timing SEI messages when fixed picture rate is signalled.

# Introduction

The current specification contains two syntax elements to indicate that the picture output rate is constant within a CVS and across CVSs. The following two issues have been identified related to these two syntax elements (namely fixed_pic_rate_within_cvs_flag and fixed_pic_rate_general_flag) and are discussed in the following:

Issue 1)		Inconsistent use of fixed_pic_rate_within_cvs_flag and fixed_pic_rate_general_flag in 7.4.6.2 and D.9.6.

Issue 2)		fixed_pic_rate_general_flag may require to rewrite the SPS and VPS at splicing points.

Also, three aspects related to pt_display_elemental_periods_minus1 in the PT SEI message and display_elemental_periods_minus1 in the frame-field information SEI message are discussed:

Issue 3)		Conflicting constraints on the value of pt_display_elemental_periods_minus1

Issue 4)		elementalOutputPeriods always set to 1 when PT SEI messages are not present in the bitstream but information conveyed through external means

PT SEI messages may be conveyed through external means as noted in D.1

In clause C.5.2, the specification for presence of SEI messages are also satisfied when those messages (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified in this Specification.

However, in setting the value of elementalOutputPeriods, this seems to be inadequately addressed and elementalOutputPeriods is incorrectly set to 1 when the PT SEI message is not present regardless of whether the value is conveyed through external means.

Issue 5)		elementalOutputPeriods set incorrectly when output layer sets in OLS have different framerates

Issue 6) 	Decoder operation without HRD timing SEI messages when fixed picture rate is signalled

Besides, during the AHG meeting, JVET-S0109 proposed to use fixed_pic_rate_within_cvs_flag for operation without timing SEI messages for HRD. During the discussion it was mentioned that in HEVC a correspondence between POC and timing in a special case was signalled to allow such an operation instead. Although, it is a further option to align to the HEVC design, it is asserted that using fixed_pic_rate_within_cvs_flag should be enough under certain conditions and might work better for splicing point, for which the POC based solution does not provide a solution.

# Proposals

Proposed text changes related to Aspect 1) and 2)

Issue 1)		Inconsistent use of fixed_pic_rate_within_cvs_flag and fixed_pic_rate_general_flag in 7.4.6.2 and D.9.6.

Issue 2)		fixed_pic_rate_general_flag may require to rewrite the SPS and VPS at splicing points.

[7.4.6.2]

elemental_duration_in_tc_minus1[ i ] plus 1 (when present) specifies, when Htid is equal to i, the temporal distance, in clock ticks, between the elemental units that specify the HRD output times of consecutive pictures in output order as specified below. The value of elemental_duration_in_tc_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

When Htid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the bitstream (in output order) that is output, the value of the variable DpbOutputElementalInterval[ n ] is specified by:

DpbOutputElementalInterval[ n ] = DpbOutputInterval[ n ]  elementalOutputPeriods	(113)

where DpbOutputInterval[ n ] is specified in Equation C.16 and elementalOutputPeriods is specified as follows:

–	If a PT SEI message is present for picture n, elementalOutputPeriods is equal to the value of pt_display_elemental_periods_minus1 + 1.

–	Otherwise, elementalOutputPeriods is equal to 1.

When Htid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the bitstream (in output order) that is output, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C.1 (using the value of ClockTick for the CVS containing picture n) when one of the following conditions is true for the following picture in output order nextPicInOutputOrder that is specified for use in Equation C.16:

–	picture nextPicInOutputOrder is in the same CVS as picture n.

–	picture nextPicInOutputOrder is in a different CVS and fixed_pic_rate_general_flag[ i ] is equal to 1 in the CVS containing picture nextPicInOutputOrder, the value of ClockTick is the same for both CVSs, and the value of elemental_duration_in_tc_minus1[ i ] is the same for both CVSs.

When Htid is equal to i and fixed_pic_rate_within_cvs_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the CVS (in output order) that is output, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C.1 (using the value of ClockTick for the CVS containing picture n) when the following picture in output order nextPicInOutputOrder that is specified for use in Equation C.16 is in the same CVS as picture n. [Ed. (GJS): Should this paragraph be an "otherwise" clause that only needs to be considered when Htid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 0?]

The current text shown above uses the syntax element fixed_pic_rate_general_flag[ i ] of both the first and second CVS to determine whether the fixed output rate applies also across CVSs. However, it is asserted that the value of fixed_pic_rate_general_flag[ i ] of the first CVS should not be taken into account for determining whether the fixed output rate applies across the two CVSs. It is asserted that the first two instances of fixed_pic_rate_general_flag highlighted in green should be fixed_pic_rate_within_cvs_flag instead. See for instance in the following Figure 1 that the value of fixed_pic_rate_general_flag for the first AU should not be of importance for the splicing point as it should apply to a previous splicing point if any CVS preceded the first CVS. It is considered that the first CVS could have a value of fixed_pic_rate_general_flag[ i ] equal to 0 or 1 and still achieve a fixed picture rate across the two CVSs.

Figure 1: two cases of splicing: same GOP sizes (top), different GOP sizes (bottom).

[D.9.6]

For purposes of interpretation of the frame-field information SEI message, the following variables are specified:

–	FixedPicRateWithinCvsFlag is set equal to fixed_pic_rate_general_flag[ Htid ]. [Ed. (GJS): Check/correct/clarify.]

–	DisplayElementalPeriods is set equal to ElementalOutputPeriods.

In D.9.6 the value of FixedPicRateWithinCvsFlag should be set to fixed_pic_rate_within_cvs_flag instead of fixed_pic_rate_general_flag, see also the editors note.

However, although these issues can be easily fixed, the bigger problem is that using fixed_pic_rate_general_flag to indicate whether the fixed picture rate is kept across CVSs requires SPS and VPS rewriting when splicing operations are carried out. The original contribution in JCTVC-K0140 pointing out cross-CVS issues for fixed picture rates had a different apporach in mind that included adding “splicing/editing point” SEI messages, while during that meeting it was decided to generate two flags from the unique flag existing back then to differenciate the picture rate within the CVS and across the CVSs.

It is asserted that the described problem does require neither an additional flag at the SPS nor SEI messages that would make splicing having to change the bitstreams either by rewriting SPS/VPS or adding SEI messages. In fact as shown in Figure 1, it is a matter of “reordering delay” of the bistreams and the information is anyway present in the bitstream in dpb_parameters().

Therefore, the proposal here is to remove fixed_pic_rate_general_flag[ i ] from the ols_hrd_parameters() and derive from parameters signalled for the two CVSs including the dpb_parameters() whether the fixed picture rate is achieved across CVSs as follows.

| ols_hrd_parameters( firstSubLayer, maxSubLayers ) { | Descriptor |
| --- | --- |
| for( i = firstSubLayer; i <= maxSubLayers; i++ ) { |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |
| else if( hrd_cpb_cnt_minus1 = = 0 ) |  |
| low_delay_hrd_flag[ i ] | u(1) |
| if( general_nal_hrd_params_present_flag ) |  |
| sublayer_hrd_parameters( i ) |  |
| if( general_vcl_hrd_params_present_flag ) |  |
| sublayer_hrd_parameters( i ) |  |
| } |  |
| } |  |

[7.4.6.2]

sps_sublayer_cpb_params_present_flag equal to 1 specifies that the ols_hrd_parameters( ) syntax structure in the SPS includes HRD parameters for sublayer representations with TemporalId in the range of 0 to sps_max_sublayers_minus1, inclusive. sps_sublayer_cpb_params_present_flag equal to 0 specifies that the ols_hrd_parameters( ) syntax structure in the SPS includes HRD parameters for the sublayer representation with TemporalId equal to sps_max_sublayers_minus1 only. When sps_max_sublayers_minus1 is equal to 0, the value of sps_sublayer_cpb_params_present_flag is inferred to be equal to 0.

When sps_sublayer_cpb_params_present_flag is equal to 0, the HRD parameters for the sublayer representations with TemporalId in the range of 0 to sps_max_sublayers_minus1 − 1, inclusive, are inferred to be the same as that for the sublayer representation with TemporalId equal to sps_max_sublayers_minus1. These include the HRD parameters starting from the fixed_pic_rate_generalwithin_cvs_flag[ i ] syntax element till the sublayer_hrd_parameters( i ) syntax structure immediately under the condition "if( general_vcl_hrd_params_present_flag )" in the ols_hrd_parameters syntax structure.

[7.4.6.2]

fixed_pic_rate_general_flag[ i ] equal to 1 indicates that, when Htid is equal to i, the temporal distance between the HRD output times of consecutive pictures in output order is constrained as specified below. fixed_pic_rate_general_flag[ i ] equal to 0 indicates that this constraint may not apply.

When fixed_pic_rate_general_flag[ i ] is not present, it is inferred to be equal to 0.

fixed_pic_rate_within_cvs_flag[ i ] equal to 1 indicates that, when Htid is equal to i, the temporal distance between the HRD output times of consecutive pictures in output order is constrained as specified below. fixed_pic_rate_within_cvs_flag[ i ] equal to 0 indicates that this constraint may not apply.

When fixed_pic_rate_general_flag[ i ] is equal to 1, the value of fixed_pic_rate_within_cvs_flag[ i ] is inferred to be equal to 1.

elemental_duration_in_tc_minus1[ i ] plus 1 (when present) specifies, when Htid is equal to i, the temporal distance, in clock ticks, between the elemental units that specify the HRD output times of consecutive pictures in output order as specified below. The value of elemental_duration_in_tc_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

When Htid is equal to i and fixed_pic_rate_generalwithin_cvs_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the bitstream (in output order) that is output, the value of the variable DpbOutputElementalInterval[ n ] is specified by:

DpbOutputElementalInterval[ n ] = DpbOutputInterval[ n ]  elementalOutputPeriods	(113)

where DpbOutputInterval[ n ] is specified in Equation C.16 and elementalOutputPeriods is specified as follows:

–	If a PT SEI message is present for picture n, elementalOutputPeriods is equal to the value of pt_display_elemental_periods_minus1 + 1.

–	Otherwise, elementalOutputPeriods is equal to 1.

When Htid is equal to i and fixed_pic_rate_generalwithin_cvs_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the bitstream (in output order) that is output, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C.1 (using the value of ClockTick for the CVS containing picture n) when one of the following conditions is true for the following picture in output order nextPicInOutputOrder that is specified for use in Equation C.16:

–	picture nextPicInOutputOrder is in the same CVS as picture n.

–	picture nextPicInOutputOrder is in a different CVS and fixed_pic_rate_ generalwithin_cvs_flag[ i ] is equal to 1 in the CVS containing picture nextPicInOutputOrder, the value of ClockTick is the same for both CVSs, and the value of elemental_duration_in_tc_minus1[ i ] is the same for both CVSs and the value of max_num_reorder_pics[ i ], max_latency_increase_plus1[ i ], and max_dec_pic_buffering_minus1[ i ] are the same for both CVSs.

When Htid is equal to i and fixed_pic_rate_within_cvs_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the CVS (in output order) that is output, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C.1 (using the value of ClockTick for the CVS containing picture n) when the following picture in output order nextPicInOutputOrder

[D.9.6]

For purposes of interpretation of the frame-field information SEI message, the following variables are specified:

–	FixedPicRateWithinCvsFlag is set equal to fixed_pic_rate_generalwithin_cvs_flag[ Htid ]. [Ed. (GJS): Check/correct/clarify.]

–	DisplayElementalPeriods is set equal to ElementalOutputPeriods.

that is specified for use in Equation C.16 is in the same CVS as picture n.

Proposed text changes related to Aspect 3)

Issue 3)		Conflicting constraints on the value of pt_display_elemental_periods_minus1

Currently the value of pt_display_elemental_periods_minus1 is constrained as follows.

[D.4.2]

When fixed_pic_rate_within_cvs_flag[ TemporalId ] is equal to 0 or sps_field_seq_flag is equal to 1, the value of pt_display_elemental_periods_minus1 shall be equal to 0.

However, this seems to contradict with the following:

[D.9.6]

For purposes of interpretation of the frame-field information SEI message, the following variables are specified:

–	FixedPicRateWithinCvsFlag is set equal to fixed_pic_rate_general_flag[ Htid ].

–	DisplayElementalPeriods is set equal to ElementalOutputPeriods.

When general_progressive_source_flag and general_interlaced_source_flag in the vui_parameters( ) syntax structure are both equal to 1, for each picture associated with the vui_parameters( ) syntax structure, a frame-field information SEI message associated with the picture shall be present.

When a frame-field information SEI message is present, the following constraints apply:

–	The value of field_pic_flag shall be equal to sps_field_seq_flag.

–	If a PT SEI message is present for picture n, elementalOutputPeriods is equal to the value of pt_display_elemental_periods_minus1 + 1.

The above means that pt_display_elemental_periods_minus1 is equal to display_elemental_periods_minus1 in the frame-field information SEI message, when present.

[8.16.2 from JVET-R2007]

Table 14 – Interpretation of frame-field information syntax elements

Table 14 in JVET-R2007, as shown above, has values (see highlighted values) for fixed_pic_rate_within_cvs_flag[ TemporalId ] equal to 0 that would require pt_display_elemental_periods_minus1 to be greater than 0. Therefore, the constraint is asserted to be incorrect. In addition, the condition on sps_field_seq_flag is layer specific although the fixed_pic_rate_within_cvs_flag corresponds to an OLS.

[D.4.2]

When fixed_pic_rate_within_cvs_flag[ TemporalId ] is equal to 0 or sps_field_seq_flag is equal to 1, the value of any of the following applies pt_display_elemental_periods_minus1 shall be equal to 0:

–	sps_field_seq_flag is equal to 1 for the output layers in the OLS.

–	sps_field_seq_flag is equal to 0 for all output layers of the bitstream and there is not frame-field information SEI message present or display_fields_from_frame_flag is equal to 0 if a frame-field SEI message is associated with the AU and fixed_pic_rate_within_cvs_flag[ TemporalId ] is equal to 0.

Proposed text changes related to Aspect 4) and 5)

Issue 4)		elementalOutputPeriods always set to 1 when PT SEI messages are not present in the bitstream but information conveyed through external means

Issue 5)		elementalOutputPeriods set incorrectly when output layer sets in OLS have different framerate

As discussed, elementalOutputPeriods is incorrectly set to 1 when the PT SEI message is not present regardless of whether the value is conveyed through external means.

Further, when considering a multi-layer CVS with output layers having different framerates, it is not possible to have the same elementalOutputPeriods for all layers. See for instance the example in Figure 2 below with two layers.

Figure 2: two-layer bitstream with different frame rates.

Therefore, it is not possible to have a PT SEI message conveying correct information for both layers at once.

It is proposed to add a gating flag for pt_display_elemental_periods_minus1, so that its presence can be controlled and when not all layers have the same value of display_elemental_periods_minus1 the information is not present in a PT SEI message but, for instance, in the frame-field information SEI message or conveyed through external means.

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| [ … ] |  |
| pt_display_elemental_periods_present_flag | u(1) |
| if( pt_display_elemental_periods_present_flag ) |  |
| pt_display_elemental_periods_minus1 | u(4) |
| } |  |

pt_display_elemental_periods_present_flag equal to 1 specifies that pt_display_elemental_periods_minus1 is present in the picture timing SEI message. pt_display_elemental_periods_present_flag equal to 0 indicates that pt_display_elemental_periods_minus1 is not present in the picture timing SEI message and either information in frame-field information SEI messages, if present, or external means are used to derive the value of elementalOutputPeriods .

Further, it is proposed to change the derivation of elementalOutputPeriods as follows:

elemental_duration_in_tc_minus1[ i ] plus 1 (when present) specifies, when Htid is equal to i, the temporal distance, in clock ticks, between the elemental units that specify the HRD output times of consecutive pictures in output order as specified below. The value of elemental_duration_in_tc_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

When Htid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the bitstream (in output order) that is output, the value of the variable DpbOutputElementalInterval[ n ] is specified by:

DpbOutputElementalInterval[ n ] = DpbOutputInterval[ n ]  elementalOutputPeriods	(113)

where DpbOutputInterval[ n ] is specified in Equation C.16 and elementalOutputPeriods is specified as follows:

–	If a PT SEI message is present for picture n and pt_display_elemental_periods_present_flag equal to 1, elementalOutputPeriods is equal to the value of pt_display_elemental_periods_minus1 + 1.

–	Otherwise, if external means are provided to set the value of elementalOutputPeriods, elementalOutputPeriods is set equal to the value provided via external means.

Otherwise , if frame-field information SEI messages are present for at least one of the output pictures in the AU of picture n, the value of elementalOutputPeriods is set to the lowest value of display_elemental_periods_minus1 + 1 among all present frame-field information SEI messages.

–	Otherwise, elementalOutputPeriods is equal to 1.

Similarly, D.9.6 has to be changed accordingly as below only requiring DisplayElementalPeriods being equal to ElementalOutputPeriods when present in the PT SEI message.

[D.9.6]

For purposes of interpretation of the frame-field information SEI message, the following variables are specified:

–	FixedPicRateWithinCvsFlag is set equal to fixed_pic_rate_generalwithin_cvs_flag[ Htid ].

–	DisplayElementalPeriods is set equal to ElementalOutputPeriods.

When general_progressive_source_flag and general_interlaced_source_flag in the vui_parameters( ) syntax structure are both equal to 1, for each picture associated with the vui_parameters( ) syntax structure, a frame-field information SEI message associated with the picture shall be present.

When a frame-field information SEI message is present, the following constraints apply:

–	The value of field_pic_flag shall be equal to sps_field_seq_flag.

–	If a PT SEI message is present for picture n and pt_display_elemental_periods_present_flag equal to 1, elementalOutputPeriods and DisplayElementalPeriods isare equal to the value of pt_display_elemental_periods_minus1 + 1.

–	When general_progressive_source_flag is equal to 0 or general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be constrained as follows:

–	If general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, source_scan_type shall be equal to 0 (interlaced).

–	Otherwise, if general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, source_scan_type shall be equal to 1 (progressive).

–	Otherwise (general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0), source_scan_type shall be equal to 2 (unknown or unspecified).

Proposed text changes related to Aspect 6)

Issue 6) Decoder operation without HRD timing SEI messages when fixed picture rate is signalled

Although, it was discussed in review of JVET-S0109 that a solution as for HEVC could be used using the POCs to derive the output timing, it is considered that a solution using fixed_pic_rate_within_cvs_flag might be better than a HEVC-style solution as in such a case the output times can be also derived across CVSs, for which splicing does not result in contiguous POC values.

Following the proposal in JVET-S0109, i.e. using general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag to control derivation of output timing information without timing SEI message in the bitstream, further conditions are added as follows that are necessary to allow this operation mode of a decoder.

It is a bitstream requirement that when both general_nal_hrd_params_present_flag  and  general_vcl_hrd_params_present_flag are equal to 0, the following applies.

- elementalOutputPeriods is 1 in the AUs of the CVS and display_elemental_periods_minus1 is set to 0 in all frame-field information SEI messages.

- No pictures with ph_pic_output_flag equal to 0 are present in AUs of the CVS

Note that the first condition is necessary to be able to derive the output times directly from ClockTick and elemental_duration_in_tc_minus1 without further information from timing SEI messages and the second condition is required so that a constant output picture rate is achieved.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)