| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0176-v1 |
| --- | --- |

| Title: | AHG9: On the subpicture level information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the subpicture level information (SLI) SEI message syntax and semantics:

- Add sli_max_sublayers_minus1, sli_sublayer_info_present_flag, and a loop for sublayers for the signalled fractions and reference level indicators, to be consistent with the signalling of level information in PTL syntax structures and consistent with the other three HRD-related SEI messages (BP, PT, and DUI SEI messages) that all have a loop for sublayers.

  - When a piece of information for a lower sublayer is not present, it is inferred to be the same as the same piece of information for the next higher sublayer.

- Allow SLI SEI messages to be available either being in the bitstream or provided through an external means not specified in this Specification, to be consistent with parameter sets and the other three conformance/HRD related SEI messages.

- Change the persistency scope from one CVS to one or more CVSs, to be consistent with VPSs and SPSs wherein level information are or may be signalled.

- Change the definition of subpicture sequence to cover the case when sn_subpic_flag is equal to 1 and there is one or more layers with single subpicture per picture.

- Require that, when an SLI SEI message is present for a CVS, the value of sps_num_subpics_minus1 shall be the same for all the SPSs referenced by the pictures in the layers with multiple subpictures per picture.

- Specify the semantics of sli_num_subpics_minus1 in a way such that the syntax element is about the subpictures of the layers with multiple subpictures per picture.

- Add an array index, which identifies a subpicture sequence, to both the variables SubpicLevelIdc and SubpicLevelIdx, as well as to the arrays SubpicCpbSizeVcl, SubpicCpbSizeNal, SubpicBitRateVcl, and SubpicBitRateNal in the last set of constraints in the semantics of the SLI SEI message.

# Introduction

The existing design of the SLI SEI message in the latest VVC draft text has the following issues:

- The SLI SEI message only signals a single set of level information for subpicture sequences regardless of the value of the highest TemporalId. However, same as bitstreams with a single subpicture per picture, different sublayer representations may conform to different levels.

- The SLI SEI messages are specified to be available only through being in the bitstream. However, similarly as parameter sets and the other HRD related SEI messages, the SLI SEI message should also be made available through an external means.

- The persistency scope of the SLI SEI message is specified to be within one CVS. However, in most cases an SLI SEI message would apply to a number of consecutive CVSs, and often the entire bitstream.

- The definition of subpicture sequence does not cover the case when there is one or more layers with a single subpicture per picture.

- A constraint is missing to require that, when an SLI SEI message is present for a CVS, the value of sps_num_subpics_minus1 shall be the same for all the SPSs referenced by the pictures in the layers with multiple subpictures per picture. Otherwise, it does not make sense to require the value of sli_num_subpics_minus1 to be equal to the value of sps_num_subpics_minus1.

- The semantics of sli_num_subpics_minus1 does not work with the case when there is one or more layers with multiple subpictures per picture.

- The variables SubpicLevelIdc and SubpicLevelIdx need to be specified to be subpicture sequence specific, as different subpicture sequences extracted from the same original bitstream can conform to different levels. Same for the SubpicCpbSizeVcl, SubpicCpbSizeNal, SubpicBitRateVcl, and SubpicBitRateNal in the last set of constraints in the semantics of the SLI SEI message.

# Proposal

The proposal for addressing the issues above is summarized as above in the abstract. The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### D.7.1	Subpicture level information SEI message syntax

| subpic_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| sli_num_ref_levels_minus1 | u(3) |
| sli_cbr_constraint_flag | u(1) |
| sli_explicit_fraction_present_flag | u(1) |
| if( sli_explicit_fraction_present_flag ) |  |
| sli_num_subpics_minus1 | ue(v) |
| sli_max_sublayers_minus1 | u(3) |
| sli_sublayer_info_present_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| sli_alignment_zero_bit | f(1) |
| for( k = sli_sublayer_info_present_flag ? 0 : sli_max_sublayers_minus1; k <= sli_max_sublayers_minus1; k++ ) |  |
| for( i = 0; i <= sli_num_ref_levels_minus1; i++ ) { |  |
| sli_non_subpic_layers_fraction[ i ][ k ] | u(8) |
| sli_ref_level_idc[ i ][ k ] | u(8) |
| if( sli_explicit_fraction_present_flag ) |  |
| for( j = 0; j <= sli_num_subpics_minus1; j++ ) |  |
| sli_ref_level_fraction_minus1[ i ][ j ][ k ] | u(8) |
| } |  |
| } |  |

#### D.7.2	Subpicture level information SEI message semantics

The subpicture level information (SLI) SEI message contains information about the level that subpicture sequences in the set of CVSs of the OLSs to which the SEI message applies, denoted as targetCvss, bitstream conform to when testing the conformance of the extracted bitstreams containing the subpicture sequences according to Annex A. The OLSs to which the SLI message applies are also referred to as the applicable OLSs or the associated OLSs. A CVS in the remainder of this clause refers to a CVS of the applicable OLSs.

When a subpicture level information SEI message is present for any picture of a CLVS, a subpicture level information SEI message shall be present for the first picture of the CLVS. The subpicture level information SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All subpicture level information SEI messages that apply to the same CLVS shall have the same content.

When an SLI SEI message is present (either being in the bitstream or provided through an external means not specified in this Specification) for any AU of a CVS, an SLI SEI message shall be present for the first AU of the CVS. The SLI SEI message persists in decoding order from the current AU until the next AU containing an SLI SEI message for which the content differs from the current SLI SEI message or the end of the bitstream. All SLI SEI messages that apply to the same CVS shall have the same content.

Among the layers in the applicable OLSs, those for which the referenced SPSs have sps_num_subpics_minus1 greater than 0 are referred to as the multiSubpicLayers.

A subpicture sequence consists of all subpictures within a CLVS targetCvss that have the same value of subpicture index subpicIdxA and belong to the layers in the multiSubpicLayers and all subpictures within targetCvss that have subpicture index equal to 0 and belong to the layers in the applicable OLSs but not in the multiSubpicLayers. A subpicture sequence is said to be associated with and identified by the subpicture index subpicIdxA.

It is a requirement of bitstream conformance that, when a subpicture level information SEI message is present for a CLVS, when an SLI SEI message is present for a CVS, for all the SPSs referenced by the pictures in the multiSubpicLayers, the value of sps_num_subpics_minus1 shall be the same and the value of sps_subpic_treated_as_pic_flag[ i ] shall be equal to 1 for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive.

sli_num_ref_levels_minus1 plus 1 specifies the number of reference levels signalled for each of the sli_num_subpics_minus1 + 1 subpicture sequences.

sli_cbr_constraint_flag equal to 0 specifies that to decode the sub-bitstreams resulting from extraction of any subpicture sequence of the bitstream according to clause C.7 by using the HRD using any CPB specification in the extracted sub-bitstream, the hypothetical stream scheduler (HSS) operates in an intermittent bit rate mode. sli_cbr_constraint_flag equal to 1 specifies that the HSS operates in a constant bit rate (CBR) mode in such a case.

sli_explicit_fraction_present_flag equal to 1 specifies that the syntax elements sli_ref_level_fraction_minus1[ i ] are present. sli_explicit_fraction_present_flag equal to 0 specifies that the syntax elements sli_ref_level_fraction_minus1[ i ] are not present.

sli_num_subpics_minus1 plus 1 specifies the number of subpictures in the pictures of the CLVS in the multiSubpicLayers in targetCvss. When present, the value of sli_num_subpics_minus1 shall be equal to the value of sps_num_subpics_minus1 in the SPSs referred to by the pictures in the CLVS the multiSubpicLayers in targetCvss.

sli_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers in the subpicture sequences for which the level information is indicated in the SLI SEI message. The value of sli_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive.

sli_sublayer_info_present_flag equal to 1 specifies that the level information for subpicture sequences is present for sublayer representation(s) in the range of 0 to sli_max_sublayers_minus1, inclusive. sli_sublayer_info_present_flag equal to 0 specifies that the level information for subpicture sequences is present for the sli_max_sublayers_minus1-th sublayer representation. When not present, the value of sli_sublayer_info_present_flag is inferred to be equal to 0.

sli_alignment_zero_bit shall be equal to 0.

sli_non_subpic_layers_fraction[ i ][ k ] indicates the i-th fraction of the bitstream level limits associated with layers in the bitstream targetCvss that have sps_num_subpics_minus1 equal to 0 when Htid is equal to k. When vps_max_layers_minus1 is equal to 0 or when no layer in the bitstream has sps_num_subpics_minus1 equal to 0, sli_non_subpic_layers_fraction[ i ][ k ] shall be equal to 0. When k is less than sli_max_sublayers_minus1 and sli_non_subpic_layers_fraction[ i ][ k ] is not present, it is inferred to be equal to sli_non_subpic_layers_fraction[ i ][ k + 1 ].

sli_ref_level_idc[ i ][ k ] indicates a the i-th level to which each subpicture sequence conforms as specified in Annex A when Htid is equal to k. Bitstreams shall not contain values of sli_ref_level_idc[ i ][ k ] other than those specified in Annex A. Other values of sli_ref_level_idc[ i ][ k ] are reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the value of sli_ref_level_idc[ 0 ][ k ] shall be equal to the value of general_level_idc of the bitstream and that the value of sli_ref_level_idc[ i ][ k ] shall be less than or equal to sli_ref_level_idc[ m ][ k ] for any value of i greater than 0 and km greater than i. When k is less than sli_max_sublayers_minus1 and sli_ref_level_idc[ i ][ k ] is not present, it is inferred to be equal to sli_ref_level_idc[ i ][ k + 1 ].

sli_ref_level_fraction_minus1[ i ][ j ][ k ] plus 1 specifies the i-th fraction of the level limits, associated with sli_ref_level_idc[ i ] that the j-th subpicture conforms to as specified in clause A.4.1, for the subpictures with subpicture index equal to j in layers in targetCvss that have sps_num_subpics_minus1 greater than 0 when Htid is equal to k. When k is less than sli_max_sublayers_minus1 and sli_ref_level_fraction_minus1[ i ][ j ][ k ] is not present, it is inferred to be equal to sli_ref_level_fraction_minus1[ i ][ j ][ k + 1 ].

The variable SubpicSizeY[ j ] is set equal to ( sps_subpic_width_minus1[ j ] + 1 ) * CtbSizeY * ( sps_subpic_height_minus1[ j ] + 1 ) * CtbSizeY.

When not present, the value of sli_ref_level_fraction_minus1[ i ][ j ][ sli_max_sublayers_minus1 ] is inferred to be equal to Ceil( 256 * SubpicSizeY[ j ] ÷ PicSizeInSamplesY * MaxLumaPs( general_level_idc ) ÷ MaxLumaPs( sli_ref_level_idc[ i ][ sli_max_sublayers_minus1 ] ) − 1.

The variable LayerRefLevelFraction[ i ][ j ][ k ] is set equal to sli_ref_level_fraction_minus1[ i ][ j ][ k ] + 1.

The variable OlsRefLevelFraction[ i ][ j ][ k ] is set equal to sli_non_subpic_layers_fraction[ i ][ k ] + ( 256 − sli_non_subpic_layers_fraction[ i ][ k ] ) ÷ 256 * ( sli_ref_level_fraction_minus1[ i ][ j ][ k ] + 1 ).

The variables SubpicCpbSizeVcl[ i ][ j ][ k ] and SubpicCpbSizeNal[ i ][ j ][ k ] are derived as follows:

SubpicCpbSizeVcl[ i ][ j ][ k ] = Floor( CpbVclFactor * MaxCPB * OlsRefLevelFraction[ i ][ j ][ k ] ÷ 256)	(D.6)

SubpicCpbSizeNal[ i ][ j ][ k ] = Floor( CpbNalFactor * MaxCPB * OlsRefLevelFraction[ i ][ j ][ k ] ÷ 256)	(D.7)

with MaxCPB derived from sli_ref_level_idc[ i ][ k ] as specified in clause A.4.2.

The variables SubpicBitRateVcl[ i ][ j ][ k ] and SubpicBitRateNal[ i ][ j ][ k ] are derived as follows:

SubpicBitRateVcl[ i ][ j ][ k ] = Floor( CpbVclFactor * ValBR * OlsRefLevelFraction[ 0 ][ j ][ k ] ÷ 256)	(D.8)

SubpicBitRateNal[ i ][ j ][ k ] = Floor( CpbNalFactor * ValBR * OlsRefLevelFraction[ 0 ][ j ][ k ] ÷ 256)	(D.9)

Where the value of ValBR is derived as follows:

–	When bit_rate_value_minus1[ Htid ][ k ][ ScIdx ] is available in the respective HRD parameters in the VPS or SPS, ValBR is set equal to ( bit_rate_value_minus1[ Htid ][ k ][ ScIdx ] + 1) * 2( 6 + bit_rate_scale ), where Htid is the considered sublayer index and ScIdx is the considered schedule index.

–	Otherwise, ValBR is set equal to MaxBR derived from sli_ref_level_idc[ 0 ][ k ] as specified in clause A.4.2.

NOTE 1 – When a subpicture is extracted, the resulting bitstream has a CpbSize (either indicated in the VPS, SPS, or inferred) that is greater than or equal to SubpicCpbSizeVcl[ i ][ j ][ k ] and SubpicCpbSizeNal[ i ][ j ][ k ] and a BitRate (either indicated in the VPS, SPS, or inferred) that is greater than or equal to SubpicBitRateVcl[ i ][ j ][ k ] and SubpicBitRateNal[ i ][ j ][ k ].

It is a requirement of bitstream conformance that, for each value of k in the range of sli_max_sublayers_minus1, inclusive, each layer in the bitstream resulting from extracting the j-th subpicture sequence for j in the range of 0 to spssli_num_subpics_minus1, inclusive, from a layer that had sps_num_subpics_minus1 greater than 0 in the input bitstream to the extraction process, and conforming to a profile with general_tier_flag equal to 0 and level equal to sli_ref_level_idc[ i ][ k ] for i in the range of 0 to sli_num_ref_levels_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	Ceil( 256 * SubpicSizeY[ j ] ÷ LayerRefLevelFraction[ i ][ j ][ k ] ) shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1 for level sli_ref_level_idc[ i ][ k ].

–	The value of Ceil( 256 * ( sps_subpic_width_minus1[ j ] + 1 ) * CtbSizeY ÷ LayerRefLevelFraction[ i ][ j ][ k ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of Ceil( 256 * ( sps_subpic_height_minus1[ j ] + 1 ) * CtbSizeY ÷ LayerRefLevelFraction[ i ][ j ][ k ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of SubpicWidthInTiles[ j ] shall be less than or equal to MaxTileCols and of SubpicHeightInTiles[ j ] shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1 for level sli_ref_level_idc[ i ][ k ].

–	The value of SubpicWidthInTiles[ j ] * SubpicHeightInTiles[ j ] shall be less than or equal to MaxTileCols * MaxTileRows * LayerRefLevelFraction[ i ][ j ][ k ], where MaxTileCols and MaxTileRows are specified in Table A.1 for level sli_ref_level_idc[ i ][ k ].

It is a requirement of bitstream conformance that, when Htid is equal to k, the bitstream resulting from extracting the j-th subpicture sequence for j in the range of 0 to spssli_num_subpics_minus1, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to sli_ref_level_idc[ i ][ k ] for i in the range of 0 to sli_num_ref_levels_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	The sum of the NumBytesInNalUnit variables for AU 0 corresponding to the j-th subpicture sequence shall be less than or equal to FormatCapabilityFactor * ( Max(SubpicSizeY[ j ], fR * MaxLumaSr * OlsRefLevelFraction[ i ][ j ][ k ] ÷ 256 ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * OlsRefLevelFraction[ i ][ j ][ k ] ) ÷ ( 256 * MinCr ) for the value of SubpicSizeInSamplesY of AU 0, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0, at level sli_ref_level_idc[ i ][ k ], and MinCr is derived as indicated in A.4.2.

–	The sum of the NumBytesInNalUnit variables for AU n (with n greater than 0) corresponding to the j-th subpicture sequence shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) * OlsRefLevelFraction[ i ][ j ][ k ] ÷ ( 256 * MinCr ), where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to AU n, at level sli_ref_level_idc[ i ][ k ], and MinCr is derived as indicated in A.4.2.

The value of the subpicture sequence level indicator, SubpicLevelIdc[ j ][ k ], for the i-th subpicture sequence when Htid is equal to k, is derived as follows:

for( j = 0; j < sli_num_subpics_minus1; j++ )
	SubpicLevelIdc[ j ][ k ] = general_level_idc
	SubpicLevelIdx[ j ][ k ] = 0
	for( i = sli_num_ref_levels_minus1; i  >=  1; i− − )
		if( OlsRefLevelFraction[ i ][ j ][ k ]  <=  256 ) {	(D.10)
			SubpicLevelIdc[ j ][ k ] = sli_ref_level_idc[ i ][ k ]
			SubpicLevelIdx[ j ][ k ] = i
		}

The j-th subpicture bitstreamsequence conforming to a profile with general_tier_flag equal to 0 and a level equal to SubpicLevelIdc[ j ][ k ] shall obey the following constraints for each bitstream conformance test as specified in Annex C when Htid is equal to k, and the variable spLvIdx below is equal to SubpicLevelIdx[ j ][ k ]:

–	For the VCL HRD parameters, SubpicCpbSizeVcl[ i ][ spLvIdx ][ j ][ k ] shall be less than or equal to CpbVclFactor * MaxCPB, where CpbVclFactor is specified in Table A.3 and MaxCPB is specified in Table A.1 in units of CpbVclFactor bits.

–	For the NAL HRD parameters, SubpicCpbSizeNal[ i ][ spLvIdx ][ j ][ k ] shall be less than or equal to CpbNalFactor * MaxCPB, where CpbNalFactor is specified in Table A.3, and MaxCPB is specified in Table A.1 in units of CpbNalFactor bits.

–	For the VCL HRD parameters, SubpicBitRateVcl[ i ][ spLvIdx ][ j ][ k ] shall be less than or equal to CpbVclFactor * MaxBR, where CpbVclFactor is specified in Table A.3 and MaxBR is specified in Table A.1 in units of CpbVclFactor bits.

–	For the NAL HRD parameters, SubpicBitRateNal[ i ][ spLvIdx ][ j ][ k ] shall be less than or equal to CpbNalFactor * MaxBR, where CpbNalFactor is specified in Table A.3, and MaxBR is specified in Table A.1 in units of CpbNalFactor bits.

NOTE 2 – When a the j-th subpicture sequence is extracted with tIdTarget equal to k, the resulting bitstream has a CpbSize (either indicated in the VPS, SPS, or inferred) that is greater than or equal to SubpicCpbSizeVcl[ i ][ j ][ spLvIdx ][ j ][ k ] and SubpicCpbSizeNal[ i ][ j ][ spLvIdx ][ j ][ k ] and a BitRate (either indicated in the VPS, SPS, or inferred) that is greater than or equal to SubpicBitRateVcl[ i ][ j ][ spLvIdx ][ j ][ k ] and SubpicBitRateNal[ i ][ j ][ spLvIdx ][ j ][ k ], where spLvIdx is equal to SubpicLevelIdx[ j ][ k ].

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).