| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0207 |
| --- | --- |

| Title: | AHG9: Order of the sublayer_level_idc[ i ] syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Rickard Sjöberg Martin Pettersson Mitra Damghanian Jacob Ström | Email: | rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution claims that the sublayer_level_idc[ i ] syntax elements, which are used to signal level values for different temporal layers, are parsed in increasing order of i but inferred in decreasing order of i when not present.

The authors claim that these orders require a decoder to perform two scans over the sublayer_level_idc[ i ] array during decoding. A first scan to decode the syntax elements that are present in the bitstream, and then a second scan to infer the values of the missing syntax elements.

This contribution proposes that the decoding order of the sublayer_level_idc[ i ] syntax elements are inverted to be in decreasing order of i by replacing one line in the syntax table as shown below.

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) | Remove this line |
| for( i = maxNumSubLayersMinus1 1; i >= 0; i ) | Add this line |
| if( ptl_sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| ... |  |

If the change is made, the contribution additionally suggests minor editorial modifications to the semantics of sublayer_level_idc[ i ].

The authors believe that the current semantics of sublayer_level_idc[ i ] is somewhat confusing and that the proposed changes improves the clarity of the specification.

# Introduction

The current version of VVC, JVET-S0152-v5, supports signalling of level information for each temporal sublayer. This is done using the sublayer_level_idc[ i ] syntax element in the profile_tier_level() syntax structure as shown below. We then also show the semantic descriptions of general_level_idc and sublayer_level_idc[ i ].

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| ptl_num_sub_profiles | u(8) |
| for( i = 0; i < ptl_num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| } |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| ptl_sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( ptl_sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |

general_sub_profile_idc[ i ] specifies the indicator of the i-th interoperability metadata registered as specified by Rec. ITU-T T.35, the contents of which are not specified in this Specification. [Ed. (YK): Consider improving the wording of "interoperability metadata".]

The semantics of the syntax element sublayer_level_idc[ i ] is, apart from the specification of the inference of not present values, the same as the syntax element general_level_idc, but apply to the sublayer representation with TemporalId equal to i.

When not present, the value of sublayer_level_idc[ i ] is inferred as follows:

–	sublayer_level_idc[ maxNumSubLayersMinus1 ] is inferred to be equal to general_level_idc of the same profile_tier_level( ) structure,

–	For i from maxNumSubLayersMinus1 − 1 to 0 (in decreasing order of values of i), inclusive, sublayer_level_idc[ i ] is inferred to be equal to sublayer_level_idc[ i + 1 ].

# Problem statement

As can be observed in the previous section, the parsing of the sublayer_level_idc[ i ] syntax elements is in increasing order of i while the inference of them when not present is in decreasing order of i.

This conflicting order requires a decoder to perform two scans over the sublayer_level_idc[ i ] array. First one scan to decode the syntax elements that are present in the bitstream, and then a second scan to infer the values of the syntax elements that are not present. As you can see, the presence of each sublayer_level_idc[ i ] syntax element is controlled by the corresponding ptl_sublayer_level_present_flag[ i ] syntax element.

# Proposal

We propose that the decoding order of the sublayer_level_idc[ i ] syntax elements are inverted to be in decreasing order of i instead of increasing. This way, only one scan of the sublayer_level_idc[ i ] array is needed where for each value of i the corresponding syntax element is either decoded or inferred to an already available value.

## Proposed normative syntax change

Proposed text on top of JVET-S0152-v5 is shown below with added text in yellow and removed text in red strike through.

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| ptl_num_sub_profiles | u(8) |
| for( i = 0; i < ptl_num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| } |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| ptl_sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| for( i = maxNumSubLayersMinus1 1; i >= 0; i ) |  |
| if( ptl_sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |

## Proposed editorial semantics change

If the proposed normative change is made, we additionally suggest the following editorial improvement to the semantics of sublayer_level_idc[ i ] with added text in yellow and removed text in red strike through.

The semantics of the syntax element sublayer_level_idc[ i ] is, apart from the specification of the inference of not present values, the same as the syntax element general_level_idc, but apply to the sublayer representation with TemporalId equal to i.

When not present, the value of sublayer_level_idc[ i ] is inferred as follows:

–	sublayer_level_idc[ maxNumSubLayersMinus1 ] is inferred to be equal to general_level_idc of the same profile_tier_level( ) structure,

–	For i from maxNumSubLayersMinus1 − 1 to 0 (in decreasing order of values of i), inclusive, sublayer_level_idc[ i ] is inferred to be equal to sublayer_level_idc[ i + 1 ].

–	if i is equal to maxNumSubLayersMinus1, sublayer_level_idc[ i ] is inferred to be equal to general_level_idc of the same profile_tier_level( ) structure,

–	otherwise (i is less then maxNumSubLayersMinus1), sublayer_level_idc[ i ] is inferred to be equal to sublayer_level_idc[ i + 1 ]

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).