| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0208-v1 |
| --- | --- |

| Title: | AHG9: Proposed fixes to HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Ruoyang Yu Rickard Sjöberg Mitra Damghanian Jack Enhorn Jacob Ström Du Liu | Email: | martin.m.pettersson@ericsson.com ruoyang.yu@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com jacob.strom@ericsson.com du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following eight fixes to HLS:

- Remove the extra_sh_bits_struct() and extra_ph_bits_struct() structures and put their content directly in the SPS. The two structures are only used once in the VVC specification. ​ The fix is claimed to be editorial and asserted to simplify the specification text. ​

- Place the check whether pps_cu_chroma_qp_offset_list_enabled_flag is 1 within the same if-clause in the PPS syntax where pps_cu_chroma_qp_offset_list_enabled_flag is signaled. This is asserted to be a simple editorial fix.

- Modify the semantics of sh_alf_enabled_flag such that ​sh_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled and not applied to any colour component in a slice. Currently sh_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled and not applied for all colour components in a slice.

​​

- Modify the definition of "parameter" to also include VPS and APS:

parameter: A syntax element of a video parameter set (VPS), sequence parameter set (SPS), picture parameter set (PPS) or adaptation parameter set (APS), or the second word of the defined term quantization parameter.

- Replace the single occurrence of “prediction unit partition” in the VVC specification with “coding block”. The proponents believe this single occurrence of “prediction unit” is a remnant from text copied from HEVC and that this is reported in ticket #1040.  ​

- Modify the derivation process for the variable TotalNumOlss by removing the text in italics below:

if( vps_max_layers_minus1  = =  0 )
		TotalNumOlss = 1
else if( olsModeIdc == 4  | |  olsModeIdc = =  0  | |  olsModeIdc  = =  1 )
		TotalNumOlss = vps_max_layers_minus1 + 1	
else if( vps_ols_mode_idc  = =  2 )
		TotalNumOlss = vps_num_output_layer_sets_minus1 + 1

- Prefix all syntax elements in the dpb_parameters(), general_hrd_parameters(), ols_hrd_parameters(), and sublayer_hrd_parameters() structures with "hrd_"​​.

- Make sure all syntax elements for the SEI messages in the “SEI for coded bitstreams” specification have a prefix descriptive to the name of the SEI message. In particular, a suggestion is to prefix all syntax elements in the film grain characteristics SEI message with “film_grain_”, all syntax elements in the frame packing arrangement SEI message with “frame_packing_”,  all syntax elements in the mastering display colour volume SEI message with “mdcv_”, all syntax elements in the content light level info SEI message with “clli_”, all syntax elements in the region-wise packing SEI message with “rwp_”, and all syntax elements in the frame-field information SEI message with “ffi_”.

# Proposals

## Remove the extra_sh_bits_struct() and extra_ph_bits_struct() structures and put their content directly in SPS

The extra_sh_bits_struct() and extra_ph_bits_struct() structures are only used once in the VVC specification. It is proposed to simplify the specification by removing the two structures and put the content directly in SPS. The fix is claimed to be editorial.

Proposed changes on top of JVET-S0152-v5 are shown below with removed text in red strike through and added text in yellow:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_num_extra_ph_bits_bytes | u(2) |
| extra_ph_bits_struct( sps_num_extra_ph_bits_bytes ) |  |
| for( i = 0; i < (sps_num_extra_ph_bits_bytes * 8 ); i++ ) |  |
| sps_extra_ph_bit_present_flag[ i ] | u(1) |
| sps_num_extra_sh_bits_bytes | u(2) |
| extra_sh_bits_struct( sps_num_extra_sh_bits_bytes ) |  |
| for( i = 0; i < (sps_num_extra_sh_bits_bytes * 8 ); i++ ) |  |
| sps_extra_sh_bit_present_flag[ i ] | u(1) |
| … |  |
| } |  |

#### 7.3.2.17	Extra picture header bits structure syntax

| extra_ph_bits_struct( numExtraBtyes ) { | Descriptor |
| --- | --- |
| for( i = 0; i < ( numExtraBtyes * 8 ); i++ ) |  |
| extra_ph_bit_present_flag[ i ] | u(1) |
| } |  |

#### 7.3.2.18	Extra slice header bits structure syntax

| extra_sh_bits_struct( numExtraBtyes ) { | Descriptor |
| --- | --- |
| for( i = 0; i < ( numExtraBtyes * 8 ); i++ ) |  |
| extra_sh_bit_present_flag[ i ] | u(1) |
| } |  |

Move the following semantics to SPS:

#### 7.4.3.1.17	Extra picture header bits structure semantics

sps_extra_ph_bit_present_flag[ i ] equal to 1 specifies that the i-th extra bit is present in the PH syntax structures referring to the SPS. sps_extra_ph_bit_present_flag[ i ] equal to 0 specifies that the i-th extra bit is not present in the PH syntax structures referring to the SPS.

The variable NumExtraPhBits is derived as follows:

NumExtraPhBits = 0
for( i = 0; i < ( sps_num_extra_ph_bits_bytes * 8 ); i++ )
	if( sps_extra_ph_bit_present_flag[ i ] )		(92)
		NumExtraPhBits++

#### 7.4.3.1.18 	Extra slice header bits structure semantics

sps_extra_sh_bit_present_flag[ i ] equal to 1 specifies that the i-th extra bit is present in the slice headers of pictures referring to the SPS. sps_extra_sh_bit_present_flag[ i ] equal to 0 specifies that the i-th extra bit is not present in the slice headers of pictures referring to the SPS.

The variable NumExtraShBits is derived as follows:

NumExtraShBits = 0
for( i = 0; i < ( sps_num_extra_sh_bits_bytes * 8 ); i++ )
	if( sps_extra_sh_bit_present_flag[ i ] )		(93)
		NumExtraShBits++

## Move the check of pps_cu_chroma_qp_offset_list_enabled_flag

We propose to have the check whether pps_cu_chroma_qp_offset_list_enabled_flag is 1 within the same if-clause as the syntax element pps_cu_chroma_qp_offset_list_enabled_flag is signaled. This is asserted to be a simple editorial fix that saves one check for the decoder when pps_chroma_tool_offsets_present_flag is equal to 0.

Proposed changes on top of JVET-S0152-v5 are shown below with removed text in red strike through and added text in yellow:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| … |  |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pps_chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= pps_chroma_qp_offset_list_len_minus1; i++ ) { |  |
| pps_cb_qp_offset_list[ i ] | se(v) |
| pps_cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

## Modify the semantics of sh_alf_enabled_flag

The semantics of sh_alf_enabled_flag currently specifies that when sh_alf_enabled_flag is equal to 0, ALF is disabled and not applied for all color components. We believe this is incorrect since it implies that ALF may be applied to some but not all color components and that the intent is to say that that ALF is disabled and not applied to any color component.

Proposed changes on top of JVET-S0152-v5 are shown below with removed text in red strike through and added text in yellow:

sh_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be applied to Y, Cb, or Cr colour component in a slice. sh_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled and not applied for all to any colour components in a slice. When not present, the value of sh_alf_enabled_flag is inferred to be equal to ph_alf_enabled_flag.

## Modify definition of parameter to include VPS and APS

The current version of VVC defines a parameter as a syntax element of an SPS or PPS or the second word of the defined term quantization parameter. It is proposed to include VPS and APS in the definition of the term parameter.

Proposed changes on top of JVET-S0152-v5 are shown below with removed text in red strike through and added text in yellow:

3.101	parameter: A syntax element of a video parameter set (VPS), sequence parameter set (SPS), or picture parameter set (PPS) or adaptation parameter set (APS), or the second word of the defined term quantization parameter.

## Replace single occurrence of prediction unit partition with coding block

In the current version of VVC “prediction unit” is mentioned only once and never defined. We believe this single occurrence of ”prediction unit” is a remnant from text copied from HEVC. It is proposed to change “prediction unit partition” to “coding block” in the VVC specification. This has also been reported in ticket #1040.

Proposed changes on top of JVET-S0152-v5 are shown below with removed text in red strike through and added text in yellow:

#### 8.5.5.8	Derivation process for constructed affine control point motion vector prediction candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block,

- the reference index of the current prediction unit partition coding block refIdxLX, with X being 0 or 1,

## Simplify the derivation process for the variable TotalNumOlss

The current derivation of TotalNumOlss has three if-statements with the following three expressions:

if(vps_max_layers_minus1 == 0)

if(olsModeIdc == 4  | |  olsModeIdc = =  0  | |  olsModeIdc  = =  1)

if(vps_ols_mode_idc == 2)

However, when vps_max_layers_minus1 is 0, the vps_each_layer_is_an_ols_flag is inferred to be equal to 1 resulting in olsModeIdc being equal to 4. And when oldModeIdc is equal to 4, TotalNumOlss is equal to vps_max_layers_minus1 + 1 which is equal to 1 in this case. Thus, the number of if-statements can be reduced.

The proposed editorial modification on top of JVET-S0152-v5 is shown below with removed text in red strike through and added text in yellow:

vps_num_output_layer_sets_minus1 plus 1 specifies the total number of OLSs specified by the VPS when vps_ols_mode_idc is equal to 2.

The variable olsModeIdc is derived as follows:

if( !vps_each_layer_is_an_ols_flag )
	olsModeIdc = vps_ols_mode_idc		(39)
else
	olsModeIdc = 4

The variable TotalNumOlss, specifying the total number of OLSs specified by the VPS, is derived as follows:

if( vps_max_layers_minus1  = =  0 )
	TotalNumOlss = 1
else if( olsModeIdc == 4  | |  olsModeIdc = =  0  | |  olsModeIdc  = =  1 )
	TotalNumOlss = vps_max_layers_minus1 + 1	(40)
else if( vps_ols_mode_idc  = =  2 )
	TotalNumOlss = vps_num_output_layer_sets_minus1 + 1

## Prefix HRD syntax elements with “hrd_”

We propose to prefix all syntax elements in dpb_parameters(), general_hrd_parameters(), ols_hrd_parameters(), sublayer_hrd_parameters() structures in the VVC specification with "hrd_"​​ to make the scope and placement of the syntax elements clearer.

## Prefix syntax element in the “SEI for coded bitstreams” specification with descriptive prefix

Many of the SEI messages in the “SEI for coded bitstreams” specification (JVET-R2007-v2) have a prefix for their syntax elements that are descriptive for the SEI message, but many of the SEI messages are missing descriptive prefixes.

It is proposed to make sure all syntax elements for the SEI messages in the specification have a prefix descriptive to the name of the SEI message. In particular, a suggestion is to prefix all syntax elements in the film grain characteristics SEI message with “film_grain_”, all syntax elements in the frame packing arrangement SEI message with “frame_packing_”,  all syntax elements in the mastering display colour volume SEI message with “mdcv_”, all syntax elements in the content light level info SEI message with “clli_”, all syntax elements in the region-wise packing SEI message with “rwp_”, and all syntax elements in the frame-field information SEI message with “ffi_”.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).