| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0221 |
| --- | --- |

| Title: | AHG9: On virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Muhammed Coban Vadim Seregin Nan Hu Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Current specification allows virtual boundary to be located at picture boundary when the picture resolution is 8xH or Wx8, but it is disallowed for the other picture resolutions. This contribution proposes to disallow virtual boundaries located at picture boundary for all picture resolutions. Two approaches, signalling constraint or semantics restriction, are proposed as follows:

- Add explicit checking on the signaling of syntax elements as follows:

  - Condition sps_virtual_boundaries_enabled_flag on (sps_pic_width_max_in_luma_samples > 8 || sps_pic_height_max_in_luma_samples > 8), and infer sps_virtual_boundaries_enabled_flag to be equal to 0 when not present.

  - Condition sps_num_ver_virtual_boundaries on (sps_pic_width_max_in_luma_samples > 8).

  - Condition sps_num_hor_virtual_boundaries on (sps_pic_height_max_in_luma_samples > 8).

  - Condition ph_num_ver_virtual_boundaries on (pps_pic_width_in_luma_samples > 8).

  - Condition ph_num_hor_virtual_boundaries on (pps_pic_height_in_luma_samples > 8).

- Add bitstream conformance as follows:

  - It is a requirement of bitstream conformance that when the value of sps_pic_width_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_ver_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of sps_pic_height_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_hor_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of pps_pic_width_in_luma_samples is equal to or smaller than 8, the value of ph_num_ver_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of pps_pic_height_in_luma_samples is equal to or smaller than 8, the value of ph_num_hor_virtual_boundaries shall be equal to 0.

# Proposal

The virtual boundaries are restricted to be located inside a picture due to the following constraints:

- The value of sps_virtual_boundary_pos_x[ i ] shall be in the range of 1 to Ceil( sps_pic_width_max_in_luma_samples ÷ 8 ) − 1, inclusive.

- The value of sps_virtual_boundary_pos_y[ i ] shall be in the range of 1 to Ceil( sps_pic_height_max_in_luma_samples ÷ 8 ) − 1, inclusive.

In the example of 1920x1080 resolution: the value of sps_virtual_boundary_pos_x[ i ] is in the range of 1, 2, …, and 239, inclusive, and the value of sps_virtual_boundary_pos_y[ i ] is in the range of 1, 2, …, and 134, inclusive, as shown in Figure 1.

Note: After May meeting, it was agreed to change the syntax elements sps_virtual_boundary_pos_x[ i ] and sps_virtual_boundary_pos_y[ i ] to sps_virtual_boundary_pos_x_minus1[ i ] and sps_virtual_boundary_pos_y_minus1[ i ] as proposed in JVET-S0047. In the same example of 1920x1080, the value of sps_virtual_boundary_pos_x_minus1[ i ] is in the range of 0, 1, 2, …, 238, inclusive, and the value of sps_virtual_boundary_pos_y_minus1[ i ] is in the range of 0, 1, 2, …, 133, inclusive.

- (b)

Figure 1. 8x8 grid of virtual boundaries in a 1920x1080 picture (a) unit of 8 luma samples (b) unit of 1 luma sample

However, when the picture size is 8xH or Wx8, the virtual boundaries are respectively located at picture boundary and a position outside the picture. Besides, the ranges of the related syntax elements values are ordered from a larger value to a smaller value in the text, and a negative value for new syntax elements after JVET-S0047 was adopted is also discovered. These make the text not clear and not clean as follows:

- 8xH:  sps_virtual_boundary_pos_x[ i ] is in the range of 1 (outside the picture) to 0 (at picture boundary). After JVET-S0047 adoption, sps_virtual_boundary_pos_x_minus1[ i ] is in the range of 0 (outside the picture) to -1 (at picture boundary).

- Wx8: sps_virtual_boundary_pos_y[ i ] is in the range of 1 (outside the picture) to 0 (at picture boundary). After JVET-S0047 adoption, sps_virtual_boundary_pos_y_minus1[ i ] is in the range of 0 (outside the picture) to -1 (at picture boundary).

Figure 2 shows an example for 8xH picture, where the positions of vertical virtual boundaries are located at picture boundary and outside the picture when the value of sps_virtual_boundary_pos_x[ i ] is equal to 0 and 1, respectively.

- (b)

Figure 2. 8-sample grid of vertical virtual boundaries in an 8xH picture (a) unit of 8 luma samples (b) unit of 1 luma sample

Two approaches are proposed to address the issues: signalling restriction and semantics constraint.

Approach 1.1: Add conditions on the signaling of syntax elements to avoid virtual boundaries located at picture boundary and outside picture. The proposed spec change on top of VVC Draft 9, i.e., JVET-R2001-vA, is highlighted with yellow as shown below:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| if( sps_pic_width_max_in_luma_samples > 8 ¦¦ sps_pic_height_max_in_luma_samples > 8 ) |  |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| if( sps_pic_width_max_in_luma_samples > 8 ) |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_x[ i ] | ue(v) |
| if( sps_pic_height_max_in_luma_samples > 8 ) |  |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_y[ i ] | ue(v) |
| } |  |
| } |  |

sps_virtual_boundaries_enabled_flag equal to 1 specifies that disabling in-loop filtering across virtual boundaries is enabled and may be applied in the coded pictures in the CLVS. sps_virtual_boundaries_enabled_flag equal to 0 specifies that disabling in-loop filtering across virtual boundaries is disabled and not applied in the coded pictures in the CLVS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When sps_virtual_boundaries_enabled_flag is not present, the value of sps_virtual_boundaries_enabled_flag is inferred to be equal to 0.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| if( sps_virtual_boundaries_enabled_flag && !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| if( pps_pic_width_in_luma_samples > 8 ) |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_x[ i ] | ue(v) |
| if( pps_pic_height_in_luma_samples > 8 ) |  |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_y[ i ] | ue(v) |
| } |  |
| } |  |

Approach 1.2: Add bitstream conformance to avoid virtual boundaries located on picture boundary and outside picture. The proposed spec change on top of VVC Draft 9, i.e., JVET-R2001-vA, is highlighted with yellow as shown below:

sps_num_ver_virtual_boundaries specifies the number of sps_virtual_boundary_pos_x[ i ] syntax elements that are present in the SPS. When sps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance that when the value of sps_pic_width_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_ver_virtual_boundaries shall be equal to 0. sps_virtual_boundary_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundary_pos_x[ i ] shall be in the range of 1 to Ceil( sps_pic_width_max_in_luma_samples ÷ 8 ) − 1, inclusive. sps_num_hor_virtual_boundaries specifies the number of sps_virtual_boundary_pos_y[ i ] syntax elements that are present in the SPS. When sps_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance that when the value of sps_pic_height_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_hor_virtual_boundaries shall be equal to 0. sps_virtual_boundary_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundary_pos_y[ i ] shall be in the range of 1 to Ceil( sps_pic_height_max_in_luma_samples ÷ 8 ) − 1, inclusive. ph_num_ver_virtual_boundaries specifies the number of ph_virtual_boundary_pos_x[ i ] syntax elements that are present in the PH. When ph_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance that when the value of pps_pic_width_in_luma_samples is equal to or smaller than 8, the value of ph_num_ver_virtual_boundaries shall be equal to 0. ph_virtual_boundary_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundary_pos_x[ i ] shall be in the range of 1 to Ceil( pps_pic_width_in_luma_samples ÷ 8 ) − 1, inclusive. ph_num_hor_virtual_boundaries specifies the number of ph_virtual_boundary_pos_y[ i ] syntax elements that are present in the PH. When ph_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance that when the value of pps_pic_height_in_luma_samples is equal to or smaller than 8, the value of ph_num_hor_virtual_boundaries shall be equal to 0. ph_virtual_boundary_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundary_pos_y[ i ] shall be in the range of 1 to Ceil( pps_pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

This contribution requests to not only disallow virtual boundary outside picture, but also disallow virtual boundary at picture boundary. However it should be noted that allowing virtual boundary at picture boundary for all picture resolutions is another alternative approach if its use case can be justified.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).