| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0222-v1 |
| --- | --- |

| Title: | On CABAC parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

CABAC parameters comprise initValue and shiftIdx, where initValue parameters are used to determine initial probability estimates based on quantization parameter (QP) and slice type, and where shiftIdx parameters determine a learning rate for each context. These parameters have been retrained with each major release of VTM. The retraining process used the test set (common test conditions) and not a separate training set. This is not ideal as it may lead to overfitting to the test set. Furthermore, the current VVC draft text does not reflect values for initValue and shiftIdx used in any version of VTM. It is proposed to change the VVC draft to reflect values of CABAC parameters that are derived from an extended training set.

# Introduction

In recent versions of VTM, the training set was extended to include data from HDR CTC. For VTM 9.0 the set was further extended to include streams from non-4:2:0 CTC, such as to generate initial values for the contexts that are specific to non-4:2:0 cases (e.g., contexts related to the adaptive color transform (ACT)).

While the inclusion of HDR and non-4:2:0 cases lessens the dependency on SDR CTC data, it is desirable to use an even larger training set to further reduce this dependency.

# Experiments

In a first experiment, retraining of CABAC parameters is done using only bitstreams submitted for the SDR verification test (15 sequences, 7 QP values ranging from 26 to 50, for a total of 105 bitstreams). Results of this experiment are summarized below.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | -0.04% | -0.10% | -0.04% | -0.05% | 99% | 101% |
| Class A2 | -0.02% | 0.17% | 0.03% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.08% | -0.04% | 0.00% | 99% | 99% |
| Class C | 0.16% | 0.13% | -0.01% | 0.14% | 99% | 99% |
| Class E | 0.10% | -0.02% | -0.16% | 0.06% | 99% | 99% |
| Overall | 0.04% | 0.06% | -0.04% | 0.03% | 99% | 100% |
| Class D | 0.42% | 0.29% | 0.09% | 0.37% | 98% | 97% |
| Class F | 0.33% | 0.11% | 0.22% | 0.30% | 99% | 99% |
|  |  |  | Random access |  |  |  |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | -0.06% | 0.40% | 0.09% | 0.00% | 98% | 99% |
| Class A2 | -0.07% | 0.18% | 0.15% | -0.02% | 98% | 100% |
| Class B | 0.01% | 0.52% | 0.39% | 0.10% | 95% | 95% |
| Class C | 0.28% | 0.81% | 0.93% | 0.40% | 99% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.05% | 0.51% | 0.43% | 0.14% | 97% | 98% |
| Class D | 0.54% | 1.09% | 0.70% | 0.61% | 97% | 98% |
| Class F | 0.49% | 0.66% | 0.74% | 0.54% | 99% | 99% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.06% | 0.54% | 0.79% | 0.18% | 97% | 99% |
| Class C | 0.50% | 1.08% | 1.72% | 0.68% | 96% | 100% |
| Class E | 0.57% | 1.92% | 0.05% | 0.65% | 99% | 100% |
| Overall | 0.33% | 1.06% | 0.92% | 0.46% | 97% | 99% |
| Class D | 0.93% | 1.41% | 3.03% | 1.19% | 96% | 98% |
| Class F | 1.12% | 1.46% | 1.97% | 1.24% | 98% | 96% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.07% | 0.55% | 0.99% | 0.21% | 97% | 96% |
| Class C | 0.45% | 1.10% | 2.11% | 0.68% | 95% | 95% |
| Class E | 0.62% | 1.79% | 0.32% | 0.71% | 97% | 95% |
| Overall | 0.34% | 1.05% | 1.19% | 0.49% | 96% | 96% |
| Class D | 0.89% | 2.34% | 2.89% | 1.24% | 96% | 92% |
| Class F | 1.13% | 1.83% | 1.28% | 1.21% | 98% | 97% |

A few points can be made:

- The average loss for AI and RA test conditions is small (~0.05%)

- The average loss for low-delay test conditions is more substantial (~0.3%). The bitstreams in the training set were all encoded under RA test conditions, which may contribute to this higher loss.

- There is a small gain for classes A1 and A2 (~0.05%). This is unsurprising given that the bitstreams in the training set are mostly of the same resolution.

- The losses for lower resolution classes (C and D) are larger (~0.2-0.9%)

- The losses for class F are more substantial. This is unsurprising given that “screen content” tools are not used in the training set

In a second experiment, the training set is the combination of the training set used for VTM 9.0 and the training set used in the experiment above. Results are summarized below.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.04% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.11% | 0.01% | 0.01% | 100% | 99% |
| Class B | 0.00% | 0.03% | 0.03% | 0.01% | 100% | 99% |
| Class C | 0.01% | 0.01% | 0.07% | 0.01% | 100% | 100% |
| Class E | -0.01% | 0.06% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.04% | 0.03% | 0.01% | 100% | 100% |
| Class D | 0.02% | -0.06% | 0.17% | 0.03% | 99% | 99% |
| Class F | -0.01% | 0.04% | 0.09% | 0.01% | 100% | 99% |
|  |  |  | Random access |  |  |  |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | -0.01% | -0.07% | -0.14% | -0.03% | 98% | 99% |
| Class A2 | 0.00% | -0.14% | -0.12% | -0.03% | 97% | 99% |
| Class B | -0.01% | -0.09% | 0.02% | -0.01% | 96% | 95% |
| Class C | -0.01% | -0.16% | 0.01% | -0.02% | 99% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | -0.12% | -0.04% | -0.02% | 97% | 98% |
| Class D | -0.03% | -0.37% | -0.15% | -0.08% | 100% | 99% |
| Class F | 0.03% | -0.05% | 0.13% | 0.03% | 100% | 99% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.02% | -0.28% | 0.04% | -0.04% | 98% | 97% |
| Class C | -0.01% | -0.09% | 0.06% | -0.01% | 96% | 98% |
| Class E | -0.01% | -0.78% | -0.56% | -0.14% | 100% | 98% |
| Overall | -0.01% | -0.34% | -0.10% | -0.06% | 98% | 98% |
| Class D | 0.00% | 0.20% | 0.63% | 0.09% | 99% | 100% |
| Class F | 0.06% | -0.17% | 0.47% | 0.08% | 100% | 98% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.02% | -0.02% | 0.22% | 0.00% | 97% | 97% |
| Class C | 0.05% | 0.27% | 0.34% | 0.10% | 96% | 95% |
| Class E | -0.02% | -0.03% | -0.46% | -0.06% | 97% | 94% |
| Overall | 0.00% | 0.07% | 0.09% | 0.02% | 97% | 96% |
| Class D | 0.05% | 0.37% | 0.01% | 0.08% | 98% | 93% |
| Class F | 0.06% | -0.09% | 0.16% | 0.05% | 101% | 99% |

These results do not substantially deviate from VTM 9.0, even though the weight of the SDR CTC bitstreams is reduced in the training process.

# Recommendations

Tables 52 to 125 in JVET-R2001-vA describe the values of initValue and shiftIdx for each context. These tables need to be filled with appropriate values. It is recommended to use one the following sets of values:

- Set used in VTM 9.0

- Set used in the second experimented described in this document

A preference is expressed for the second set of values as it is less reliant on the test set in the training process.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).