| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0225 |
| --- | --- |

| Title: | AHG9: On AUD for sub-bitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two options to handle the AUDs for sub-bitstream extraction process,

- Option 1 proposes to add AUD or rewrite aud_irap_or_gdr_au_flag of the existing AUD when the associated extracted AU is an IRAP AU or GDR AU.

- Option 2 proposes to add AUD or rewrite aud_irap_or_gdr_au_flag  of the existing AUD when the associated AU is an IRAP AU or GDR AU and the target OLS has multi-layers; and either remove the existing AUD or rewrite aud_irap_or_gdr_au_flag  of the existing AUD when the associated AU is an IRAP AU or GDR AU and the target OLS contains only single layer.

# Proposals

JVET_S0102v1 proposed two options to handle AUD during the sub-bitstream extraction process.

Option 1 is to add an AUD when the associated extracted AU becomes an IRAP or GDR AU with two or more layers and there is no associated AUD present.

Option 2 is to mandate AUD for each AU that contains two or more IRAP or GDR pictures in any OLS and rewrite the aud_irap_or_gdr_au_flag of AUD when the associated extracted AU becomes an IRAP or GDR AU with two or more layers.

It is asserted that mandating AUD in each AU that contains two or more IRAP or GDR pictures in any OLS may not be desirable since such AU may not become an IRAP AU or GDR AU after extraction in some OLSs, or the corresponding AU may be removed in some OLSs.

It is asserted that option 1 may not be complete if option 1 and 2 are mutually exclusive, option 1 does not handle the case when an AU becomes an IRAP AU or GDR AU and there is associated AUD present but with the wrong aud_irap_or_gdr_au_flag value.

This contribution proposes two options to handle AUD for the extraction process. Option 1 handles AUD for the target OLS with single layer or multi-layers the same, while option 2 handles AUD differently depending on the target OLS constains single layer or multiple layers.

## Option 1

The proposed text is as followings:

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	When an AU is an IRAP AU or GDR AU and no associated AUD, add an AUD NAL unit of type AUD_NUT before the first VCL NAL unit of the AU with aud_irap_or_gdr_au_flag equal to 1. Otherwise for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for  an IRAP AU or GDR AU.

–	Remove from outBitstream all VCL NAL units for which all of the following conditions are true, and their associated non-VCL NAL units with nal_unit_type equal to PH_NUT, FD_NUT, SUFFIX_SEI_NUT, and PREFIX_SEI_NUT with PayloadType not equal to 0, 1, or 130:

–	nal_unit_type is equal to TRAIL_NUT, STSA_NUT, RADL_NUT, or RASL_NUT, or nal_unit_type is equal to GDR_NUT and the associated ph_recovery_poc_cnt is not equal to 0.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

## Option 2

In case it is desirable not to add AUD for each IRAP AU or GDR AU when the target OLS contains a single layer, the text is proposed as follows:

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	When the targte OLS contains more than one layer,

- For each AU that is an IRAP AU or GDR AU with no associated AUD, add a NAL unit of type AUD_NUT before the first VCL NAL unit of the AU with aud_irap_or_gdr_au_flag equal to 1.

- Otherwise for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for an IRAP AU or GDR AU.

–	Otherwise (the target OLS contains only one layer), remove the associated AUD from outBitstream for each AU, or for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for an IRAP AU or GDR AU.

–	Remove from outBitstream all VCL NAL units for which all of the following conditions are true, and their associated non-VCL NAL units with nal_unit_type equal to PH_NUT, FD_NUT, SUFFIX_SEI_NUT, and PREFIX_SEI_NUT with PayloadType not equal to 0, 1, or 130:

–	nal_unit_type is equal to TRAIL_NUT, STSA_NUT, RADL_NUT, or RASL_NUT, or nal_unit_type is equal to GDR_NUT and the associated ph_recovery_poc_cnt is not equal to 0.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).