| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0226-v1 |
| --- | --- |

| Title: | AHG9: On high level syntax for longer tap deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Nan Hu Muhammed Coban Vadim Seregin Jianle Chen Marta Karczewicz | Tel: Email: | akotra@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add:

- High level syntax flag to switch the application of long deblocking filters

- GCI flag to constrain long deblocking filter

# Introduction

One of the main differences between HEVC and VVC deblocking filter designs is the long deblocking filters which were mainly introduced in VVC to remove blocking artifacts belonging to larger blocks. Although the longer tap deblocking filter successfully removes blocking artifacts belonging to larger blocks, in some cases it may result in oversmoothing resulting in loss of structure/details.

JVET-R0300 reports that the current design of longer tap filters may result in overfiltering for few HDR sequences. Reportedly, the authors of this contribution also observed some oversmoothing for Marathon sequence which is currently a candidate sequence in the SDR verification test.

VTM-8.0 Frame 56, RA configuration              Longer filter turned off, Frame 56, RA configuration

One may also argue that the strength of long deblocking filters can be controlled by tuning the Tc and Beta offsets, but this may have the disadvantage of reducing the filtering strength also for normal strong and weak filters. This is because, in the current VVC deblocking design both long, strong and weak filters share the same Tc and Beta offsets.

Therefore, this contribution suggests including a high level syntax flag and a constraint flag which can turn off the application of long deblocking filters.

# Proposal

It is proposed to include a high level syntax flag either in the SPS (or) in the PPS which could further be overridden in the PH, SH. Moreover, a constraint flag is also proposed to be included which constraints the application of longer filters. It is reported that the changes to the low-level deblocking process are very minimal as can be seen in the proposed text changes of section 8.8.3.3 below.

## High level syntax flag in SPS

A high level syntax flag called as “sps_ltdf_enabled_flag” is introduced. The long filters are only selected when the sps_ltdf_enabled_flag is set to equal to 1. Also, a constraint flag called “no_ltdf_constraint_flag” is introduced.

The proposed text changes are added on top of JVET-S0152-v5 [1]

7.3.3.2	General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| …. |  |
| no_ladf_constraint_flag | u(1) |
| no_ltdf_constraint_flag | u(1) |
| …. |  |

no_ltdf_constraint_flag equal to 1 specifies that sps_ltdf_enabled_flag shall be equal to 0. no_ltdf_constraint_flag equal to 0 does not impose such a constraint.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| …. |  |
| sps_ladf_enabled_flag | u(1) |
| if( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| sps_ltdf_enabled_flag | u(1) |
| …. |  |

sps_ltdf_enabled_flag equal to 0 specifies that the longer filters for deblocking are disabled and not applied in decoding of pictures in the CLVS. sps_ltdf_enabled_flag equal to 1 specifies that the longer filters for deblocking are enabled and may be applied in decoding of pictures in the CLVS.

8.8.3.3	Derivation process of transform block boundary

….

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable cIdx specifying the colour component of the current coding block,

- a variable filterEdgeFlag,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs and maxFilterLengthPs,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are:

- the modified two-dimensional (nCbW)x(nCbH) array edgeFlags,

- the modified two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs, maxFilterLengthPs.

Depending on edgeType, the arrays edgeFlags, maxFilterLengthPs and maxFilterLengthQs are derived as follows:

- The variable gridSize is set as follows:

gridSize = cIdx  = =  0 ? 4 : 8		(1247)

- If edgeType is equal to EDGE_VER, the following applies:

- The variable numEdges is set equal to Max( 1, nCbW / gridSize ).

- For xEdge = 0..numEdges − 1 and y = 0..nCbH − 1, the following applies:

- The horizontal position x inside the current coding block is set equal to xEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If x is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32 and sps_ltdf_enabled_flag is equal to 1 then, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4 , maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or greater than 32 and sps_ltdf_enabled_flag is equal to 1 then, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If the width in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the width at chroma location ( xCb + x − 1,  yCb + y ) are both equal to or greater than 8 and sps_ltdf_enabled_flag is equal to 1 then, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

- The variable numEdges is set equal to Max( 1, nCbH / gridSize ).

- For yEdge = 0..numEdges − 1 and x = 0..nCbW − 1, the following applies:

- The vertical position y inside the current coding block is set equal to yEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If y is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32 and sps_ltdf_enabled_flag is equal to 1 then, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or greater than 32 and sps_ltdf_enabled_flag is equal to 1 then, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If the height in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the height in chroma samples of the transform block at chroma location ( xCb + x,  yCb + y − 1 ) are both equal to or greater than 8 and sps_ltdf_enabled_flag is equal to 1 then, the following applies:

    - If ( yCb + y ) % CtbHeightC is greater than 0, i.e. the horizontal edge do not overlap with the upper chroma CTB boundary, both maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3

    - Otherwise ( ( yCb + y ) % CtbHeightC is equal to 0, i.e. the horizontal edge overlaps with the upper chroma CTB boundary), maxFilterLengthPs[ x ][ y ] is set equal to 1 and maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

## High level syntax flag in PPS, PH and SH

7.3.3.2	General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| …. |  |
| no_ladf_constraint_flag | u(1) |
| no_ltdf_constraint_flag | u(1) |
| …. |  |

no_ltdf_constraint_flag equal to 1 specifies that pps_ltdf_enabled_flag, ph_ltdf_enabled_flag and sh_ltdf_enabled_flag shall be equal to 0. no_ltdf_constraint_flag equal to 0 does not impose such a constraint.

7.3.2.4	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | u(6) |
| …. |  |
| pps_deblocking_filter_control_present_flag | u(1) |
| if( pps_deblocking_filter_control_present_flag ) { |  |
| pps_deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_no_pic_partition_flag && pps_deblocking_filter_override_enabled_flag ) |  |
| pps_dbf_info_in_ph_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_luma_beta_offset_div2 | se(v) |
| pps_luma_tc_offset_div2 | se(v) |
| pps_ltdf_enabled_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| ….. |  |

pps_ltdf_enabled_flag equal to 0 specifies that the longer filters for deblocking are not applied for slices referring to the PPS unless the longer filter application is overridden by the picture header or slice header. pps_ltdf_enabled_flag equal to 1 specifies that the longer filters for deblocking are may be enabled for one or more slices referring to the PPS. When not present the value of pps_ltdf_enabled_flag is inferred to be equal to zero.

7.3.2.7	Picture header structure syntax

| …. |  |
| --- | --- |
| if( pps_deblocking_filter_override_enabled_flag && pps_dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_luma_beta_offset_div2 | se(v) |
| ph_luma_tc_offset_div2 | se(v) |
| ph_ltdf_enabled_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| ….. |  |

ph_ltdf_enabled_flag equal to 0 specifies that the longer filters for deblocking are disabled and not applied for the slices associated with the PH. ph_ltdf_enabled_flag equal to 1 specifies that the longer filters for deblocking are enabled and may be applied for the slices associated with the PH. When not present the value of ph_ltdf_enabled_flag is inferred to be equal to pps_ltdf_enabled_flag.

7.3.7.1	General slice header syntax

| …. |  |
| --- | --- |
| if( pps_deblocking_filter_override_enabled_flag && !pps_dbf_info_in_ph_flag ) |  |
| sh_deblocking_filter_override_flag | u(1) |
| if( sh_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| sh_deblocking_filter_disabled_flag | u(1) |
| if( !sh_deblocking_filter_disabled_flag ) { |  |
| sh_luma_beta_offset_div2 | se(v) |
| sh_luma_tc_offset_div2 | se(v) |
| sh_ltdf_enabled_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| sh_cb_beta_offset_div2 | se(v) |
| sh_cb_tc_offset_div2 | se(v) |
| sh_cr_beta_offset_div2 | se(v) |
| sh_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| …. |  |

sh_ltdf_enabled_flag equal to 0 specifies that the longer filters for deblocking are disabled and not applied in decoding of current slice. sh_ltdf_enabled_flag equal to 1 specifies that the longer filters for deblocking are enabled and may be applied in decoding of current slice. When not present the value of sh_ltdf_enabled_flag is inferred to be equal to ph_ltdf_enabled_flag.

8.8.3.3	Derivation process of transform block boundary

….

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable cIdx specifying the colour component of the current coding block,

- a variable filterEdgeFlag,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs and maxFilterLengthPs,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are:

- the modified two-dimensional (nCbW)x(nCbH) array edgeFlags,

- the modified two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs, maxFilterLengthPs.

Depending on edgeType, the arrays edgeFlags, maxFilterLengthPs and maxFilterLengthQs are derived as follows:

- The variable gridSize is set as follows:

gridSize = cIdx  = =  0 ? 4 : 8		(1247)

- If edgeType is equal to EDGE_VER, the following applies:

- The variable numEdges is set equal to Max( 1, nCbW / gridSize ).

- For xEdge = 0..numEdges − 1 and y = 0..nCbH − 1, the following applies:

- The horizontal position x inside the current coding block is set equal to xEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If x is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32 and sh_ltdf_enabled_flag is equal to 1 then, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4 , maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or greater than 32 and sh_ltdf_enabled_flag is equal to 1 then, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If the width in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the width at chroma location ( xCb + x − 1,  yCb + y ) are both equal to or greater than 8 and sps_ltdf_enabled_flag is equal to 1 then, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

- The variable numEdges is set equal to Max( 1, nCbH / gridSize ).

- For yEdge = 0..numEdges − 1 and x = 0..nCbW − 1, the following applies:

- The vertical position y inside the current coding block is set equal to yEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If y is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32 and sh_ltdf_enabled_flag is equal to 1 then, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or greater than 32 and sh_ltdf_enabled_flag is equal to 1 then, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If the height in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the height in chroma samples of the transform block at chroma location ( xCb + x,  yCb + y − 1 ) are both equal to or greater than 8 and sh_ltdf_enabled_flag is equal to 1 then, the following applies:

    - If ( yCb + y ) % CtbHeightC is greater than 0, i.e. the horizontal edge do not overlap with the upper chroma CTB boundary, both maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3

    - Otherwise ( ( yCb + y ) % CtbHeightC is equal to 0, i.e. the horizontal edge overlaps with the upper chroma CTB boundary), maxFilterLengthPs[ x ][ y ] is set equal to 1 and maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

# Conclusion

It is proposed to include a high level syntax flag and a constraint flag to control the application of long deblocking filters.

# References

- AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome                    Document JVET-S0152-v5, Y.K. Wang

- Additional fix for long luma deblocking decision Document JVET-R0300, K.Andersson, J. Enhorn

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).