| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0228 |
| --- | --- |

| Title: | Transform coefficients range extension for high bit depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Takeshi Chujoh Eiichi Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a format range extension design for high bit depth coding. In HEVC format range extension (RExt), the extended precision flag (extended_precision_processing_flag) is specified. When extended precision flag is equal to 1, it extends the transform coefficient range from 15 bit to BitDepth + 6 bit and modifies a bdShift value from 20 – BitDepth to 11. However, this extended precision flag was only supported in 16 bit profiles (Monochrome 16, Main 4:4:4 16 Intra, and Main 4:4:4 16 Still Picture) and variables in 10 bit case are also changed according to the flag. This contribution proposes to use BitDepth + 5 and bdShift to 10 in the range of 10 to 16 bit case in VVC. It is reported that high precision design provides better coding performance compared with the current bit depth precision in 12, 14 bit case as well as 16 bit case.

Problem Statement

In the current VVC, decoder processes scaling (IQ), LFNST, and IDCT to derive residual. In this process, the intermediate value range for transform is restricted to the so-called max transform coefficients range (called log2TransformRange) and several shifting operations are carried out to adjust the value range. Specifically >> bdShift (bdShift1) in scaling, >> 7 in LFNST, >> 7 in vertical IDCT and 20 – BitDepth (bdShift2) in horizontal IDCT.

The bdShift1 and bdShift2 are derived as follows.

bdShift1 = BitDepth + rectNonTsFlag +	( ( Log2(nTbW) + Log2(nTbH) ) / 2 ) − 5 + sh_dep_quant_used_flag

bdShift2 = Max (20 – BitDepth, 0)

These shift keeps the intermediate bit depth to 15 (log2TransformRange) as the first shift of bdShift1 changes bit depth dependent coefficient range to log2TransformRange before transform (LFNST, DCT) and the second shift of bdShift2 restores the log2TransformRange range to output bit depth. This design is beneficial in processing efficiency and proved sufficient for 10 bit source.

In HEVC, 16 bit profile modifies the designs for 10 to 16 bit source in the case that extended precision flag is equal to 1 in SPS extension.

log2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthY + 6 ) : 15

bdShift1 = BitDepthY + Log2( nTbS ) + 10 − log2TransformRange

bdShift2 = Max( 20 − BitDepth, extended_precision_processing_flag ? 11 : 0 )

However, there are two problems.

- This extended precision flag was only supported in 16 bit profile (Monochrome 16, Main 4:4:4 16 Intra, and Main 4:4:4 16 Still Picture). Thus 12, 14 bit profile doesn’t have sufficient precision (not-optimal).

- The range and bdShift value in 10 bit case are also changed according to the flag.

This contribution proposes to use BitDepth + 5 and bdShift to 10 in the range of 10 to 16 bit case.

Compared to the HEVC extended prediction design where the range is BitDepth + 6 and bdShift is equal to 11, it is asserted that 1 bit less range would be beneficial to reduce the multiplier burdens when we enable this feature in 12, 14 bit profile as well.

The transform coefficient decoding process of current VTM software shows in figure 1.

Figure 1: VVC transform coefficient decoding process

Proposal

This contribution proposes a design where higher bit depth is supported by bdShift2 being equal to Max (20 – BitDepth, 10) and changing the max transform coefficients  range to Max(BitDepth + 5, 15) This ensures that when input bit depth value is equal to 8, 9, and 10 bits, the shifting and coefficients range values are the same as current VVC spec. Table 1 shows the differences in clipping and shift values between current VVC, HEVC Extension , and proposed design.

log2TransformRange = extended_precision_processing_flag ? BitDepth + 5: 15

bdShift1 = 10 − log2TransformRange + rectNonTsFlag + ( ( Log2(nTbW) + Log2(nTbH) ) / 2 ) + sh_dep_quant_used_flag

bdShift2 = extended_precision_processing_flag ? 10 : Max(20 – BitDepth, 0)

Experiment

We implemented the proposed high precision coding in VTM and compared coding gain with the current basic precision. Additionally, we also conducted HEVC high precision coding for information.

In the testing, palette mode (PLT), LMCS, and Dependent Quantization (DepQuant) were disabled in the test followed by [1].

Test Condition:

Low QP (2, 7, 12, 17) Short test (17 frames for ClassA, 49 frames for others), CTC Class A, B, C.

Table 1. Proposed high precision design

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -0.13% | -0.03% | 0.00% |
| 14-bit | -0.14% | -0.03% | 0.00% |
| 16-bit | -0.22% | -0.05% | -0.01% |

Table 2. HEVC high precision mode (information)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -0.14% | -0.05% | -0.03% |
| 14-bit | -0.15% | -0.04% | -0.02% |
| 16-bit | -0.22% | -0.06% | -0.01% |

Anchor:  VTM-9.0 with RExt__HIGH_BIT_DEPTH_SUPPORT = 1.

Proposal: VTM-9.0 with RExt__HIGH_BIT_DEPTH_SUPPORT = 1, ExtendedPrecision = 1 and maxLog2TrDynamicRange = bitDepth + 5

HEVC high precision mode (information):

VTM-9.0 with RExt__HIGH_BIT_DEPTH_SUPPORT = 1, ExtendedPrecision = 1 and maxLog2TrDynamicRange = bitDepth + 6

As shown, the proposed high precision provides coding gain in 12, 14 bit depth as well as 16 bit depth.

Reference

[1]	Browne, S. Keating and K. Sharman, “High bit depth coding”, Joint Video Experts Team Document, JVET-R0351, teleconference, April 2020.

[2]	B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, Joint Video Experts Team Document, JVET-R2001, teleconference, April 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).