| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0232 |
| --- | --- |

| Title: | Removal of redundant clipping operations on inter prediction samples in forward luma mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Tsung-Chuan Ma, Che-Wei Kuo, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
|  | Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei | Email: | peter.chuang@mediatek.com |
| Source: | Kwai Inc. and MediaTek Inc. |  |  |

_____________________________

- Abstract

When LMCS is applied, for inter and CIIP modes, forward luma mapping is applied to convert the inter prediction samples of luma component before they are added with the corresponding residual samples. In this contribution, one mismatch is identified between VVC draft 9 and VTM-9.0 software on the clipping operations that are applied to luma inter prediction samples when the LMCS is applied. Specifically, in VVC draft 9, the luma inter prediction samples of the CIIP mode are clipped to the full range of internal bit-depth after the forward luma mapping is applied. Meanwhile, such clipping operation is not applied to luma prediction samples of inter modes. On the other hand, in VTM-9.0, the inter prediction samples of luma component for both inter and CIIP modes are always clipped after the forward luma mapping is applied. It is asserted that all the above clipping operations are redundant. For a more consistent and clean design, it is proposed to remove all those unnecessary clipping operations from the current specification and the reference software. Simulation results prove the redundancy of the removed clipping operations by providing bit-exact BD-rate performance.

Introduction

In VVC draft 9 [1], when luma mapping with chroma scaling (LMCS) is applied, luma motion compensated prediction samples generated from temporal reference pictures are maintained in the original domain (i.e., non-mapped). Because luma reconstruction is performed in the mapped domain, the luma inter prediction samples of inter modes and combined merge and intra prediction (CIIP) mode needs to be converted to the mapped domain before they are added with their residual samples. Additionally, in the current specification, one clipping is used to clip the luma inter prediction samples of the CIIP mode to the full dynamic range of inter coding bit-depth, i.e., [0, (1<<BitDepth)-1]. However, such clipping operation is not applied to the luma prediction samples of the inter modes. This is reflected in the VVC draft 9 as follows:

8.5.6.7 Weighted sample prediction process for combined merge and intra prediction

……

When cIdx is equal to 0 and sh_lmcs_used_flag is equal to 1, predSamplesInter[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are modified as follows:

idxY = predSamplesInter[ x ][ y ]  >>  Log2( OrgCW )
predSamplesInter [ x ][ y ] = Clip1( LmcsPivot[ idxY ] +	(1028)
	( ScaleCoeff[ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ idxY ] ) +
	( 1  <<  10 ) )  >>  11 )

8.7.5.2 Picture reconstruction with mapping process for luma samples

……

- Otherwise (CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ]  >>  Log2( OrgCW )
predMapSamples[ i ][ j ] = LmcsPivot[ idxY ] +
		( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + ( 1  <<  10 ) )  >>  11	(1232)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

On the other hand, in VTM-9.0 [2], the luma inter prediction samples for both inter and CIIP modes are always clipped based on the internal bit-depth after the forward luma mapping is applied, i.e., the function Clip1() is applied to both equations (1028) and (1232). It is notice that ticket #1027 was submitted in bug tracker for reporting the same mismatch.

Proposal

In the existing LMCS design, there is one bit-stream conformance constraint applied in the LMCS semantics, which requires that the total length of the 16 segments in the luma mapping function should not exceed the maximum value within the internal bit-depth range, as described as below:

7.4.3.20 Luma mapping with chroma scaling data semantics

……

It is a requirement of bitstream conformance that the following condition is true:

i=015lmcsCW[ i ]  <=  ( 1  <<  BitDepth ) − 1	(101)

The above constraint can guarantee that the resulting samples from the forward LMCS are always within the range of the internal bit-depth. This means that the mapped luma inter prediction samples of the inter and CIIP modes cannot go beyond such dynamic range. Therefore, all the existing clipping operations used for the forward luma mapping process of the inter and CIIP modes are redundant. For a more consistent and clean design, this contribution proposes to remove those unnecessary clipping operations from both text specification and reference software.

Specifically, for the text specification, the clipping function Clip1() is removed from equation (1028) for the luma mapping process of the CIIP mode, as described as:

8.5.6.7 Weighted sample prediction process for combined merge and intra prediction

……

When cIdx is equal to 0 and sh_lmcs_used_flag is equal to 1, predSamplesInter[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are modified as follows:

idxY = predSamplesInter[ x ][ y ]  >>  Log2( OrgCW )
predSamplesInter [ x ][ y ] = Clip1( LmcsPivot[ idxY ] +	(1028)
	( ScaleCoeff[ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ idxY ] ) +
	( 1  <<  10 ) )  >>  11 )

For the reference software, three lines of changes are made to avoid unnecessary clipping operations in the luma forward mapping functions Reshape::constructReshaper(), EncReshape::initLUTfromdQPModel() and EncReshape::constructReshaperLMCS(). The implementation of the proposed software changes based on VTM-9.0 are provided as attachment of the contribution.

Simulation results

The proposed changes were implemented on top of VTM-9.0 reference software and tested by following the common test conditions (CTCs) as specified in [3]. As seen below, the proposed change produces bit-exact BD-rate results as the anchors.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 98% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 104% | 102% |

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).