| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0232_r1 |
| --- | --- |

| Title: | Removal of redundant clipping operations on inter prediction samples in forward luma mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Tsung-Chuan Ma, Che-Wei Kuo, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
|  | Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Jie Chen, Ruling Liao, Yan Ye | Email: Email: | peter.chuang@mediatek.com jiechen.cj@alibaba-inc.com |
| Source: | Kwai Inc. and MediaTek Inc. and Alibaba Inc. |  |  |

_____________________________

In the update of the contribution, one overflow problem was identified for the forward luma mapping in 16-bit coding scenario, where the existing bit-stream conformance constraint of luma mapping function cannot always guarantee the output from the forward luma mapping function within the 16-bit dynamic range. Therefore, one additional solution (Solution #2) is proposed to align the current text specification with the reference software, i.e., applying the clipping operation to the mapped luma prediction samples of both CIIP and inter modes.

- Abstract

When LMCS is applied, for inter and CIIP modes, forward luma mapping is applied to convert the inter prediction samples of luma component before they are added with the corresponding residual samples. In this contribution, one mismatch is identified between VVC draft 9 and VTM-9.0 software on the clipping operations that are applied to luma inter prediction samples when the LMCS is applied. Specifically, in VVC draft 9, the luma inter prediction samples of the CIIP mode are clipped to the full range of internal bit-depth after the forward luma mapping is applied. Meanwhile, such clipping operation is not applied to luma prediction samples of inter modes. On the other hand, in VTM-9.0, the inter prediction samples of luma component for both inter and CIIP modes are always clipped after the forward luma mapping is applied. For a more consistent and clean design, two solutions are proposed in this contribution. In solution #1 it is proposed to remove all the clipping operations from the current specification and the reference software. In solution #2, it is proposed to align the current text specification to be same as reference software, i.e., clipping the output luma prediction samples of both the CIIP and inter modes.

Introduction

In VVC draft 9 [1], when luma mapping with chroma scaling (LMCS) is applied, luma motion compensated prediction samples generated from temporal reference pictures are maintained in the original domain (i.e., non-mapped). Because luma reconstruction is performed in the mapped domain, the luma inter prediction samples of inter modes and combined merge and intra prediction (CIIP) mode needs to be converted to the mapped domain before they are added with their residual samples. Additionally, in the current specification, one clipping is used to clip the luma inter prediction samples of the CIIP mode to the full dynamic range of inter coding bit-depth, i.e., [0, (1<<BitDepth)-1]. However, such clipping operation is not applied to the luma prediction samples of the inter modes. This is reflected in the VVC draft 9 as follows:

8.5.6.7 Weighted sample prediction process for combined merge and intra prediction

……

When cIdx is equal to 0 and sh_lmcs_used_flag is equal to 1, predSamplesInter[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are modified as follows:

idxY = predSamplesInter[ x ][ y ]  >>  Log2( OrgCW )
predSamplesInter [ x ][ y ] = Clip1( LmcsPivot[ idxY ] +	(1017)
	( ScaleCoeff[ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ idxY ] ) +
	( 1  <<  10 ) )  >>  11 )

8.7.5.2 Picture reconstruction with mapping process for luma samples

……

- Otherwise (CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ]  >>  Log2( OrgCW )
predMapSamples[ i ][ j ] = LmcsPivot[ idxY ] +
		( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + ( 1  <<  10 ) )  >>  11	(1219)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

On the other hand, in VTM-9.0 [2], the luma inter prediction samples for both inter and CIIP modes are always clipped based on the internal bit-depth after the forward luma mapping is applied, i.e., the function Clip1() is applied to both equations (1028) and (1232). It is notice that ticket #1027 was submitted in bug tracker for reporting the same mismatch.

Proposal

In the existing LMCS design, there is one bit-stream conformance constraint applied in the LMCS semantics, which requires that the total length of the 16 segments in the luma mapping function should not exceed the maximum value within the internal bit-depth range, as described as below:

7.4.3.20 Luma mapping with chroma scaling data semantics

……

It is a requirement of bitstream conformance that the following condition is true:

i=015lmcsCW[ i ]  <=  ( 1  <<  BitDepth ) − 1	(101)

The above constraint can guarantee that the resulting samples from the forward LMCS are always within the range of the internal bit-depth when the internal bit-depth is from 8-bit to 15-bit. This means that the mapped luma inter prediction samples of the inter and CIIP modes cannot go beyond such dynamic range. However, when the internal bit-depth is 16-bit, the existing bit-stream conformance constraint cannot always guarantee that the output of the forward luma mapping is within the range of the internal bit-depth. The reason is due to the value of the left shift (which is equal to 11) that is currently applied to calculate the slope of the forward luma mapping function is insufficient for 16-bit, as shown below:

ScaleCoeff[ i ] = ( lmcsCW[ i ] * (1 << 11 ) + ( 1 << ( Log2( OrgCW ) − 1 ) ) ) >> ( Log2( OrgCW ) )       (104)

Specifically, for 16-bit coding, the value of OrgCW is equal to (1<<12) such that left shift of 11-bit is smaller than the right shift of Log2(OrgCW) = 12. In such case, the rounding offset (1  <<  ( Log2( OrgCW ) − 1 )) could lead to the distance between the mapped luma sample and its starting pivot in the corresponding segment being larger than lmcsCW[i], as shown below:

ScaleCoeff[ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ idxY ] ) + ( 1  <<  10 ) )  >>  11

In details, when the value of predSamplesInter[ x ][ y ] − InputPivot[ idxY ] is equal to (OrgCW - 1) and lmcsCW[ i ] is an odd number, the value of the above equation is equal to (lmcsCW[i] + 1). If such case happens in the last valid segment, the mapped luma sample could be equal to (1<<16), i.e., beyond the range of [0, (1<<16)-1], according to the following equation.

predMapSamples[ i ][ j ] = LmcsPivot[ idxY ] +
		( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + ( 1  <<  10 ) )  >>  11	(1219)

The table below illustrates one example when such overflow problem happens.

| idxY | LmcsCW[idxY] | LmcsPivot[idxY] | InputPivot[idxY] |
| --- | --- | --- | --- |
| 0 | 7678 | 0 | 0 |
| 1 | 4096 | 7678 | 4096 |
| 2 | 4096 | 11774 | 8192 |
| 3 | 4096 | 15870 | 12288 |
| 4 | 4096 | 19966 | 16384 |
| 5 | 4096 | 24062 | 20480 |
| 6 | 4096 | 28158 | 24576 |
| 7 | 4096 | 32254 | 28672 |
| 8 | 4096 | 36350 | 32768 |
| 9 | 4096 | 40446 | 36864 |
| 10 | 4096 | 44542 | 40960 |
| 11 | 4096 | 48638 | 45056 |
| 12 | 4096 | 52734 | 49152 |
| 13 | 4096 | 56830 | 53248 |
| 14 | 4096 | 60926 | 57344 |
| 15 | 513 | 65022 | 61440 |

In the above example, when predSamples[ i ][ j ] is equal to 65535, the corresponding value of idxY is equal to 15 and the values of InputPivot[idxY] and LmcsPivot[idxY] are equal to 61440 and 65022, respectively. Based on equation (104), the value of ScaleCoeff[ idxY ] is equal to 257. Correspondingly, the calculated value of predMapSamples[ i ][ j ] is equal to 65536 based on equation (1219).

For a more consistent and clean design, this contribution proposes two solutions in the following.

Solution #1

In the first solution, it is proposed to remove those clipping operations from both text specification and reference software.

Specifically, for the text specification, the clipping function Clip1() is removed from equation (1028) for the luma mapping process of the CIIP mode, as described as:

8.5.6.7 Weighted sample prediction process for combined merge and intra prediction

……

When cIdx is equal to 0 and sh_lmcs_used_flag is equal to 1, predSamplesInter[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are modified as follows:

idxY = predSamplesInter[ x ][ y ]  >>  Log2( OrgCW )
predSamplesInter [ x ][ y ] = Clip1( LmcsPivot[ idxY ] +	(1028)
	( ScaleCoeff[ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ idxY ] ) +
	( 1  <<  10 ) )  >>  11 )

For the reference software, three lines of changes are made to avoid unnecessary clipping operations in the luma forward mapping functions Reshape::constructReshaper(), EncReshape::initLUTfromdQPModel() and EncReshape::constructReshaperLMCS(). The implementation of the proposed software changes based on VTM-9.0 are provided as attachment of the contribution.

Solution #2

In the second solution, it is proposed to align the text specification to be same as the reference software VTM-9.0 by applying clipping operations to the output of forward luma mapping process for both the CIIP and inter modes. Specifically, the function Clip1() is added to the equation () for the luma mapping process of inter modes, as described as:

8.7.5.2 Picture reconstruction with mapping process for luma samples

……

- Otherwise (CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ]  >>  Log2( OrgCW )
predMapSamples[ i ][ j ] = Clip1(LmcsPivot[ idxY ] +
		( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + ( 1  <<  10 ) )  >>  11)	(1219)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

Simulation results

The proposed changes of Solution #1 were implemented on top of VTM-9.0 reference software and tested by following the common test conditions (CTCs) as specified in [3].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 98% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 104% | 102% |

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).