| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0235 |
| --- | --- |

| Title: | AHG9: On pps_cabac_init_flag signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Vadim Seregin Yong He Yao-Jen Chang Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to move the flag indicating the presence of sh_cabac_init_flag from PPS to SPS.

# Proposal

In current VVC draft [1], presence of sh_cabac_init_flag is indicated in PPS. Encoders typically will choose to exercise this option at sequence level rather than picture level. It is proposed to move the signalling of presence of sh_cabac_init_flag to SPS.

## Proposed text changes

The changed texts are based on the latest VVC text in JVET-R2001-vA. Parts that have been added or modified are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts.

=========================VVC draft 9 changes BEGIN==============================

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| sps_bit_depth_minus8 | ue(v) |
| sps_cabac_init_present_flag | u(1) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| sps_entry_point_offsets_present_flag | u(1) |
| …. |  |

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | u(6) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| pps_cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| pps_num_ref_idx_default_active_minus1[ i ] | ue(v) |
| pps_rpl1_idx_present_flag | u(1) |
| … |  |

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| sh_picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( sh_slice_type != I ) { |  |
| if( ppssps_cabac_init_present_flag ) |  |
| sh_cabac_init_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && !pps_rpl_info_in_ph_flag ) { |  |
| … |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_cabac_init_present_flag equal to 1 specifies that sh_cabac_init_flag is present in slice headers referring to the SPS. sps_cabac_init_present_flag equal to 0 specifies that sh_cabac_init_flag is not present in slice headers referring to the SPS.

sps_entropy_coding_sync_enabled_flag equal to 1 specifies that a specific synchronization process for context variables is invoked before decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the SPS, and a specific storage process for context variables is invoked after decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the SPS. sps_entropy_coding_sync_enabled_flag equal to 0

…

#### 7.4.3.4 Picture parameter set RBSP semantics

…

pps_cabac_init_present_flag equal to 1 specifies that sh_cabac_init_flag is present in slice headers referring to the PPS. pps_cabac_init_present_flag equal to 0 specifies that sh_cabac_init_flag is not present in slice headers referring to the PPS.

pps_num_ref_idx_default_active_minus1[ i ] plus 1, when i is equal to 0, specifies the inferred value of the variable NumRefIdxActive[ 0 ] for P or B slices with sh_num_ref_idx_active_override_flag equal to 0,

…

=========================VVC draft 9 changes END==============================

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-P2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 18th Meeting: by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).