| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0237-v8 |
| --- | --- |

| Title: | Agenda and [Draft] report of the 19-21 June 2020 HLS AHG pre-meeting |  |  |
| --- | --- | --- | --- |
| Status: | Agenda and report document from the coordinators of the AHG meeting |  |  |
| Purpose: | Agenda and report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Ye-Kui Wang Bytedance Inc. 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +1 908 803 3888 yekui.wang@bytedance.com |
| Source: | Coordinators of the AHG pre-meeting |  |  |

_____________________________

# Summary

The high-level syntax (HLS) AHG pre-meeting, mostly in the scopes of AHGs 8, 9, and 12, held three days of teleconference meeting sessions on June 19-21, 2020 (in UTC time).

The sessions on each of the three days were held during the following time slots (in UTC time):

- 1300 (2 hours)

- [break – 20 minutes]

- 1520 (2 hours)

- [break – 3 hours 40 minutes]

- 1900 (2 hours)

- [break – 20 minutes]

- 2120 (2 hours)

- [break – 11 hours 40 minutes to the next session (when available)]

In addition to the breaks between sessions as noted above, five-to-ten-minute breaks were held at approximately the one-hour midpoint within each meeting session.

Altogether, 11 sessions were held (2 days with 4 sessions per day plus 3 sessions for the last day). The sessions are numbered as Sessions 9 to 19 in this report.

Approximately XXX people attended the AHG pre-meeting, and approximately 120+84 input documents were considered to be within the scope of the AHG, although 81 were (initially) discussed earlier at the May 27-28 HLS AHG meeting, and not all of the remaining ones were discussed at the AHG pre-meeting.

By the end of Day 3 of the AHG pre-meeting, approximately 157 (77%) of the 120+84 input documents had been reviewed, resulted in 76 recommendations for normative action, 24 editorial action items, and 23 revisits/deferred:

- (5) 3.1.1 High level tool control (24/(16+11)): 4 recommendations, 4 editorial action items, 2 revisits

- (3) 3.1.2 General and misc. HLS topics (31/(16+15)): 12+9 recommendations, 1 editor action item, 3 deferred/revisits

- (3) 3.1.3 General constraints information (GCI) (32/(25+7)): 11 recommendations, 6 editor action items, 3 revisits

- (0) 3.1.4 SPS, PPS, and APS cleanups (17/(9+8)): 8 recommendations, 1 editor action item

- (7) 3.1.5 PH and SH cleanups (10/(10+4)): 3 recommendations, 3 deferred/revisits

- (1) 3.1.6 Reference picture lists cleanups (10/(8+2)): 7 recommendations, 2 editor action items, 1 revisit

- (1) 3.1.7 Signalling of virtual boundaries (7/(2+5)): 4 recommendations, 2 editor action items, 1 revisit

- (23) 3.1.8 Hypothetical reference decoder (HRD) (5/(12+14)): 10 recommendations, 1 editor action item, 2 deferred

- (6) 3.1.9 DCI, VUI, and SEI (1/(3+3)): 1 recommendation, 1 revisit

- (7) 3.1.10 HLS editorial inputs (0/(4+3))

- (2) 3.2 Subpictures, slices, and tiles (12/(7+5)): 2 recommendations, 4 editor action items, 2 revisits

- (7) 3.3 Scalability and RPR cleanups (8/(8+5)): 5 recommendations, 3 editor action items, 3 deferred

# Attendance

Lists of the participants who attended the AHG meeting can be found in spreadsheet records attached with this report. The spreadsheets have not been fully pruned to identify unique participants, so the estimate of about XXX total participants is only a rather rough one. Each meeting session had an average of roughly AAA active connections on the call at any particular moment. This included a few participants who were using multiple connections in order to have dual screen display capability (e.g. for viewing or sharing meeting notes and a presentation, the participant list and/or chat session messages).

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

In order to manage the teleconference resources effectively and reduce potential problems with security and attendance recording, a password was used for the meeting sessions. The password was distributed only to those who had registered for the JVET meeting.

# High-level syntax (HLS) proposals (120+84)

## AHG9: General high-level syntax (105+74)

### High level tool control (16+11)

JVET-S0144 AHG9: A summary of proposals on high level tool control [L. Zhang, Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0835 UTC on 27 May and in the HLS AHG pre-meeting at 1305 UTC on 19 June.

This contribution provides a summary of the 25 proposals on High level tool control (the agenda item 3.1.1 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following actions are proposed:

- Removal of SPS BDOF/DMVR/PROF control present flag

Remove the syntax elements sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag and instead condition the presence of ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag on the existing syntax elements sps_bdof_enabled_flag, sps_dmvr_enabled_flag and sps_affine_prof_enabled_flag, respectively. (S0044)

Each of these flags was said to save at most one bit per picture. This would increase overhead in some common uses. No clear need for action was evident, so no action was recommended.

- On TMVP derivation considering inter-layer reference picture

  - Disable TMVP when either RefPic or ColRefPic is inter-layer reference picture (ILRP), but not both. (S0089 #1)

  - Enable TMVP with combination of STRP and ILRP on RefPic or ColRefPic, and disable TMVP with combination of LTRP and ILRP. (S0089 #2)

  - Enable TMVP when either RefPic or ColRefPic is ILRP. (S0089 #3)

| RefPic (target reference picture) | ColRefPic (reference picture of collocated block) | Current VVC | Solution #1 | Solution #2 | Solution #3 |
| --- | --- | --- | --- | --- | --- |
| STRP | ILRP | X | X | O | O |
| ILRP | STRP | X | X | O | O |
| LTRP | ILRP | O | X | X | O |
| ILRP | LTRP | O | X | X | O |

Much of the behaviour was originally from HEVC.

It was commented that this has a low-level impact and we should avoid any potentially unnecessary changes in this area. No clear need for action was evident, so no action was recommended.

- On maximum number of subblock merge candidates

Change the range of the value of sps_five_minus_max_num_subblock_merge_cand from 0..5 − sps_sbtmvp_enabled_flag to 0..5 and change the derivation of maximum number of subblock merge candidates in PH as follows: (S0088)

if( sps_affine_enabled_flag && sps_five_minus_max_num_subblock_merge_cand < 5 )

MaxNumSubblockMergeCand = 5 − sps_five_minus_max_num_subblock_merge_cand (88)

else

MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag  &&  ph_temporal_mvp_enabled_flag

Some participants said that it was intentional not to want to consider ph_temporal_mvp_enabled_flag.

There was no agreement that the proposal was needed, so no action was recommended.

- On PDPC control

  - Add an SPS flag to control PDPC on/off and check it in the intra prediction process. (S0072, S0136)

    - For all color formats, add an SPS PDPC disabled flag (named sps_pdpc_disabled_flag) (S0072 method 1), or an SPS PDPC enabled flag (named sps_pdpc_enabled_flag). (S0136)

    - Only for 4:4:4 and 4:2:2 cases, add an SPS PDPC disabled flag (named sps_pdpc_disabled_flag). (S0072 method 3)

  - Add a PDPC constraint flag in the general constraint information (GCI) syntax structure. (S0136, S0072 v2)

There is currently no high-level control syntax for PDPC, although there is some form of on-off switching for it. The proponent indicated that being able to disable it could provide a benefit in some cases (e.g., for SCC and 4:4:4 – verbally commented to be as much as 2% for an SCC sequence with some specialized configuration that had some tools disabled – ordinarily a much smaller impact on PSNR, but asserted to be a visual improvement for still pictures in some cases).

It was commented that adding this control would involve a low-level change.

It was noted that approach a.ii would not affect the Main 10 (4:2:0) profile.

It was commented that HMVP also doesn’t have a high-level flag.

See also the notes for the next item.

- On intra reference sample filtering control

  - Add an SPS flag to control intra reference sample filtering on/off and check it in intra prediction process. (S0072, S0150)

    - For all color formats, add an SPS disabled flag (named sps_intra_reference_filter_disabled_flag). (S0072 method 2, S0150)

      - Furthermore, add sps_intra_smoothing_disabled_present_in_sh_flag when sps_intra_reference_filter_disabled_flag is equal to 0, and add sh_intra_smoothing_disabled_flag when sps_intra_smoothing_disabled_present_in_sh_flag is equal to 1. (S0150)

    - Only for 4:4:4 and 4:2:2 cases, add an SPS disabled flag (named sps_intra_reference_filter_disabled_flag). (S0072 method 3)

  - Add an intra reference filtering constraint flag in the GCI syntax structure to constrain the added SPS flag. (S0072 v2)

It was commented that adding this control would involve a low-level change.

This was further discussed in the HLS AHG pre-meeting on Friday 19 June at 1305 UTC, together with the previous item (note that S0150 is a submission submitted after the AHG meeting in May).

This item and the previous one raise similar issues and for similar reasons – trying to avoid using spatial neighbours for some sort of blending operation that may not be beneficial for SCC usage.

There was discussion of whether a benefit with still pictures for high QP is really important.

Mixed content was mentioned, in which camera captured regions would be better with the blending enabled and SCC regions perhaps being better without it.

There is some disabling of PDPC already in the design, but not for the same cases, so it was said that some low-level change would be involved.

No action was recommended due to the late stage of the process and some questioning of the degree of benefit.

AHG discussion stopped here at 0920 UTC on 27 May.

This topic was discussed in the HLS AHG at 0030 UTC on 28 May.

- On TSRC

  - Remove no_tsrc_constraint_flag. (S0073, option 3 in section 3.3)

  - Change the semantics of no_tsrc_constraint_flag (removal: italic; addition: bold):

    - Add "When no_transform_skip_constraint_flag is 1, the value of no_tsrc_constraint_flag shall be one." (S0062, method 3)

    - Change the semantics to be as follows (S0069, option 1):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 or sps_transform_skip_enabled_flag shall be equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

    - Change the semantics to be as follows (S0069, option 2):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 when sps_transform_skip_enabled_flag is equal to 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

    - Change the semantics to be as follows (S0105):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

    - Add a semantic constraint to no_tsrc_constraint_flag to disable DQ and SDH in SPS level (S0073, option 1 in section 3.1)

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 and sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag are both equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

    - Add a semantic constraint to no_tsrc_constraint_flag to disable DQ and SDH in all slices (S0073, option 2 in section 3.2)

  - Infer sh_ts_residual_coding_disable_flag to be equal to !sps_transform_skip_enabled_flag instead of 0. (S0062, method 1)

  - Add an SPS flag for TSRC, use it to control the presence of sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag, and change the semantics of no_tsrc_constraint_flag to constrain the SPS flag instead of the SH flag. (S0062, method 2)

  - Change both semantics of no_tsrc_constraint_flag and sh_ts_residual_coding_disabled_flag: (S0069, option 3) as follows (removal: italic; addition: bold)

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

sh_ts_residual_coding_disabled_flag equal to 1 specifies that the syntax structure residual_coding( ) is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the syntax structure residual_ts_coding( ) is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.the following applies:

- If sps_transform_skip_enabled_flag is equal to 0, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 1.

- Otherwise, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 0.

A suggested principle was to not have GCI constraint flags that constrain combinations.

The no_tsrc_constraint_flag was suggested to be problem in this regard (as it disables three things: TSRC and SDH and DQ). Disabling SDH and DQ has a significant coding efficiency impact. It was thus suggested to remove this constraint flag (item “a” above).

The other aspects (some of which relate to bugs in the specification of the implications of that flag) would also be resolved by that action.

AHG Recommendation (cleanup/bug fix): Remove the no_tsrc_constraint_flag.

Discussion in AHG sessions 1-8 stopped here.

Discussion in AHG pre-meeting session 9 began here on 19 June at 1330.

- On residual coding

Add the following SPS and PH flags, and revise SH gating conditions and inferences: (S0093)

  - Addition of the SPS flag sps_residual_control_present_in_sh_flag.

  - Addition of picture header flags for DQ and SDH.

  - Modification of gating conditions for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to ensure that at most one of these flags are signalled at the slice level.

  - Modifications of inference rules for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to match the desired behaviour.

It was commented that when SDH or DQ is enabled, it is now necessary to repeat the enabling indication in every SH. The proposal is to implicitly disable these when TSRC is disabled in the SH. It was commented that the interaction between these features is relevant only for very high quality coding, where saving something in the SH is not so relevant.

There had been extensive discussion of the SDH/DQ interaction with TSRC at the previous meeting. It was commented that we should not further tinker with this unless we find some aspect that is really a bug, and that is not the case here. Thus, no action was recommended on this.

- On presence of explicit scaling list

Allow the aps_chroma_present_flag of the scaling list APS NAL unit referenced by the PH to be equal to 0 when ChromaArrayType is greater than 0. (S0049, aspect 9)

This flag is already in the syntax, but is required to be 1 for colour formats that have chroma.

It was commented that ALF and LMCS act similarly to what is proposed, and that a previous problem with allowing this had gone away due to an action of the previous meeting.

In the current design, there is no provision to use a scaling list for chroma without using a scaling list for luma.

It was commented that the flag was only added to avoid sending scaling lists that are completely irrelevant, and sending flat chroma scaling lists does not cost many bits.

Although there may be some arguable inconsistency with ALF and LMCS, it was said that we should not change this since there is no real bug. Thus, no action was recommended on this.

- On chroma QP table signalling

- Change the range of sps_num_points_in_qp_table_minus1[ i ] from 0..63 + QpBdOffset to 0..62 + QpBdOffset. (S0130 aspect 1)

AHG Recommendation (spec bug fix/expression of existing intent): Adopt this aspect.

- Add an SPS flag (named sps_chroma_qp_identical_to_luma_flag) to skip signalling of chroma QP information when chroma QPs are set equal to luma QPs. (S0130 aspect 2)

This would be a syntax shortcut for generating a chroma QP table that indicates chroma QP to be identical to the luma QP. Relative to what the VTM currently does, this would save 6 bits when the tables are identical and add one bit for when they are not, at the SPS level. The proponent noted that this is motivated as a shortcut for a reasonable case rather than really as a bit savings issue. It was commented that in the 4:2:0 case the QP table would typically not be identical. No clear need for action was identified, so no action was recommended on this.

- On ALF APS constraints

Add an SPS flag (named sps_one_aps_per_slice_one_filter_per_aps_flag) to support the following case and change the signalling of aps_adaptation_parameter_set_id, sh_alf_aps_id_luma, sh_alf_aps_id_chroma from u(5)/u(3) to u(v). (S0134):

- Each ALF APS contains up to 1 luma filter, up to 1 chroma filter, and no cross-component filters, and each slice in the CLVS uses 0 or 1 ALF APSs for luma adaptive loop filtering, 0 or 1 ALF APSs for chroma adaptive loop filtering, and applies no cross-component adaptive loop filtering.

- Allow maximum of 115 ALF APSs.

The use case is BEAMing with different encoders using different ALF parameters in different slices.

As far as the syntax is concerned, this would be only a high-level change.

However, the memory usage for the proposal would assume the decoder stores filters in a particular way to save memory. If the decoder does not do this, there would be a problem for the decoder memory. It was thus commented that this would actually have a low-level impact.

No action was recommended on this, as it would be a late change that has a low-level impact, and we do not have evidence that the current design has a problem for most use cases.

- On deblocking filter parameters

- Inference of chroma DB offsets

    - If pps_chroma_tool_offsets_present_flag is equal to 1, infer the values of the chroma DB offsets in the PH and SH, when not present, to be equal to the chroma DB offsets in the PPS and PH, respectively, instead of inferring the values from the luma DB offsets in the same header (as in the current VVC text). Otherwise, infer the values from the luma DB offsets in the same header (as in the current VVC text) (S0053, aspect 1)

    - Infer the chroma DB offsets in the PH and SH to be equal to the values of the chroma DB offsets in the PPS and PH, respectively, when not present, regardless of the value of pps_chroma_tool_offsets_present_flag. But this solution is not recommended. (S0053, aspect 1, alternative solution)

It was commented that aspect a.i is more sensible than the current design, as it makes use of information sent for chroma at a higher level.

AHG Recommendation (logical cleanup): Adopt S0053 as proposed (approach a.i).

- Removal the condition check of pps_deblocking_filter_override_enabled_flag for signalling the ph_deblocking_filter_override_flag. (S0121, aspect 2)

It was noted that this is a purely editorial change, to avoid checking an unnecessary condition in the syntax logic.

Editor action item: The editor is strongly suggested to remove this unnecessary condition check.

- Semantics of pps_chroma_tool_offsets_present_flag

Change the semantics such that the chroma deblocking tc and β offset syntax elements may be (instead of "are") present in the PHs or the SHs of pictures referring to the PPS when the flag is equal to 1. (S0053, aspect 2)

It was noted that this is an editorial bug fix, simply to make the semantics accurate.

AHG Recommendation (editorial bug fix): Adopt S0053 aspect 2 to fix the semantics error.

- Change the inference rules of non-present syntax elements PH/SH SAO, ALF, CCALF flags. (S0121, aspect 1)

- Infer ph_sao_luma_enabled_flag to be equal to sps_sao_enable_flag instead of 0.

- Infer ph_alf_enabled_flag to be equal to sps_alf_enable_flag instead of 0.

- Infer ph_sao_chroma_enabled_flag to be as follows, instead of 0:

- If sps_sao_enable_flag is equal to 1 and ChromaArrayType is not equal to 0, ph_sao_chroma_enabled_flag is inferred to be equal to 1.

- Otherwise, ph_sao_chroma_enabled_flag is inferred to be equal to 0.

- Infer ph_alf_cb_flag/ph_alf_cr_flag to be 0 if one of the following two conditions are true, and to be 1 otherwise:

- ChromaArrayType is equal to 0.

- ph_alf_enabled_flag is equal to 0.

- Infer ph_cc_alf_cb_enabled_flag/ph_cc_alf_cr_enabled_flag to be 0 if one of the following two conditions are true, and to be 1 otherwise:

- sps_ccalf_enabled_flag is equal to 0.

- ph_alf_enabled_flag is equal to 0.

- Infer sh_alf_cb(r)_flag as follows:

- If sh_alf_enabled_flag or ph_alf_cb(r)_flag is equal to 0, sh_alf_cb(r)_flag is inferred to be equal to 0.

- Otherwise (both sh_alf_enabled_flag and ph_alf_cb(r)_flag are equal to 1), sh_alf_cb(r)_flag is inferred to be equal to 1.

- Infer sh_cc_alf_cb(r)_enabled_flag as follows:

- If sh_alf_enabled_flag or ph_cc_alf_cb(r)_enabled_flag is equal to 0, sh_cc_alf_cb(r)_enabled_flag is inferred to be equal to 0.

- Otherwise (both sh_alf_enabled_flag and ph_cc_alf_cb(r)_enabled_flag are equal to 1), sh_cc_alf_cb(r)_enabled_flag is inferred to be equal to 1.

In the discussion, it was said that these changes are just to better express the existing design intent.

It was agreed that there is a problem in the existing semantics.

However, it was said that the proposed semantics should be carefully studied and there may be a simpler way to address the problem.

Revisit after offline study.

- Change the semantics of some PH syntax elements related to ALF and SAO as follows, asserted to address the inconsistency in the semantics reported in JVET-S0121: (S0252 late)

(Convention for text changes proposed: “quoted text” is proposed to be removed and text inside delimiters < > is proposed to be added by proposals)

ph_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be used for the current picture. ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter <may or may not be enabled> "is disabled and not used" for the current picture. When not present, ph_alf_enabled_flag is inferred to be equal to 0.

ph_alf_cb_flag "equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cb colour component for the current picture. ph_alf_cb_flag" equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cb colour component for the current picture. <ph_alf_cb_flag equal to 0 specifies that the adaptive loop filter may or may not be enabled for the Cb colour component of the current picture.> When ph_alf_cb_flag is not present, it is inferred to be equal to 0.

ph_alf_cr_flag "equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cr colour component for the current picture. ph_alf_cr_flag" equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cr colour component for the current picture. <ph_alf_cr_flag equal to 0 specifies that the adaptive loop filter may or may not be enabled for the Cr colour component of the current picture.> When ph_alf_cr_flag is not present, it is inferred to be equal to 0.

ph_cc_alf_cb_enabled_flag equal to 1 specifies that cross-component adaptive loop filter for the Cb colour component is enabled and may be used for the current picture. ph_cc_alf_cb_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cb colour component <may or may not be enabled> "is disabled and not used" for the current picture. When not present, ph_cc_alf_cb_enabled_flag is inferred to be equal to 0.

ph_cc_alf_cr_enabled_flag equal to 1 specifies that cross-compoent adaptive loop filter for the Cr colour component is enabled and may be used for the current picture. ph_cc_alf_cr_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cr colour component <may or may not be enabled> "is disabled and not used" for the current picture. When not present, ph_cc_alf_cr_enabled_flag is inferred to be equal to 0.

ph_sao_luma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the luma component of the current picture. ph_sao_luma_enabled_flag equal to 0 specifies that SAO <may or may not be enabled> "is disabled and not used" for the luma component of the current picture. When ph_sao_luma_enabled_flag is not present, it is inferred to be equal to 0.

ph_sao_chroma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the chroma component of the current picture. ph_sao_chroma_enabled_flag equal to 0 specifies that SAO <may or may not be enabled> "is disabled and not used" for the chroma component of the current picture. When ph_sao_chroma_enabled_flag is not present, it is inferred to be equal to 0.

- Change the semantics of PH syntax elements related to ALF and SAO as follows, asserted to address the inconsistency in the semantics reported in JVET-S0121: (S0255 late)

(Convention for text changes proposed: “quoted text” is proposed to be removed and text inside delimiters < > is proposed to be added by proposals)

<When pps_alf_info_ph_flag is equal to 1,> ph_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be used for the current picture. ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled and not used for the current picture. When not present, ph_alf_enabled_flag is inferred to be equal to 0."

<When pps_alf_info_ph_flag is equal to 1,> ph_alf_cb_flag equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cb colour component for the current picture. ph_alf_cb_flag equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cb colour component for the current picture. When ph_alf_cb_flag is not present, it is inferred to be equal to 0.

<When pps_alf_info_ph_flag is equal to 1,> ph_alf_cr_flag equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cr colour component for the current picture. ph_alf_cr_flag equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cr colour component for the current picture. When ph_alf_cr_flag is not present, it is inferred to be equal to 0.

<When pps_alf_info_ph_flag is equal to 1,> ph_cc_alf_cb_enabled_flag equal to 1 specifies that cross-component adaptive loop filter for the Cb colour component is enabled and may be used for the current picture. ph_cc_alf_cb_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cb colour component is disabled and not used for the current picture. When not present, ph_cc_alf_cb_enabled_flag is inferred to be equal to 0.

<When pps_alf_info_ph_flag is equal to 1,> ph_cc_alf_cr_enabled_flag equal to 1 specifies that cross-compoent adaptive loop filter for the Cr colour component is enabled and may be used for the current picture. ph_cc_alf_cr_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cr colour component is disabled and not used for the current picture. When not present, ph_cc_alf_cr_enabled_flag is inferred to be equal to 0.

<When pps_alf_info_ph_flag is equal to 1,> ph_sao_luma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the luma component of the current picture. ph_sao_luma_enabled_flag equal to 0 specifies that SAO is disabled and not used for the luma component of the current picture. When ph_sao_luma_enabled_flag is not present, it is inferred to be equal to 0.

<When pps_alf_info_ph_flag is equal to 1,> ph_sao_chroma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the chroma component of the current picture. ph_sao_chroma_enabled_flag equal to 0 specifies that SAO is disabled and not used for the chroma component of the current picture. When ph_sao_chroma_enabled_flag is not present, it is inferred to be equal to 0.

- Invocation of the decoding process for symmetric motion vector difference reference indices specified in clause 8.3.5

In clause 8.1.2, add the following text as a step after the invocation of the decoding process for generating unavailable reference pictures specified in clause 8.3.4: "At the beginning of the decoding process for each B slice, the decoding process for symmetric motion vector difference reference indices specified in clause 8.3.5 is invoked for derivation of the variables RefIdxSymL0 and RefIdxSymL1.", and at the beginning of clause, add the following: "This process is invoked at the beginning of the decoding process for each B slice, after decoding of the slice header as well as the invocation of the decoding process for reference picture list construction for the slice as specified in clause 8.3.2, but prior to the parsing and decoding of any coding unit." (S0191)

It was commented that this is really an editorial bug fix proposal, and some correction is needed, but that it is editorially preferable to avoid having parsing specified in a way that is dependent on the decoding process specification (when feasible).

It was suggested that something also be done for 8.3.6.

Revisit after offline study.

- Make either of the following changes regarding the inferences of sps_scaling_matrix_for_lfnst_disabled_flag and sps_scaling_matrix_designated_colour_space_flag: (S0196)

  - Remove the inferences of both flags, as they are asserted not used when not present.

  - Change the inferred value from 1 to 0 for both flags.

It was commented that these are simply editorial suggestions.

Editor action item: The editor is suggested to remove the unnecessary inferences.

- Make one of the following changes to disallow the ACT and LFNST combination: (S0197)

  - When sps_lfnst_enabled_flag is equal to 1, sps_act_enabled_flag is not signalled and is inferred to be 0.

  - When sps_act_enabled_flag is equal to 1, sps_lfnst_enabled_flag is not signalled and inferred to be 0.

  - Add a constraint: The values of sps_act_enabled_flag and sps_lfnst_enabled_flag shall not both be equal to 1.

The proponent indicated that this is the only case with three cascaded transforms in the decoding process, and that experiments with RGB content showed no benefit for the combination.

Another participant said that on the OldTownCross test sequence in the RA case, the combination of ACT and LFNST provides 2.8% gain, and that overall it also provides gain on some other content.

Test results were shown, with different test results for different test sequences. It was commented that an encoder could hypothetically make a smart decision about when to enable the combination.

It was commented that the cascading of ACT with LFNST is not really an implementation problem.

Some participants preferred to leave this to the encoder to be able to choose.

It was not clear that there is a real problem that needs to be solved by this change, so no action was recommended.

- Signal an SPS flag to specify whether ph_joint_cbcr_sign_flag is present in PHs. When not present, the value of ph_joint_cbcr_sign_flag is inferred to be equal to 1. (S0201)

The proponent indicated that only 3 test sequences out of 26 in the CTC use the sign flag. One of these is SlideShow, which is screen content.

The argument, aside from 1 bit per picture bit savings, is consistency with how BDOF is handled. Another participant said there had been a proposal to change BDOF signalling, for a similar consistency reason, with no action recommended.

It was not clear that there is a real problem that needs to be solved by this change, so no action was recommended.

- Make the following syntax and semantics changes to infer the values of ph_dmvr_disabled_flag and ph_bdof_disabled_flag to be equal to 1 (DMVR/BDOF disabled) when the picture is not qualified to use bi-prediction, to fix the asserted issue that currently they are inferred to be equal to 0 (DMVR/BDOF enabled) when the picture is not qualified to use bi-prediction: (S0205)

Change the picture header structure syntax and semantics as follows (additions in bold fonts, removals in italic fonts):

| presenceFlag = 0 |  |
| --- | --- |
| if( ph_inter_slice_allowed_flag ) { |  |
| ... |  |
| if( sps_mmvd_fullpel_only_flag ) |  |
| ph_mmvd_fullpel_only_flag | u(1) |
| presenceFlag = 0 |  |
| if( !pps_rpl_info_in_ph_flag ) /* This condition is intentionally not merged into the next, to avoid possible interpretation of RplsIdx[ i ] being unspecified. */ |  |
| presenceFlag = 1 |  |
| else if( num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 0 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) |  |
| presenceFlag = 1 |  |
| if( presenceFlag ) { |  |
| ph_mvd_l1_zero_flag | u(1) |
| if( sps_bdof_control_present_in_ph_flag ) |  |
| ph_bdof_disabled_flag | u(1) |
| if( sps_dmvr_control_present_in_ph_flag ) |  |
| ph_dmvr_disabled_flag | u(1) |
| } |  |

Change the picture header structure syntax as follows:

ph_bdof_disabled_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_bdof_disabled_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_bdof_disabled_flag is inferred as follows:

- If presenceFlag is equal to 1 sps_bdof_control_present_in_ph_flag is equal to 0, the value of ph_bdof_disabled_flag is inferred to be equal to 1 − sps_bdof_enabled_flag.

- Otherwise (presenceFlag is equal to 0 sps_bdof_control_present_in_ph_flag is equal to 1), the value of ph_bdof_disabled_flag is inferred to be equal to 1.

ph_dmvr_disabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_dmvr_disabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_dmvr_disabled_flag is inferred as follows:

- If presenceFlag is equal to 1 sps_dmvr_control_present_in_ph_flag is equal to 0, the value of ph_dmvr_disabled_flag is inferred to be equal to 1 − sps_dmvr_enabled_flag.

- Otherwise (presenceFlag is equal to 0 sps_dmvr_control_present_in_ph_flag is equal to 1), the value of ph_dmvr_disabled_flag is inferred to be equal to 1.

This is only editorial. Currently there is an inference rule that sometimes infers values that are not used. The current expression is done this way just to make the inference rule simple.

It was suggested to change the expression of the inference rule so that values are not inferred when they are not used.

Editor action item: The editor may consider ways of improving the expression, although no clear bug has been identified.

- Add sps_ltdf_enabled_flag for controlling the on/off of long deblocking filters, and add a GCI flag (no_ltdf_constraint_flag) to constrain the new sps_ltdf_enabled_flag. (S0226)

R0300 had reported that there was sometimes an over-smoothing phenomenon and suggested to be able to control the long deblocking filter. It was commented that the long DBF is the main difference between VVC and HEVC deblocking.

It was commented that there is already a way to control overall DBF strength.

There had been a proposal at the previous meeting for separate control of the long filter at the slice level, but it had not been determined necessary at the time. There had also been analysis of the question at the meeting before that.

This would have a low-level impact, as it would be necessary to support the switching within the DBF process.

It was not clear that there is a real problem that needs to be solved by this change, so no action was recommended.

- Make either of the following changes to resolve an asserted inconsistency between the semantics of sps_mmvd_fullpel_only_flag and the related syntax parsing process: (S0227)

  - Change the syntax condition for ph_mmvd_fullpel_only_flag from "if( sps_mmvd_fullpel_only_flag )" to " if( !sps_mmvd_fullpel_only_flag )".

  - Rename sps_mmvd_fullpel_only_flag to sps_mmvd_fullpel_only_enabled_flag, and switch the phrases "uses" and "may use" for the value 1 and 0 in the semantics of this SPS flag.

It was commented that option a has a problem.

AHG recommendation (logical cleanup): Adopt option b. (The naming of the flag is an editorial matter left to the discretion of the editor – removing “_only” was suggested.)

- Change the LMCS data syntax and semantics as well as the related decoding process texts as in S0245 (S0245 aspect 1 late)

TBP: Proponent not present at the time of AHG discussion.

- Change scaling list data semantics as well as the related decoding process texts as in S0245 (S0245 aspect 2 late)

TBP: Proponent not present at the time of AHG discussion.

JVET-S0044 AHG9: Cleanup of high-level signalling of BDOF, DMVR and PROF [M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, Z. Zhang, J. Ström, R. Yu, D. Liu (Ericsson)]

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 9 of this contribution belongs to this category.

JVET-S0053 AHG9: Inference of chroma deblocking offset values in the PH and SH [Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, K. Andersson, J. Ström, R. Sjöberg, R. Yu, D. Liu (Ericsson)]

JVET-S0062 AHG9: On no_tsrc_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0072 AHG9: On PDPC and reference sample filtering of non-420 sequences [M. G. Sarwer, Y. Ye (Alibaba)]

JVET-S0069 AHG9: General constraint information on features related to transform skip mode [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspect 2 of this contribution belongs to this category.

JVET-S0073 AHG9: Bug-fix of the constraint flag of VVC [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 2 of this contribution belongs to this category.

JVET-S0088 AHG9: Clean-up on derivation of the MaxNumSubblockCand [N. Park, J. Nam, H. Jang, J. Lim, S. Kim(LGE)]

JVET-S0089 AHG8: On TMVP derivation using inter-layer reference picture [N. Park, J. Nam, H. Jang, J. Lim, S. Kim(LGE)]

JVET-S0093 On slice header flags related to residual coding [J. Samuelsson, S. Deshpande, F. Bossen, A. Segall (Sharp)]

JVET-S0148 Crosscheck of JVET-S0093 (On slice header flags related to residual coding) [J. Gan (Canon)]

JVET-S0105 AHG9: Modification of general constraint information [S. McCarthy, P. Yin, T. Lu, F. Pu, W. Husak, T. Chen (Dolby)]

The aspect on changing the semantics of no_tsrc_constraint_flag belongs to this category.

JVET-S0121 AHG9: On deblocking filter, ALF, and SAO enabling flags in PH and SH [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

JVET-S0130 On chroma QP mapping [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0134 AHG9/AHG12: Relaxing an ALF APS constraint [A. Aminlou, M. M. Hannuksela (Nokia)]

JVET-S0136 AHG9: On signaling of PDPC enabling/disabling flag in SPS [X. Xiu, Y.-W. Chen, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai)]

JVET-S0150 AHG9: On high-level syntax for smoothing intra prediction tools [A. Filippov, V. Rufitskiy, E. Alshina (Huawei)]

JVET-S0191 AHG9: On decoding process for symmetric motion vector difference reference indices [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0196 AHG9: On inference rules of the SPS scaling matrix related flags [M. G. Sarwer, Y. Ye (Alibaba)]

JVET-S0197 AHG9: On ACT and LFNST [M. G. Sarwer, Y. Ye (Alibaba)]

JVET-S0201 AHG9: On signaling the JCCR sign information in PH [C.-W. Kuo, X. Xiu, H.-J. Jhu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0205 AHG9: Inference value of DMVR/BDOF disabled flag in PH [K. Unno, K. Kawamura (KDDI)]

JVET-S0226 AHG9: On high level syntax for longer tap deblocking filter [A. M. Kotra, N. Hu, M. Coban, V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

JVET-S0227 AHG9: On MMVD flag in SPS and PH [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

JVET-S0245 AHG9: Editorial clarifications of APS ID for LMCS_APS and SCALING_APS [Y. Ahn, J. Lee, D. Sim (Digital Insights), S. Park (Hyundai Motors)] [late]

JVET-S0252 PH semantics of ALF and SAO syntax elements [J. Samuelsson, S. Deshpande, A. Segall (Sharp), L. Li, X. Li, B. Choi (Tencent)] [late]

JVET-S0255 AHG9: Semantics fix to ALF and SAO enabling flags in PH [N. Hu, V. Seregin, M. Karczewicz (Qualcomm), O. Chubach, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

### General and misc. HLS topics (16+16)

JVET-S0139 AHG9: A summary of proposals on general and misc. HLS topics [Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0515 UTC on 27 May and in the HLS AHG pre-meeting at 1630 UTC on 19 June and 1300 UTC on 21 June.

This contribution provides a summary of the 31 proposals on general and misc. HLS topics (the agenda item 3.1.2 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following actions are proposed:

- Remove the support of the separate colour plane coding from VVCv1, by removing the syntax elements sps_separate_colour_plane_flag and sh_colour_plane_id and the related text. If such functionality is desired in the future, the SPS and SH extension mechanisms could be used. Another option is to rename these syntax elements to be reserved bits. (S0052)

AHG Recommendation (cleanup): Remove these syntax elements and the associated decoding process as proposed.

- Add a constraint: The value of sps_field_seq_flag shall be the same in all SPSs that are referred to by CLVSs in a CVS. (S0056 aspect 1)

No action seemed necessary on this, as the decoding process is not affected.

- Add a constraint: When sps_field_seq_flag is equal to 1, all fields that compose the same frame shall refer to the same PPS. (S0056 aspect 2)

The concept of fields belonging to the same frame did not seem adequately defined, and no action was recommended on this.

- Add a constraint: When sps_video_parameter_set_id is equal to 0, parameter sets referred to by all VCL NAL units shall have the same value of nuh_layer_id as the VCL NAL units. (S0075 aspect 1 – also covered in another summary document S0141)

This was deferred for consideration in relation to S0141 regarding inference of an OLS when there is no VPS – adding a NOTE may be sufficient.

- Do the following for setting of PictureOutputFlag after receiving the first slice of a picture and before receiving other slices of the picture: (S0077)

- Add a constraint that when a picture contains a mix of RASL_NUT with NAL unit type(s) other than RADL_NUT or a mixed of RADL_NUT with other NAL unit type(s) other than RASL_NUT, then the following applies:

    - If interlace coding is not used (i.e., sps_field_seq_flag is equal to 0), there shall be at least one non-leading picture between the picture and its associated IRAP picture.

    - Otherwise, if interlace coding is used (i.e., sps_field_seq_flag is equal to 1), there shall be at least two non-leading pictures between the picture and its associated IRAP picture.

- Add a NOTE to clarify the derivation of PictureOutputFlag that when decoder received the first slice of the current picture and the NAL unit type of the slice is either RASL_NUT or RADL_NUT and the value of pps_mixed_nalu_types_in_pic_flag is equal to 1, if decoder has not received any non-leading picture from the last IRAP picture, then decoder can determine that the current picture is a RASL picture.

It was noted that PictureOutputFlag is not really needed until all slices of the picture have been decoded, and thus suggested to move the derivation of this flag to the end of the decoding process of the picture.

AHG Recommendation (cleanup): Move the derivation of this flag to the end of the decoding process of the picture.

- Invoke the derivation process for NoOutputOfPriorPicsFlag only once per AU, that is, before the decoding process of the first picture in the CVSS AU but after parsing the slice header of the first slice of the current picture. When NoOutputOfPriorPicsFlag is equal to 1, the process of removing pictures stored in the DPB without outputting them is invoked only once per access unit. (S0078)

AHG Recommendation (BF/expression of existing intent): Adopt.

- Fix an asserted bug (the derivation of prevTid0Pic that is used in the POC decoding process does not consider the value of ph_non_ref_pic_flag), by adding the following constraint: (S0081)

Let picture picA and picB be two pictures with TemporalId equal to 0, it is a constraint of conformance bitstream that DiffPicOrderCnt( picB, picA ) shall not be greater than MaxPicOrderCntLsb / 2 when all of the following conditions are satisfied:

- The values of nuh_layer_id of picA and picB are the same.

- picA and picB are not RADL picture or RASL picture.

- The value of ph_non_reference_picture_flag of picA and picB are both equal to 0.

- picB is the first picture with TemporalId equal to 0 that follows picA in output order.

An alternative approach was suggested, to modify the derivation of prevTid0Pic to consider only pictures with ph_non_ref_pic_flag equal to 0. It was commented that this approach was taken in HEVC.

It was also commented that we should do the same with prevNonDiscardablePic and in C.4 and the semantics of delta_poc_msb_cycle_present_flag.

AHG Recommendation (BF/expression of existing intent): Make the alternative suggested changes.

- When the current picture is a RADL picture, allow RASL pictures with pps_mixed_nalu_types_in_pic_flag is equal to 1 in active entries in RefPicList[ 0 ] or RefPicList[ 1 ]. (S0084, spec text ticket #1035)

AHG Recommendation (BF/expression of existing intent): Adopt (the editor may also add a NOTE to explain that such RADL pictures would still be correctly decodable when random accessing from the CRA picture, due to the existence of subpicture-level constraints).

- Do one of the following to the NOTE in the definition of RASL picture: (S0110 aspect 1)

- Remove the sentence "RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures."

- Change the sentence to be: "RASL pictures with mixed_nalu_types_in_pic_flag equal to 0 are not used as reference pictures for the decoding process of non-RASL pictures."

AHG Recommendation (BF/expression of existing intent): Adopt “b” in principle; editor to produce the precise wording.

- When aud_irap_or_gdr_au_flag is equal to 1, in the AUD syntax, optionally signal the OLS index of the OLS represented by the CVS (aud_cvs_ols_idx) and the highest temporal sublayer present in the CVS (aud_cvs_htid_plus1), each conditioned by its own presence flag, with the following additional details: (S0087)

- aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0. aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

- aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

- When aud_cvs_htid_plus1 is present, it is used to derive the value of Htid. Otherwise, Htid is set by external means (like in VVC Draft 9).

- When aud_cvs_ols_idx is present, it is used to derive the value of TargetOlsIdx. Otherwise, TargetOlsIdx is set by external means (like in VVC Draft 9).

- The sub-bitstream extraction process is modified so that the highest temporal sublayer and the target OLS given as inputs to the process are used in modifying the AUD NAL units in the output sub-bitstream.

It was noted that this requires AUD syntax rewriting for extraction.

A participant suggested to consider using the AUD content only if such information is not provided by external means. Others said that any external means and the AUD content (if present) should be required to match each other. Another participant said this could have a similar concept as for parameter sets – it needs to be available – either in the bitstream or by external means.

Another participant commented that dropping of some NAL units may cause mismatch, although we would not need to specify a decoder response to accidental dropping that produces non-conforming bitstreams.

The spirit is to allow the information to be signalled “in band”. If not present, the current scheme would apply such that external means is needed.

Something similar had been proposed at the January 2020 (Brussels) meeting in JVET-Q0256, using a different NAL unit rather than the AUD. This proposal would make the AUD have variable length.

It was commented that having a different NAL unit might help deal with the rewriting issue.

This was initially deferred for further study until the main meeting to propose an approach with a distinct NAL unit and text such that the information needs to be available – either in the bitstream or by external means. See the notes for item 27 for the later resolution of this.

- Add a rule to select among OLSs matching BitstreamToDecode: When TargetOlsIdx is not provided via external means, it is set equal to the OLS with the lowest index that contains all layers of AU 0 of BitstreamToDecode and the highest number of output layers. (S0103 aspect 1)

Currently, there is no default, and TargetOlsIdx needs to be provided.

This was initially deferred for further study until the main meeting in conjunction with the previous item above. See the notes for item 27 for the later resolution of this.

- Do one of the following to enable setting of PictureOutputFlag equal to 0 for pictures in CVSS AUs (for correct picture output behavior when random accessing from a DRAP AU): (S0104)

- Add sps_pic_in_cvss_au_no_output_flag, which equal to 1 specifies that a picture in a CVSS AU referring to the SPS is not output (regardless of the ph_pic_output_flag value in the PH).

- Add an external-means-determined variable NoOutputBeforeDrapFlag for CVSS AUs. PictureOutputFlag is set equal to 0 for pictures in CVSS AUs with NoOutputBeforeDrapFlag equal to 1.

It was asked whether this proposed behaviour is consistent with DRAP being only an SEI message, since this is proposing a change in the main body. DRAP was also an SEI message in HEVC.

It was commented that the distinction between “output” and “display” might allow a system to not display an IRAP picture although the decoder would conceptually “output” it.

It was commented that DRAP could be used for various purposes – not just recovery but also for fast-forward.

Option “b” might be considered similar in spirit with some of these suggestions.

It was commented that it had been intended for DRAP to not affect the main spec, and it seemed too late to change that intent.

No action was recommended for this (within the scope of v1).

- In order to enable signalling the picture rate information without requiring signalling a complete HRD model, allow general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag to be both equal to 0, and when that condition is true, only signal the following HRD parameters syntax elements: num_units_in_tick, time_scale, general_same_pic_timing_in_all_ols_flag, fixed_pic_rate_general_flag[ i ], fixed_pic_rate_within_cvs_flag[ i ], and elemental_duration_in_tc_minus1[ i ]. (S0109)

It was asked whether these can be sent in AVC and HEVC without HRD parameters, and the proponent said this was possible. It was also asked what these would mean if fixed_pic_rate_general_flag[ i ] and fixed_pic_rate_within_cvs_flag[ i ] are equal to 0.

HEVC has a specification of a correspondence between POC and timing in a special case.

It was commented that the semantics of some of these parameters depend on the HRD specification, so the parameters would not make sense without the HRD model (as currently specified).

This was deferred for further study to determine whether it is feasible and desirable to signal picture rate information in the bitstream without signalling HRD parameters in the bitstream. See S0175 aspect 6 in section 3.1.8.

- Add a prediction constraint on the pictures following a recovery point picture in both decoding and output order, as follows: When the current picture follows a recovery point picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that recovery point picture in output order or decoding order of the associated GDR picture in the layer with the nuh_layer_id equal to the nuh_layer_id of the reference picture. (S0114)

It was commented that this would prohibit using “clean” areas of “partially dirty” pictures prior to the recovery point. This seems against the spirit of the intent of the GDR design, so no action was recommended on this.

- Make the following changes: (S0124)

- Add derivation of TemporalId, ph_non_ref_pic_flag, and parameter sets to the decoding process for generating unavailable reference pictures (in order to enable checking of some constraints for them).

- Remove the constraint that each inter-layer reference picture entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

- Invoke the decoding process for generating unavailable reference pictures for ILRPs or constrain that ILRPs shall not be equal to "no reference picture".

It was commented that aspects b and c are not necessary. Part of the motivation for item b is to enable more use of signalling the RPL in the PH or SPS rather than SH – e.g., for bit savings.

As proposed “parameter set information” was vague, and specifying inference of ph_pic_parameter_set_id instead was suggested.

AHG Recommendation (BF/expression of existing intent): Adopt aspect a, modified as described.

(Note: This may also need to be an errata item for HEVC.)

- Move the signalling of WPP enabling flag and the entry point offsets present flag from SPS to PPS, and enable the signalling of only the tile entry points when both tiles and WPP are enabled. (S0128)

Part of the motivation of the first proposed change is to enable the second proposed change.

No strong need for action was identified on this, so no action was recommended.

Discussion of this topic area stopped here at the mid-term AHG meeting and resumed at 1630 UTC on 19 June in the AHG pre-meeting.

- Make the following changes on end of sequence (EOS) NAL units (S0155):

  - In clause 3 (Definitions), the NOTE as part of the definition of CLVS picture, the phrase "the first PU in the layer of the bitstream that follows an EOS NAL unit in decoding order", change "EOS NAL unit" to "EOS NAL unit in the layer".

  - In clause 7.4.2.2 (NAL unit header semantics), instead of requiring the nuh_layer_id of an EOS NAL unit to be equal to the nuh_layer_id of the associated VCL NAL unit, it is specified that the nuh_layer_id of an EOS NAL unit shall be equal to one of the nuh_layer_id values of the layers present in the CVS.

  - In clause 7.4.2.4.3 (Order of PUs and their association to AUs), it is specified that, when present, the next PU of a particular layer after an EOS NAL unit that belongs to the same layer shall be an IRAP or GDR PU.

  - In clause 7.4.2.4.4 (Order of NAL units and coded pictures and their association to PUs), it is specified that, when an EOS NAL unit is present in a PU, it shall be the last NAL unit among all NAL units within the PU other than other EOS NAL units (when present) or an EOB NAL unit (when present).

  - In clause 7.4.3.10 (End of sequence RBSP semantics), it is specified that, when present, the EOS RBSP specifies that the next subsequent PU that belongs to the same layer as the EOS NAL unit in the bitstream in decoding order (if any) is an IRAP or GDR PU.

AHG Recommendation (cleanup/BF/expression of existing intent): Adopt.

- Add either of the following constraints related to EOS NAL units in multi-layer bitstreams (S0174 aspect 2):

- The picture referred to by each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall not be a RASL picture when the associated CRA has NoOutputBeforeRecoveryFlag equal to 1 and the current picture is not a RASL picture.

- It is a requirement of bitstream conformance that when a layer k that depends on a layer m and the layer m contains an EOS NAL unit in AU n, layer k shall also contain an EOS NAL in AU n or the next AU n + 1 must contain a CLVSS picture for layer k.

A wording clarification for this was suggested: When an AU auA contains an EOS NAL unit in a layer layerA, for each layer layerB that is present in the CVS and has layerA as a reference layer, the first picture in layerB in decoding order in an AU following auA in decoding order shall be a CLVSS picture.

It was commented that option “a” doesn’t cover some GDR cases and that option “b” seemed more aligned with practical use cases.

AHG Recommendation (cleanup/BF/expression of existing intent): Adopt option b, as clarified.

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of slices i.e., items c and d, from being based on MaxSlicesPerPicture and the maximum picture size to be based on MaxSlicesPerPicture * ( the number of pictures in the AU ) and the maximum AU size. (S0156 aspect 1)

- Alternatively, change the two level limits to be based on MaxSlicesPerAu, which is rename of the existing MaxSlicesPerPicture without changing its value, and change "the number of slices in each picture in AU" to "the number of slices in AU". (S0156 aspect 1 alt, late)

AHG Recommendation (cleanup/BF/expression of existing intent): Adopt alternative “a”.

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of tiles i.e., items i and j, from being based on MaxTileCols * MaxTileRows and the maximum AU size to be based on MaxTileCols * MaxTileRows * ( the number of pictures in the AU ) and the maximum AU size. (S0156 aspect 2)

- Alternatively, change the two level limits to be based on MaxTilesPerAu, which replaces the existing MaxTileRows, where the value of MaxTilesPerAu is set equal to MaxTileRows * MaxTileCols, and change "the number of tiles in each picture in AU" to "the number of tiles in AU". (S0156 aspect 2 alt, late)

AHG Recommendation (cleanup/BF/expression of existing intent): Adopt alternative “a”.

It was noted that this implies that the minimum tile size can be as small as a single CTU in certain cases.

- In clause C.4 (Bitstream conformance), the 6th constraint, change "DpbOutputTime[ n ] greater than DpbOutputTime[ currPic ]" to "DpbOutputTime[ n ] greater than CpbRemovalTime[ currPic ]". This is asserted to be a bug, because all decoded pictures in the DPB are always decoded earlier than decoding of the current picture, and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ]. This is also asserted to be a bug in the latest HEVC spec. (S0156 aspect 3)

AHG Recommendation (BF/expression of existing intent): Adopt. Also clarify that the number of decoded pictures in the DPB is the number that is computed in the sentence. This is noted to also affect HEVC.

Discussion in AHG session 10 stopped here in the HLS AHG pre-meeting at 1720 UTC on 19 June.

Discussion in AHG session 17 began here in the HLS AHG pre-meeting at 1300 UTC on 21 June.

- Change "less than" in the following condition (that's part of the specification of the first picture of an AU) to "less than or equal to": The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order. (S0160 aspect 5)

AHG Recommendation (editorial bug fix/expression of existing intent): Adopt.

- Remove the following constraint from the definition of the two still picture profiles: The referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0. (S0160 aspect 6)

It was commented that this affects the ability to extract a picture from a Main profile bitstream.

AHG Recommendation (editorial bug fix/expression of existing intent): Adopt. (Also affects HEVC)

- Make sure that each VUI syntax element that describes a property of the bitstream has an inferred/default value when sps_vui_parameters_present_flag is equal to 0. (S0160 aspect 8)

Revisit after offline study to determine whether action is needed. Defaults should indicate unspecified and/or unconstrained indications.

- Change an existing constraint such that the value of ph_poc_msb_cycle_present_flag is required to be equal to 0 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is an ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture (instead of "and there is a picture in the current AU in a reference layer of the current layer"). (S0160 aspect 9, late)

Several participants confirmed the need for this.

AHG Recommendation (editorial bug fix/expression of existing intent): Adopt.

- Add the following constraints to resolve an asserted bug that may cause POC derivation process not working correctly for IRAP pictures in an independent non-output layer in which pictures other than IRAP pictures are not used for inter-layer prediction and hence would be removed during sub-bitstream extraction: (S0241 late)

  - It is a requirement of bitstream conformance that the value of sps_poc_msb_cycle_flag shall be equal to 1 when both of the following conditions are true:

- The value of sps_video_parameter_set_id is not equal to 0.

- The SPS is referred to by at least a VCL NAL unit with layer id equal to nuh_layer_id and value of NumSubLayersInLayerInOLS[ i ][ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 for any value of i in the range 0 TotalNumOlss − 1, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1.

  - It is a requirement of bitstream conformance that the value of ph_poc_msb_cycle_present_flag shall be equal to 1 when all the following conditions are true:

- The picture associated with the picture header is an IRAP picture or GDR picture with associated ph_recovery_poc_cnt equal to 0 that is not a CLVSS picture.

- The POC difference between the current picture and the previous IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0 in the same layer in decoding order is equal to or greater than MaxPicOrderCntLsb / 2.

- The value of sps_video_parameter_set_id is greater than 0 and the value of NumSubLayersInLayerInOLS[ i ][ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 for i in the range 0 TotalNumOlss − 1, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1.

It was commented that the constraint that the output of the bitstream extraction process shall be a conforming bitstream imposes this requirement indirectly. It was also commented that the second bullet of aspect “b” is somewhat circular. Adding a NOTE was suggested.

Editor action item: The editor is strongly suggested to add a NOTE to explain this issue. This affects such scenarios as single-layer intra-picture-only trick play.

- Enable signalling of TargetOlsIdx and Htid in the bitstream as follows: (S0163)

- Add operation point information (OPI) NAL unit for carrying syntax elements indicating TargetOlsIdx and Htid.

- Including an extension mechanism in OPI NAL unit similar to that in DCI and all parameter sets.

- Allowing OPI NAL unit in all AUs with IRAP or GDR slices (and thus may become IRAP or GDR AUs as a result of layer and/or subpicture extraction), and requiring the content of the OPI NAL unit to be the same within a CVS.

- TargetOlsIdx and Htid remain valid for one or more CVSs, until they are updated by another OPI NAL unit or through external means.

- When both AUD and OPI NAL units are present in an AU, the OPI NAL unit shall be the next NAL unit after the AUD NAL unit. When no AUD NAL unit is present in an AU and the OPI NAL unit is present in the AU, the OPI NAL unit shall be the first NAL unit in the AU. No more than one instance of the OPI NAL unit shall be present in an AU.

- Allowing to have neither external means nor the OPI NAL unit present for the first AU of the bitstream.

- When no target OLS index is provided in the OPI NAL unit or through external means for the first AU of the bitstream, TargetOlsIdx is inferred to be equal to the lowest OLS index that contains the largest number of layers and the largest number of output layers among all OLSs.

- When no highest temporal ID is provided in the OPI NAL unit or through external means for the first AU of the bitstream, Htid is inferred equal to vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ].

- Assigning nal_unit_type equal to 12 (which was earlier used for RSV_IRAP_12) for the OPI NAL unit.

- TemporalId of OPI NAL unit shall be equal to 0 and TemporalId of the containing AU shall also be 0.

- Allowing any nuh_layer_id value for the OPI NAL unit. Consequently, in the sub-bitstream extraction process, the OPI NAL unit is added among the NAL units that are not removed based on nuh_layer_id values.

If item h above is adopted, the following items are also proposed:

- In the current VVC draft, if no VPS is present, then vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ] is not inferred. This document proposes to infer it equal to sps_max_sublayers_minus1 when no VPS is present.

- It is proposed to consider changing the syntax element name of vps_ptl_max_tid[ i ] and change its semantics to be more generic.

The proposed semantics are (the bolded italics text at the end of the sentence added): vps_ptl_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure in the VPS and present in the OLSs with index olsIdx such that vps_ols_ptl_idx[ olsIdx ] is equal to i.

This contribution was the result of offline work after the May AHG meeting. The proposal is to add an “operation point information NAL unit”. The information could be in the bitstream or provided by external means. It would be at the beginning of an AU when present.

A default would be specified if this is not present – see aspect “g”.

It was commented that this is a substantial action, but appears to resolve a key open issue in the specification.

It was asked why it is necessary to be able to have the additional NAL unit in the bitstream; this is to provide some in-bitstream means of specifying non-default OLS behaviour.

After discussion, it was suggested that if external means is available, it would override the in-bitstream information.

AHG Recommendation (cleanup): Adopt, modified as suggested to specify the ability to override by external means (text with the modification to be provided in a revision).

- Make the following changes to the constraints on RPLs for RADL pictures and subpictures, considering multi-layer coding as well as its combination with subpictures (S0167) (additions in bold, removals in italic):

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture with pps_mixed_nalu_types_in_pic_flag is equal to 0

NOTE 4 – This means that an active entry of the RPLs of a RADL picture can refer to a RASL picture with pps_mixed_nalu_types_in_pic_flag equal to 1 and the referenced RASL picture may either belong to the same layer or a different layer than the layer containing the current RADL picture. However, when decoding starts from the associated CRA picture, such a RADL picture can still be correctly decoded, because the RADL subpicture(s) in that referenced RASL picture with sps_subpic_treated_as_pic_flag equal to 1 would be correctly decoded, as the RADL picture would only refer to the RADL subpictures in the referenced RASL picture, as imposed by the next constraint that disallows RADL subpictures referring to a RASL subpicture.

  - A picture that precedes the associated IRAP picture in decoding order

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx and sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, is a RADL subpicture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A picture [Removing: with nuh_layer_id equal to layerId] for which the value of nuh_layer_id may or may not be equal to layerId containing a RASL subpicture with subpicture index equal to subpicIdx

  - A RASL picture for which the value of nuh_layer_id is not equal to layerId and the value of sps_num_subpics_minus1 is equal to 0

  - A picture that precedes the picture containing the associated IRAP subpicture in decoding order

The first part of the proposal was supported as a clarification.

It was agreed that we should not insert “with sps_subpic_treated_as_pic_flag equal to 1” and “and sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1” because if the RASL pictures discussed in the NOTE have all I slices, they can be correctly decoded, and the subpicture-level constraint would apply even if the sps_subpic_treated_as_pic_flag[ subPicIdx ] is equal to 0.

  - AHG Recommendation (expression of existing intent): Adopt, modified to not insert two phrases as noted above.

- Make the following changes, considering the combination of GDR, subpictures, and virtual boundaries (S0168):

- (Option 1) Do nothing or rewrite virtual boundary information in the SPS in the subpicture sub-bitstream extraction process (without actual proposed changes).

- (Option 2) Add a requirement of bitstream conformance that the extracted first subpicture for the sub-bitstream shall contain an intra-coded area and the following processing are required in sub-bitstream extraction:

    - Locate the first subpicture with an intra-coded area in the GDR picture or the following recovering pictures. This located subpicture becomes the new GDR subpicture.

    - Change the value of nal_unit_type of the new GDR subpicture to GDR_NUT when the new GDR subpicture is not in the current GDR picture.

    - Rewrite the virtual boundary information in the SPS in terms of the new GDR subpicture.

    - Adjust the value of ph_recovery_poc_cnt in the PH when the new GDR subpicture is not the current GDR picture.

- (Option 3, which is on top of option 2) When sps_gdr_enabled_flag is equal to 1, add sps_gdr_subpic_intra_starting_present_flag in the SPS. In the PH, when ph_recovery_poc_cnt is greater than 0 and sps_gdr_subpic_intra_starting_present_flag is equal to 1, add ph_subpic_intra_starting_poc_cnt[ i ] for each subpicture, which specifies "the intra-coded starting point of the i-th decoded subpicture in output order".

The question is whether we need some kind of subpicture-level information about GDR. We do not have area-specific information about what areas have been refreshed. It was commented that this concept had been discussed before. The contribution considers the implication of this for subpictures.

The proposed additional information is essentially metadata that would provide some extra information but would not be completely necessary and would not affect the decoding process. Such information – if determined helpful – could be developed as something to be considered for later specification, e.g., as an SEI message.

It was noted that we do not have any constraint that the picture must contain some intra-coded area in order to be identified as a GDR picture. How the refresh is done is a matter left up to the encoder.

It was asked whether it is really important to try to do something special for the combination of subpictures with GDR.

Further study of the need for a potential SEI approach (after version 1) is encouraged. No current action was recommended.

- Add a constraint: The recovering pictures of a GDR picture shall precede the recovery point picture in decoding order. (S0188 aspect 1)

The draft standard says that all pictures that follow the recovery point picture in output order (and that picture itself) will be correct.

A “recovering picture” is defined as a picture (that follows the GDR picture in decoding order) that precedes the recovery point picture in output order.

An example is a CRA. A CRA is basically a GDR picture with a zero recovery period, but it may be followed in decoding order by RASL pictures, which are pictures that follow the GDR picture in decoding order and precede it in output order. RASL pictures would violate the proposed constraint.

Otherwise, a GDR picture could be used as a CRA replacement.

It was not clear that there is a need to establish the proposed constraint, so no action was recommended.

- Add a constraint: When pps_mixed_nalu_types_in_pic_flag is equal to 1 for a picture, the picture shall not be a recovering picture or the recovery point picture associated with a GDR picture. (S0188 aspect 2)

It was noted that GDR NAL unit types cannot be mixed with other types.

It was commented that the proposed constraint would probably be ordinarily applied by encoders, but it was said that the constraint may not really be necessary.

An example that would violate the constraint was expressed as a GDR followed by a picture that mixes the “dirty area” with something else and then has a later recovery picture.

It was noted that we allow mixing of trailing and STSA pictures and it was said that this could be OK in a recovering picture.

It was not clear that there is a need to establish the proposed constraint, so no action was recommended.

- Add a constraint: The value of recoveryPointPocVal of a layer shall be equal to or greater than the recoveryPointPocVal of a reference layer of the layer. (S0188 aspect 3)

It was commented that R0274 had also proposed this and it was determined not to be necessary.

It was not clear that there is a need to establish the proposed constraint, so no action was recommended.

- Move the syntax element ph_recovery_poc_cnt to be signalled after ph_poc_msb_cycle_val. (S0188 aspect 4)

This is proposed due to resolve a semantics dependency issue, to order the syntax so that the variables computed from a syntax element do not depend on a later syntax element.

This change would require parsing the SPS before using the ph_recovery_poc_cnt to identify the distance to a recovery point. A system could scan through the data for recovery point identification without parsing the SPS.

Because of this consideration, no action was recommended on this.

It was remarked that in the semantics of ph_recovery_poc_cnt and possibly in a NOTE in 8.3.4.2, there is a lack of consideration of the case with recovery POC distance of 0.

AHG Recommendation (editorial BF/expression of existing intent): Fix the description to consider the case with a recovery POC distance of 0. (Possibly also affecting HEVC and AVC.)

- Modify the decoding process for generating unavailable reference pictures to set the values of nuh_layer_id and PicOrderCntVal of unavailable inter-layer reference pictures and mark those pictures as “used for long-term reference”. (S0192 aspect 1)

It was commented that this aspect had been discussed at the May AHG meeting, and that inter-layer reference pictures cannot be unavailable.

It was said that the reason for this existing constraint is for picture loss detection.

There was an editorial issue discussed – in the RPL derivation there is a process that considers the forbidden possibility of an ILRP that is not present in the DPB. This is not an error, but is not strictly necessary. There is a NOTE below that describes the potential inference of an unintentional loss.

- Change the definition of still picture profiles to contain one AU only instead of one picture only. (S0091 aspect 1, S0202 aspect 2)

- If yes, add the following two constraints: (S0202 aspect 2)

    - When sps_video_parameter_set_id is not equal to 0, there shall be one OLS specified in the VPS that contains all the layer present in the bitstream and the OLS has only one output layer.

    - The value of max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] in the DPB parameters shall not be less than the maximum of direct reference layers of any layer in the bitstream.

This would include inter-layer prediction, motion vectors, etc.

See also item 46 of S0138, discussion whether to either change general_one_picture_only_constraint_flag to general_one_au_only_constraint_flag. (S0091 #1) or remove general_one_picture_only_constraint_flag (S0160 #7).

    - The AHG recommended to revisit this in the main meeting, together with the parent bodies.

- Add either of the following constraint for a bitstream conforming to the Main 10 Still Picture profile or Main 4:4:4 10 Still Picture profile (S0202 aspect 1):

- When sps_video_parameter_set_id is greater than 0, there shall be an OLS specified in the VPS that contains only the layer that is present in the bitstream.

- The value of sps_video_parameter_set_id shall be equal to 0.

It was asked whether the picture output flag would also need to be 1 for a picture in the Main 10 Still Picture profile. It was commented that perhaps we should have a constraint that applies to all bitstreams that requires that the bitstream must have at least one picture that is in an output layer that has pic_output_flag equal to 1.

AHG Recommendation (sensibility constraint): Require that the bitstream shall contain at least one picture with pic_output_flag equal to 1 that is in an output layer.

- In the PTL syntax structure, change the signalling of sublayer_level_idc[ i ] to be in descending order of i, i.e., from the maxNumSubLayersMinus1 − 1 to 0, inclusive. (S0203 aspect 1, S0207)

This is for semantics purposes, so that the syntax order would be the same order as the inference dependency order.

AHG Recommendation (cleanup): Adopt, together with the next item.

- In the PTL syntax structure, change the signalling of ptl_sublayer_level_present_flag[ i ] to be in descending order of i. (S0203 aspect 2)

This would match the order for item 37 and was agreed; see the notes for that item.

- In the PTL syntax structure, move the sub-profile fields to be immediately before general_level_idc, which saves two lines of syntax, "if( profileTierPresentFlag ) {" and "}". (S0216)

It was commented that the general PTL info is the most important to systems and decoders and should be kept together, so no action was recommended on this.

- Add an inference rule: When not present, ph_poc_msb_cycle_present_flag is inferred to be equal to 0. (S0239 late, bug ticket #1118)

It was commented that we could either do this inference or avoid using the flag when it is not present.

AHG Recommendation (editorial bug fix/expression of existing intent): either infer ph_poc_msb_cycle_present_flag to be equal to 0 when it is not present or avoid referring to the flag when it is not present.

JVET-S0052 AHG9: Removing separate colour plane coding from VVCv1 [Y.-K. Wang, Z. Deng (Bytedance), J. Boyce (Intel)]

JVET-S0056 AHG9: On field coding [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0075 AHG9: On constraints and inference values when VPS is not present [Hendry, S. Paluri, S. Kim (LGE)]

Aspect 1 of this contribution belongs to this category.

JVET-S0077 AHG9: On derivation of PictureOutputFlag for RASL [Hendry, H. Jang, J. Nam, S. Paluri, S. Kim, J. Lim (LGE)]

JVET-S0078 AHG9: On derivation of NoOutputOfPriorPicsFlag [Hendry, S. Paluri, S. Kim (LGE)]

JVET-S0081 AHG9: On non-referenced picture and POC derivation [Hendry, H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-S0084 AHG9: On RPL constraint of RADL picture [Hendry (LGE)]

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 1 of this contribution belongs to this category.

JVET-S0087 AHG9: On target OLS and sublayers for decoding [M. M. Hannuksela (Nokia)]

JVET-S0103 AHG9: On OLS identification [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 1 of this contribution belongs to this category.

JVET-S0104 AHG9: On DRAP output [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-S0109 AHG9: Signalling of picture rate [B. Heng, W. Wan (Broadcom)]

JVET-S0114 AHG9: On GDR RPL constraint [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0124 AHG9: On unavailable reference pictures [V. Seregin, Y. He, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-S0128 AHG12: On entry point signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

JVET-S0155 AHG9: On EOS NAL units [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0174 AHG9: Miscellaneous cleanups [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 2 of this contribution belongs to this category.

JVET-S0156 AHG9: On level definitions and bitstream conformance [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspects 5, 6, 8, and 9 of this contribution belong to this category.

JVET-S0163 AHG9: On target OLS and sublayers for decoding [M. M. Hannuksela (Nokia), R. Skupin (HHI), Hendry (LG Electronics), B. Choi (Tencent)]

JVET-S0167 AHG9: Constraints on reference picture lists of a RADL picture [L. Chen, S.-T. Hsiang, C.-W. Hsu, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0168 AHG9: On GDR subpictures for sub-bitstream extraction [L. Chen, C.-Y. Chen, C.-W. Hsu, O. Chubach, S.-T. Hsiang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0188 AHG9: On gradual decoding refresh [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0192 AHG9: On reference picture list with generating unavailable reference picture [B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 1 of this contribution belongs to this category.

JVET-S0091 AHG8/AHG9: On constraints of still picture [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

Aspect 1 of this contribution belongs to this category.

JVET-S0202 AHG9: On Still Picture Profile and multi layers aspect [H. Jang, Hendry, J. Nam, N. Park, S. Paluri, S. Kim, J. Lim (LGE)]

JVET-S0203 AHG9: On signalling of sublayer level idc [Hendry (LGE)]

JVET-S0207 AHG9: Order of the sublayer_level_idc[ i ] syntax elements [Z. Zhang, R. Sjöberg, M. Pettersson, M. Damghanian, J. Ström (Ericsson)]

JVET-S0216 AHG9: Parsing order of profile_tier_level( ) [K. Naser, F. Le Leannec, T. Poirier, M. Kerdranvat (InterDigital)]

JVET-S0239 AHG9: Clean-up on derivation of POC [N. Park, Hendry, J. Nam, H. Jang, J. Lim, S. Kim(LGE)] [late]

JVET-S0241 AHG9: On signalling of POC MSB information in picture header [Hendry, H. Jang, N. Park, S. Kim, J. Lim (LGE) [late]]

### General constraints information (GCI) (25+7)

JVET-S0138 AHG9: A summary of proposals on general constraints information [Y. He (Qualcomm), Y.-K. Wang, Z. Deng (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 2110 UTC on 27 May and in the HLS AHG pre-meeting at 1900 UTC on 19 June.

This contribution provides a summary of the 31 proposals on general constraints information (GCI) (the agenda item 3.1.3 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following actions are proposed:

On the GCI syntax structure

- Approaches to enable skipping all GCI syntax elements

  - Add a presence flag for general_constraint_info( ) syntax structure. (S0050 #1, S0127)

    - Infer the values of the GCI flags to 0. (S0050 #1a, S0127)

    - Infer the values of max_bitdepth_constraint_idc to 8 and the value of max_chroma_format_constraint_idc value to 3. (S0050 #1). Or infer the value of max_bitdepth_constraint_idc to 2 and max_chroma_format_constraint_idc to 1. (S0127)

Editor action item: Consider the name of max_bitdepth_constraint_idc to clarify its semantics.

    - Move the alignment bits outside the GCI syntax structure and change the GCI reserved bytes to bits (S0050 #1b).

  - Move the extension length indicator (gci_num_reserved_bytes) from last to first (gci_num_constraint_bytes) in the GCI syntax structure to enable skip all GCI SE signaling when all GCI SEs values are 0. The value of gci_num_reserved_bytes shall be equal to either 0 or 9. (S0092)

It was commented, and agreed, that inference rules should not really be needed; the flags would just have semantics when they are present (just an editorial matter).

It was questioned whether people would really use the flags if they can be skipped.

It was noted that the number of flags is substantial, so having some ability to skip them is desirable.

AHG Recommendation: Agree in spirit to allow some form of skipping the GCI syntax. The exact method was agreed to be for further study to be determined later.

This was further discussed in the HLS AHG pre-meeting at 1900 UTC on 19 June after offline discussion.

A side activity had been conducted for the above. The result from that side activity was a design proposed in S0179: Signal the GCI syntax structure between the general PTL fields and sub-profile fields in the PTL syntax structure. Signal the gci_present_flag at the beginning of the GCI syntax structure and use it to condition the GCI fields, move the byte alignment to the end of the GCI syntax structure, and change the reserved bytes to reserved bits.

The semantics were reviewed and refined (essentially editorially) in group discussion, to be uploaded in a v2 of S0179.

AHG Recommendation (cleanup): Adopt S0179-v2.

Editor action item: The editor is requested to review the clarity of semantics for reserved syntax elements and reserved values of syntax elements, to not only prohibit use in bitstreams conforming to this version of the Specification, but also to clarify that decoders shall allow presence of reserved syntax elements and ignore values of reserved syntax elements when they are present. Where relevant, decoders shall also allow reserved values of existing syntax elements that do not affect the decoding process and picture output (e.g., transfer function and matrix coefficients) and interpret those as having an unspecified meaning, and encoders for the current version shall not use such values.

- Categorize GCI SEs into substructures

- Into 11 sub-structures (format, functionality, NALU, partition, pred, intra, inter, transform/quantization/residual, loop filter, layer, SEI) and add inference value in the semantics (S0105), each substructure is gated by a present flag and, when not present, the value of the corresponding constraint flag is inferred to be 0 (S0105 alt)

- Into 3 sub-structures (capability, coding tool, NALU) (S0111)

- Rearrange GCI SEs to put intra-related GCI flags together, and inter-related GCI flags together (S0129 #3)

Approach c was said to be essentially a subset of approach a.

AHG Recommendation: Adopt approach “a”.

It was later recommended that the order of GCI syntax elements should be aligned with the SPS syntax elements. This was included into the side activity for items 4 to 22, and the report of that side activity is in S0161 (see notes under item 4) below).

It was commented that we should not have category-wise skipping. Although some interest was expressed by a proponent, no action was recommended on that aspect.

New GCI SEs

- Weighted prediction

- Two GCI flag, one for each of sps_weighted_pred_flag and sps_weighted_bipred_flag (S0050 #2, S0105, S0131)

- One GCI flag for both sps_weighted_pred_flag and sps_weighted_bipred_flag (S0058)

There are currently no weighted prediction constraint flags.

AHG Recommendation: Adopt approach “b”.

- no_virtual_boundaries_constraint_flag to constrain sps_virtual_boundaries_enabled_flag (S0050 #2, S0105, S0131)

AHG Recommendation: J. Boyce was asked to coordinate further study offline and make a recommendation for items 4-22.

The side activity also included an effort trying to align the order of GCI syntax elements with the SPS syntax elements, related to item 2) above.

The outcome of the side activity was included in S0161.

This was further discussed in the HLS AHG pre-meeting at 1930 UTC on 19 June after offline discussion.

Additional recommendations in S0161 included the following:

  - Remove any constraints that existing constraint flags impose on other constraint flags (as previously agreed)

  - Rename max_bitdepth_constraint_idc to max_bitdepth_minus8_constraint_idc (a matter of editorial clarity)

  - Remove no_aps_constraint_flag (since there are already constraint flags for each tool that uses an APS and since the flag only controlled presence within the bitstream without controlling presence by external means, which is hard to control).

An alternative is proposed in S0073-v2 aspect 1 option 1, which disables presence in the bitstream and also disables the use of any APS references. It was commented that S0050 item 5 proposes the same modification.

AHG Recommendation (cleanup): Adopt S0161 except additional recommendation “c”, for which we instead adopt S0073-v2 aspect 1 option 1 and editorial recommendation “b”.

There was further discussion of general_non_packed_constraint_flag and general_non_‌projected_‌constraint_‌flag. It was suggested to either move them to VUI or move them outside the GCI structure.

AHG Recommendation (cleanup): Move general_non_packed_constraint_flag and general_non_‌projected_‌constraint_‌flag to the VUI. After the general_interlaced_source_flag.

There was further discussion of single_layer_constraint_flag. It was suggested to just remove the flag. Another suggestion was to move it outside the GCI structure into the PTL structure.

It was noted that the value of the VPS ID being 0 is another clear indication of a lack of layering usage, and those are observable in the bitstream earlier than the PTL information. Another suggestion was to have a no_vps_nal_unit_constraint_flag.

Revisit.

- no_explicit_scaling_list_constraint_flag/no_scaling_list_constraint_flag to constrain

- sps_explicit_scaling_list_enabled_flag shall be equal to 0 (S0050 #2, S0058 S0066, S0067, S0105)

- and aps_params_type shall not be equal to 2 (S0066)

- no_mtt_constraint_flag to constrain SPS MTT SEs sps_max_mtt_hierarchy_depth_intra_slice_luma, sps_max_mtt_hierarchy_depth_inter_slice and sps_max_mtt_hierarchy_depth_intra_slice_chroma (S0058)

- max_luma_transform_size_32_constraint_flag or no_luma_transform_size_64_constraint_flag to constrain sps_max_luma_transform_size_64_flag (S0058, S0066)

- no_long_term_ref_pic_constraint_flag to constrain sps_long_term_ref_pics_flag (S0058)

- max_log2_ctu_size_constraint_idc (u(2)) to constrain sps_log2_ctu_size_minus5 (S0066)

- max_layers_constraint_idc to constrain vps_max_layers_minus1 (S0113)

- max_sublayers_constraint_idc to constrain vps_max_sublayers_minus1 (S0113)

- max_subpics_constraint_idc to constrain sps_num_subpics_minus1 (S0113)

- no_pic_partition_constraint_flag to constrain pps_no_pic_partition_flag (S0113)

- no_rectangular_slice_constraint_flag to constrain pps_rect_slice_flag (S0113)

- one_slice_per_subpicture_constraint_flag to constrain pps_single_slice_per_subpic_flag (S0113)

- no_conformance_window_constraint_flag to contrain sps_conformance_window_flag (S0131)

- general_single_ols_per_layer_set_constraint_flag to constraint no two OLSs contain the same layer set (S0103#2)

- no_scalability_constraint_flag to constraint scalable and layered coding (S0105)

- no_360_video_constraint_flag to constrain sps_ref_wraparound_enabled_flag, and equirectangular projection SEI messages or generalized cubemap projection SEI messages (S0105)

- lossless_coding_constraint_flag to constrain all picture are losslessly coded (S0054 #8)

- no_scc_constraint_flag to constrain sps_ibc_enabled_flag, sps_bdpcm_enabled_flag and sps_palette_enabled_flag (S0105)

- no_VUI_constraint_flag to constrain sps_vui_parameters_present_flag (S0113)

- SEI related new GCI flags

- no_scalable_nesting_SEI_constraint_flag to constrain scalable nesting SEI (S0105)

- no_subpic_level_SEI_constraint_flag to constrain subpicture level SEI (S0105)

- no_filler_payload_SEI_constraint_flag to constrain filler payload SEI (S0105)

- no_user_data_reg_SEI_constraint_flag to constrain user data registered by Recommendation ITU T.35 SEI (S0105)

- no_user_data_unreg_SEI_constraint_flag to constrain data unregistered SEI (S0105)

- no_film_grain_SEI_constraint_flag to constrain film grain characteristics SEI (S0105)

- no_parameter_set_incl_SEI_constraint_flag to constrain parameter sets inclusion indication SEI (S0105)

- no_decoded_picture_hash_SEI_constraint_flag to constrain decoded picture hash SEI (S0105)

- no_mcdv_SEI_constraint_flag to constrain mastering display colour volume SEI (S0105)

- no_cll_SEI_constraint_flag to constrain content light level SEI (S0105)

- no_drap_sei_constraint_flag to constrain DRAP indication SEI (S0105, S0113)

- no_alt_transfer_char_SEI_constraint_flag to constrain alternative transfer characteristics SEI (S0105)

- no_ambient_view_envir_SEI_constraint_flag to constrain ambient viewing environment SEI (S0105)

- no_ccv_SEI_constraint_flag to constrain content colour volume SEI (S0105)

- no_omni_video_SEI_constraint_flag to constrain ERP, GCMP, sphere rotation, region-wise packing and omnidirectional viewport SEI (S0105, S0113)

- no_field_frame_SEI_constraint_flag to constrain frame-field information SEI (S0105)

- no_sar_SEI_constraint_flag to constrain sample aspect ratio SEI (S0105)

- general_non_HRD_constraint_flag to constrain BP, PT and DU SEI (S0113)

No action was recommended on these SEI-message-related constraints.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Subpicture, slices, and tiles related

- When one_tile_per_pic_constraint_flag is equal to 1, NumTilesInPic shall be equal to 1. (S0050 #3b)

This proposal seemed editorial.

Editor action item: Check/clarify as appropriate.

- When one_subpic_per_pic_constraint_flag is equal to 1,

- sps_num_subpics_minus1 shall be equal to 0. (S0050 #3c, S0112)

Aspect “a” seemed editorial.

Editor action item: Check/clarify aspect “a” as appropriate.

- sps_subpic_info_present_flag shall be equal to 0. (S0105)

It was commented that aspect “b” would interfere with some BEAM usage involving the sh_subpic_id, so no action was recommended on that.

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0054 #3, S0106 #5, S0112, S0131)

Some reluctance was expressed for aspect “c” in regard to BEAM. We often allow tools to be not used without requiring associated constraint flags to be equal to 1, so no action was recommended on that.

It was noted that the software has some configuration issues for the constraint flags, such that it tends to set the constraint flags whenever some feature is not being used. This could cause splicing/concatenation problems. The software coordinator suggested to have configuration support for multiple modes for these flags in the software – e.g. a) set the flag if the feature is disabled, b) explicitly control the value of the flag. Volunteers for this software work were solicited.

- When one_slice_per_pic_constraint_flag is equal to 1, pps_rect_slice_flag shall be equal to 1. (S0050#5c, S0106 #6)

Such a change did not seem necessary, so no action was recommended on this.

- When one_slice_per_pic_constraint_flag is equal to 1, pps_num_slices_in_pic_minus1 shall be equal to 0. (S0050#5c)

Such a change did not seem necessary, so no action was recommended on this.

Editor action item: Consider clarifying semantics – e.g., when pps_rect_slice_flag is 1, then pps_num_slices_in_pic_minus1 needs to be equal to 0 (which is already required).

- When one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint flag are both equal to 1, the value of pps_no_pic_partition_flag shall be equal to 1. (S0050 #5, S0112)

Such a change did not seem necessary, so no action was recommended on this.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Single-layer, single-picture related

- Change the semantics of single_layer_constraint_flag equal to 1 from "single_layer_constraint_flag equal to 1 specifies that sps_video_parameter_set_id shall be equal to 0" to one of the following: (S0090)

- Option 1: single_layer_constraint_flag equal to 1 specifies that one of the following conditions shall be fulfilled (1) sps_video_parameter_set_id is equal to 0; (2) The value of vps_max_layers_minus1 in the referred VPS is equal to 0 and all VCL NAL units in the CVS have the same value of nuh_layer_id.

- Option 2: single_layer_constraint_flag equal to 1 specifies that all VCL NAL units in the CVS shall have the same value of nuh_layer_id.

Currently this flag can only be set to 1 if there is no VPS. These proposals allow it to be set to 1 under additional circumstances. Option 2 fits the name of the flag, but there was the question of what is the intent for the use of the flag. Is it for not using the VPS or is it for just indicating having one layer? Some participants said the intent for adoption of the flag was consistent with option 2.

AHG Recommendation: Adopt option 2.

- When single_layer_constraint_flag is equal to 1, all_layers_independent_constraint_flag shall be equal to 1. (S0050 #4, S0054 #6)

Such a change did not seem necessary, so no action was recommended on this.

- When general_one_picture_only_constraint_flag is equal to 1,

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0050 #4)

- no_trail_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

- no_stsa_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

- no_rasl_constraint_flag shall be equal to 1 (S0050 #4, S0054#5)

- no_radl_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

The first picture in the bitstream needs to be IRAP or GDR.

In the current draft, we cannot indicate conformance to the Main 10 Still Picture profile without sending the constraint flag. There is also a max dec pic buffering issue.

In HEVC, the Main Still Picture profile has its own value of profile_idc, and there were profile compatibility flags, which are not present in VVC.

AHG Recommendation: Use a different profile_idc for Main 10 Still Picture (we already require Main 10 profile decoders to decode such bitstreams). Do not require the constraint flag to be 1 as part of the profile definition.

It was asked whether we need the general_one_picture_only_constraint_flag.

Further study was recommended to study the final signalling relationship for the still picture profile(s).

It was noted that there is also a proposal S0129 to have a one-picture-only indication in the SPS.

It was commented that all expressed constraints on the values of GCI flags should be removed, as these should just be specified with one-way meaning.

AHG Recommendation: Remove all constraints that require GCI flags to be equal to a value that imposes a constraint.

AHG discussion for Sessions 1-8 stopped here 2315 UTC Wednesday 27 May.

AHG discussion for Sessions 11-12 started here 20:40 UTC Friday 19 June.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Intra only related

- When intra_only_constraint_flag is equal to 1,

- all_layers_independent_constraint_flag shall be equal to 1 (S0050 #4, S0106 #4. S0112)

- no_ref_pic_resampling_constraint_flag shall be equal to 1 (S0050 #4, S0112, S0131)

- no_res_change_in_clvs_constraint_flag shall be equal to 1 (S0050 #4)

- no_sbt_constraint_flag shall be equal to 1 (S0050 #4, S0060, S0106 #1, S0112, S0129 #3, S0131)

- no_trail_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_stsa_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_rasl_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_radl_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_cra_constraint_flag shall be equal to 1 (S0106 #3)

- no_gdr_constraint_flag shall be equal to 1 (S0106 #3)

- no_idr_constraint_flag shall be equal to 0 (S0106 #3)

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0112)

- sps_weighted_pred_flag shall be equal to 0. (S0061)

- sps_weighted_bipred_flag shall be equal to 0. (S0061)

- sps_long_term_ref_pics_flag shall be equal to 0. (S0061)

- sps_inter_layer_ref_pics_present_flag shall be equal to 0. (S0061)

- sps_idr_rpl_present_flag shall be equal to 0. (S0061)

- sps_rpl1_same_as_rpl0_flag shall be equal to 0. (S0061)

- sps_num_ref_pic_lists[ i ] shall be equal to 0. (S0061)

- sps_sbtmvp_enabled_flag shall be equal to 0. (S0061)

- sps_six_minus_max_num_merge_cand shall be equal to 0. (S0061)

- sps_log2_parallel_merge_level_minus2 shall be equal to 0. (S0061)

- sps_explicit_mts_inter_enabled_flag shall be equal to 0. (S0061)

- pps_rpl1_idx_present_flag shall be equal to 0 (S0061)

- pps_rpl_info_in_ph_flag shall be equal to 0 (S0061)

- pps_ref_wraparound_enabled_flag shall be equal to 0 (S0061)

- pps_output_flag_present_flag shall be equal to 0 (S0112)

- ph_pic_output_flag shall be equal to 1 (S0112)

- ph_inter_slice_allowed_flag shall be equal to 0 (S0061, S0112)

- ph_non_ref_pic_flag shall be equal to 1 (S0112)

No action on a-l, per previous discussions about constraining constraint flags. For the others, it was commented that it is desirable to be able to extract all the I pictures from a bitstream and set the intra_only_constraint_flag to 1 without needing to rewrite headers. No action was recommended for this, to avoid the need for header rewriting just to avoid constraint violations.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Random access related

- When no_cra_constraint_flag is equal to 1, no_rasl_constraint_flag shall be equal to 1. (S0054 #4, S0112)

No action, per previous discussions about constraining constraint flags.

- When no_idr_constraint_flag is equal to 1 and no_cra_constraint_flag is equal to 1, no_radl_constraint_flag shall be equal to 1. (S0054 #4)

No action, per previous discussions about constraining constraint flags.

- At least one of the no_gdr_constraint_flag, no_idr_constraint_flag, and no_cra_constraint_flag shall be equal to 0. (S0050 #5, S0112)

No action, per a general desire to avoid constraining constraint flags.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: APS related

- When no_alf_constraint_flag is equal to 1, aps_params_type shall not be equal to 0. (S0066)

No action was recommended on this, as it would prohibit presence of non-referenced APSs (which could, e.g., be referenced in a later CVS).

- When no_lmcs_constraint_flag equal to 1, aps_params_type shall not be equal to 1. (S0066)

No action was recommended on this, as it would prohibit presence of non-referenced APSs (which could, e.g., be referenced in a later CVS).

- When no_aps_constraint_flag is equal to 1,

- no_lmcs_constraint_flag shall be equal to 1. (S0050 #4, S0067#1, S0105)

- no_ccalf_constraint_flag shall be equal to 1. (S0050 #4)

- no_explicit_scaling_list_constraint_flag shall be equal to 1. (S0050 #4, S0067 #2, S0105)

- sps_ccalf_enabled_flag, ph_num_alf_aps_ids_luma, ph_alf_cb_flag, ph_alf_cr_flag, sh_num_alf_aps_ids_luma, sh_alf_cb_flag, and sh_alf_cr_flag all be equal to 0. (S0050 #5, S0073 #1.1)

- (alternative to 38.d and 38.f) sps_chroma_alf_enabled_flag (newly added as part of this item), sps_chroma_alf_enabled_flag and sh_num_alf_aps_ids_luma shall be equal to 0 (S0073 #1.2)

- (alternative to 38.d and 38.e) sps_alf_enabled_flag and sps_ccalf_enabled_flag shall both be equal to 0 (S0073 #1.3)

No additional action was needed on this, per previous discussions about constraining constraint flags and per action recommended earlier for sub-item d as noted elsewhere in the notes.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Video format related

- The value of the u(4)-coded max_bitdepth_minus8_constraint_idc shall be in the range of 0 to 8, inclusive. (S0046 #1, S0050 #3a).

AHG Recommendation (sensibility constraint): Adopt.

- When max_chroma_format_constraint_idc is equal to 0, no_chroma_qp_offset_constraint_flag shall be equal to 1. (S0046 #3, S0060, S0112, S0131).

No action, per previous discussions about constraining constraint flags.

- When max_chroma_format_constraint_idc is equal to 0, no_act_constraint_flag shall be equal to 1. (S0131)

No action, per previous discussions about constraining constraint flags.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Others

- When no_ref_pic_resampling_constraint_flag to 1, no_res_change_in_clvs_constraint_flag shall be equal to 1. (S0054 #7, S0105, S0112, S0131)

No action, per previous discussions about constraining constraint flags.

- When no_transform_skip_constraint_flag is equal to 1, no_bdpcm_constraint_flag shall be equal to 1. (S0050 #4, S0059, S0069#1, S0105, S0112, S0131)

No action, per previous discussions about constraining constraint flags.

- When no_affine_motion_constraint_flag is equal to 1, no_prof_constraint_flag shall be equal to 1. (S0046 #2, S0112, S0131)

No action, per previous discussions about constraining constraint flags.

- When all_layers_independent_constraint_flag is equal to 1, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 0. (S0050 #5)

It was noted that this constraint already expressed (indirectly), so no action was needed on this.

GCI and profiles

- Either change general_one_picture_only_constraint_flag to general_one_au_only_constraint_flag. (S0091 #1) or remove general_one_picture_only_constraint_flag (S0160 #7)

It was noted that a profile_idc value already indicates one picture only.

It was commented that extraction from a Main profile bitstream might be a potential use of the flag.

Revisit after consideration of proposals to change the Still Picture profile.

- Constrain the values of max_chroma_format_constraint_idc and max_bitdepth_constraint_idc in main 10 profile. (S0094)

It was noted that these syntax elements are defined in a backwards way from the way other constraints are expressed.

AHG Recommendation (consistency cleanup): Make the maximum bit depth to be 16 − max_bitdepth_‌constraint_idc and the maximum chroma format be 3 − max_chroma_format_constraint_idc.

No other action on this.

GCI editorial changes

Editor action item: The editor is asked to consider the below suggestions.

- Rename "max_bitdepth_constraint_idc" to "max_bitdepth_minus8_constraint_idc". (S0105)

No action on this per the decision on the previous item.

- Rename "general_non_packed_constraint_flag" to "general_non_packed_SEI_constraint_flag", and "general_non_projected_constraint_flag" to "general_non_projected_SEI_constraint_flag". (S0105)

No action (capital letters, not very necessary, being moved anyway).

- Move the constraint "When general_frame_only_constraint_flag is equal to 1, the value of sps_field_seq_flag shall be equal to 0" from SPS semantics to GCI semantics (S0054 #1, S0105, S0209 #2) and/or rename general_frame_only_constraint_flag" to "no_field_seq_constraint_flag" and change the semantics to constrain sps_field_seq_flag (S0195 #2).

- Move the constraint "When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of pps_mixed_nalu_types_in_pic_flag shall be equal to 0" from PPS semantics to GCI semantics. (S0054 #2, S0209 #2)

- Replace the while-loop for GCI byte alignment with 6 zero-valued bits. (S0209 #3)

- Prefix all syntax elements in the general constraint information with 'gci_'. (S0209 #4)

New submissions (for the June 10th submission deadline) on adding new GCI fields, or removing or changing existing GCI fields

- Add a "no_ts_rrc_constraint_flag" (S0238 late)

This was not just an editorial proposal. The prior related flag had been agreed to be removed in earlier discussion. The proposal suggested to add a flag to require sh_tsrc_disabled_flag to be equal to 0. The CTC use would match this flag being 1.

It was commented that this does not constrain a tool but rather a combination. The regular residual coding would still be present in the decoder.

No action was recommended on this.

- Remove general_non_packed_constraint_flag and general_non_projected_constraint_flag (S0195 #1)

See notes elsewhere, agreeing to move these flags.

- Add new GCI flags

  - no_subpic_info_constraint_flag to constrain sps_subpic_info_present_flag (S0195 #3)

  - no_idr_rpl_constraint_flag to constrain sps_idr_rpl_present_flag (S0195 #3)

  - no_hrd_constraint_flag to constrain vps_general_hrd_params_present_flag and sps_general_hrd_params_present_flag (S0195 #3)

  - no_bidirectional_prediction_constraint_flag to constrain pps_rpl1_idx_present_flag and num_ref_entries[ 1 ][ rplsIdx ] (S0209 #1)

  - no_parallel_merge_constraint_flag to constrain sps_log2_parallel_merge_level_minus2 (S0209 #1)

  - no_switchable_cabac_init_constraint_flag to constrain pps_cabac_init_present_flag (S0209 #1)

  - no_entropy_coding_sync_constraint_flag to constrain sps_entropy_‌coding_sync_enabled_flag (S0230)

J. Boyce was asked to coordinate offline study of these proposals for revisit during the main meeting.

JVET-S0046 AHG9: General constraint information semantics constraints [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, Z. Zhang, J. Ström, D. Liu (Ericsson)]

JVET-S0050 AHG9: On general constraints information [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, K. Fan (Bytedance)]

JVET-S0054 AHG9: Clean-ups on general constraint flags [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0058 AHG9: Additional General Constraint Flags [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0059 AHG9: Fixing the Semantics of no_bdpcm_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0060 AHG9: Fixing the Semantics of no_chroma_qp_offset_constraint_flag and no_sbt_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0061 AHG9: HLS Cleanup of All-Intra Profile [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0066 AHG9: On constraint flags [C. Rosewarne, J. Gan (Canon)]

JVET-S0067 AHG9: General constraint information on features using APS [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0069 AHG9: General constraint information on features related to transform skip mode [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspect 1 of this contribution belongs to this category.

JVET-S0073 AHG9: Bug-fix of the constraint flag of VVC [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 1 of this contribution belongs to this category.

JVET-S0090 AHG8/AHG9: On single_layer_constraint_flag [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

JVET-S0091 AHG8/AHG9: On constraints of still picture [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

Aspect 2 of this contribution belongs to this category.

JVET-S0092 On constraint info signalling [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-S0094 AHG9: Profile-level constraints on chroma format and bit depth constraint flags [K. Sharman, S. Keating, A. Browne (Sony)]

JVET-S0103 AHG9: On OLS identification [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 2 of this contribution belongs to this category.

JVET-S0105 AHG9: Modification of general constraint information [S. McCarthy, P. Yin, T. Lu, F. Pu, W. Husak, T. Chen (Dolby)]

All aspects other than the aspect on changing the semantics of no_tsrc_constraint_flag belong to this category.

JVET-S0106 AHG9: Some cleanups on general constraint flags [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

JVET-S0111 AHG9: On GCI syntax structure [Y. He, Y.-J. Chang, N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0112 AHG9: On GCI semantic constraints [Y. He, N. Hu, Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0113 AHG9: On new GCI flags [Y. He, Y.-J. Chang, N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0127 AHG9: On general constraint info signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 3 of this contribution belongs to this category.

JVET-S0131 AHG9: On general constraint information syntax [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspect 7 of this contribution belongs to this category.

JVET-S0161 AHG9: Report of side activity on GCI syntax elements [J. Boyce]

JVET-S0179 AHG9: On conditional signalling of GCI fields [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang, K. Fan (Bytedance), J. Samuelsson, S. Deshpande, A. Segall (Sharp), M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm), Hendry, J. Nam (LGE), S. Wenger, B. Choi (Tencent), M. Zhou (Broadcom)]

JVET-S0195 AHG9: On GCI cleanups [Y. He, Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0209 AHG9: On General Constraint Information [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, J. Enhorn, J. Ström, D. Liu (Ericsson)]

JVET-S0230 AHG9: New GCI flag [T. Ikai (Sharp)]

JVET-S0238 AHG9: GCI for RRC [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)] [late]

### SPS, PPS, and APS cleanups (9+8)

JVET-S0142 AHG9: A summary of proposals on SPS, PPS, and APS cleanups [Hendry (LGE)]

This contribution was discussed in the HLS AHG meeting at 2330 UTC on Wednesday 27 May and in the HLS AHG pre-meeting at 2225 UTC on 19 June.

This contribution provides a summary of the 16 proposals on SPS, PPS, and APS cleanups (the agenda item 3.1.4 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following changes are proposed:

- It is asserted that when extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, it is allowed that no extension_data_flag syntax elements be present. The following options for change are proposed (JVET-S0042)

  - Option 1: When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, enforce the presence of at least one extension_data_flag syntax element in the syntax.

From the following design in the current spec:

| dci_extension_flag | u(1) |
| --- | --- |
| if( dci_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |

To the following:

| dci_extension_flag | u(1) |
| --- | --- |
| if( dci_extension_flag ) { |  |
| do |  |
| dci_extension_data_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| } |  |

  - Option 2: Relax the semantics so that an extension_flag in DCI, VPS, SPS, PPS, or APS equal to 1 specifies that specifies extension_data_flag syntax elements may be present. (Currently, it says these flags are present.)

  - Option 3: Do nothing

It was commented that option 2 seems like just a better editorial expression of the what the current syntax already specifies.

AHG Recommendation (editorial clarification): Option 2.

Note: This should be also considered as an errata report for HEVC semantics clarification.

- Rearrange some syntax elements in SPS so that the syntax elements of similar coding tools are grouped together.

  - All of the transform related SPS syntax are placed together (JVET-S0074 aspect 1)

  - sps_lmcs_enabled_flag is signalled right after signalling of sps_ccalf_enabled_flag (JVET-S0074 aspect 1)

  - Group the dual tree related syntax elements together in the SPS (JVET-S0132 aspect 1)

  - Rearrange the partition constraint related syntax elements in the SPS to be consistent with the ordering of the corresponding syntax elements in the picture header (PH). More specifically, group such syntax in the SPS based on whether they are intra or inter related (JVET-S0132 aspect 2)

AHG Recommendation (minor cleanup of logical syntax order): Adopt. It is also desirable to have the constraint flag order match SPS order (to be considered by J. Boyce in further study).

- Not signalling sps_max_sublayers_minus1 when sps_ptl_dpb_hrd_params_present_flag is equal to 0 (JVET-S0079)

  - Move the signalling of sps_max_sublayers_minus1 and sps_reserved_zero_4bits immediately before the signalling of profile_tier_level( 1, sps_max_sublayers_minus1 )

  - Condition the presence of the two syntax elements such that they are present only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

The proponent reported that the sps_max_sublayers_minus1 value is not actually used when sps_ptl_dpb_hrd_params_present_flag is equal to 0.

It was commented that there may be some interaction with other proposals or the extraction process.

It was commented that sps_max_sublayers_minus1 could be useful to know even if not necessary for parsing and the decoding process.

No clear need for action was evident, so no action was recommended.

- Add a constraint such that when sps_video_parameter_set_id is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of sps_max_sublayers_minus1 shall be equal to the value of vps_max_sublayers_minus1 (JVET-S0115 aspect 2).

It was commented that this had been proposed in JVET-R0125 item 2. At that time a concern had been expressed about what would happen if some sublayers are removed and the SPS is rewritten.

It has been intended that it should be allowed to removed sublayers without rewriting the VPS.

AHG Recommendation (bug fix/expression of existing intent): Change the name and semantics of vps_all_layers_same_num_sublayers_flag to only control VPS syntax without requiring the number of sublayers to be the same for all layers.

No action on the proposed change, due to the agreement recorded above.

- Add new flag sps_one_picture_only_constraint_flag and use the flag to skip signalling of some syntax elements in SPS. The skipped syntax elements include inter prediction, RPL, and POC related syntax elements in both SPS and PH (JVET-S0129 aspect 2).

It was commented that this would require PH rewriting when extracting a single picture from the bitstream (e.g., to remove RPL and POC).

Concern was expressed about POC, as we allow sending POC for IDR pictures.

A similar proposal was considered at the previous meeting, for intra-only rather than for single-picture-only. The basic issues seemed similar.

It was commented that it is an encoder choice how to set the values of non-relevant syntax elements if the bitstream will contain only intra pictures or only one picture.

No clear need for action was identified, so no action was recommended.

- Constrain the value of pps_alf_info_in_ph_flag to be equal to 0 when the PH is in the SH (JVET-S0049 aspect 4)

It was asked why we hadn’t already done something like this, since we did something like this for five other similar syntax elements (see the next item for a list). There had been some argument of wanting the ALF info to be early in the header. However, it was commented that the ALF info can be in the SH already.

It was noted that another proposal has this as one of two options for action (see item 9a in the S0143 summary for S0120).

AHG Recommendation (consistency/cleanup): Adopt.

- When pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, constrain the values of the 6 PPS flags (i.e., pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag) to be all equal to 0 (JVET-S0049 aspect 5).

The motivation / justification of the proposed changes is as follows:

  - To be better aligned with the inference of the value to 0 when pps_no_pic_partition_flag is equal to 1.

  - For more optimal syntax in the case of pps_rpl_info_in_ph_flag or pps_wp_info_in_ph_flag.

  - For the case of pps_alf_info_in_ph_flag, to enable the use of ALF for the same use case that applies independent parallel encoding of different subpictures as above.

It was commented that this could require SH rewriting if there is an original bitstream and one subpicture is extracted as one picture with one slice. These flags could be in the PPS and would need to be moved to the SH.

It was commented that there is a similar issue for when pps_no_pic_partition_flag is equal to 1. However, it was commented that this flag could be set to 0.

After offline further study, the proposal was withdrawn in HLS AHG pre-meeting at 2225 UTC on 19 June.

- Rearrange the syntax elements related to inter slices in the picture parameter set (PPS) in a similar manner as the grouping of the inter-slice related syntax elements in the PH. More specifically, group syntax elements in PPS based on whether they are intra or inter related (JVET-S0132 aspect 3).

It was commented that this change is pretty minor and logical, and this had been tested with software. See also item 2 above.

AHG Recommendation (consistency/cleanup): Adopt.

- Currently due to existing constraints, only 3 bits out of 5 bits of APS Id are needed in current profiles. The following is proposed: split the u(5)-coded aps_adaptation_parameter_set_id into u(2)-coded aps_reserved_zero_2bits followed by u(3)-coded aps_adaptation_parameter_set_id (JVET-S0049 aspect 6).

Currently we signal the APS ID first, followed by the APS type, but the value spaces for different APS types are distinct.

It was commented that in some future profile we might want a larger range of values.

It was concluded that, as proposed, this would really be only an editorial change.

No clear need for action was evident, so no action was recommended.

- If aps_chroma_present_flag is equal to 0, due to an existing constraint, alf_luma_filter_signal_flag must be equal to 1. Based on that, it is proposed that when aps_chroma_present_flag is equal to 0, alf_luma_filter_signal_flag is skipped and inferred to be equal to 1. Otherwise, when alf_chroma_filter_signal_flag and alf_cc_cb_filter_signal_flag are both equal to 0, alf_cc_cr_filter_signal_flag is skipped and inferred to be equal to 1 (JVET-S0049 aspect 7).

No clear need for action was evident, so no action was recommended.

- It is asserted that the memory to store APSs may exceed the max memory used to store the max number of APSs in a PU due to the fact that in a PU, prefix and suffix APS NAL units with particular APS identifier and type can have different contents. The following two constraints are proposed (JVET-S0122)

  - In a PU, for an APS type, a signalled suffix APS shall not have the same identifier as a referenced APS.

It was commented that this topic had been raised the previous meeting.

It was commented that this depends on whether the decoding process is applied after receiving the VCL NAL units or after also receiving the suffix APS NAL units. If the picture is decoded before parsing the suffix APS NAL units, the memory of the prior APS content can be re-used in the on-chip memory.

No clear need for action was evident, so no action was recommended.

  - Any two suffix (prefix resp.) APS NAL units with particular APS identifier and type signalled in a PU cannot have the same APS identifier

This would disallow repetition. It was commented that such repetition is not really a problem.

No clear need for action was evident, so no action was recommended.

Review of the above items in this category completed ~0120 UTC 28 May.

Discussion resumed here in the HLS AHG pre-meeting at 2225 UTC on 19 June.

- Allow inferring some SPS syntax elements from SPS in the reference layer. It is asserted that in HEVC such mechanism is allowed (i.e., for scaling list data structure). If we allow this mechanism, the following syntax elements are proposed to be inferred from SPS in direct reference layer (JVET-S0166):

  - RPLs of the current SPS from the SPS referred to by the direct reference layer.

  - Subpicture information of the current SPS from the SPS referred to by the direct reference layer

Mechanism detail: for each category of syntax elements that can be inferred, add a new flag to specify whether the syntax elements are inferred or not. If inferred, signal additional syntax element to specify the index of the direct reference layer.

Concern was expressed about introducing such a prediction across layers at this late stage.

A benefit of coding efficiency could be argued, but did not seem like an important consideration.

The proponent said the proposal might help encoders avoid conformance problems.

It was noted that SPS sharing is allowed. This would be a form of partial SPS sharing.

No action was recommended on this, to be conservative about considering late changes.

- It is asserted that the current semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag are inaccurate. It currently says that it control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements. However, it actually controls the presence of those syntax elements for temporal sub-layers that are not the highest. It is proposed to change the semantics to be more accurate (JVET-S0169 aspect 1). (added or changed in bold)

sps_sublayer_dpb_params_flag is used to control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements in the dpb_parameters( ) syntax strucure in the SPS for i in range from 0 to sps_max_sublayers_minus1 − 1, inclusive, when sps_max_sublayers_minus1 is larger than 0. When not present, the value of sps_sublayer_dpb_params_flag is inferred to be equal to 0.

vps_sublayer_dpb_params_present_flag is used to control the presence of max_dec_pic_buffering_minus1[ j ], max_num_reorder_pics[ j ], and max_latency_increase_plus1[ j ] syntax elements in the dpb_parameters( ) syntax strucure in the VPS for j in range from 0 to vps_dpb_max_tid[ i ] − 1, inclusive, when vps_dpb_max_tid[ i ] in VPS is larger than 0. When not present, vps_sub_dpb_params_info_present_flag is inferred to be equal to 0.

It is noted that j instead of i is used for specifying the range of syntax element array max_dec_pic_buffering_minus1[ ], max_num_reorder_pics[ ], and max_latency_increase_plus1[ ] in order to avoid confusion of the i in vps_dpb_max_tid[ i ].

It was commented that this is just editorial and should be considered by the editor.

Editor action item: The editor is asked to consider this potential clarification.

- Move the two syntax elements (sps_chroma_format_idc and sps_log2_ctu_size_minus5) to take the place of sps_reserved_zero_4bits. (JVET-S0186 aspect 1)

The reserved bits were said to have been there only for byte alignment.

AHG Recommendation (cleanup): Adopt.

- Reserve sps_seq_parameter_set_id value equal to 15 for future extensibility. Additionally, do the same for value 63 of pps_pic_parameter_set_id. (JVET-S0186 aspect 2)

No clear need for action on this seemed evident.

- Move the location of decoded picture buffer (DPB) parameters syntax structure and hypothetical reference decoder (HRD) parameters syntax structure to directly follow sps_ptl_dpb_hrd_params_present_flag. (JVET-S0199).

Currently only the signalling of PTL syntax structure directly follows sps_ptl_dpb_hrd_params_present_flag (i.e., early in SPS), the signalling of DPB syntax structure is in the middle of SPS, and HRD syntax structure is near the end of SPS. The proposal would move DPB and HRD syntax structures to early position in the SPS.

It was commented that moving DBP and HRD syntax earlier could put it ahead of other syntax that may be more important to access from the system perspective and that saving some condition checks in the parsing is not so important, so no action was recommended on this.

- Change the coding of syntax element sps_log2_transform_skip_max_size_minus2 from ue(v) to u(2). (JVET-S0204).

It was asked whether some future profile might want to allow a value greater than 3, and this seemed possible, although it was commented that such a future profile would probably have other, much more significant, changes in it.

In HEVC, RExt added larger TS block sizes without a syntax change for that, and used the same type of coding, although in the PPS rather than SPS.

It was noted that this would increase the number of bits used for coding the value 0.

Since no clear need for action was identified, no action was recommended on this.

- Add additional conditions for the presence of the six PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag, as follows (S0164) (additions in italic):

| ... |  |
| --- | --- |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) && pps_deblocking_filter_override_enabled_flag ) |  |
| pps_dbf_info_in_ph_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |

| ... |  |
| --- | --- |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) ) { |  |
| pps_rpl_info_in_ph_flag | u(1) |
| pps_sao_info_in_ph_flag | u(1) |
| pps_alf_info_in_ph_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && pps_rpl_info_in_ph_flag ) |  |
| pps_wp_info_in_ph_flag | u(1) |
| pps_qp_delta_info_in_ph_flag | u(1) |
| } |  |
| … |  |

The contribution was said to be similar in spirit to item 7 above (S0049).

The desire is that when there is only one slice in the picture, to avoid signalling some information in the PPS.

It was commented that if an original bitstream has multiple slices and has this signalling in the PH, then extraction of a single slice could cause a rewriting requirement. The proponent indicated that there is a provision to avoid the rewriting based on the single slice per subpicture flag equal to 1 (at some expense of bits).

No clear need for action was identified, and thus no action was recommended.

- Enable removal of parameter sets (i.e., PPS and APS) in non-output layer that is not used as reference for decoding of pictures in output layers during extraction process, as follows: (JVET-S0219 aspect 1)

- Modify the semantics of vps_max_tid_il_ref_pics_plus1[ i ][ j ] such that when it is greater than 0 it means for decoding pictures of the i-th layer, no parameter set and picture from the j-th layer with TemporalId greater than vps_max_tid_il_ref_pics_plus1[ i ][ j ] − 1 is used as reference.

- In extraction process, in addition to removing VCL NAL unit, also remove PPS and APS with TemporalId that is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

It was commented that this makes more sense for the APS than the PPS and that APSs are a bigger problem. This would treat APS more similar to PH for the extraction.

“Smart” extractors were discussed, and whether the extraction process model is sufficient.

Several participants saw value in being able to identify and remove useless APS from the bitstream based on this proposal. The change is also small.

AHG Recommendation (cleanup): Adopt APS aspect only.

- Move the signalling of aps_params_type to an earlier position and change the signalling of aps_adaptation_parameter_set_id from u(5) to u(v). The length of the syntax element depends on the type of the APS. (JVET-S0219 aspect 2)

A participant said this (or swapping the order of the ID and type) may be useful for extensibility, as some future APS type might need more ID values. The fact that the type is before the ID seemed to just be for historical reasons – it was originally proposed that way and never modified.

It was commented that we took no action on something similar previously.

AHG Recommendation (cleanup): Just change the order to put the type before the ID.

- Move the flag indicating the presence of sh_cabac_init_flag from PPS to SPS. (JVET-S0235)

The location is a carry-over from HEVC.

No action seemed clearly necessary, so no action was recommended on this.

JVET-S0042 JVET-S0042 AHG9: On the parameter set extension mechanism [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)]

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspects 4 to 7 of this contribution belong to this category.

JVET-S0074 AHG9: On SPS, PH, SH syntax order [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 1 of this contribution belongs to this category.

JVET-S0079 AHG9: On signalling of sps_max_sublayers_minus1 [Hendry (LGE)]

JVET-S0115 AHG9: On SPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 2 of this contribution belongs to this category.

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 2 of this contribution belongs to this category.

JVET-S0132 AHG9: On syntax signaling order in SPS and PPS [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0122 AHG9: On APS memory constraint [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-S0164 AHG9: On signalling six PPS flags for indicating the presence of syntax in PH/SH [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0166 AHG9: Inferring information of the SPS from the reference layer [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0169 AHG9: Miscellaneous Cleanups for HLS [B. Wang, S. Esenlik, H. Gao, E. Alshina (Huawei)]

Aspect 1 of this contribution belongs to this category.

JVET-S0186 AHG9: On SPS Cleanup [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0199 AHG9: On syntax grouping in SPS [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, C.-W. Kuo, X. Wang (Kwai Inc.)]

JVET-S0204 AHG9: On signalling of the maximum transform skip size [K. Unno, K. Kawamura (KDDI)]

JVET-S0219 AHG8/AHG9: On APS NAL unit clean-up [H. Jang, J. Nam, J. Lim, S. Paluri, Hendry, S. Kim (LGE)]

JVET-S0235 AHG9: On pps_cabac_init_flag signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

### PH and SH cleanups (10+4)

JVET-S0143 AHG9: A summary of proposals on picture header and slice header cleanups [Hendry (LGE)]

This topic was discussed in the HLS AHG at 2330 on 28 May and in the HLS AHG pre-meeting at 1900 on 21 June.

This contribution provides a summary of the 13 proposals on picture header and slice header cleanups (the agenda item 3.1.5 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following changes are proposed:

- Add a constraint such that the value of ph_inter_slice_allowed_flag is required to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 (JVET-S0049 aspect 8).

This proposal seemed undesirable, as it would enforce undesirable behaviour on the picture header based on layer-level characteristics.

- It is asserted there may be issue with the signalling of ph_no_output_of_prior_pics_flag. When ph_gdr_or_irap_pic_flag is equal to 0, but the associated picture the associated picture may or may not be an IRAP picture but the ph_no_output_of_prior_pics_flag is not present but there is no inference value for it. The following options are proposed to fix this asserted issue (JVET-S0055 proposal 1, JVET-S0070 proposal 1)

  - Option 1: Change the current semantics of ph_gdr_or_irap_pic_flag to be a two-way (i.e., if ph_gdr_or_irap_pic_flag is equal 1 the associated picture is a GDR or an IRAP picture, otherwise, if ph_gdr_or_irap_pic_flag is equal to 0, the associated picture is not a GDR nor an IRAP picture) (JVET-S0055 proposal 1 option 1).

  - Option 2: Apply the following (JVET-S0070 proposal 1):

    - Replace the current flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag

    - Semantics of ph_irap_pic_flag and ph_gdr_pic_flag are two-way.

    - ph_no_output_of_prior_pics_flag is present when at least ph_irap_pic_flag or ph_gdr_pic_flag is equal to 1

  - Option 3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 0. (JVET-S0055 proposal 1 option 1)

It was agreed that there is a problem. It was commented that the idea of using one-way semantics was to avoid PH rewriting for BEAM usage. It was commented that a two-flag approach had also been previously discussed.

Moving the no_output_of_prior_pics_flag to the slice header was suggested. It had previously been there.

See item 13 for later developments on this issue.

- Add a constraint such that when aud_irap_or_gdr_au_flag is present and the value of aud_irap_or_gdr_au_flag is equal to 1, the value of ph_gdr_or_irap_pic_flag shall be equal to 1. (JVET-S0055 proposal 2)

It was commented that this interacts with BEAM usage. This is related to item 2 above.

The AHG deferred this topic for further study.

- Regarding the signalling of ph_gdr_or_irap_pic_flag when pps_mixed_nalu_types_in_pic_flag is equal to 1 (JVET-S0135):

  - Option 1: add a constraint such that the value of ph_gdr_or_irap_pic_flag shall be equal to 0 when pps_mixed_nalu_types_in_pic_flag is equal to 1

  - Option 2: condition the presence of ph_gdr_or_irap_pic_flag on pps_mixed_nalu_types_in_pic_flag being equal to 0.

  - Option 3: add a constraint to the definition of GDR picture that pps_mixed_nalu_types_in_pic_flag is equal to 0.

For option 3, the proponent provided the text as a NOTE, instead of a constraint.

This is related to items 2 and 3 above.

The AHG deferred this topic for further study.

- Change coding of ph_pic_parameter_set_id from ue(v) to u(6) (JVET-S0076 item b), JVET-S0110 aspect 3)

This would add 5 bits to every picture when only 1 PPS is being used.

No strong need for action was identified, so no action was recommended for this.

- Rearrange the position of syntax elements in PH and SH.

  - Move syntax elements related to SAO and de-blocking process to earlier position in PH and in SH (i.e., signal them right after ALF and LMCS related syntax) JVET-S0074 aspect 2)

This suggestion had been discussed at the previous meeting. This would move the APS ID for the scaling list APS to later in the syntax, which is undesirable. No strong need for action was identified, so no action was recommended for this.

  - Move ph_non_ref_pic_flag to earlier position, i.e., before any syntax element whose presence is conditioned on the value of other syntax element(s). Immediately after ph_gdr_or_irap_pic_flag (JVET-S0076 item a) )

AHG Recommendation (cleanup): Adopt.

  - Move ph_pic_parameter_set_id to earlier position (i.e., after ph_non_ref_pic_flag) (JVET-S0076 item c) )

This was withdrawn due to lack of action on item 5.

  - Move down the location of syntax elements ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag and condition the presence of ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture and non-gdr picture when ph_recovery_poc_cnt is equal to 0 (JVET-S0070 proposal 2)

It was commented that this would require some SPS parsing for interpretation. There was discussion over whether this would be of interest to the system level.

It could save one or two bits per picture under some circumstances.

There seemed to be no clear need for action on this, so no action was recommended.

Discussion in the HLS AHG stopped here at 0020 UTC on 29 May.

Discussion in the HLS AHG pre-meeting resumed here at 1900 on 21 June.

- Condition the presence of ph_gdr_pic_flag such that it may be present only when sps_gdr_enabled_flag is equal to 1 (JVET-S0076 item d)

This proposal is somewhat based on the idea that ph_gdr_pic_flag is not needed/used by the system level, which was not considered necessarily true, so no action was recommended on this item.

- Change the constraint on the value of sh_picture_header_in_slice_header_flag as follows: the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when sps_subpic_info_present_flag is equal to 1 and pps_no_pic_partition_flag is equal to 0 (JVET-S0120 aspect 1)

The idea is that if there is only one slice in the picture, it should be OK to put the PH in the SH.

This would not be friendly to parallel encoding with merging.

In the discussion, it was said that the only purpose of having sps_subpic_info_present_flag equal to 1 is to support BEAM usage, and putting the PH in the SH would not be consistent with that purpose. No action was thus recommended on this.

- Change the constraint on the value of sh_picture_header_in_slice_header_flag as follows: (JVET-S0120 aspect 2 and 3)

  - Option 1: the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when the value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, pps_alf_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1.

  - Option 2: remove the following constraint: “the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when  The value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1”

This topic was closed; see notes for S0142.

- Change VTM encoder to signal the override flag (i.e., ph_partition_constraints_override_flag) as zero when the derived partition constraints for a PH are identical to the ones in the SPS (JVET-S0133 aspect 1)

This aspect is a non-normative software-only encoder modification proposal. The software was attached to the proposal.

Some coding gain was reported under CTC conditions (overall 0.14% in LB, with 0.46% in Class E).

It was commented that this could have just been fixed in response to a bug track report.

AHG Recommendation (non-normative encoder software improvement): Adopt.

- Change the semantics of the signalled values of partition constraints in PH from values for overriding the corresponding values signalled in SPS to delta values relative to the corresponding values signalled in the SPS (JVET-S0133)

The results reportedly show that the encoder-only change to remove the redundant signalling introduces -0.10% (RA) and -0.06% (LD) BD-rate reduction for class D. On top of the encoder-only change, the proposed syntax modifications introduce -0.18% (RA), -0.19% (LD) BD-rate reduction for class D. In average, the encoder-only change introduces 0.00 (AI), -0.03% (RA) and -0.14% (LDB) BD-rate changes, and when combined with the encoder-only change, the proposed syntax modification introduces 0.00 (AI), -0.03% (RA) and -0.15% (LDB) BD-rate changes under CTC.

The additional average gain from the syntax change across all classes is negligible, although there is a little gain for class D.

It is noted that the results show no impact on AI because the current VTM always disables the override of the partition constraints for intra pictures.

The proposal assumes the SPS-level information is an upper bound or lower bound. The proposal use the upper bound convention for some SEs and the lower bound convention for others, based on which would be more efficient than the CTC.

It was commented that this seems to complicate the design for basically negligible benefit, so no action was recommended.

- It is asserted that reference rule for the value of ph_collocated_from_l0_flag when L0 empty and L1 non-empty is missing. Update the current inference rule for ph_collocated_from_l0_flag as follows: (JVET-S0174 aspect 3) (added/changed in bold)

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When not present and ph_temporal_mvp_enabled_flag and pps_rpl_info_in_ph_flag are both equal to 1, the value of ph_collocated_from_l0_flag is inferred to be equal to ( num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]  = =  0 ? 1 : 0 ).

This is proposed as a bug fix.

It was commented that either of the lists can be empty. However, per the notes for S0140 item 11 in 3.1.6 that discuss S0096, List 0 could only be empty for an I slice.

Revisit with S0096.

- Further study result for the item 2 above (i.e., harmonization of JVET-S0055 proposal 1 and JVET-S0070 proposal 1). Change IRAP or GDR flag in PH with the following options (JVET-S0193 aspect 1):

  - Option 1: Change the semantics of ph_gdr_or_irap_pic_flag to be two-way.

  - Option 2: Replace the flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag with two-way semantics.

  - Option 3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 1.

  - Option 4: Move no_output_of_prior_pics_flag to the slice header of IRAP and GDR slices (but require it to have the same in all IRAP and GDR slices of the AU).

It was commented that option 1 would be harmful to merging bitstreams that have different IRAP periods. Option 2 has the same issue.

Option 3 would lose the ability to set the flag to 0 in some cases and might require some rewriting to put the flag in.

Option 4 was thus suggested. It was noted that the flag in HEVC is at the slice level.

Putting the no_output_of_prior_pics_flag in the AUD was mentioned as a possible option 5.

An option 6 was mentioned of putting the flag in all PHs.

  - AHG Recommendation (bug fix): Adopt option 4. (It was noted that no inference rule is needed for this case and the text should reflect that.)

- If option 3 or option 4 from the above item is agreed, change the syntax element ph_gdr_or_irap_pic_flag to ph_irap_pic_flag and send a GDR flag in more cases. (JVET-S0193 aspect 1)

It was commented that the detecting a random-access point would be easier without the change. However, detecting an IRAP would be easier with the change, and this is anticipated to be the more common use – e.g. for trick play. Another participant said that the current semantics is difficult to understand.

Since the technical merits seem relatively unimportant, no action was recommended on this aspect.

Discussion in the HLS pre-meeting stopped here at 2100 UTC on 21 June.

- Do one of the following (JVET-S0200):

  - Option 1: Add a constraint such that ph_inter_slice_allowed_flag shall be equal to 1 when ph_gdr_pic_flag is equal to 1.

  - Option 2: Condition the presence of ph_inter_slice_allowed_flag on ph_gdr_pic_flag being equal to 0.

- Add a constraint such that when vps_max_layers_minus1 is equal to 0, an AUD is present for the current AU, and aud_irap_or_gdr_au_flag is equal to 0, the value of ph_gdr_or_irap_pic_flag shall be equal to 0 (JVET-S0160 aspect 4).

- Add a constraint such that when an AUD is present for the current AU and aud_pic_type is equal to 0, the value of sh_slice_type shall be equal to 2 (JVET-S0160 aspect 2).

- Add a constraint such that when an AUD is present for the current AU and aud_pic_type is equal to 1, the value of sh_slice_type shall be equal to 1 or 2 (JVET-S0160 aspect 3).

- Add the following constraint for CTU size when using ILRP as collocated reference picture as follows (JVET-S0174 aspect 1) (addition in bold):

sh_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

[…]

Let colPicList be set equal to sh_collocated_from_l0_flag ? 0 : 1. It is a requirement of bitstream conformance that the picture referred to by sh_collocated_ref_idx shall be the same for all non-I slices of a coded picture and the value of RprConstraintsActiveFlag[ colPicList ][ sh_collocated_ref_idx ] shall be equal to 0 and the value of sps_log2_ctu_size_minus5 in the SPS referred to by the picture referred to by sh_collocated_ref_idx shall be equal to or greater than the value of sps_log2_ctu_size_minus5 in the SPS referred to by the current picture.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution. the same scaling window offsets and same or larger CTU size as the current picture.

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 8 of this contribution belongs to this category.

JVET-S0055 AHG9: On signaling IRAP and GDR pictures [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0070 AHG9: On flag ph_gdr_or_irap_pic_flag [L. Chen, C.-W. Hsu, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0074 AHG9: On SPS, PH, SH syntax order [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 2 of this contribution belongs to this category.

JVET-S0076 AHG9: On syntax elements in the beginning of picture header [Hendry (LGE)]

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 3 of this contribution belongs to this category.

JVET-S0120 AHG9: Constraints on sh_picture_header_in_slice_header_flag [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

JVET-S0133 AHG9: On partition constraints override in picture header [Y.-W. Chen, X. Xiu, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0135 AHG9: On IRAP and GDR picture signaling in picture header [X. Xiu, Y.-W. Chen, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspects 2 to 4 of this contribution belong to this category.

JVET-S0174 AHG9: Miscellaneous cleanups [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspects 1 and 3 of this contribution belong to this category.

JVET-S0193 AHG9: Bugfix and Cleanup on ph_no_output_of_prior_pics_flag and ph_gdr_or_irap_pic_flag [B. Choi, S. Wenger, S. Liu (Tencent), L. Chen, C.-W. Hsu, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei(MediaTek)]

JVET-S0200 AHG9: On ph_inter_slice_allowed_flag in GDR picture [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, C.-W. Kuo, X. Wang (Kwai Inc.)]

### Reference picture lists cleanups (8+2)

JVET-S0140 AHG9: A summary of proposals on reference picture lists [Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0500 UTC on Thursday 28 May and in the HLS AHG pre-meeting at 1300 UTC on 20 June.

This contribution provides a summary of the 9 proposals on reference picture lists cleanups (the agenda item 3.1.6 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following changes were proposed:

- Modify the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], such that it (S0045)

- swaps the meaning of positive and negative (consequently swaps the sign assignment of DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] in Equation 155, and change the subtraction to addition when assigning RefPicPocList[ i ][ j ] using DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] in Equation 205.

The concept is that in other place in the spec, a sign flag value ‘1’ is associated with a negative value and ‘0’ is associated with a positive value.

- associates strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] with DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] instead of with an "i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )".

Both aspects are only editorial. The “bits on the wire” do not change.

Editor action item: The editor is strongly suggested to make the proposed changes as a more precise and natural expression of the intent.

- When sps_inter_layer_ref_pics_present_flag is equal to 1, sps_idr_rpl_present_flag is skipped and inferred to be equal to 1. (S0049 aspect 2)

The motivation is to eliminate a combination that is asserted to not make sense.

This is closely related to items 4 and 5.

It was suggested to remove the sps_idr_rpl_present_flag, as a simplification of the RPL design for IDR pictures. This would require RPL to be present even for all-intra usage, costing about one or two bits per picture to signal an empty RPL.

Another participant said there is not really a problem in the current syntax.

No clear need for action was evident, so no action was recommended.

- Add a constraint: When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_idr_rpl_present_flag shall be equal to 1. (S0049 aspect 2)

The motivation is to eliminate a combination that is asserted to not make sense.

It was commented that it might be desirable to be able to detect the self-contained nature of such an IDR picture.

No clear need for action was evident, so no action was recommended.

- Remove sps_idr_rpl_present_flag (i.e., always signal RPLs for IDR slices, like for non-IDR I slices), and invoke generating unavailable reference picture for IDR pictures: (S0123 approach 1)

See also items 2 and 5. No clear need for action was evident, so no action was recommended.

- As an alternative to 4), do the following and invoke generating unavailable reference picture for IDR pictures: (S0123 approach 2)

  - Do one of the following:

- Change the SH syntax to signal RPL for each IDR slice in a non-independent layer.

- Require sps_idr_rpl_present_flag or pps_rpl_info_in_ph_flag to be equal to 1 for an IDR slice when the current layer is a non-independent layer.

- Require sh_slice_type to be equal to 2 for IDR slice when sps_idr_rpl_present_flag and pps_rpl_info_in_ph_flag are both equal to 0.

  - Change the process for deriving empty RPLs when sps_idr_rpl_present_flag is equal to 0 and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP to involve pps_rpl_info_in_ph_flag.

See also items 2 and 4. For the aspect “a”, no clear need for action was evident, so no action was recommended.

However, for the basic concept of needing to generate unavailable reference picture for IDR pictures, it was agreed there is an issue.

It was commented that this may only be relevant/necessary for a merge/extract scenario.

It was noted that some validity checks for entries of the RPL apply even if the picture is not an active entry.

AHG Recommendation (bug fix/expression of existing intent): Invoke the generation of unavailable reference picture for an IDR picture that has RPLs, and change the RPL constraints in clause 8.3.2 accordingly.

For aspect “b” it was agreed that there is a spec bug relating to when the RPL is in the PH.

AHG Recommendation (bug fix/expression of existing intent): Adopt aspect “b”. Editor may consider the exact expression.

- Add sps_ref_layer_idx_plus1 under the condition "if( sps_inter_layer_ref_pics_present_flag )" to enable skipping ilrp_idx[ listIdx ][ rplsIdx ][ i ] in the RPL syntax structures when the layer has only one direct reference layer, as follows: (S0082)

  - The value of ps_ref_layer_idx_plus1 shall be equal to

( NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] > 1 ) ? 0:
		DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ 0 ] + 1

  - Consequently, when sps_ref_layer_idx_plus1 is greater than 0, ilrp_idx[ listIdx ][ rplsIdx ][ i ] is skipped and inferred to be equal to sps_ref_layer_idx_plus1 − 1.

It is common for there to be only one direct reference layer. In this case it would be possible to skip the ILRP index signalled in the PH/SH (whichever has the RPL structure, if one of them does).

It was noted that something similar is done in HEVC.

It was commented that this seemed like something we would do if it was proposed a couple of meetings earlier. It would be basically a shortcut for a common use case.

The bit savings for this was not so clear, and software had not been tested.

No clear need for action was evident, so no action was recommended.

- Change ilrp_idx[ listIdx ][ rplsIdx ][ i ] to delta_ilrp_idx_minus1[ i ][ RplsIdx ][ i ] as well as the semantics such that the following part of Equation 205 (S0083):

layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]
refPicLayerId = vps_layer_id[ layerIdx ]

is proposed to be changed to be as follows:

ilrpIdx = GeneralLayerIdx[ nuh_layer_id ] − ( delta_ilrp_idx_minus1[ i ][ RplsIdx ][ j ] + 1 )
layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrpIdx ]
refPicLayerId = vps_layer_id[ layerIdx ]

The motivation is partly to make it more possible to be able to share SPSs.

It was commented that some sharing is already enabled since the index is an index into a list of direct reference layers.

Again it was commented that this seemed like something we would do if it was proposed a couple of meetings earlier.

There was a bug noted in the proposed equations, that they should instead be more like this:

ilrpIdx = GeneralLayerIdx[ nuh_layer_id ] − ( delta_ilrp_idx_minus1[ i ][ RplsIdx ][ j ] + 1 )
refPicLayerId = vps_layer_id[ ilrpIdx ]

No clear need for action was evident, so no action was recommended.

- Fix an asserted bug (when sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag is equal to 0, and rpl_idx[ 1 ] is less than sps_num_ref_pic_lists[ 0 ], the value of rpl_idx[ 1 ] may be out of range), with either of the following two options: (S0085 aspect 1)

  - Add a constraint: When sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag shall be equal to 1.

  - Add the following to the semantics of rpl_idx[ i ]: When pps_rpl1_idx_present_flag is equal to 0, the syntax element rpl_idx[ 0 ] shall be in the range of 0 to min(sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ]) − 1 and the syntax element rpl_idx[ 0 ] is represented by Ceil( Log2(min( sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ] ) ) ) bits, otherwise (pps_rpl1_idx_present_flag is equal to 1) the syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits.

It was commented that there is a constraint, on the value range of rpl_idx[ 1 ], that would be violated by the example for describing of the asserted but. Therefore, the asserted bug is not valid.

It was noted that a value range for a syntax element, unless explicitly said to apply only when the syntax element is present, it applies also to inferred values.

No need for an action was identified. Thus no action was recommended on this.

- Fix an asserted bug in the syntax condition "if( !pps_rpl_info_in_ph_flag  | |  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 )" in the PH syntax (when pps_rpl_info_in_ph_flag is equal to 0 the variable RplsIdx[ 1 ] in the context is used undefined), with the following changes: (S0085 aspect 2)

  - Add ", otherwise rpl_sps_flag[ i ] is inferred to be equal to 0" to the end of inference of rpl_sps_flag[ i ].

  - Add "num_ref_entries[ listIdx ][ sps_num_ref_pic_lists[ listIdx ] ] is infered to be equal to 0." to end of the semantics of num_ref_entries[ listIdx ][ rplsIdx ].

It was commented that " if( !pps_rpl_info_in_ph_flag  | |  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 )" is equivalent to "if( !pps_rpl_info_in_ph_flag  | | ( pps_rpl_info_in_ph_flag  &&  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) )". Therefore, the asserted bug is not valid.

Generally, a value of a syntax element or variable, when not present, is not inferred/derived when not needed, unless it is referred in a syntax condition or other condition.

It was commented that the text has some issue.

It was initially agreed to add the derivation of the value of RplsIdx[ 1 ] in the PH semantics when pps_rpl_info_in_ph_flag is equal to 0. In later discussion, adding an “if” condition was suggested as an alternative.

Editor action item: The editor is asked to rephrase the text to avoid the problem. The suggested method is to add an extra “if” condition with a comment to explain why it is there.

- Fix an asserted bug that the inference of RplsIdx[ 0 ] is missing, by adding the following inference rule (in addition to the existing inference of RplsIdx[ 1 ]): The value of rpl_idx[ 0 ] is inferred to be equal to 0 when sps_num_ref_pic_lists[ 0 ] is equal to 1 and rpl_sps_flag[ 0 ] is equal to 1. (S0085 aspect 3, S0096 aspect 3)

AHG Recommendation (bug fix/expression of existing intent): Adopt in spirit; infer a value only when needed. Exact expression to be determined offline by the editor.

- Additionally check num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] for conditional signalling of ph_collocated_from_l0_flag and ph_mvd_l1_zero_flag to reduce the unnecessary signalling, by changing "num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0" in the syntax condition to "num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 0  &&  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0". (S0096 aspect 1)

If list 0 is empty, we should not be able to indicate that we are getting the collocated picture from that list.

List 0 could only be empty for an I slice. Usually, for an I slice, if we have a non-empty list, we would make that list 0, but we do not have a constraint that requires that. It was remarked that we might not want to establish such a constraint.

The existing syntax condition seems to have a built-in assumption that list 0 will always be non-empty. This is not a valid assumption.

The initial

AHG discussion thought that, as a bug fix/expression of existing intent, this should be adopted.

This was further discussed in the context of discussion of S0174 on 21 June at 1940. The prior understanding seemed to be incorrect. If the RPL is sent in the PH, it is used for all slices, and therefore list 0 can only be empty if the whole picture contains only I slices.

It was suggested to require that ph_inter_slice_allowed_flag be 0 when the RPL is sent in the PH and list 0 is empty, but this could be unfriendly to merging.

See also the notes for related contribution S0174 in 3.1.5 for S0143 item 12.

Offline study and revisit was suggested to determine the appropriate course of action.

- Consider the number of active entries in the reference picture list when inferring the value of sh_collocated_ref_idx so that encoder can have more flexibility to change the number of active entries in slice headers, by changing "sh_collocated_ref_idx is inferred to be equal to ph_collocated_ref_idx" to "sh_collocated_ref_idx is inferred to be equal to min( ph_collocated_ref_idx, NumRefIdxActive[ !sh_collocated_from_l0_flag ] − 1 )". (S0096 aspect 2)

The issue is not actually a bug, because the problem case is prohibited.

The usefulness of the extra flexibility would be to avoid needing to signal the RPL in the SH by allowing it to be sent in the PH in such a case. However, it was remarked that it is not clear that this would be a common case. No clear need for action was evident, so no action was recommended.

- Change the constraint that the collocated picture "shall be the same for all slices of a picture" to "shall be the same for all non-I slices of a picture". (S0096 aspect 4).

It was remarked that this seems necessary to resolve the lack of a collocated picture definition for an I slice, and that this is the same way it was done in HEVC.

AHG Recommendation (bug fix/expression of existing intent): Adopt.

The discussion of the above items of this section in the HLS AHG meeting was completed at 0830 UTC on 28 May.

Discussion of the below items in the HLS pre-meeting began at 1300 UTC on 20 June.

- Add a constraint such that when sps_idr_rpl_present_flag is equal to 0, pps_mixed_nalus_in_pic_flag is equal to 1, and pps_rpl_info_in_ph_flag is equal to 0, the value of nal_unit_type shall not be equal to IDR_W_RADL or IDR_N_LP. (S0159 aspect 1)

There was discussion of whether this would be purely editorial (due to existing indirect constraints) and whether it is necessary to specify. A possible violation would be an IDR slice without RPL information mixed with an I slice with a trail NAL unit type in which the RPL is empty, which would not seem to cause a problem in principle (although it would be strange).

No action was recommended on this.

- Add a constraint such that when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 1. (S0159 aspect 2)

It was asked whether this constraint would really be needed. It did not seem to really be needed. It was mentioned that since some semantics are “one-way”, an encoder could set flags to indicate that something may happen that would not actually occur in the bitstream, and there seemed to be no clear need to prohibit this. Thus, no action was recommended on this.

- Add GDR picture with ph_recovery_poc_cnt equal to 0 (in addition to IRAP picture) to the constraint in clause 8.3.2 on RPLs based on the value of vps_max_tid_il_ref_pics_plus1[ ][ ]. (S0159 aspect 3)

The idea of this is to treat GDR picture with ph_recovery_poc_cnt equal to 0 consistently the same as an IRAP picture. This seemed to be just a forgotten, essentially editorial, issue.

AHG Recommendation (editorial BF/expression of existing intent): Adopt as proposed.

- Mark each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ], of a CLVSS picture as "used for long-term reference". (S0159 aspect 4)

The proponent said that this is an essentially editorial bug fix of the existing intent of the spec.

It was noted that the proposal would put the same text in both an “if” and “otherwise”, so it could just be applied unconditionally.

AHG Recommendation (editorial BF/expression of existing intent): Adopt (with suggested editorial refinement).

- When num_ref_entries[ listIdx ][ rplsIdx ] in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is equal to 0, ltrp_in_header_flag[ listIdx ][ rplsIdx ] is skipped and inferred to be equal to 0. (S0182)

In this case, the ltrp_in_header_flag[ listIdx ][ rplsIdx ] has no purpose or meaning.

The inference aspect is not proposing a change, since there is an existing inference specification that would naturally include this case.

AHG Recommendation (cleanup): Adopt.

JVET-S0045 AHG9: On the sign of DeltaPocValSt [D. Liu, R. Sjöberg, Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, J. Ström, R. Yu(Ericsson)]

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 2 of this contribution belongs to this category.

JVET-S0082 AHG8/AHG9: On signaling of inter-layer reference picture layer index [H. Jang, Hendry, S. Paluri, J. Nam, S. Kim, J. Lim (LGE)]

JVET-S0083 AHG8/AHG9: On signaling of ILRP layer index using delta value [Hendry, H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-S0085 AHG9: On semantics related to reference picture lists [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-S0096 AHG9: On RPL syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

Aspects 1 to 4 of this contribution belong to this category.

JVET-S0123 AHG9: On reference picture list constraints [V. Seregin, Y. He, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-S0159 AHG9: Reference picture list (RPL) cleanups [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

JVET-S0182 AHG9: On Reference Picture List Sgnalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

### Signalling of virtual boundaries (2+5)

JVET-S0047 AHG9: Signalling of Virtual Boundary Positions [M. Damghanian, R. Sjöberg, M. Pettersson, Z. Zhang, J. Enhorn, J. Ström, R. Yu, D. Liu]

This contribution was discussed in the HLS AHG meeting at 0837 UTC on Thursday 28 May.

At the 18th JVET meeting, it was decided (in JVET-R0266 item 6) to code virtual boundary positions using ue(v) for the sps_virtual_boundary_pos_x, sps_virtual_boundary_pos_y, ph_virtual_boundary_pos_x and ph_virtual_boundary_pos_y syntax elements instead of fixed length coding using u(13). It is observed that the value of 0 is not allowed for these four syntax elements so this contribution proposes to align the semantics of these four syntax elements with other syntax elements in the specification and use the minus1 mechanism. The following changes to the syntax and semantics are proposed:

- Add “_minus1” to each of the four syntax elements sps_virtual_boundary_pos_x, sps_virtual_boundary_pos_y, ph_virtual_boundary_pos_x and ph_virtual_boundary_pos_y.

- Update the corresponding ranges and derivations in the semantics of the four syntax elements accordingly.

AHG Recommendation (cleanup): Adopt.

JVET-S0065 AHG9: On Virtual Boundary [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

This contribution was discussed in the HLS AHG meeting at 0840 UTC on Thursday 28 May.

Modifications are proposed related to virtual boundary.

The following changes were proposed:

- Proposal 1: An assertedly missing inference rule is specified for sps_virtual_boundaries_present_flag to be inferred to be equal to 0 when not present.

AHG Recommendation (bug fix/expression of existing intent): Adopt.

- Proposal 2: An asserted simplification of text is proposed for virtual boundary derivation.

This aspect is only editorial.

Editor action item: The editor is requested to consider improving the expression. It was remarked that there may be some issues with the text that was proposed.

The following topics were discussed in the HLS pre-meeting at 1340 UTC on 20 June.

JVET-S0165 AHG9: Cleanup on signalling virtual boundaries [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution following high-level syntax cleanups are proposed, related to signalling information on virtual boundaries:

- Proposal 1: The presence of sps_virtual_boundaries_present_flag is conditioned on sps_res_change_in_clvs_allowed_flag.

- Proposal 2: The presence of virtual boundaries is further conditioned on sps_subpic_info_present_flag.

  - Proposal 2a: The presence of ph_virtual_boundaries_present_flag is further conditioned on sps_subpic_info_present_flag

  - Proposal 2b: The presence of sps_virtual_boundaries_present_flag is further conditioned on sps_subpic_info_present_flag

This is a proposal for syntax conditioning to enforce existing constraints that disable the combination of virtual boundaries with resolution changes and subpictures.

It was commented that it would be hypothetically possible to make these tools work together (e.g. in some future revision), although that is prohibited, and that we ordinarily do not use syntax conditioning in such cases. In some prior contributions (see JVET-Q0417), these features were proposed to work together. Thus, no action was recommended on this.

JVET-S0171 AHG9: Cleanup on virtual boundaries for SAO [S.-Y. Lin, Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju (MediaTek)]

This contribution proposes editorial changes for disabling SAO filtering operations across virtual boundaries by moving the specification text to the condition of edge offset mode to make it clearer.

This does not propose technical modifications.

Editor action item: The editor is requested to consider the proposed improvements.

JVET-S0184 AHG9: On Virtual Boundary Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

Modification is proposed related to a virtual boundary constraint.

It is proposed to specify a conformance constraint regarding virtual boundary signalling when subpictures are present, directly on related SPS syntax elements.

Revisit after offline study.

JVET-S0211 AHG9: Extensibility for the number of virtual boundaries [M. Damghanian, M. Pettersson, R. Sjöberg, J. Enhorn, J. Ström, R. Yu, D. Liu (Ericsson)]

This contribution proposes to signal the number of virtual boundaries using ue(v) instead of the current signalling using u(2) for the four syntax elements sps_num_ver_virtual_boundaries, sps_num_hor_virtual_boundaries, ph_num_ver_virtual_boundaries and ph_num_hor_virtual_boundaries. The motivation for the proposed change was reported to be future extensibility. The value range of the four syntax elements are proposed to be kept as 0 to 3, inclusive, for version 1 of VVC.

The current “hard coding” of the constraint seemed only to try to prohibit violation, but forecloses future uses. It was commented that we generally do not use syntax solely to enforce constraints that could be violated in a reasonable future use.

AHG Recommendation (extensibility cleanup): Adopt (while retaining the current value range).

JVET-S0221 AHG9: On virtual boundaries [Y.-J. Chang, M. Coban, V. Seregin, N. Hu, M. Karczewicz (Qualcomm)]

The current specification allows a virtual boundary to be located at the picture boundary when the picture resolution is 8xH or Wx8, but it is disallowed for other picture resolutions. This contribution proposes to disallow virtual boundaries located at picture boundary for all picture resolutions. Two approaches, signalling constraint or semantics restriction, are proposed as follows:

- Add explicit checking on the signalling of syntax elements as follows:

  - Condition sps_virtual_boundaries_enabled_flag on (sps_pic_width_max_in_luma_samples > 8 | | sps_pic_height_max_in_luma_samples > 8), and infer sps_virtual_boundaries_enabled_flag to be equal to 0 when not present.

  - Condition sps_num_ver_virtual_boundaries on (sps_pic_width_max_in_luma_samples > 8).

  - Condition sps_num_hor_virtual_boundaries on (sps_pic_height_max_in_luma_samples > 8).

  - Condition ph_num_ver_virtual_boundaries on (pps_pic_width_in_luma_samples > 8).

  - Condition ph_num_hor_virtual_boundaries on (pps_pic_height_in_luma_samples > 8).

- Or add bitstream conformance constraints as follows:

  - It is a requirement of bitstream conformance that when the value of sps_pic_width_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_ver_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of sps_pic_height_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_hor_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of pps_pic_width_in_luma_samples is equal to or smaller than 8, the value of ph_num_ver_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of pps_pic_height_in_luma_samples is equal to or smaller than 8, the value of ph_num_hor_virtual_boundaries shall be equal to 0.

In the current draft spec, there is a case where a syntax element has a specified range from 0 to −1 under certain conditions.

The proponent said that subpicture layout has a similar syntax treatment as approach #1.

AHG Recommendation (spec bug fix/expression of existing intent): Adopt approach 2 (or editorial equivalent). It also seems desirable to make sure the range constraint never specifies a non-valid range even in disallowed cases.

### Hypothetical reference decoder (HRD) (12+15)

See also JVET-S0098 aspects c and d in section 3.3.

JVET-S0141 AHG9: A Summary of Proposals Related to HRD [S. Deshpande (Sharp)]

This contribution was discussed in the HLS AHG meeting at 0850 UTC on Thursday 28 May and in the HLS AHG pre-meeting at 1530 on 20 June.

This contribution intends to provide a summary of proposals on core aspects of HRD (including HRD operation, related SEI message signalling and sub-bitstream extraction).

Following proposals listed in JVET-S0137-v2 section 3.1.8 are covered in this summary: JVET-S0064, JVET-S0075 (aspect 2), JVET-S0097, JVET-S0080, JVET-S0086, JVET-S0099, JVET-S0101, JVET-S0102, JVET-S0117, JVET-S0118. Additionally JVET-S0049 Item 1 is covered.

Additionally the 14 new proposals listed below in this Section starting from JVET-S0154 are also covered in this summary.

It is suggested that this summary be used for the reviewing of these proposals, such that the discussions may be done in a more structured and efficient manner. The following changes were proposed:

Related to HRD signalling and operation:

- Conditionally signal bp_sublayer_dpb_output_offsets_present_flag (and bp_dpb_output_tid_offset[ i ]) only when bp_max_sublayers_minus1 is greater than 0 and infer it as 0 otherwise?

  - Alternatively, add following constraint: when bp_max_sublayers_minus1 is equal to 0, bp_sublayer_dpb_output_offsets_present_flag shall be equal to 0. (JVET-S0064)

AHG Recommendation (bug fix/expression of existing intent): Adopt the conditional signalling.

- When sps_video_paramater_set_id is equal to 0:

AHG Recommendation (bug fix/expression of existing intent): Adopt inferences a, b, c, e, f, and g.i.

  - Infer TotalNumOlss to be equal to 1 (JVET-S0075, JVET-S0097)

  - Infer NumLayersInOls[ 0 ] to be equal to 1 (JVET-S0075, JVET-S0097)

  - Infer vps_layer_id[ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Require that parameter sets referred to shall have same nuh_layer_id value (JVET-S0075)

It was commented that this aspect is not necessary since the extraction process indirectly requires this.

Also, sharing of PSs is only allowed only if all layers are present in the OLS, which would be violated unless the requirement is true.

Editor action item: The editor is asked to consider adding some explanation to point out that this is the case. Item e below may help to make this clear.

  - Infer LayerIdInOls[ 0 ][ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Infer NumSubLayersInLayerInOLS[ 0 ][ 0 ] to be equal to sps_max_sublayers_minus1 + 1. (JVET-S0097)

  - Modify the inference rule for vps_max_sub_layers_minus1 and range of values for sps_max_sublayers_minus1 by adding quoted text and not inferring vps_max_layers_minus1 to be equal to 6? (JVET-S0097)

    - “When sps_video_parameter_set_id is greater than 0,” the value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. “When sps_video_parameter_set_id is equal to 0, the value of sps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive, and vps_max_sublayers_minus1 is inferred to be equal to sps_max_sublayers_minus1.”

or

    - Modify only the inference for vps_max_sub_layers_minus1 (JVET-S0049 Item 1, JVET-S0097) as: When sps_video_parameter_set_id is equal to 0, the value of vps_max_sublayers_minus1 is inferred to be equal to “sps_max_sublayers_minus1” (instead of 6).

It was commented that option ii has a circularity issue in the semantics of sps_max_sublayers_minus1, so option i is preferred.

- Modify the semantics of bp_max_sublayers_minus1 to change the range of values from 0 to vps_max_sublayers_minus1 to instead require it to be equal to maxSubLayers (given as input to the ols_hrd_parameters() syntax structure)? (JVET-S0097)

It was commented that this has a relationship to S0100.

See the notes for S0100 in section 3.3.

- Modify the additional bumping process for DPB by doing following? (JVET-S0080)

  - Move the process for setting the value of PicLatencyCount from within additional bumping process to within the output and removal of pictures from the DPB (i.e., sub clause C.5.2.2) after the invocation of bumping process.

It was commented that no action is needed on this due to the recommended action for PictureOutputFlag for S0077.

  - Remove the additional bumping process or removal of the output of pictures in the additional bumping process.

Defer this for further study.

Discussion in the HLS AHG meeting stopped here at ~0220 on 28 May.

Discussion continued here in the HLS AHG pre-meeting at 1530 on 20 June.

- Fix an asserted bug in Equation C.10 (also applicable to HEVC)? (JVET-S0101)

AHG Recommendation (bug fix/expression of existing intent): Adopt (also applicable to HEVC).

- Add a constraint on the sum of InitialCpbRemovalDelay and InitialCpbRemovalOffset as follows? (JVET-S0101)

When bp_concatenation_flag is equal to 0, the sum of InitCpbRemovalDelay[ Htid ][ ScIdx ] and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] shall be the same as in the previous buffering period.

In the discussion, it was commented that we need to consider the possibility of splicing without the use of bp_concatenation_flag equal to 1. It was commented that this constraint could be a very serious imposition for such a splicing operation, and would be unlikely to be fulfilled in practice.

No action was recommended on this.

- Include PH and AUD NAL units into the bitstream subject to HRD conformance checking using either of the following alternative options? (JVET-S0118)

  - Include PH NAL unit and AUD in Type I bitstream

  - Include PH NAL unit and AUD in Type II bitstream

Currently, Type I includes VCL (i.e., slices) and filler data NAL units only. Everything goes in Type II (possibly needing editorial clarification, but this is clearly what is specified).

Since PH is sometimes in the SH, it will be in Type I when it is not in a separate NAL unit.

When rewriting / BEAMing, we do not ordinarily expect PH content to be rewritten.

AHG Recommendation (consistency cleanup): Include PH NAL unit in Type I (but keep AUD in Type II as it was).

Related to general and subpicture sub-bitstream extraction processes:

Review of this group of contributions was deferred TBP, and the next topic discussed was item 12.

- Perform the asserted bug-fixes for the general sub-bitstream extraction process for determining which AUD NAL units of the input bitstream are kept in the output sub-bitstream as follows? (JVET-S0086)

  - Add AUD_NUT among the NAL unit types that are kept in the output sub-bitstream regardless of its nuh_layer_id value.

  - Add removal of an AUD NAL unit for which all VCL NAL units of an AU are removed by the sub-bitstream extraction.

- Make the following changes related to the general sub-bitstream extraction process? (JVET-S0102)

  - Add a constraint that the BP/PT/DUI SEI messages for any two OLSs with the same layer set are the same.

  - Modification regarding AUs that become IRAP or GDR AU after extraction by doing one of the two options:

    - Option 1: Add AUD writing to the extraction process for AUs that become IRAP AUs or GDR AUs.

    - Option 2: Mandate AUDs to be placed in all AUs that contain IRAP or GDR pictures and add rewriting of aud_irap_or_gdr_au_flag to extraction process.

Item 46) is related.

  - Place PT SEI messages in individual SEI NAL units when general_same_pic_timing_in_all_ols_flag is equal to 1.

  - Require that the scalable-nested and non-scalable-nested BP SEI messages in a CVS have the same values for the five syntax elements which specify lengths of various u(v) syntax elements.

- Modify the subpicture sub-bitstream extraction process regarding handling of scalable-nested SEI messages by performing following? (JVET-S0099)

  - Removal of scalable-nested SEI messages for non-target subpictures

Item 66) is related.

  - Removal before replacing non-scalable-nested SEI messages

Item 72) is related.

- Modify the subpicture sub-bitstream extraction process by performing following? (JVET-S0117)

  - Remove rewriting of PPS conformance window syntax elements since pps_conformance_window_flag is asserted to be equal to 0.

  - Remove subpicture ID mapping in SPS and PPS as follows:

    - Remove the syntax elements sps_subpic_id[ j ] in all the referenced SPS NAL units and for each j that is not equal to subpicIdx.

    - Remove the syntax elements pps_subpic_id[ j ] in all the referenced PPS NAL units and for each j that is not equal to subpicIdx.

  - Rewrite virtual boundary syntax elements in SPS.

New submissions (for the June 10th submission deadline)

Discussion in the AHG pre-meeting began here at 1635 on 20 June.

(Convention for text changes proposed: “quoted text” is proposed to be removed and text inside delimiters < > is proposed to be added by proposals)

Related to HRD signalling and operation:

- Change HRD operation description from being picture-specific to be AU-specific as follows? (JVET-S0157 item 1)

  - In clauses C.2.3, D.3.2, and D.4.2, rename prevNonDiscardablePic to prevNonDiscardableAu, firstPicInPrevBuffPeriod to firstAuInPrevBuffPeriod, notDiscardablePic to notDiscardableAu, and change their descriptions from being picture-specific to AU-specific.

  - In clause D.3.2, change the description of the semantics of bp_concatenation_flag and bp_cpb_removal_delay_delta_minus1 from being picture-specific to be AU-specific.

  - In clause D.3.2, change the description of the condition for the constraint requiring bp_alt_cpb_params_present_flag to be equal to 0 from being picture-specific to be AU-specific.

  - In clause D.4.2, change the descriptions of BpResetFlag, CpbRemovalDelayMsb[ i ], CpbRemovalDelayVal[ i ] and the constraint requiring pt_cpb_alt_timing_info_present_flag to be equal to 0 from being picture-specific to be AU-specific.

  - In clause D.4.2, change the description of the semantics of pt_dpb_output_delay, pt_dpb_output_du_delay, and pt_display_elemental_periods_minus1 from being picture-specific to be AU-specific.

  - In clause D.5.2 (DU information SEI message semantics), change the description of the semantics of dui_dpb_output_du_delay from being picture-specific to be AU-specific.

It was commented that the design intent should already be clear that the HRD is intended to operate on an AU basis. These changes are intended just as editorial expression of this existing design intent.

AHG Recommendation (editorial BF/expression of existing intent): Adopt.

- In clause D.3.2, modify the constraint related to bp_alt_cpb_params_present_flag as follows: (JVET-S0157 item 2):

When <the associated AU is not an IRAP or GDR AU> “the associated picture is neither a CRA picture nor an IDR picture”, the value of bp_alt_cpb_params_present_flag shall be equal to 0.

The idea of this is to treat GDR picture with ph_recovery_poc_cnt equal to 0 consistently the same as an IRAP picture. This seemed to be just a forgotten, essentially editorial, issue.

AHG Recommendation (editorial BF/expression of existing intent): Adopt as proposed.

- In clause D.4.2, in the constraint related to requiring pt_cpb_alt_timing_info_present_flag to be equal to 0, add that the RASL pictures in the constraint are RASL pictures that do not have mixed NAL unit types as follows? (JVET-S0157 item 4).

When <all pictures in the associated AU are RASL pictures with pps_mixed_nalu_types_in_pic_flag equal to 0> “the associated picture is a RASL picture”, the value of pt_cpb_alt_timing_info_present_flag shall be equal to 0.

AHG Recommendation (editorial BF/expression of existing intent): Adopt as proposed.

- In clause D.5.2, in the semantics of pt_display_elemental_periods_minus1, change fixed_pic_rate_within_cvs_flag[ TemporalId ] (3 instances) to fixed_pic_rate_within_cvs_flag[ Htid ]? (JVET-S0153 item 5)

AHG Recommendation (editorial BF/expression of existing intent): Adopt as proposed.

- Modify constraints as follows: (JVET-S0175 aspect 3)?

When “fixed_pic_rate_within_cvs_flag[ TemporalId ] is equal to 0 or sps_field_seq_flag is equal to 1, the value of” <any of the following applies> pt_display_elemental_periods_minus1 shall be equal to 0.<:

–	sps_field_seq_flag is equal to 1 for the output layers in the OLS.

–	<sps_field_seq_flag is equal to 0 for all output layers of the bitstream and there is not frame-field information SEI message present or display_fields_from_frame_flag is equal to 0 if a frame-field SEI message is associated with the AU and fixed_pic_rate_within_cvs_flag[ TemporalId ] is equal to 0.>

It was noted that the frame-field SEI message is in the SEI message specification, and it contains the display_fields_from_frame_flag. It was said that the expressed constraint on pt_display_elemental_periods_minus1 is wrong in both specs.

AHG Recommendation (editorial bug fix/expression of existing intent): Replace the constraint above with “When sps_field_seq_flag is equal to 1, the value of pt_display_elemental_periods_minus1 shall be equal to 0” and to say this (or equivalent) in both specs.

- Change the condition for fixed picture rate across CVSs, by removing the max_dec_pic_buffering_minus1 (asserted not to be needed) and adding BP SEI message syntax related to concatenation_flag. (JVET-S0175 aspect 1, 2)

AHG Recommendation (bug fix/expression of existing intent): Adopt first aspect. (This also affects HEVC in one place.)

The second aspect would impose an extra burden that is not currently imposed when the general flag is set equal to 0. It was also commented that it may not consider picture decoding time appropriately.

No action was recommended on the second aspect.

Discussion in the HLS AHG pre-meeting stopped here with the remaining items TBP.

- Make either of the following changes (JVET-S0175 aspects 4 and 5)?

  - Add a gating flag for pt_display_elemental_periods_minus1, so that its presence can be controlled and change the derivation of elementalOutputPeriod as follows and change D.9.6 accordingly?

–	If a PT SEI message is present for picture n <and pt_display_elemental_periods_present_flag equal to 1>, elementalOutputPeriods is equal to the value of pt_display_elemental_periods_minus1 + 1.

–<	Otherwise, if external means are provided to set the value of elementalOutputPeriods, elementalOutputPeriods is set equal to the value provided via external means.

–	Otherwise , if frame-field information SEI messages are present for at least one of the output pictures in the AU of picture n, the value of elementalOutputPeriods is set to the lowest value of display_elemental_periods_minus1 + 1 among all present frame-field information SEI messages.>

–	Otherwise, elementalOutputPeriods is equal to 1.

  - An alternative option to a), with the key difference being that herein the main VVC specification does not depend on the frame-field information SEI message that is specified in the SEI specification.

- Enable decoder operation without HRD timing SEI messages when fixed picture rate is signalled? (JVET-S0175 aspect 6). This is related to S0109 in section 3.1.2.

- Conditionally signal bp_sublayer_initial_cpb_removal_delay_present_flag, only when bp_max_sublayers_minus1 is greater than 0 and infer it to 0 otherwise? (JVET-S0181 proposal 1, JVET S0157 item 3)

- Move the signalling of bp_max_sublayers_minus1 to be before the signalling of bp_cpb_removal_delay_deltas_present_flag and conditionally signal bp_cpb_removal_delay_deltas_present_flag and bp_num_cpb_removal_delay_deltas_minus1 and bp_cpb_removal_delay_delta_val[ i ] syntax elements, only when bp_max_sublayers_minus1 is greater than 0 and infer bp_cpb_removal_delay_deltas_present_flag equal to 0 otherwise? (JVET-S0181 proposal 2)?

- Move the syntax element pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ]  to locate it near the other CPB removal delay syntax elements, in particular other pt_cpb_removal_delay_minus1[ i ] syntax elements (and pt_cpb_removal_delay_delta_idx[ i ] syntax elements) for i in the range of TemporalId to bp_max_sub_layers_minus1 − 1, inclusive? (JVET-S0185 proposal 1)

- Move the signalling of syntax element bp_alt_cpb_params_present_flag to locate it near the syntax element bp_use_alt_cpb_params_flag where it is used? (JVET-S0185 proposal 2)

- Modify the output and removal of picture from DPB as follows? (JVET-S0198)

C.5.2.2 Output and removal of pictures from the DPB

…

–	Otherwise (the current AU is not a CVSS AU or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture of the current AU), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. “When one or more of the following conditions are true, t”<T>he "bumping" process specified in clause C.5.2.4 is invoked repeatedly “while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied,” until <the number of pictures in the DPB is less than max_dec_pic_buffering_minus1[ Htid ] + 1>. “none of the following conditions are true:

- The number of pictures in the DPB that are marked as "needed for output" is greater than max_num_reorder_pics[ Htid ].

- max_latency_increase_plus1[ Htid ] is not equal to 0 and there is at least one picture in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyPictures[ Htid ].

- The number of pictures in the DPB is greater than or equal to max_dec_pic_buffering_minus1[ Htid ] + 1.”

…

- In clause 8.1.1, add the derivation of the variables DuHrdPreferredFlag and DecodingUnitHrdFlag, simialrly as in HEVC. (S0248 aspect 2 late)

- In clauses C.2.2 and C.2.3, change the local variable decodingUnitParamsFlag to be a global variable DecodingUnitParamsFlag, to address the issue that decodingUnitParamsFlag was used in clause C.2.2 without being defied/initialized. (S0248 aspect 3 late)

- In clause C.3.2, change the condition on when to remove decoded pictures neither needed for referecne nor needed for output such that it is applied to all pictures except for the first picture of AU 0. (S0248 aspect 4 late)

- In clause C.5.2.2, change "Otherwise (the current AU is not a CVSS AU or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture of the current AU)" to either of the following, as the condition in the parentheses is assertedly incorrect.

  - "Otherwise (the current AU is not a CVSS AU, <the current AU is CVSS AU 0,> or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture of the current AU)" (S0223)

  - "Otherwise" (S0248 aspect 5 late)

- In clause C.5.2.3, remove "AU n containing " from the first sentence, i.e., "The processes specified in this clause happen instantaneously when the last DU of AU n containing the current picture is removed from the CPB." (S0223, S0248 aspect 6 late)

- In clause D.3.2, add the following constraint to disallow the combination of alternative timing for the CRA/DRAP case and the decoding unit HRD mode: When bp_alt_cpb_params_present_flag is equal to 1, the value of bp_du_hrd_params_present_flag shall be equal to 0. (S0248 aspect 7 late)

Related to sub-bitstream extraction

General SEI semantics and constraints:

- Fix an asserted missing integration of an agreement (from JVET-Q0394) that specifies that non-scalable-nested HRD-related SEI messages apply to the OLSs that include the same set of layers as the entire bitstream (instead of only to the 0th OLS)? (JVET-S0178 aspect 1)

  - Consequently, add a constraint such that when there is no OLS that includes the set of layers same as the entire bitstream, there shall be no non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI)? (JVET-S0178 aspect 1.a)

  - And remove the constraint requiring the value of nuh_layer_id for an SEI NAL unit containing non-scalable-nested HRD-related SEI messages to be equal to vps_layer_id[ 0 ], such that the value of nuh_layer_id for such SEI NAL units is unconstrained, same as for the nuh_layer_id for DCI, VPS, AUD, and EOS NAL units? (JVET-S0178 aspect 1.b)

- Remove the payloadType value 203 (SLI) from the VclAssociatedSeiList? (JVET-S0178 aspect 3)

- Add the clarification (that was recently added to the HEVC specification as an corrigendum item) on the value ranges of tone_map_id, frame_packing_arrangement_id, knee_function_id, and colour_remap_id, including potential collisions of the interpretation for values of these syntax elements? (JVET-S0178 aspect 4)

- Add a constraint such that when general_same_pic_timing_in_all_ols_flag is equal to 1, there shall be no SEI NAL units that contain a scalable-nested SEI message with payloadType equal to 1 (PT) ? (JVET-S0178 aspect 5)

- Modify an existing constraint, to add the SLI SEI message, such that when an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, or 203? (JVET-S0178 aspect 6)

- Modify an existing constraint, to add the SLI SEI message, that when an SEI NAL unit contains a scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, 203, or 133 (scalable nesting) ? (JVET-S0178 aspect 7)

- Add a constraint such that when an SEI NAL unit contains an SEI message with payloadType equal to 3 (filler payload), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 3? (JVET-S0178 aspect 8)

- Add a constraint such that when an SLI SEI message and a BP SEI message that apply to a particular OLS are present within an AU, the SLI SEI messages shall precede the BP SEI message in decoding order? (JVET-S0178 aspect 9)

Scalable nesting SEI message:

- Modify semantics of sn_num_subpics_minus1 (based on following options)?

  - Clarify the association of scalable-nested SEI messages for subpictures to either layers or OLS as follows? (JVET-S0173 item 2)

sn_num_subpics_minus1 plus 1 specifies the number of subpictures to which the scalable-nested SEI messages apply. <When sn_ols_flag is equal to 0,> the value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS referred to by the pictures <of the indicated layer. Otherwise, when sn_ols_flag is equal to 1, the value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS referred to by the pictures of the highest layer in the OLS with sps_num_subpics_minus1 greater than 0.>

  - Specify the semantics of sn_num_subpics_minus1 and sn_subpic_idx[ i ] in a way such that the syntax elements are about the subpictures of the layers with multiple subpictures per picture? (JVET-S0177 aspect 7)

<Among the layers in the OLSs (when sn_ols_flag is equal to 1) to which the scalable-nested SEI messages apply, or among the layers (when sn_ols_flag is equal to 0) to which the scalable-nested SEI messages apply, those for which the referenced SPSs have sps_num_subpics_minus1 greater than 0 are referred to as the multiSubpicLayers.>

sn_num_subpics_minus1 plus 1 specifies the number of subpictures <in each picture in the multiSubpicLayers> “to which the scalable-nested SEI messages apply”. The value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS<s> referred to by the pictures in the <multiSubpicLayers> “CLVS”.

- Change sn_subpic_id[ i ] to sn_subpic_idx[ i ] and remove sn_subpic_id_len_minus1 as follows? (JVET-S0177 aspect 1)

sn_subpic_id<x>[ i ] “indicates” <specifies the subpicture index of> the i-th subpicture “ID associated with the scalable-nested SEI messages” <in each picture in the multiSubpicLayers. The value of sn_subpic_idx[ i ] shall be less than or equal to the value of sps_num_subpics_minus1 in the SPSs referred to by the pictures in the multiSubpicLayers.> “The length of the sn_subpic_id[ i ] syntax element is sn_subpic_id_len_minus1 + 1 bits.” <The scalable-nested SEI messages also apply to the single subpicure in each picture in the layers that are not in the multiSubpicLayers but are among the layers in the OLSs (when sn_ols_flag is equal to 1) to which the scalable-nested SEI messages apply or among the layers (when sn_ols_flag is equal to 0) to which the scalable-nested SEI messages apply.>

“sn_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sn_subpic_id[ i ]. The value of sn_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of sn_subpic_id_len_minus1 shall be the same for all scalable nesting SEI messages that are present in a CLVS.”

Or change semantics of sn_subpic_id_len_minus1 by changing CLVS to CVS (JVET-S0173 item 2):

sn_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sn_subpic_id[ i ]. The value of sn_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of sn_subpic_id_len_minus1 shall be the same for all scalable nesting SEI messages that are present in a C“L”VS.

- Disallow containing of filler payload SEI messages in a scalable nesting SEI message? (JVET-S0177 aspect 2)

- Constrain that, when a scalable nesting SEI message contains one or more subpicture level information (SLI) SEI messages, the value of sn_ols_flag shall be equal to 1, and the value of sn_subpic_flag shall be equal to 0? (JVET-S0177 aspect 3)

- Modify an existing constraint, to include the SLI message, such that when a scalable nesting SEI message contains a BP, PT, DUI, or SLI SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI)? (JVET-S0177 aspect 4)

- Modify an existing constraint, to include recently added SEI messages, such that when a scalable nesting SEI message contains an SEI message that has payloadType not equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT? (JVET-S0177 aspect 5)

- Add a constraint such that when a scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to SUFFIX_SEI_NUT? (JVET-S0177 aspect 6)

General sub-bitstream extraction:

- Handling of AUD in sub-bitstream extraction process (JVET-S0102 item 5, JVET-S0225)

  - Restrict aud_irap_or_gdr_au_flag to a one-way constraint in single-layer bitstreams as follows? (JVET-S0102 item 5).

aud_irap_or_gdr_au_flag equal to 1 specifies that the AU containing the AU delimiter is an IRAP or GDR AU. aud_irap_or_gdr_au_flag equal to 0 specifies that the AU containing the AU delimiter is not an IRAP or GDR AU <when NumLayersInOls[ TargetOlsIdx ] is greater than 1>.

  - And/or modify sub-bitstream extraction process using either one of the following two options: (JVET-S0225)

    - Option 1: Add AUD or rewrite aud_irap_or_gdr_au_flag of the existing AUD when the associated extracted AU is an IRAP AU or GDR AU as follows:

<When an AU is an IRAP AU or GDR AU and no associated AUD, add an AUD NAL unit of type AUD_NUT before the first VCL NAL unit of the AU with aud_irap_or_gdr_au_flag equal to 1. Otherwise for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for  an IRAP AU or GDR AU.>

    - Option 2: Add AUD or rewrite aud_irap_or_gdr_au_flag of the existing AUD when the associated AU is an IRAP AU or GDR AU and the target OLS has multi-layers; and either remove the existing AUD or rewrite aud_irap_or_gdr_au_flag of the existing AUD when the associated AU is an IRAP AU or GDR AU and the target OLS contains only single layer as follows:

–	<When the targte OLS contains more than one layer,

- For each AU that is an IRAP AU or GDR AU with no associated AUD, add a NAL unit of type AUD_NUT before the first VCL NAL unit of the AU with aud_irap_or_gdr_au_flag equal to 1.

- Otherwise for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for an IRAP AU or GDR AU.

–	Otherwise (the target OLS contains only one layer), remove the associated AUD from outBitstream for each AU, or for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for an IRAP AU or GDR AU.>

Item 9).b) is related.

- In the general sub-bitstream extraction process, specify the conditions under which an output sub-bitstream is required to be a conforming bitstream such that the value of tIdTarget is specified to be in the range of 0 to vps_ptl_max_tid[ vps_ols_ptl_idx[ targetOlsIdx ] ], inclusive (instead of 0 to 6 inclusive)? (JVET-S0158 aspect 1)

- Specify the general sub-bitstream extraction process such that it would remove, from the output bitstream outBitstream, SEI NAL units that contain a scalable nesting SEI message with sn_ols_flag equal to 0 while the applicable layers as indicted in the scalable nesting SEI message does not include any layer in the target OLS? (JVET-S0158 aspect 2)

- Make the following changes to the 4th step in the general sub-bitstream extraction process? (JVET-S0158 aspect 3)

  - Remove the redundant condition "nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive". (This is asserted to be editorial.)?

  - When removing from the output bitstream the VCL NAL units, instead of removing the associated SEI NAL units containing SEI messages other than the BP, PT, or DUI SEI messages, remove all SEI NAL units that contain SEI messages that become not applicable to any picture or subpicture remaining in the final output bitstream? (Since this is not easy to do, another option is not to remove SEI NAL units in this step.)

- Make the following changes to the last step in the general sub-bitstream extraction process? (JVET-S0158 aspect 4)

  - Change the condition as follows?

When the list LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all <VCL> NAL units in the bitstream <inBitstream>.

  - When at least one layer is removed by the extraction process, also remove all SEI NAL units that contain a non-scalable-nested SLI SEI message (same as for BP, PT, and DUI SEI messsages)?

  - Insert SEI NAL units to directly contain those SEI messages that were scalable-nested HRD-related SEI messages that apply to the output bitstream, and remove their original container SEI NAL units from the output bitstream. When the target OLS includes only one layer, apply the same for scalable-nested non-HRD-related SEI messages?

Subpicture level information (SLI) SEI message:

- Clarify the scope of Subpicture Level Information (SLI) SEI to be CVS instead of CLVS? (JVET-S0173 item 1)

- Include all layers of the OLS in MinCr related constraint for subpictures by making the following changes? (JVET-S0173 item 3):

- The sum of the NumBytesInNalUnit variables for AU 0 corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * ( Max(“SubpicSizeY[ j ]”<AuSizeMaxInSamplesY[ 0 ]>, fR * MaxLumaSr <) >* OlsRefLevelFraction[ i ][ j ] ÷ 256 “)” + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * OlsRefLevelFraction[ i ][ j ] ) ÷ ( 256 * MinCr ), where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0, at level sli_ref_level_idc[ i ], and MinCr and AuSizeMaxInSamplesY[ 0 ] are derived as indicated in A.4.2.

- Modify the inference rule for sli_ref_level_fraction_minus1[ i ][ j ]? (JVET-S0173 item 5):

When not present, the value of sli_ref_level_fraction_minus1[ i ][ j ] is inferred to be equal to <Max( 256,> Ceil( 256 * SubpicSizeY[ j ] ÷ PicSizeInSamplesY * MaxLumaPs( general_level_idc ) ÷ MaxLumaPs( sli_ref_level_idc[ i ] ) <)>− 1.

- Resolve editorial notes related to subpictures as follows? (JVET-S0173 item 6):

  - Refer to minCR and fR derivation in A.4.2.

  - Replace PicSizeInSamplesY with PicSizeMaxInSamplesY and clarify that the picture size of the highest layer with subpictures in the OLS is taken.

- Replace sps_num_subpics_minus1 with sli_num_subpics_minus1 in semantics of sli_num_ref_levels_minus1? (JVET-S0173 item 7)

- Modify alignment constraint of intendent subpictures by adding sps_subpic_treated_as_pic_flag[ i ] syntax element to the clause as follows? (JVET-S0173 item 4):

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and <sps_log2_ctu_size_minus5 and> shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

- Add sli_max_sublayers_minus1, sli_sublayer_info_present_flag, and a loop for sublayers for the signalled fractions and reference level indicators, to support sublayers? (JVET-S0176 item 1)

- Allow SLI SEI messages to be available either in the bitstream or provided through an external means not specified in this Specification? (JVET-S0176 item 2)

- Change the persistency scope for SLI SEI from one CVS to one or more CVSs, to be consistent with VPSs and SPSs wherein level information are or may be signalled? (JVET-S0176 item 3)

- Change the definition of subpicture sequence to cover the case when there is one or more layers with single subpicture per picture? (JVET-S0176 item 4)

- Require that, when an SLI SEI message is present for a CVS, the value of sps_num_subpics_minus1 shall be the same for all the SPSs referenced by the pictures in the layers with multiple subpictures per picture. (JVET-S0176 item 5)?

- Specify the semantics of sli_num_subpics_minus1 in a way such that the syntax element is about the subpictures of the layers with multiple subpictures per picture? (JVET-S0176 item 6)

- Make variable derivations subpicture sequence specific as follows: Add an array index, which identifies a subpicture sequence, to both the variables SubpicLevelIdc and SubpicLevelIdx, as well as to the arrays SubpicCpbSizeVcl, SubpicCpbSizeNal, SubpicBitRateVcl, and SubpicBitRateNal in the last set of constraints in the semantics of the SLI SEI message? (JVET-S0176 item 7)

Subpicture sub-bitstream extraction:

- Modify the specification of subpicture sequences, which when extracted, are required to be a conforming bitstream? (JVET-S0154 aspect 1)?

- Remove VCL NAL units and their associated filler data NAL units and associated filler payload SEI messages etc. regardless of whether there is an external means for replacing of the parameter sets? (JVET-S0154 aspect 2)

- Remove SEI NAL units containing scalable-nested SEI messages that do not apply to the output bitstream from the output bitstream? (JVET-S0154 aspect 3)

- Specify the subpicture index for identifying the subpicture sequence as the subpicture index of the to-be-extracted subpictures in the layers with multiple subpictures per picture, not the layers with only one subpicture per picture? (JVET-S0154 aspect 4)

- Rewrite both general_level_idc and sublayer_level_idc[ k ] for k in the range of 0 to tIdTarget − 1, inclusive, in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOLsIdx ] is equal to 0? (JVET-S0154 aspect 5)

- Rewrite cpb_size_value_minus1[ k ][ j ] and bit_rate_value_minus1[ k ][ j ] for all values of k in the range of 0 to tIdTarget, inclusive, in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOLsIdx ] is equal to 0? (JVET-S0154 aspect 6)

- Specify the calculation and rewriting of the scaling window offset parameters as part of the subpicture sub-bitstream extraction process?

  - As per JVET-S0154 aspect 7?

  - As per JVET-S0189?

- Rewrite cbr_flag[ tIdTarget ][ j ] in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOLsIdx ] is equal to 0? (JVET-S0154 aspect 8)

- Insert SEI NAL units to directly contain those SEI messages that were scalable-nested HRD-related SEI messages that apply to the output bitstream, and remove their original container SEI NAL units from the output bitstream. When the target OLS includes only one layer, apply the same for scalable-nested non-HRD-related SEI messages? (JVET-S0154 aspect 9)

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 1 of this contribution belongs to this category.

JVET-S0064 AHG9: On Buffering Period Message Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0075 AHG9: On constraints and inference values when VPS is not present [Hendry, S. Paluri, S. Kim (LGE)]

Aspect 2 of this contribution belongs to this category.

JVET-S0097 AHG8/AHG9: Clarifications to HRD specification for single-layer and multi-layers bitstreams [V. Drugeon, T. Nishi, K. Abe, T. Toma (Panasonic)]

JVET-S0080 AHG9: On additional bumping process in the DPB [S. Paluri, Hendry, S. Kim (LGE)]

JVET-S0086 AHG9: On AUD NAL units in the sub-bitstream extraction process [M. M. Hannuksela (Nokia)

JVET-S0099 AHG12: SEI message handling in subpicture extraction [R. Skupin, Y. Sanchez, K. Suehring (HHI), V. Drugeon (Panasonic)]

JVET-S0101 AHG9: On HRD [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-S0102 AHG9: On OLS extraction [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-S0117 AHG12: On Subpicture sub-bitstream exaction [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0118 AHG9: On HRD bitstream [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0154 AHG9/AHG8/AHG12: On the subpicture sub-bitstream extraction process [Y.-K. Wang, Z. Deng, K. Zhang, L. Zhang (Bytedance)]

JVET-S0157 AHG9: HRD and related cleanups [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0158 AHG9/AHG8: On the general sub-bitstream extraction process [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0173 AHG9/AHG12: Subpicture related cleanups [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-S0175 AHG9: Cleanup of picture rate info and HRD operation without timing SEI messages [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-S0176 AHG9: On the subpicture level information SEI message [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0177 AHG9: On the scalable nesting SEI message [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0178 AHG9: General SEI semantics and constraints [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0181 AHG9: On Buffering Period [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0185 AHG9: On HRD Cleanups [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0189 AHG9/AHG8/AHG12: On subpicture bitstream extraction process [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0198 AHG9: On removal of picture from DPB process [S. Paluri, Hendry, S. Kim (LGE)]

JVET-S0223 AHG9: On picture bumping process [V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0225 AHG 9: On AUD for sub-bitstream extraction [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

S0102-v2 item 5 is related.

JVET-S0248 AHG9: HRD text cleanups [Y.-K. Wang (Bytedance), Y. Sanchez, R. Skupin (HHI), S. Deshpande (Sharp), V. Seregin (Qualcomm), Hendry (LGE), J. Chen (Alibaba)] [late]

Aspects 2 to 7 this contribution belong to this category.

### DCI, VUI, and SEI (3+3)

JVET-S0172 AHG9: On generalized cubemap projection SEI message [Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju (MediaTek)]

This contribution was discussed in the HLS AHG pre-meeting at 1900 on 20 June. It proposes the following modifications and constraints related to the generalized cubemap projection SEI message:

- Add a condition check of gcmp_guard_band_flag when deriving gcmpGuardBandSamples to prevent from accessing an uninitialized variable.

- Add constraints on gcmp_face_index[ i ] and gcmp_face_rotation[ i ] for the hemisphere cubemap projection to guarantee that the four half faces are the ones connecting to the full face and can represent 180°×180° omnidirectional image/video. (The “if” part of the “otherwise” should be removed.)

- Include an offset the remapping of sample locations on the half faces of the hemisphere cubemap projection.

AHG Recommendation (bug fix and sensibility constraints): Adopt.

It was asked whether each half-height / half-width is required to be an integer multiple of 2 in the 4:2:0 / 4:2:2 cases, to avoid half-chroma samples.

Revisit after offline work by the proponent was requested to establish this constraint.

The remaining items in this category are TBP.

JVET-S0051 AHG9: Digital signature SEI message [J. Xu, Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

JVET-S0107 AHG9/AHG12: Recommended multi-layer composite picture SEI messages [J. Boyce (Intel)]

JVET-S0108 AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers [E. Thomas (TNO)]

JVET-S0213 AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers with example bitstreams [E. Thomas (TNO)]

JVET-S0214 AHG8/AHG9: Updates on the implementation of multi-layer decoding and output independent layer arrangement in VTM [E. Thomas (TNO)]

### HLS editorial inputs (4+3)

TBP.

JVET-S0068 AHG9: Editorial improvements on high-level syntax [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0096 AHG9: On RPL syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

Aspect 5 of this contribution belongs to this category.

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 2 of this contribution belongs to this category.

JVET-S0119 AHG9: Expression of existing intent for VPS/SPS/PPS syntax elements [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

JVET-S0192 AHG9: On reference picture list with generating unavailable reference picture [B. Choi, S. Wenger, S. Liu (Tencent)]

Aspects 2 and 3 of this contribution belong to this category.

JVET-S0194 AHG9: On editorial cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0208 AHG9: Proposed fixes to HLS [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, J. Enhorn, J. Ström, D. Liu (Ericsson)]

## AHG12: High-level parallelism and coded picture regions (7+5)

JVET-S0145 AHG12: A summary of proposals on tile, slices, and subpictures [Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0050 UTC on Friday 29 May and in the HLS AHG pre-meeting at 1945 on 20 June.

This contribution provides a summary of the 11 proposals on subpictures, slices, and tiles (the agenda item 3.2 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

On tiles and slices

- Allow the value of pps_no_pic_partition_flag to be different for different PPSs that are referred to by coded pictures within a CLVS. (S0049 aspect 3, S0095 aspect 4)

  - Alternatively, move pps_no_pic_partition_flag from the PPS to the SPS (and rename it to sps_no_pic_partition_flag). (S0095 aspect 4)

It was said that it is not clear why this constraint is in the draft and that the constraint does not seem logical. We have different flags regarding slices and tiles.

AHG Recommendation (cleanup): Remove the constraint.

- Do either of the following to reduce the current two options for signalling of a tile containing exactly one rectangular slice to one option only: (S0095 aspect 1)

  - Replace pps_num_exp_slices_in_tile[ i ] with pps_num_exp_slices_in_tile_minus1[ i ].

  - Change the upper limit of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 2.

There isn’t really a bug or problem, but there are two ways to signal the same thing.

Approach “a” would have a bit cost penalty.

No clear need for action was evident, so no action was recommended on this.

- For a more precise value range, change the upper limit of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − pps_num_exp_slices_in_tile[ i ]. (S0095 aspect 2)

This is purely editorial.

Editor action item: The editor may wish to consider whether this would improve clarity.

- Change the syntax condition for pps_rect_slice_flag from "if( NumTilesInPic > 1 )" to "if( NumTilesInPic > 3 )", because when the number of tiles in a picture is less than 4, the shape of each slice has to be rectangular. (S0095 aspect 3)

It was commented that this seems somewhat similar in spirit to item 2. This would force the encoder to decide the slice layout at the PPS level in advance when there are fewer than 4 tiles per picture. It was commented that this imposition would be undesirable, so no action was recommended on this.

- Change the semantics of pps_rect_slice_flag to cover the case that rectangular slice is smaller than the tile. (S0095 aspect 5)

This is a correction of an error in the current semantics expression.

AHG Recommendation (editorial bug fix): Adopt.

- The inference condition of pps_single_slice_per_subpic_flag is changed from "When not present" to "When the value of pps_no_pic_partition_flag is equal to 1" so that pps_single_slice_per_subpic_flag is inferred only when it is needed. (S0095 aspect 6)

This is purely editorial. (It makes no technical difference whether we make this change or not.)

Editor action item: The editor may wish to consider whether this would be an editorial improvement.

Discussion in the HLS AHG meeting stopped here at 0120 UTC on 29 May.

Discussion in the HLS AHG pre-meeting began here at 1945 on 20 June.

- When pps_pic_width_in_luma_samples is less than or equal to CtbSizeY, pps_num_exp_tile_columns_minus1 and pps_tile_column_width_minus1[ i ] are skipped and inferred. (S0116 aspect 1)

  - Alternatively, when the condition is true, the values of pps_num_exp_tile_columns_minus1 and pps_tile_column_width_minus1[ i ] are both constrained to be equal to 0. (S0116 aspect 1)

It was commented that for subpicture position and width / height, there is a syntax condition check to optimize for the single-CTU case. In that case it was necessary to prevent a syntax problem.

It was commented that we generally avoid optimizing syntax for such corner cases.

It was asked whether the constraint is already indirectly required, and the answer was yes, so no action was recommended for this.

- When pps_pic_height_in_luma_samples is less than or equal to CtbSizeY, pps_num_exp_tile_row_minus1 and pps_tile_row_height_minus1[ i ] are skipped and inferred. (S0116 aspect 1)

  - Alternatively, when the condition is true, the values pps_num_exp_tile_row_minus1 and pps_tile_row_height_minus1[ i ] are both constrained to be equal to 0. (S0116 aspect 1)

See item 7; this is basically the same issue.

- When NumTilesInPic is equal to 1 and either pps_pic_width_in_luma_samples or pps_pic_height_in_luma_samples is less than or equal to CtbSizeY, pps_tile_idx_delta_present_flag is skipped and inferred to be equal to 0. (S0116 aspect 2)

This raises similar issues as items 7 and 8. It was commented that a sufficient indirect constraint is already expressed.

On subpictures

- Add a shortcut for signalling of subpictures layout, as follows: (S0071)

  - Add sps_subpic_same_res_flag, equal to 1 specifies that all subpictures have the same width and height, respectively.

  - When sps_subpic_same_res_flag is equal to 1, sps_subpic_width_minus1[ i ] and sps_subpic_height_minus1[ i ] are only signalled when i is equal to 0, and sps_subpic_ctu_top_left_x[ i ] and sps_subpic_ctu_top_left_y[ i ] are skipped for all i values.

Item 17) is related.

The proponent indicated that a uniform grid would be a typical use case.

A cross-check is in S0247. It was reported that in one OMAF example, 1500 bits could be saved in the SPS. Two other participants said they had closely checked the text and software.

A concern was expressed about adding a special case at this late stage, potentially introducing some problem.

It was asked how this handles the case where the width/height is not divisible by the expressed uniform subpicture width/height. Such a case is disallowed in the proposed shortcut mode. It was noted that in the uniform tile width/height syntax shortcut case, that issue is handled differently, by creating “leftover” areas.

It was commented that various other shortcuts had been proposed before and was suggested that it is too late to introduce a new one.

A participant commented that the use case would be common and said there is not really a need for “leftover” consideration, as this would not be expected in the use of the scheme.

It was commented that the need for repetition in the syntax is rather annoying from the encoder perspective to need to repeat the same data over and over, and that the use case would actually be common.

It was commented that at this meeting we have resisted other temptations to introduce new shortcuts.

Revisit was suggested.

- When sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples are both less than or equal to CtbSizeY, sps_num_subpics_minus1 is skipped and inferred to be equal to 0. (S0115 aspect 1)

This raises similar issues as items 7 and 8. It was commented that a sufficient constraint is already expressed and special handling in the syntax is not necessary.

- Merge the three sets of constraints on the combination of subpictures and scalability, in the semantics of sps_subpic_treated_as_pic_flag[ i ], the semantics of pps_subpic_id[ i ], and in the end of clause 8.3.2, respectively, to be placed in the end of clause 8.3.2, as follows (removal of those constraints from the semantics of sps_subpic_treated_as_pic_flag[ i ] and the semantics of pps_subpic_id[ i ] not shown below): (S0125)

- When sps_num_subpics_minus1 is greater than 0 and the current subpicture with subpicture index subpicIdx has sps_subpic_treated_as_pic_flag[ subpicIdx ] equal to 1, it is a requirement of bitstream conformance that exactly one and not both of the following two conditions shall be true:

- The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] and the current picture have the same value for each of the following:

  - pps_pic_width_in_luma_samples

  - pps_pic_height_in_luma_samples

  - sps_num_subpics_minus1

  - sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ], sps_subpic_height_minus1[ i ], sps_subpic_treated_as_pic_flag[ i ], respectively, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive

  - SubpicIdVal[ subpicIdx ]

- The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

Some constraints in the current text are expressed the same regardless of whether the subpictures are treated as pictures or not. The proponent said it was not clear that this was intentional.

It was commented that there is an important bug fix in the proposal, when the dependent layer has independent subpictures and the reference layer has non-independent subpictures. This should be disallowed and was not in the current text.

However, a different issue is having non-independent subpictures in an enhancement layer with independent subpictures in the reference layer. This is currently disallowed (intended but not fully expressed in the Brussels output), and would be allowed in the proposal, although it is not supported by the specified extraction process. It was said that this characteristic of the proposal is undesirable, so it was agreed to disallow this case.

Revisit for modified text.

New submissions (for the June 10th submission deadline)

- Add the following constraint to the semantics of sps_subpic_treated_as_pic_flag[ ] (S0190 aspect 1):

When sps_num_subpics_minus1 is equal to 0, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of all the layers that have the current layer as a reference layer:

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

It is proposed that all reference layers need to have the same subpicture layout. There seemed to be a problem with the proposed expression of this concept, so the proposed text was edited above to reflect this intent. It was commented that it is not clear that such a constraint needs to be imposed. No action was recommended unless offline study determines otherwise.

- Add a constraint: When the value of sps_subpic_info_present_flag of the current layer is equal to 1, the value of sps_res_change_in_clvs_allowed_flag of the reference layers of the current layer shall be equal to 0. (S0190 aspect 2).

In the discussed case, the size of the pictures in the reference layer could change from AU to AU. It was commented that this is not necessarily a problem, as we already have resampling invoked between the base layer and enhancement layer, and changing the resampling ratio from AU to AU should not be a problem. No action was recommended unless offline study determines otherwise.

- Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present, for consistency with the inferences of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag. (S0160 aspect 1)

It was commented that this is purely editorial, since the flag is only inferred when there is only one tile in the picture. And since the tile boundary is also a picture boundary in that case, no loop filtering is applied for that boundary anyway.

Editor action item: The editor is asked to consider this suggested consistency improvement.

- Add a subpicture merging software, which includes the following setup, into the VTM source code package: (S0162)

- Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder.

- Merging selected encoded bitstreams into a bitstream with multiple subpictures, using provided subpicture merging software.

- Decoding a bitstream having multiple subpictures, using the VTM decoder.

The contribution was greatly appreciated for demonstration and experimentation with merging functionality. Software was provided with the contribution, but the understanding was that the merging part was the only change – the encoder and decoder were merely a copy of the existing VTM software. It was agreed to add the merging software into the VTM software package.

It was commented that the type of encoding that is done in this experiment does not show the full potential of VVC for an application, as it disables several tools that are useful.

Having more functionality added in future work is highly desirable.

It was commented that this software will need to be tested and maintained as the rest of the VTM package evolves.

- Add a flag, sps_raster_scan_order_subpics_flag, to the SPS, the value equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS, and when this flag is equal to 1, derive the top-left CTU positions of the subpictures instead of signalling them. (S0206)

Item 10) is related.

We have a similar scheme for rectangular slices. Software was provided, and it had been cross-checked, as reported in S0250.

Relative to item 10, this would save roughly half of the bits in that special case.

It was noted that this is only an SPS-level issue, where bits are not so precious.

It was commented that other schemes for SPS-level savings were not adopted.

It was commented that the derivation process seems nontrivial.

Due to the expressed concerns and late stage of the project, no action was recommended on this.

- Specify that the value of sps_num_subpics_minus1 shall be less than MaxSlicesPerPicture. (S0236 aspect 1)

It was noted that this is purely editorial.

Editor action item: The editor is strongly suggested to consider this change (MaxSlicesPerAu rather than MaxSlicesPerPicture).

- When pps_subpic_id_mapping_present_flag is equal to 1, skip pps_rect_slice_flag and infer it to be equal to 1. (S0236 aspect 2)

It was asked whether it would be possible to define subpicture mapping to be compatible with rectangular slices. Yes, this would be possible. However, it was reported that we already have an assumption of this constraint that is built into other parts of the syntax.

It was commented that this is too trivial a refinement to consider at this stage, so no action was recommended for this.

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 3 of this contribution belongs to this category.

JVET-S0071 AHG12: Cleanup of subpicture layout signalling [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

JVET-S0247 Crosscheck of JVET-S0071 (AHG12: Cleanup of subpicture layout signalling) [R. Sjöberg, D. Liu, M. Damghanian (Ericsson)] [late]

JVET-S0095 AHG12: On tile and slice partitioning related syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

JVET-S0115 AHG9: On SPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 1 of this contribution belongs to this category.

JVET-S0116 AHG9: On PPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0125 AHG9: On sub-picture constraints [V. Seregin, Y. He, Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0190 AHG8/AHG9/AHG12: On reference picture resampling with scalability [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspect 1 of this contribution belongs to this category.

JVET-S0162 AHG3/AHG12: Subpicture merging software [A. Hallapuro, M. M. Hannuksela (Nokia)]

JVET-S0206 AHG12: Raster scan order subpictures [M. Damghanian, D. Liu, R. Sjöberg, M. Pettersson, J. Enhorn, J. Ström, R. Yu (Ericsson)]

JVET-S0250 Crosscheck of JVET-S0206 (Raster scan order subpictures) [S. Paluri, Hendry(LGE)] [late]

JVET-S0236 AHG9/AHG12: High-level syntax cleanups on subpictures [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

## AHG8: Layered coding and resolution adaptivity (8+5)

### General (1+0)

JVET-S0147 AHG8: A summary of proposals on scalability and RPR [Y.-K. Wang (Bytedance)]

This contribution provides a summary of the 12 proposals on scalability and RPR (the agenda item 3.3 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

The following changes were proposed:

On scalability

- When vps_max_layers_minus1 is equal to 0, vps_num_ptls_minus1 is skipped and inferred to be equal to 0. (S0063)

It was commented that this may not be strictly necessary but seems like a very small nice little cleanup and that since we are skipping nearby syntax elements under this condition.

AHG Recommendation (cleanup/consistency): Adopt.

- Fix an asserted bug, that the specification allows to extract sublayers with TemporalId higher than the vps_ptl_max_temporal_id[ 0 ] + 1 for an OLS, by setting NumSubLayersInLayerInOLS[ i ][ k ] to be equal to vps_ptl_max_temporal_id[ i ] + 1, instead of vps_max_sublayers_minus1 + 1, for output layers in Equation 40. (S0100 aspect 1)

AHG Recommendation (bug fix/expression of existing intent): Adopt.

- Add the following constraint to disallow certain indirect reference layers when the value of vps_max_tid_il_ref_pics_plus1[ i ][ j ] is 0 for all the direct reference layers: (S0100 aspect 2)

For any two layers k and j within an OLS i, with k > j, when both NumSubLayersInLayerInOLS[ i ][ j ] and NumSubLayersInLayerInOLS[ i ][ k ] are equal to 0, j is a (direct or indirect) reference layer of k and there is a picture in layer k that is an IRAP picture or a GDR picture with ph_recovery_poc_cnt equal to 0 in an AU, the picture in layer j shall also be an IRAP picture or a GDR picture with ph_recovery_poc_cnt equal to 0, respectively.

It was commented that a similar constraint is already enforced by the requirement that the extracted bitstream shall be a conforming bitstream. The proposed constraint is a bit tighter than the indirect existing constraint.

Editor action item: The editor is requested to consider adding some NOTE/explanation for this aspect.

- Do one of the following, with option b being preferred: (S0100 aspect 3)

  - The values of vps_ptl_max_temporal_id[ i ], vps_dpb_max_temporal_id[ i ] and vps_hrd_max_tid[ i ] shall be the same.

  - The values of vps_ols_dpb_max_temporal_id[ vps_ols_dpb_params_idx[ i ] ] and vps_hrd_max_tid[ vps_ols_hrd_idx[ i ] ] shall be greater than or equal to vps_ptl_max_temporal_id[ vps_ols_ptl_idx[ i ] ].

  - Infer the DPB and HRD parameters for sublayers from vps_ols_dpb_max_temporal_id[ dpbIdx ] and vps_hrd_max_tid[ hrdIdx ] up to vps_ptl_max_temporal_id[ ptlIdx ].

The idea is to have a relationship of these three values for any particular OLS. (The index variables above may not express this concept quite correctly.)

It was noted that S0097 aspect 3 is related (see section 3.1.8).

A participant commented that approach “a” is too restrictive, and indeed that approach “b” is preferred. Another participant considered approach “a” to be better.

Approach “b” would allow to send less unnecessary syntax for HRD and DPB parameters for the various OLSs than approach “a”.

It was asked whether S0097 aspect 3 would take care of the issue by itself.

The AHG agreed to defer this issue and S0097 aspect 3 for further study.

- When not present, the value of vps_max_tid_ref_present_flag[ i ] is inferred to be equal to 0. (S0129 aspect 1)

The contributor said this is not needed after review of the text and withdrew this aspect.

- When vps_all_independent_layers_flag is equal to 1 and vps_each_layer_is_an_ols_flag is equal to 1, the value of vps_ols_mode_idc is inferred to be equal to 0. (also in S0129 aspect 1)

It was noted that this is purely editorial, as the value that is inferred has no effect; this is just a matter of editorial completeness – wanting to always have some value if something is checked in a condition.

It was commented that inferring the value 0 might be confusing or incorrect, since 0 ordinarily has a semantic interpretation that is not intended here. Inferring the value 3 was suggested. Alternatively, adding an extra “if” with a comment seemed better.

Editor action item: The editor is asked to rephrase the text to avoid the problem. The suggested method is to add an extra “if” condition with a comment to explain why it is there.

On RPR

- Change the value ranges of the scaling window offsets such that the scaling window width and height can be up to 16 times the picture width and height, respectively. (S0048 aspect 1)

It was asked whether this might have a memory bandwidth impact and concluded that it does not.

However, it was commented that there could be a bit width impact on some variables.

This is closely related to item 8 below.

The HLS AHG deferred this topic for further study.

- Change the conformance specification for subpicture sequences for the case where higher layers of an OLS contain multiple subpictures while lower layers of OLS do not use subpicture partitioning, as follows: (JVET-S0098)

  - Support extraction of subpictures that have reference layers with only one subpicture.

This is closely related to item 7 above.

The HLS AHG deferred this topic for further study.

  - Change the derivation of RprConstraintsActive[ i ][ j ] to incorporate the number of subpictures in reference picture and current picture, and set RprConstraintsActive[ i ][ j ] to 1 if the two values differ.

This is to fix a clear problem in the text.

AHG Recommendation (bug fix/expression of existing intent): Adopt this aspect.

  - Signal a new syntax element non_subpic_layers_fraction[ i ] that specifies the fraction of the bitstream level limits associated with layers in the bitstream that have sps_num_subpics_minus1 equal to 0, for each i.

The current derivation is asserted to not be sensible for cases where different layers have different subpicture layouts.

AHG Recommendation (bug fix): Adopt this aspect.

  - Handle accumulated levels for subpicture sets by using one of the following options:

    - Option 1: Remove the accumulation and adjust equation D.11 to use a single subpicture.

    - Option 2: Modify equation D.10 (and D.11).

AHG Recommendation (bug fix): Adopt option 1.

- Change the semantics of the sps_ref_pic_resampling_enabled_flag

- to be aligned with the derivation of the RprConstraintsActive variable, which would be set equal to 1 when one or more of the following 6 parameters differ between the current picture and the reference picture: 1) picture width, 2) picture height, 3) scaling window left offset, 4) scaling window right offset, 5) scaling window top offset, and 6) scaling window bottom offset. (S0048 aspect 2)

- such that when sps_ref_pic_resampling_enabled_flag equal to 1, a slice may refer to a reference picture with a different spatial resolution or a different scaling window. (S0057 aspect 1)

This is editorial – a matter of accurately describing the semantics.

Approach “a” refers to PPS content in SPS semantics, which was suggested to not be appropriate. However, it is more explicit regarding what is involved, and this was suggested to be helpful.

The phrase “spatial resolution” in approach “b” was questioned.

Editor action item: The editor is requested to clarify the semantics as appropriate.

- Add pps_res_change_in_clvs_allowed_flag, and when pps_res_change_in_clvs_allowed_flag is equal to 0, skip pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples and infer their values to be equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively. (S0057 aspect 2)

It was commented that this could introduce a parsing dependency problem for the tile configuration in the PPS. The proposal had not been tested with an implementation. Thus, no action was recommended for this.

- Add either of the following constraints (S0126):

- When sps_res_change_in_clvs_allowed_flag is equal to 0, all pictures in the CLVS shall have the same values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset, respectively.

- When sps_res_change_in_clvs_allowed_flag is equal to 0, a current picture in the CLVS and a reference picture having the same nuh_layer_id shall have the same values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset, respectively.

It was commented that this constraint would limit some use cases and that a similar constraint had previously been considered and agreed to be undesirable (see JVET-R0058 versions 1-3 aspect 2.c).

No clear problem was agreed to exist, so no action was recommended on this.

New submissions (for the June 10th submission deadline)

TBP here onwards.

- Change Equation 37 as follows, considering that a reference layer can only be a lower layer: (S0169 aspect 2) (additions in bold and bigger fonts, removals in italic and smaller fonts)

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0; j < i  <=  vps_max_layers_minus1; j++ ) {
		dependencyFlag[ i ][ j ] = vps_direct_ref_layer_flag[ i ][ j ]
		for( k = j+1 0 ; k < i; k++ )
			if( vps_direct_ref_layer_flag[ i ][ k ]  &&  dependencyFlag[ k ][ j ] )
				dependencyFlag[ i ][ j ] = 1
	}
	LayerUsedAsRefLayerFlag[ i ] = 0
}
for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0, d = 0, r = 0; j < i  <=  vps_max_layers_minus1; j++ ) {	(37)
		if( vps_direct_ref_layer_flag[ i ][ j ] ) {
			DirectRefLayerIdx[ i ][ d++ ] = j
			LayerUsedAsRefLayerFlag[ j ] = 1
		}
		if( dependencyFlag[ i ][ j ] )
			RefLayerIdx[ i ][ r++ ] = j
	}
	NumDirectRefLayers[ i ] = d
	NumRefLayers[ i ] = r
}

- Do either of the following, considering that assertedly vps_num_output_layer_sets_minus1 is signalled when vps_max_layers_minus1 is greater than 0 (and when vps_ols_mode_idc is equal to 2) and in this case there are at least two OLSs: (S0183 aspect 1)

  - Change vps_num_output_layer_sets_minus1 to vps_num_output_layer_sets_minus2.

  - Add a constraint: The value of vps_num_output_layer_sets_minus1 shall be greater than 0.

- Add a flag for each multi-layer OLS (except the first one) to indicate whether the OLS DPB picture width, height, chroma format, and bit depth values are the same as the values for the previous multi-layer OLS, and if the flag is equal to 1, these fields are skipped and inferred. (S0183 aspect 2)

- Change to code the vps_ols_dpb_pic_width[ i ] and vps_ols_dpb_pic_height[ i ] in units of 8 luma samples, considering that assertedly the decoded picture width and height in a CLVS are constrained to be an integer multiple of Max( 8, MinCbSizeY ). (S0212 aspect 1)

- Instead of signalling a set of width, height, chroma format, and bit depth of a DPB picture storage buffer for each multi-layer OLS in the VPS, signal these DPB parameters in the dpb_parameters( ) syntax strucures, each of which (per the existing VVC text) can be shared by two or more OLSs. (S0160 aspect 10 late)

- Specify that the ue(v)-coded vps_ols_dpb_bitdepth_minus8[ i ] shall be in the range of 0 to 8, inclusive, considering that the value range is currently missing. (S0212 aspect 2)

- In Equation 41, replace vps_ptl_max_tid[ 0 ] with vps_ptl_max_tid[ vps_ols_ptl_idx[ 0 ] ] and vps_ptl_max_tid[ i ] with vps_ptl_max_tid[ vps_ols_ptl_idx[ i ] ] (for all the instances). (S0248 aspect 1 late)

### Scalability cleanups (3+5)

JVET-S0063 AHG9: On VPS Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0100 AHG9: On OLS and sublayers [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI), Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 1 of this contribution belongs to this category.

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspect 10 of this contribution belongs to this category.

JVET-S0169 AHG9: Miscellaneous Cleanups for HLS [B. Wang, S. Esenlik, H. Gao, E. Alshina (Huawei)]

Aspect 2 of this contribution belongs to this category.

JVET-S0183 AHG9: On VPS Information Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0212 AHG9: Modifications on VPS OLS DPB related parameters [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, J. Ström, D. Liu (Ericsson)]

JVET-S0248 AHG9: HRD text cleanups [Y.-K. Wang (Bytedance), Y. Sanchez, R. Skupin (HHI), S. Deshpande (Sharp), V. Seregin (Qualcomm), Hendry (LGE), J. Chen (Alibaba)] [late]

Aspect 1 of this contribution belongs to this category.

### Reference picture resampling (RPR) cleanups (4+0)

JVET-S0048 AHG9/AHG8: On reference picture resampling [Y.-K. Wang, Z. Deng, K. Zhang, L. Zhang (Bytedance)]

JVET-S0098 AHG9/12: On subpicture conformance [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-S0057 AHG8/AHG9: On signaling reference picture resampling [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0126 AHG8: On scaling window constraint [Y.-J. Chang, V. Seregin, Y. He, A. K. Ramasubramonian, M. Coban, M. Karczewicz (Qualcomm)]

– End of AHG report –