| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0252 |  |
| --- | --- | --- |

| Title: | PH semantics of ALF and SAO syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall Ling Li Xiang Li Byeongdoo Choi | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc., Tencent |  |  |

_____________________________

# Abstract

This contribution presents a proposal for modifications to the semantics of picture header syntax elements related to ALF and SAO. The changes are asserted to address the inconsistency in the semantics reported in JVET-S0121.

# Introduction

This contribution is the result of an offline discussion related to document JVET-S0121 and includes proposed changes to the semantics of PH syntax elements related to ALF and SAO. In total there are currently seven flags in the PH for which the semantics says that the value 0 specifies that the corresponding tool is disabled. However, since there are corresponding flags in the SH that may enable the tools, it is proposed to change from “disabled” to “may or may not be enabled”. For two of the syntax elements an editorial change is proposed to reorder so that the text for when the value is equal to “1” always precedes the text for when the value is “0”.

No changes are proposed to the slice header.

# Proposal

ph_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be used for the current picture. ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter may or may not be enabled is disabled and not used for the current picture. When not present, ph_alf_enabled_flag is inferred to be equal to 0.

ph_alf_cb_flag equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cb colour component for the current picture. ph_alf_cb_flag equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cb colour component for the current picture. ph_alf_cb_flag equal to 0 specifies that the adaptive loop filter may or may not be enabled for the Cb colour component of the current picture. When ph_alf_cb_flag is not present, it is inferred to be equal to 0.

ph_alf_cr_flag equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cr colour component for the current picture. ph_alf_cr_flag equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cr colour component for the current picture. ph_alf_cr_flag equal to 0 specifies that the adaptive loop filter may or may not be enabled for the Cr colour component of the current picture. When ph_alf_cr_flag is not present, it is inferred to be equal to 0.

ph_cc_alf_cb_enabled_flag equal to 1 specifies that cross-component adaptive loop filter for the Cb colour component is enabled and may be used for the current picture. ph_cc_alf_cb_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cb colour component may or may not be enabled is disabled and not used for the current picture. When not present, ph_cc_alf_cb_enabled_flag is inferred to be equal to 0.

ph_cc_alf_cr_enabled_flag equal to 1 specifies that cross-compoent adaptive loop filter for the Cr colour component is enabled and may be used for the current picture. ph_cc_alf_cr_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cr colour component may or may not be enabled is disabled and not used for the current picture. When not present, ph_cc_alf_cr_enabled_flag is inferred to be equal to 0.

ph_sao_luma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the luma component of the current picture. ph_sao_luma_enabled_flag equal to 0 specifies that SAO may or may not be enabled is disabled and not used for the luma component of the current picture. When ph_sao_luma_enabled_flag is not present, it is inferred to be equal to 0.

ph_sao_chroma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the chroma component of the current picture. ph_sao_chroma_enabled_flag equal to 0 specifies that SAO may or may not be enabled is disabled and not used for the chroma component of the current picture. When ph_sao_chroma_enabled_flag is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Sharp Labs of America, Inc. and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).