| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0258 |
| --- | --- |

| Title: | AHG9: On subpicture constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Yong He, Yao-Jen Chang, Muhammed Coban, Marta Karczewicz (Qualcomm) Ye-Kui Wang (Bytedance) Robert Skupin (HHI) | Tel: Email: | vseregin@qti.qualcomm.com yekui.wang@bytedance.com firstname.lastname@hhi.fraunhofer.de |
| Source: | Qualcomm Incorporated, Bytedance Inc., Fraunhofer HHI |  |  |

_____________________________

# Abstract

There are multiple constraints applied for subpictures, some constraints are applied for subpictures treated as pictures, some other constraints are applied regardless of sps_subpic_treated_as_pic_flag value. These constraints are expressed in three different places in the specification, namely the semantics of related syntax elements in SPS, PPS and in the decoding process for reference picture list construction.

This contribution is a continuation of JVET-S0125, where additional changes are included based on JVET-S0125 proposal review in HLS AHG and offline discussion. The contribution proposes to add a condition of sps_subpic_treated_as_pic_flag[ i ] equal to 1 for subpicture constraints to have the same picture size, subpicture layout, subpicture ID, and sps_subpic_treated_as_pic_flag for a current and active reference pictures, including inter-layer reference pictures.

The subpicture constraints are consolidated in the section 8.3.2 “Decoding process for reference picture lists construction”.

# Introduction

In VVC specification, there are three places where constraints are expressed for subpictures.

### 7.4.3.3 Sequence parameter set RBSP semantics

...

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

...

### 7.4.3.4 Picture parameter set RBSP semantics

...

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( sps_subpic_id_mapping_explicitly_signalled_flag )
		SubpicIdVal[ i ] = pps_subpic_id_mapping_present_flag ? pps_subpic_id[ i ] : sps_subpic_id[ i ]	(81)
	else
		SubpicIdVal[ i ] = i

It is a requirement of bitstream conformance that both of the following constraints apply:

–	For any two different values of i and j in the range of 0 to sps_num_subpics_minus1, inclusive, SubpicIdVal[ i ] shall not be equal to SubpicIdVal[ j ].

–	For each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, when the value of SubpicIdVal[ i ] of a current picture is not equal to the value of SubpicIdVal[ i ] of a reference picture, the active entries of the RPLs of the coded slices in the i-th subpicture of the current picture shall not include that reference picture.

...

### 8.3.2 Decoding process for reference picture lists construction

…

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and sps_num_subpics_minus1 is greater than 0, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

...

# Proposed modifications

The proposed changes are highlighted in yellow and are as follows.

### 7.4.3.3 Sequence parameter set RBSP semantics

...

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

...

### 7.4.3.4 Picture parameter set RBSP semantics

...

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( sps_subpic_id_mapping_explicitly_signalled_flag )
		SubpicIdVal[ i ] = pps_subpic_id_mapping_present_flag ? pps_subpic_id[ i ] : sps_subpic_id[ i ]	(81)
	else
		SubpicIdVal[ i ] = i

It is a requirement of bitstream conformance that both of the following constraints apply:

–	For any two different values of i and j in the range of 0 to sps_num_subpics_minus1, inclusive, SubpicIdVal[ i ] shall not be equal to SubpicIdVal[ j ].

–	For each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, when the value of SubpicIdVal[ i ] of a current picture is not equal to the value of SubpicIdVal[ i ] of a reference picture, the active entries of the RPLs of the coded slices in the i-th subpicture of the current picture shall not include that reference picture.

...

### 8.3.2 Decoding process for reference picture lists construction

…

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, and sps_num_subpics_minus1 is greater than 0 and the current subpicture with subpicture index subpicIdx has sps_subpic_treated_as_pic_flag[ subpicIdx ] equal to 1, it is a requirement of bitstream conformance that both of the following conditions shall be true:

  - Exactly one and not both of the following two conditions shall be true:

    - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] and the current picture have the same value for each of the following:

      - pps_pic_width_in_luma_samples

      - pps_pic_height_in_luma_samples

      - sps_num_subpics_minus1

      - sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ], sps_subpic_height_minus1[ i ], sps_subpic_treated_as_pic_flag[ i ], respectively, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive

      - SubpicIdVal[ subpicIdx ]

    - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

  - The current picture and any picture that is in a different layer than the current layer and has the current picture in an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] have the same value for each of the following:

    - pps_pic_width_in_luma_samples

    - pps_pic_height_in_luma_samples

    - sps_num_subpics_minus1

    - sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ], sps_subpic_height_minus1[ i ], sps_subpic_treated_as_pic_flag[ i ], SubpicIdVal[ i ], respectively.

# Patent rights declaration(s)

Qualcomm Incorporated, Bytedance Inc., and Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).