| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0105 |
| --- | --- |

| Title: | AHG12: On the Rice parameter derivation for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van Dmytro Rusanovskyy Marta Karczewicz | Email: | lphamvan@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution proposes a modification to the Rice parameters selection method for regular residual coding (RRC), for consideration in VVC version 2. In current VVC, the rice parameter has been extensively tested for encoding video sources of 8 bit-depth or 10 bit-depth and is limited in the range from 0 to 3 by clipping. It is asserted that the residual coefficient values become larger for high bit depths and low QPs. Therefore, the current restriction of the rice parameter range leads to performance degradation of VTM in high bit-depth coding. To address this issue, this contribution introduces a minor modification in the rice parameter derivation of VVC: a down scaling of the locSumAbs prior to rice parameter derivation.

It is reported x.xx% and 24.80% in bdrateY in AI-12 qp=-13, -8, -3, 2, 7, and AI-16 -33, -28, -23, 18, 13, and xx.xx% and xx.xx% in bdrateY in LDB-12 qp=-13, -8, -3, 2, 7, and LDB-16 -33, -28, -23, 18, 13 respectively.

# Introduction

In the current VVC [1], rice parameter for RRC is derived using look-up table with considering of coefficient value of the neighboring transform coefficients in a template. The template of the neighbouring coefficients is presented in Figure 1.

First, locSumAbs which is the sum of absolute of five available neighbouring coefficients in the template is calculated.

The locSumAbs is then normalized (subtraction and clip) as following:

locSumAbs = Clip3( 0, 31, locSumAbs - baseLevel * 5 )

The rice parameter is derived using this locSumAbs with look-up table (Table 1). With the current design, the rice parameter range is constrained from 0 to 3.

Figure 1 Neighbouring coefficients which are using in calculation of the localSumAbs of the current  coefficient

Table 1. Look-up table for Rice Parameter based on locSumAbs in current spec

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

When the bitdepth of the input video is increased, or the extended precision is enabled, or the quantization parameter is set very low, the range of the coefficients and therefore locSumAbs value may significantly increase. In such cases, the current VVC range of allowed rice parameter may not be sufficient. This limitation causes a coding performance drop of VVC in high bit-depth coding.

# Proposed methods

This contribution proposes a derivation method for rice parameter which inherits the design of current VVC design with a minor change and does not affect to the performance of VVC version 1.

Proposed modification can be expressed as following.

Firstly, the locSumAbs is scalled/normalized by a scalling factor which depends on the input bitdepth. The localSumAbs is then used to derive the rice parameter using look-up table as in current VVC. Finally, the parameter cRiceParam is added by the scalling factor:

Proposed method1: The down-scaling factor depends on the input bitdepth

The specification text is accordingly modified as follows:

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } shift = (Bitdepth – 10) > 0) && (localSumAbs − baseLevel * 5) >= 32 ? Floor(Log2(4*(Bitdepth - 10))) : 0 localSumAbs = (shift == 0) ? localSumAbs : (localSumAbs + (1 << (shift -1) )) >> shift locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

Proposed method2: The down-scaling factor does not depend on the input bitdepth

The specification text is accordingly modified as follows (only updated line compare to Method 1 is listed):

shift = (localSumAbs − baseLevel * 5) >= 32 ? 4 : 0

# Simulation results

The proposed modification has been implemented on top of the latest VTM-10.0 with JVET_R0351_HIGH_BIT_DEPTH_ENABLED is set equal to 1. The test sequences and condition follow AHG12 CTC [2].

Anchor:

Anchor0: VTM-10.0 with ExtendedPrecision = 0

Anchor1: VTM-10.0 with ExtendedPrecision = 1

Tests:

Test1: VTM-10.0 with ExtendedPrecision = 1, scaling factor for localSumAbs depends on the input bitdepth

Test2: VTM-10.0 with ExtendedPrecision = 1, scaling factor for localSumAbs does not depend on the input bitdepth

The experimental results are summarized as in the following Tables.

Test1 vs Anchor1

|  | AI-12 |  |  |  |  |  |  |  |  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10 |  |  |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  | Y | U | V | EncT | DecT |
| SVT |  |  |  |  |  |  |  | SVT |  | -24.80% | -23.66% | -23.66% | 110% | 95% |
| PQ444 | -0.53% | -0.56% | -0.58% | 85% | 90% |  |  |  |  |  |  |  |  |  |
| PQ422 | -0.63% | -0.69% | -0.71% | 90% | 90% |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  | Overall |  | -24.80% | -23.66% | -23.66% | 110% | 87% |
|  | LDB-12 |  |  |  |  |  |  |  |  | LDB-16 |  |  |  |  |
|  | Over VTM-10 |  |  |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  | Y | U | V | EncT | DecT |
| SVT |  |  |  |  |  |  |  | SVT |  |  |  |  |  |  |
| PQ444 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PQ422 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  | Overall |  |  |  |  |  |  |
|  |  |  | RA-12 |  |  |  |  |  |  |  |  | RA-16 |  |  |
|  | Over VTM-10 |  |  |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  | Y | U | V | EncT | DecT |
| SVT |  |  |  |  |  |  |  | SVT |  |  |  |  |  |  |
| PQ444 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PQ422 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  | Overall |  |  |  |  |  |  |

## Additional results (extended_precision_flag = 1 in test, 0 in anchor)

Test1 vs Anchor0

|  | AI-12 |  |  |  |  |  |  |  |  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10 |  |  |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  | Y | U | V | EncT | DecT |
| SVT |  |  |  |  |  |  |  | SVT |  | -22.13% | -28.66% | -27.05% | 110% | 95% |
| PQ444 | -1.90% | -2.51% | -2.68% | 99% | 97% |  |  |  |  |  |  |  |  |  |
| PQ422 | -1.78% | -2.09% | -2.21% | 96% | 95% |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  | Overall |  | -22.13% | -28.66% | -27.05% | 110% | 95% |
|  | LDB-12 |  |  |  |  |  |  |  |  | LDB-16 |  |  |  |  |
|  | Over VTM-10 |  |  |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  | Y | U | V | EncT | DecT |
| SVT |  |  |  |  |  |  |  | SVT |  |  |  |  |  |  |
| PQ444 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PQ422 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  | Overall |  |  |  |  |  |  |
|  |  |  | RA-12 |  |  |  |  |  |  |  |  | RA-16 |  |  |
|  | Over VTM-10 |  |  |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  | Y | U | V | EncT | DecT |
| SVT |  |  |  |  |  |  |  | SVT |  |  |  |  |  |  |
| PQ444 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PQ422 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  | Overall |  |  |  |  |  |  |

# Conclusion

This contribution proposes the modification on the derivation of the rice parameter to support high bit-depth coding. Proposed modification shows significant coding efficiency compared to current VTM in high bit-depth coding, while introducing no changes to processing on the 10bits and below. It is recommended to conduct core experiment for this technique.

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[2] T. Ikai, A. Browne, X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).