| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 – 16 October 2020 | Document: JVET-T0116 |
| --- | --- |

| Title: | Crosscheck of AHG10: Extension of rate control to support random access configuration with GOP size of 32 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Yang Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-T0062 [1], which proposes the extension of the rate control in VTM to support the configuration with the GOP size of 32. There is a mismatch between the simulation results and those provided in JVET-T0062.

Experimental results

JVET-T0062 was implemented on top of VTM-10.0x (git hash: 16bc143c). The simulations are conducted in the condition of Random Access. The crosscheck results of test #1 and test #2 are shown in Table 1 and Table 2. Most simulation results match with those provided in JVET-T0062 except the case highlighted in yellow in Table 2, while the average BD-rate of luma component for class D is 5.73% provided in JVET-T0062. The mismatch is caused by rounding difference from BQSquare with QP32 in VTM anchor, wherein the summary output of PSNR-Y for sequential encoding is 32.8795 dB provided in JVET-T0062 and the PSNR-Y for parallel encoding calculated by averaging results from all frames is 32.8794 dB (i.e., 32.879449 dB) in this crosscheck report.

Table 1. Results of test #1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0-16bc143c (fix bit ratio) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -17.62% | -19.56% | -20.71% | 93% | 98% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -19.52% | -18.99% | -18.82% | 89% | 95% |
| Class F |  |  |  |  |  |

Table 2. Results of test #2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0-16bc143c (fix QP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 3.16% | 6.99% | 7.84% | 103% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 5.72% | 15.30% | 16.82% | 102% | 92% |
| Class F |  |  |  |  |  |

References

F. Liu, Z. Liu, Y. Li, and Z. Chen, “AHG10: Extension of rate control to support random access configuration with GOP size of 32,” JVET-T0062, October 2020, by teleconference.