| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 – 16 October 2020 | Document: JVET-T0116-v2 |
| --- | --- |

| Title: | Crosscheck of AHG10: Extension of rate control to support random access configuration with GOP size of 32 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Yang Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-T0062 [1], which proposes the extension of the rate control in VTM to support the configuration with the GOP size of 32. There is a mismatch between the simulation results and those provided in JVET-T0062.

Experimental results

JVET-T0062 was implemented on top of VTM-10.0x (git hash: 16bc143c). The simulations are conducted in the condition of Random Access. The crosscheck results of test #1 and test #2 are shown in Table 1 and Table 2. Most simulation results match with those provided in JVET-T0062 except the cases highlighted in yellow in Table 1 and Table 2. In Table 1, the average BD-rate of luma component for ClassF is -7.75% provided in JVET-T0062, and in Table 2 the average BD-rate of luma component for class D is 5.73% provided in JVET-T0062. In Table 1, the mismatch is caused by rounding difference from ArenaOfValor with QP32 in JVET-T0062, wherein the summary output of PSNR-Y for sequential encoding is 37.2412 dB provided in JVET-T0062 and the PSNR-Y for parallel encoding calculated by averaging results from all frames is 37.2413 dB in this crosscheck report. In Table 2, the mismatch is caused by rounding difference from BQSquare with QP32 in VTM anchor, wherein the summary output of PSNR-Y for sequential encoding is 32.8795 dB provided in JVET-T0062 and the PSNR-Y for parallel encoding calculated by averaging results from all frames is 32.8794 dB (i.e., 32.879449 dB) in this crosscheck report.

Table 1. Results of test #1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0-16bc143c (fix bit ratio) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.09% | -13.74% | -16.39% | 98% | 98% |
| Class A2 | -21.78% | -24.61% | -23.96% | 89% | 96% |
| Class B | -20.02% | -23.00% | -22.72% | 91% | 99% |
| Class C | -17.62% | -19.56% | -20.71% | 93% | 98% |
| Class E |  |  |  |  |  |
| Overall | -17.75% | -20.55% | -21.17% | 93% | 98% |
| Class D | -19.52% | -18.99% | -18.82% | 89% | 95% |
| Class F | -7.76% | -8.64% | -8.18% | 99% | 100% |

Table 2. Results of test #2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0-16bc143c (fix QP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 16.83% | 17.73% | 15.18% | 103% | 99% |
| Class A2 | 6.68% | 11.89% | 10.30% | 105% | 99% |
| Class B | 5.37% | 9.44% | 9.63% | 105% | 98% |
| Class C | 3.16% | 6.99% | 7.84% | 103% | 97% |
| Class E |  |  |  |  |  |
| Overall | 7.33% | 10.94% | 10.39% | 104% | 98% |
| Class D | 5.72% | 15.30% | 16.82% | 102% | 92% |
| Class F | 16.21% | 12.92% | 13.57% | 117% | 98% |

References

F. Liu, Z. Liu, Y. Li, and Z. Chen, “AHG10: Extension of rate control to support random access configuration with GOP size of 32,” JVET-T0062, October 2020, by teleconference.