| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0007-v1 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. Francois W. Husak S. Iwamura D. Rusanovskyy | Email: | asegall@sharplbas.com Edouard.Francois@InterDigital.com WJH@dolby.com iwamura.s-gc@nhk.or.jp> dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 20th meeting by teleconference (7 – 16 October 2020) and the 21st Meeting, by teleconference, 6–15 Jan. 2021.

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Generate CTC anchors for the VTM according to JVET-T2011 within two weeks of availability of SDR CTC anchors.

- Study the luma/chroma bit allocation in the HDR CTC, especially for HLG content.

- Coordinate with AHG4 in preparation for verification testing for HDR video content.

- Study additional aspects of coding HDR/WCG content.

# Activities

The primary activity of the AhG was related to the mandates of (i) study and evaluate available HDR/WCG test content, (ii) comparing the performance of the VTM for HDR/WCG content, and (iii) coordinating with AHG4 in preparation for verification testing for HDR video content. This work is described in the following subsection.

## Anchor Generation

The AhG generated CTC anchors for the VTM according to JVET-T2011.  A summary of the performance is provided below, and more detailed information may be found in the included XLS data.

### VTM 10.0 versus VTM 9.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 102% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -19.12% | -15.43% | -9.47% | -11.80% | -11.26% | -9.49% | -11.54% | -11.01% | 100% | 101% |
| Class H2 |  |  |  |  |  | -7.58% | -9.84% | -11.45% | 99% | 103% |
| Overall | -19.12% | -15.43% | -9.47% | -11.80% | -11.26% | -8.79% | -10.92% | -11.17% | 99% | 101% |

### VTM 11.0 versus HM 16.18

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.23% | -26.74% | -26.21% | -56.67% | -52.06% | -23.54% | -52.57% | -45.24% |  |  |
| Class H2 |  |  |  |  |  | -19.90% | -53.76% | -48.39% |  |  |
| Overall | -41.23% | -26.74% | -26.21% | -56.67% | -52.06% | -22.22% | -53.00% | -46.39% |  |  |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -44.26% | -41.84% | -37.61% | -53.40% | -46.46% | -34.85% | -48.93% | -39.86% |  |  |
| Class H2 |  |  |  |  |  | -31.84% | -66.00% | -60.17% |  |  |
| Overall | -44.26% | -41.84% | -37.61% | -53.40% | -46.46% | -33.76% | -55.14% | -47.24% |  |  |

## Study and Evaluate HDR/WCG Content

The HLG content in the CTC was updated during the October meeting to include type-2 versions of the HLG content that was provided in JVET-S0218.

During the interim period, a chroma artifact was identified in two sequences provided in JVET-S0218 that are not in the HDR CTC but are being considered for the verification test.  These chroma artifacts had not appeared in the original, type-0 versions.  The proponents of JVET-S0218 graciously regenerated the type-2 versions of the two sequences from the original content.  And, the chroma artifacts did not appear in the regenerated content.  Thus, the working conclusion is that there had been some issue during the file conversion.

Several sequences from JVET-S0218 are also used in the HDR CTC.  Due to the previously described chroma artifact, these CTC sequences were studied and visually evaluated for chroma artifacts.  Currently, no visual artifact has been identified.

## Coordinating with AHG4

The AHG had significant coordination with AHG4, including sharing results on the new HDR content and expert viewing procedures.

# Contributions

There are two contributions related to HDR video coding.

| JVET-U0042 | Status Report on HDR Verification Test Preparation | M. Wien, V. Baroncini and A. Segall |
| --- | --- | --- |
| JVET-U0046 | AHG4: Agenda and report on the AHG meeting on the HDR verification test preparation on 2020-12-10 | A. Segall, M. Wien and V. Baroncini |

# Recommendations

The AHG recommends the following:

- Review all input contributions