| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0011-v2 |
| --- | --- |

| Title: | JVET AHG report: Neural Network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Shan Liu Andrew Segall Yan Ye Elena Alshina Jianle Chen Franck Galpin Jonathan Pfaff Shanshe Wang Mathias Wien Ping Wu Jizheng Xu | Email: | shanl@tencent.com asegall@sharplabs.com yan.ye@alibaba-inc.com elena.alshina@huawei.com cjianle@qti.qualcomm.com Franck.Galpin@InterDigital.com firstname.lastname@hhi.fraunhofer.de sswang@pku.edu.cn wien@lfb.rwth-aachen.de pwuzte@gmail.com xujizheng@bytedance.com |
| Source: | AHG 11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural Network-based video coding between the 19th 20th meeting (7 – 16 Oct. 2020) and the 21st meeting (6 – 15 Jan. 2021), both held by teleconference.

# Introduction

The AHG was established with the following mandates:

- Evaluate and quantify performance improvement potential of NN based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures.

- Finalize, conduct and discuss the EE on neural network-based video coding JVET-T2023.

- Solicit input contributions on NN based video coding technologies.

- Continue to refine the test conditions for neural network-based video coding, and develop supporting software as needed.

- Investigate technical aspects specific to NN-based video coding, such as encoding and decoding complexity of neural networks, design network representation, operation, tensor, on-the-fly network adaption (e.g. updating during encoding) etc;

- Study the impact of training on the performance of candidate technology.

- Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology.

- Identify video test materials, training set materials, and testing methods for assessment of the effectiveness and complexity of considered technology.

- Generate and distribute anchor encodings and develop improvements of the JVET-T2006 common test conditions for NNVC technology.

- Particularly consider the suitability of sequences from the YouTube UGC data set for future inclusion in the test set.

- Coordinate with other relevant groups, including SC29/AG5 on visual quality assessment.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email exchange with AHG11 included in the subject lines.  A kick-off message was sent on December 7, 2020, and four additional emails were exchanged on the reflector.  While not official AHG activity, the AHG11 coordinators also observed significant side activity to progress the work of the AHG and the corresponding EE on Neural Network-based video coding.

## Test and Training Material

The AHG made meaningful progress on the mandate to identify training content for the NNVC activity.  102 sequences were identified from the JVET ftp site that are asserted to be available for use by the NNVC activity.  Additionally, another 764 sequences were identified from the BVI-DVC dataset that are asserted to be available for use by the NNVC activity.

To facilitate the study and use of the training data, a git repository was created.  The repository can be accessed at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc, and it includes the URL for each file in the dataset, as well as information about how to extract the training data.  Example screen shots are shown below.  It is anticipated that the git repository will be used for ongoing management of the test and training material.

Figure 1 – Screen shot of the NNVC git repository providing information about the training data and extraction procedures.

Figure 2 – Screen shot of the NNVC git repository providing information about the training data URL and filename.

## Finalize and Conduct EE on NNVC

The EE on NNVC was finalized and conducted during the AHG period.  And, a summary report is provided separately at this meeting as:

| JVET-U0023 | EE Summary Report: Neural Network-based Video Coding | E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma, H. Wang |
| --- | --- | --- |

It is noted that a primary goal of the EE was to test the NNVC Common Test Condition procedure provided in JVET-T2006.  During the evaluation process, at least three issues were identified:

- The NNVC CTC defines that VTM10.0 shall be used for the anchor, while the use of more recent versions of the VTM are encouraged.  However, at the last meeting, it was decided to make non-normative changes to the VTM that were included in the VTM11.0 release.  These non-normative changes include using a larger GOP size and enabling the temporal filter.  The NNVC CTC is unclear if these non-normative changes should be included in the anchor.

- The NNVC CTC requests reporting of MS-SSIM using HDRTools.  This currently requires storing the decoded YUV file and then generating the MS-SSIM data in a separate processing pass.  This did not directly fit into many of the proponents workflows.

- Additionally, while the NNVC requests reporting MS-SSIM using HDRTools, VTM11.0 now supports the calculation of MS-SSIM directly.  However, the MS-SSIM calculation used in VTM11.0 is not the same as what is used in HDRTools.  This created additional confusion in the EE process.

As a results of these issues, it is noted that the simulations reported in the EE contain a mixture of GOP size configurations (16 vs 32).  And, they do not report MS-SSIM data.  To be clear, this is explicitly captured in the summary report.  And, it is asserted to still allow for comparison and evaluation of the studied methods.

Using the obsersations of the EE process, it is recommended to update the NNVC Common Test Conditions at the January meeting to:

- Clarify that VTM11.0 (or later) should be used as the anchor

- Clarify that the GOP size should be 32

- Clarify that temporal filtering should be enabled

- Change the MS-SSIM calculation to use the algorithm provided in VTM11.0

Additionally, volunteers are requested to create a stand alone implementation of the MS-SSIM algorithm that appears in VTM11.0 to better support proposals considering end-to-end solutions (and, thus, not using the VTM11.0 code base.)

## Anchor Encoding

Due to the three issues identified in the previous sub-section, the generation and cross-check of anchor encodings was delayed.  However, the AHG was ultimately able to complete the anchor generation and cross-check.  The data for VTM10.0 (GOP Size = 16) and VTM10.0 (GOP Size = 32) are attached to this report.

As part of the anchor encoding process, the AHG was also able to study the impact of the GOP size change and temporal filter changes incorporated into VTM11.0.  This is summarized in the two tables below.  In the first table, the VTM10.0 (GOP Size=16) anchor is compared to the VTM10.0 (GOP Size = 32) anchor.  As can be seen in Table 1, the increase in GOP size results in an average gain of approximately 3.8% for the Random Access configuration.

|  | Random access Main10 (QP22,27,32,37) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -1.94% | -4.82% | -5.04% | -2.60% | -7.34% | -9.57% | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | -3.32% | -6.67% | -5.88% | -2.35% | -6.41% | -5.93% | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -4.35% | -7.77% | -7.97% | -5.34% | -8.30% | -8.48% | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | -5.08% | -7.02% | -7.12% | -5.77% | -7.13% | -7.25% | #DIV/0! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -3.85% | -6.76% | -6.74% | -4.31% | -7.41% | -7.86% | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -4.50% | -6.22% | -6.65% | -5.28% | -6.37% | -7.09% | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | -11.36% | -11.56% | -11.36% | -11.30% | -15.58% | -13.45% | #DIV/0! | #DIV/0! | #DIV/0! |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 1 – Comparison of VTM10.0 (GOP Size =32) to VTM10.0 (GOP Size = 16).  As can be seen from the Table, the change in GOP Size results in approximately a 3.8% improvement in BD-rate for the random access configuration.

Additionally, it was noted that the comparison of VTM-10.0 vs VTM-11.0 was shared on the JVET reflector on December 1, 2020, and re-produced in Table 2.  As can be seen in the Table, VTM-11.0 provides approximately 7.9% improvement in BD-rate for the random access configuration.  Since the major changes between VTM-10.0 and VTM-11.0 are (i) an increase in GOP Size to 32 and (ii) enabling the temporal filter, the following was concluded:

- Approximately 3.8% of the gain can be attributed to the change in GOP Size.

- The remainder (approximately 4%) of the gain can likely be attributed to enabling the temporal filter.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.83% | -11.43% | -10.74% | 93% | 101% |
| Class A2 | -8.31% | -14.93% | -14.18% | 92% | 99% |
| Class B | -10.53% | -15.03% | -15.35% | 96% | 99% |
| Class C | -6.72% | -10.55% | -10.40% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -7.93% | -13.09% | -12.87% | 96% | 99% |
| Class D | -3.88% | -9.44% | -9.79% | 100% | 97% |
| Class F | -11.36% | -11.57% | -11.35% | 99% | 99% |

Table 2 – Comparison of VTM11.0 (Temporal Filter Enabled and GOP Size = 32) to VTM10.0 (Temporal Filter Disabled and GOP Size = 16).  As can be seen from the Table, VTM11.0 results in approximately a 7.9% improvement in BD-rate for the random access configuration.

## Software Support

An implementation of MS-SSIM was ported to the VTM to better support the NNVC activity.  The commit id is 18f28ecf5c4de1548612f858284606e57905b797, and the implementation is reported to correspond to the MS-SSIM implementation available in HM-16.19.

## Evaluation of YouTube UGC Content

The AHG performed further evaluation of the YouTube User Generated Content (UGC) dataset to determine its suitability for future inclusion in the test (or training) set.  The dataset is available at https://media.withyoutube.com and contains 1380 sequences, each with a duration of 20 seconds.  The sequences in the dataset are asserted “to be distributed under the Creative Commons license”, according to the website description.  The dataset is divided into 15 categories on content type, as shown below.

Figure 3 – Example clips corresponding to each category in the YouTube UGC dataset.

The dataset also includes Mean Opinion Scores for all of the video clips.  Each sequence is reported to be rated by 100+ subjects using crowsourcing methods.  These scores are reported for each complete sequence, as well as 10 second sub-sections of the sequence.

The AHG studied the MOS scores for the full sequences, and the distribution for the dataset is as follows:

Table 3 – Histogram of MOS scores in the YouTube UGC data set.  Each MOS scores correspond to a full, 20s sequence in the dataset.

It is observed that approximately 185 of the sequences in the dataset have a MOS score of 4.3 or larger, which is anticipated to correspond to a high quality video suitable for the NNVC activity.

A study of the distribution of these potential high quality videos is shown in Table 4.  As can be seen from the Table, a larger number of the high quality videos correspond to the Gaming and Sports categories.

Table 4 – Histogram of the categories of sequences in the YouTube UGC data set with a MOS score greater than or equal to 4.3, which is asserted to be high quality.

It is anticipated that the above information will be useful for future discussion of the NNVC CTC during the January meeting.

# Input contributions

There are 18 input contriubtions related to the AHG mandates.  Eight of the contributions are directly relatd to the EE activity, while the remaining 10 contributions are related to AHG11 but not directly part of the EE.  (As a reminder, the purpose of the EE was mainly to evaluate the NNVC CTC, and so there is not an asserted difference in status of the doucments.)  The list of input contributions is provided below.

## EE Related Input Contributions

| JVET-U0023 | EE Summary Report: Neural Network-based Video Coding | E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma, H. Wang |
| --- | --- | --- |
| JVET-U0054 | EE: Neural network based in-loop filtering | Z. Wang, R.-L. Liao, C.Y. Ma, Y. Ye (Alibaba) |
| JVET-U0060 | EE-1.1: A comparison of depthwise separable convolution and regular convolution with the JVET-T0057 neural network based in-loop filter | C. Auyeung, W. Wang, W. Jiang, X. Li, S. Liu (Tencent) |
| JVET-U0061 | EE-1.1-related: BD-Rate improvements to JVET-T0057 neural network based in-loop filter using depthwise separable convolution and regular convolution | C. Auyeung, X. Li, S. Liu (Tencent) |
| JVET-U0074 | EE: SSIM based CNN model for in-loop filtering | T. Ouyang, H. Zhu, Z. Chen (Wuhan Unvi.), X. Xu, S. Liu (Tencent) |
| JVET-U0094 | EE: Tests on Neural Network-based In-Loop Filter | H. Wang, M. Karczewicz, J. Chen, A. M. Kotra (Qualcomm) |
| JVET-U0096 | EE: Tests on Decomposition, Compression and Synthesis (DCS)-based Technology | M. Lu, Z. Ma (Nanjing Univ.), L. Xu, D. Wang (OPPO) |
| JVET-U0101 | EE-2.1.5: In-loop filtering based on neural network | W. Chen, X. Xiu, Y.-W. Chen, H.-J. Jhu, C.-W. Kuo, X. Wang (Kwai) |

## Non-EE Input Contributions

| Loop Filtering |  |  |
| --- | --- | --- |
| JVET-U0055 | AHG11: Multi-density network for in-loop filtering | Z. Wang, R.-L. Liao, C.Y. Ma, Y. Ye (Alibaba) |
| JVET-U0068 | AHG11: Convolutional Neural Network-based In-Loop Filter with Adaptive Model Selection | Y. Li, L. Zhang, K. Zhang (Bytedance) |
| JVET-U0077 | AHG11: Revisiting SAO in-loop filter with Neural Networks | P. Bordes, F. Galpin, T. Dumas, P. Nikitin (Interdigital) |
| JVET-U0104 | AHG11: In-loop filtering with convolutional neural network and large activation | J. Chen, H. Wang, A. M. Kotra, M. Karczewicz (Qualcomm) |
| JVET-U0115 | AHG11: Neural Network-based In-Loop Filter Performance with No Deblocking Filtering stage | H. Wang, J. Chen, A. M. Kotra, M. Karczewicz (Qualcomm) |
| Super Resolution |  |  |
| JVET-U0053 | AHG9/AHG11: Level information for super-resolution neural network | T. Chujoh, E. Sasaki, T. Suzuki, T. Ikai (Sharp) |
| JVET-U0099 | AHG11: Neural Network-based Super Resolution | A. M. Kotra, K. Reuzé, J. Chen, H. Wang, M. Karczewicz, J. Li (Qualcomm) |
| Inter Prediction |  |  |
| JVET-U0087 | AHG11: Updated information on inter-prediction coding tool with deep neural network | Z. Li, B. Choi, W. Wang, W. Jiang, X. Xu, S. Liu (Tencent) |
| Intra Prediction |  |  |
| JVET-U0105 | AHG11-related: Investigation on CNN-based Intra Prediction | M. Meyer, C. Rohlfing (RWTH Aachen Univ.) |
| End-to-End |  |  |
| JVET-U0102 | AHG11: Variable rate end-to-end image compression | C. Lin, F. Chen, L. Wang (Hikvision) |
| High Level Syntax |  |  |
| JVET-U0091 | AHG9/AHG11: SEI message for carriage of neural network information for post filtering | B. Choi, Z. Li, W. Wang, W. Jiang, X. Xu, S. Wenger, S. Liu (Tencent) |
| Test Procedures and Content |  |  |
| JVET-U0116 | A video dataset for training in neural network based video coding | X. Xu, S. Liu, R. Yao, L. Wang, S. Tian (Tencent), D. Wu (Shenzhen Boyan Technology Ltd.), Y. Hu, J. Li, J. Xia, W. Qi, J. Zhang, J. Wen (Tsinghua University), |

# Recommendations

The AHG recommends:

- Review all input contributions

- Release an updated version of the NNVC CTC clarifying the GOP size and temporal filter configuration for the anchor.

- Release an update version of the NNVC CTC modifying the MS-SSIM calculation procedure to use the method available in VTM11.0.

- Further discussion and definition of common test conditions and exploration experiments as appropriate

- Continue investigating neural-network-based video coding tools, including coding performance and complexity;