| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0022 |
| --- | --- |

| Title: | CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomonori Hashimoto Hong-Jheng Jhu Dmytro Rusanovskyy | Email: | adrian.browne@sony.com tomonori.hashimoto@sharp.co.jp jhuhong-jheng@kwai.com dmytror@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Core Experiment on entropy coding for high bit depth and high bit rate coding. A total of 14 tests have been completed within the CE between the JVET T and U meetings to study and evaluate technologies related to Rice parameter derivation. Both regular residual coding (RRC) (5 tests) and transform skip residual coding (TSRC) (3 tests) have been tested. In addition 6 tests have been completed in a second stage on combinations of the previous tests.

In this report, coding performance and complexity are reported and analyzed. In particular, test results against VTM-11.0rc1 anchors are provided to show the coding efficiency and complexity trade-off of each tool. Where appropriate results are also provided for lossless conditions. Crosschecking results for the performed tests are submitted as separate documents and their summaries are integrated in this contribution.

# Test specification

## CE-1 (RRC)

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE-1.1 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0064 | On the Rice parameter derivation for high bit-depth coding | Adrian Browne (Sony) |
| CE-1.2 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0064 | On the Rice parameter derivation for high bit-depth coding | Adrian Browne (Sony) |
| CE-1.3 | Tomonori Hashimoto (Sharp) | JVET-U0050 | Rice parameter derivation by using a formula for RRC | Hong-Jheng Jhu (Kwai) |
| CE-1.4 | Adrian Browne (Sony) | JVET-U0057 | Modification of Rice parameter selection for high bit depths using the value of previously coded coefficients within the slice. | Dmytro Rusanovskyy (Qualcomm) |
| CE-1.5 | Adrian Browne (Sony) | JVET-U0057 | Simplification of CE-1.4 | Dmytro Rusanovskyy (Qualcomm) |

## CE-2 (TSRC)

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE-2.1 | Hong-Jheng Jhu (Kwai) | JVET-U0075 | Slice based Rice parameter selection for transform skip residual coding | Tomonori Hashimoto (Sharp) |
| CE-2.2 | Adrian Browne (Sony) | JVET-U0057 | Rice parameter selection using the value of previously coded coefficients both locally (within the TB) and globally (within the slice). | Hong-Jheng Jhu (Kwai) |
| CE-2.3 | Adrian Browne (Sony) | JVET-U0057 | Rice parameter selection using the value of previously coded coefficients within the slice. Simplification of CE-2.2 | Hong-Jheng Jhu (Kwai) |

## CE-3 (Combinations)

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE-3.1 | Tomonori Hashimoto (Sharp) | JVET-U0050 | Combination of CE-1.3 and CE-2.1 | Kenji Kondo (Sony) |
| CE-3.2 | Adrian Browne (Sony) | JVET-U0058 | Combination of CE-1.5 and CE-2.1 | Tomonori Hashimoto (Sharp) |
| CE-3.3 | Adrian Browne (Sony) | JVET-U0059 | Combination of CE-1.5 and CE-2.3 | Dmytro Rusanovskyy (Qualcomm) |
| CE-3.4 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0065 | Combination of CE-1.12 and CE-2.1 | Adrian Browne (Sony) |
| CE-3.5 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0066 | Combination of CE-1.2 and CE-1.4/1.5 | Hong-Jheng Jhu (Kwai) |
| CE-3.6 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0066 | Combination of CE-1.3 and CE-3.5 | Hong-Jheng Jhu (Kwai) |

# Test descriptions

## CE-1.1: Method of JVET-T0105 without locally adaptivity

In this test, method proposed in JVET-T0105 with globally derived adjusment for rice parameter derivation is being evaluated. To extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a right shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value.

Adjustment parameters are determined from signalled syntax elemments, without local adaptation within a block (no dependency on locSumAbs). Two subtests were planned:

- Test CE-1.1.a: SPS-level control information is used to derive adjustment parameters.

- Test CE-1.1.b: Syntax elements controlling adjustment parameters are signalled at the sub-SPS levels, e.g. at slice level.

During CE development, CE-1.1.a test was omitted. Only sub-test with slice level signaling of the derivation parameter was conducted. Detailed description of this method and specification text are provided in JVET-U0064.

## CE-1.2: Method of JVET-T0105 with local adaptivity

In this test, method proposed in JVET-T0105 with locally adaptive adjusment for Rice parameter derivation is being evaluated. Similarly to the Test CE-1.1, to extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value. A shift parameter for locSumAbs scaling and cRiceParam increment is derived at the encoder side and signalled at the slice header. Motivated by the history usage of CE-1.4 and CE-1.5, additional sub-test was conducted (CE-1.2.b) targeting improving accuracy of the Rice derivation for coeffitients with no accract template information available. Detailed description of this method and specification text are provided in JVET-U0064.

## CE-1.3: JVET-U0050

The proposal of JVET-U0050 introduces a modification to the VVC Rice parameters derivation method for regular residual coding (RRC). It is proposed to use a formula instead of conventional look-up table. Specifically, the rice parameter value is predicted based on an adjusted value of the locSumAbs using linear prediction with log2 operation. The linear prediction calicurate floorLog2(a * locSumAbs + b) + c. The linear prediction parameter (a, b and c) depend on syntax (i.e. abs_reminder or dec_abs_level). An enablied flag for the proposed method is signalled in SPS. This flag doesn’t affect the process if bitdepth is equal or less than 10.

## CE-1.4: RRC technique in JVET-U0057

The RRC component of the method previously proposed in JVET-T0072. For this experiment the TSRC component of the modification described in JVET-T0072 is disabled and standard VVC TSRC is used.

The implementation has been improved from that in JVET-T0072 by computing the parameters for updating counters and deriving Rice parameters at the point the counters are updated. In addition, the counters and associated parameters are stored within the structure used for CABAC contexts (as proposed but not implemented in JVET-T0072). Finally a fix has been implemented to automatically adjust the Rice parameter selected based on whether a parity bit has been coded.

## CE-1.5: RRC simplification in JVET-U0057

The RRC component of the simplification previously proposed in JVET-T0072. For this experiment the TSRC component of the modification described in JVET-T0072 is disabled and standard VVC TSRC is used.

This experiment simplifies CE-1.4 as the parameters for updating counters and deriving Rice parameters are computed at the end of each transform block. In addition, the counters are only updated when the coefficient being coded is on the leading diagonal of the transform block.

## CE-2.1: TSRC technique in JVET-U0075

Method proposed in JVET-U0075, it is proposed to explicitly signal the Rice parameter for each slice to indicate the Rice parameter for the binary codewords of abs_remainder. In the proposed method, one control flag is signaled in sequence parameter set to indicate the signaling of Rice parameter for the transform skip blocks is enabled or disabled. When the control flag is signaled as enabled, one syntax element is further signaled for each transform skip slice to indicate the Rice parameter of that slice. When the control flag is signaled as disabled (e.g. set equal to “0”), no further syntax element is signaled at lower level to indicate the Rice parameter for the transform skip slice and a default Rice parameter (e.g. 1) is used for all the transform skip slice. Detailed description of this method and specification text are provided in JVET-U0075.

## CE-2.2: TSRC technique in JVET-U0057

The TSRC component of the method previously proposed in JVET-T0072. For this experiment the RRC component of the modification described in JVET-T0072 is disabled and standard VVC RRC is used.

The implementation has been improved from that in JVET-T0072 by computing the parameters for updating counters and deriving Rice parameters at the point the counters are updated. In addition, the counters and associated parameters are stored within the structure used for CABAC contexts (as proposed but not implemented in JVET-T0072). Finally a fix has been implemented to automatically adjust the Rice parameter selected based on whether a parity bit has been coded.

## CE-2.3: TSRC simplification in JVET-U0057

The TSRC component of the simplification previously proposed in JVET-T0072. In addition, this experiment replaces the usage of locSumAbs and g_auiGoRiceParsCoeff with an offset derived from the value of the selected counter. For this experiment the RRC component of the modification described in JVET-T0072 is disabled and standard VVC RRC is used.

This experiment simplifies CE-1.4 as the parameters for updating counters and deriving Rice parameters are computed at the end of each transform block. In addition, the counters are only updated when the coefficient being coded is on the leading diagonal of the transform block.

## CE-3.1: Combination of CE-1.3 and CE-2.1

This experiment combines the RRC coding modifications from CE-1.3 with the TSRC coding modifications in CE-2.1 and is described in JVET-U0050.

## CE-3.2: Combination of CE-1.5 and CE-2.1

This experiment combines the RRC coding modifications from CE-1.5 with the TSRC coding modifications in CE-2.1 and is described in JVET-U0058.

## CE-3.3: Combination of CE-1.5 and CE-2.3

This experiment combines the RRC coding modifications from CE-1.5 with the TSRC coding modifications in CE-2.3 and is described in JVET-U0059.

## CE-3.4: Combination of CE-1.2 and CE-2.1

This experiment will blend the RRC coding modifications from CE-1.2.a with the TSRC coding modifications in CE-2.1. Detailed description of this method and specification text are provided in JVET-U0065.

## CE-3.5: Combination of CE-1.2 and CE-1.4/1.5

This experiment will investigate integration of the classification ellements of CE-1.4/1.5 with rice parameter inheritance mechanism and the Rice parameter derivation of the CE-1.2.b. Detailed description of this method and specification text are provided in JVET-U0066.

## CE-3.6: Combination of CE-1.3 and CE-3.5

This experiment will investigate integration of Rice parameter derivation method of CE1.3 with with Rice parameter inheritance mechanism of the CE1.2.b modified with classification ellements of CE-1.4/1.5. Detailed description of this method and specification text are provided in JVET-U0066.

# Simulation results

| AI | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | R | G | B | Enc | Dec |
| CE-1.1 | -0.60% | -0.67% | -0.67% | 82% | 90% | -0.22% | -0.24% | -0.24% | 83% | 87% | -13.86% | -13.23% | -13.23% | 90% | 83% |
| CE-1.2a | -0.76% | -0.79% | -0.79% | 92% | 83% | -0.31% | -0.31% | -0.32% | 90% | 82% | -14.54% | -13.82% | -13.82% | 97% | 91% |
| CE-1.2b | -0.76% | -0.79% | -0.79% | 92% | 83% | -0.31% | -0.31% | -0.32% | 90% | 82% | -14.79% | -13.97% | -13.98% | 98% | 91% |
| CE-1.3 | -0.92% | -0.91% | -0.90% | 100% | 96% | -0.50% | -0.41% | -0.41% | 102% | 99% | -14.33% | -13.76% | -13.78% | 102% | 87% |
| CE-1.4 | -0.94% | -0.89% | -0.91% | 104% | 100% | -0.60% | -0.45% | -0.45% | 103% | 101% | -16.00% | -14.70% | -14.71% | 107% | 106% |
| CE-1.5 | -0.92% | -0.85% | -0.85% | 102% | 100% | -0.63% | -0.47% | -0.46% | 102% | 102% | -15.96% | -14.57% | -14.60% | 107% | 110% |
| CE-2.1 | -0.12% | -0.09% | -0.08% | 99% | 97% | -0.10% | -0.05% | -0.05% | 100% | 100% | -11.84% | -11.48% | -11.43% | 101% | 96% |
| CE-2.2 | -0.18% | -0.20% | -0.19% | 102% | 102% | -0.07% | -0.05% | -0.05% | 101% | 101% | -11.99% | -11.58% | -11.53% | 106% | 104% |
| CE-2.3 | -0.15% | -0.14% | -0.13% | 101% | 100% | -0.17% | -0.12% | -0.12% | 100% | 100% | -8.74% | -8.30% | -8.26% | 103% | 99% |
| CE-3.1 | -0.92% | -0.87% | -0.86% | 108% | 103% | -0.54% | -0.43% | -0.43% | 104% | 100% | -14.62% | -13.95% | -13.94% | 106% | 90% |
| CE-3.2 | -0.92% | -0.81% | -0.81% | 103% | 100% | -0.63% | -0.47% | -0.46% | 102% | 101% | -15.54% | -14.39% | -14.37% | 105% | 106% |
| CE-3.3 | -0.92% | -0.84% | -0.84% | 102% | 100% | -0.67% | -0.51% | -0.51% | 101% | 102% | -15.96% | -14.60% | -14.61% | 107% | 110% |
| CE-3.4 | -0.75% | -0.74% | -0.75% | 88% | 99% | -0.36% | -0.33% | -0.33% | 85% | 96% | -14.66% | -13.95% | -13.93% | 95% | 89% |
| CE-3.5 | -0.95% | -0.87% | -0.88% | 105% | 104% | -0.46% | -0.32% | -0.32% | 104% | 102% | -15.88% | -14.77% | -14.77% | 98% | 97% |
| CE-3.6 | -1.05% | -0.92% | -0.91% | 103% | 101% | -0.56% | -0.37% | -0.37% | 103% | 101% | -15.76% | -14.79% | -14.80% | 115% | 96% |

| LDB | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | R | G | B | Enc | Dec |
| CE-1.1 | -0.05% | -0.06% | -0.07% | 99% | 95% | -0.03% | -0.04% | -0.04% | 94% | 101% | -8.43% | -7.79% | -7.92% | 101% | 94% |
| CE-1.2a | -0.03% | -0.04% | -0.04% | 89% | 89% | 0.03% | 0.02% | 0.03% | 99% | 99% | -8.85% | -8.37% | -8.42% | 103% | 100% |
| CE-1.2b | -0.03% | -0.04% | -0.04% | 89% | 89% | 0.03% | 0.02% | 0.03% | 99% | 99% | -8.94% | -8.57% | -8.58% | 114% | 106% |
| CE-1.3 | 0.07% | 0.08% | 0.09% | 89% | 86% | -0.03% | 0.01% | 0.01% | 101% | 99% | -8.86% | -8.15% | -8.31% | 104% | 94% |
| CE-1.4 | -0.09% | -0.10% | -0.13% | 103% | 101% | -0.08% | -0.08% | -0.07% | 103% | 102% | -9.38% | -8.96% | -8.97% | 107% | 109% |
| CE-1.5 | -0.09% | -0.10% | -0.12% | 101% | 100% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.40% | -8.95% | -8.96% | 104% | 108% |
| CE-2.1 | 0.01% | -0.01% | 0.01% | 98% | 100% | 0.00% | -0.01% | -0.01% | 99% | 100% | -7.19% | -6.91% | -6.89% | 99% | 100% |
| CE-2.2 | -0.02% | -0.03% | -0.06% | 102% | 101% | 0.00% | -0.01% | -0.01% | 102% | 100% | -7.10% | -6.79% | -6.78% | 101% | 104% |
| CE-2.3 | -0.01% | -0.01% | -0.01% | 100% | 100% | 0.00% | -0.01% | -0.01% | 101% | 100% | -4.65% | -4.37% | -4.37% | 100% | 99% |
| CE-3.1 | 0.06% | 0.08% | 0.10% | 105% | 104% | -0.03% | 0.01% | 0.01% | 104% | 102% | -8.99% | -8.60% | -8.62% | 109% | 97% |
| CE-3.2 | -0.07% | -0.09% | -0.12% | 101% | 104% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.43% | -8.96% | -8.96% | 103% | 107% |
| CE-3.3 | -0.10% | -0.10% | -0.11% | 101% | 101% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.43% | -8.94% | -8.95% | 104% | 109% |
| CE-3.4 | -0.03% | -0.02% | -0.03% | 88% | 89% | 0.03% | 0.03% | 0.03% | 86% | 93% | -8.97% | -8.59% | -8.60% | 105% | 96% |
| CE-3.5 | -0.03% | -0.05% | -0.06% | 101% | 100% | 0.05% | 0.06% | 0.06% | 103% | 105% | -9.53% | -9.14% | -9.15% | 102% | 107% |
| CE-3.6 | 0.10% | 0.12% | 0.12% | 99% | 97% | 0.01% | 0.05% | 0.05% | 108% | 110% | -9.46% | -9.10% | -9.12% | 127% | 115% |

| RA | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | R | G | B | Enc | Dec |
| CE-1.1 | -0.12% | -0.14% | -0.16% | 89% | 95% |  |  |  |  |  | -7.90% | -7.98% | -8.00% | 91% | 87% |
| CE-1.2a | -0.13% | -0.14% | -0.14% | 77% | 78% | 0.00% | 0.00% | 0.00% | 96% | 103% | -8.41% | -8.49% | -8.47% | 85% | 85% |
| CE-1.2b | -0.13% | -0.14% | -0.14% | 77% | 78% | 0.00% | 0.00% | 0.00% | 96% | 103% | -8.56% | -8.62% | -8.60% | 91% | 90% |
| CE-1.3 | -0.05% | -0.01% | 0.01% | 100% | 102% | -0.04% | -0.01% | 0.00% | 109% | 108% | -8.30% | -8.33% | -8.37% | 98% | 88% |
| CE-1.4 | -0.20% | -0.19% | -0.21% | 103% | 102% | -0.09% | -0.08% | -0.07% | 103% | 101% | -9.00% | -8.99% | -8.97% | 107% | 108% |
| CE-1.5 | -0.19% | -0.17% | -0.20% | 102% | 101% | -0.10% | -0.07% | -0.07% | 101% | 101% | -9.06% | -9.01% | -8.98% | 103% | 107% |
| CE-2.1 | 0.00% | -0.01% | 0.00% | 99% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% | -6.75% | -7.04% | -6.99% | 100% | 100% |
| CE-2.2 | -0.05% | -0.04% | -0.05% | 102% | 101% | 0.00% | 0.00% | 0.00% | 101% | 101% | -6.67% | -6.95% | -6.90% | 101% | 103% |
| CE-2.3 | -0.02% | -0.02% | -0.02% | 101% | 100% | 0.00% | -0.01% | 0.00% | 100% | 100% | -4.10% | -4.37% | -4.34% | 100% | 99% |
| CE-3.1 | -0.03% | 0.00% | 0.01% | 105% | 125% | -0.04% | -0.01% | 0.00% | 100% | 89% | -8.57% | -8.57% | -8.56% | 106% | 94% |
| CE-3.2 | -0.18% | -0.17% | -0.19% | 102% | 104% | -0.09% | -0.07% | -0.06% | 97% | 100% | -9.04% | -9.00% | -8.96% | 104% | 106% |
| CE-3.3 | -0.19% | -0.18% | -0.19% | 102% | 101% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.09% | -9.00% | -8.97% | 104% | 107% |
| CE-3.4 | -0.14% | -0.12% | -0.14% | 86% | 93% | 0.00% | 0.00% | 0.01% | 105% | 109% | -8.56% | -8.62% | -8.59% | 88% | 88% |
| CE-3.5 | -0.17% | -0.16% | -0.16% | 104% | 103% | 0.02% | 0.03% | 0.03% | 110% | 113% | -9.15% | -9.18% | -9.15% | 95% | 98% |
| CE-3.6 | -0.01% | 0.03% | 0.04% | 107% | 107% |  |  |  |  |  | -9.05% | -9.08% | -9.06% | 110% | 97% |

# Crosschecker report

| Test | Crosschecker | Crosscheck report |
| --- | --- | --- |
| CE-1.1 | Sony | results: Cross-check results match proponent’s with neglible discrepancies at QP=-13 and below code verification: Consistent with CE description. draft text verification: further comments: Encoder / decoder run-times did not match |
| CE-1.2 | Sony | results: Three small discrepancies in SVT LDB (2 in subtest a, 1 in subtest b) code verification: Consistent with CE description. draft text verification: further comments: Encoder / decoder run-times did not match |
| CE-1.3 | Kwai | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-1.4 | Qualcomm | results: Preliminary results match proponent’s. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-1.5 | Qualcomm | results: Preliminary results match proponent’s. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-2.1 | Sharp | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-2.2 | Kwai | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-2.3 | Kwai | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-3.1 | Sony | results: code verification: draft text verification: further comments: |
| CE-3.2 | Sharp | results: code verification: draft text verification: further comments: a |
| CE-3.3 | Qualcomm | results: Preliminary results match proponent’s. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-3.4 | Sony | results: Cross-check results completely match proponent’s code verification: Consistent with CE description. draft text verification: further comments: Encoder / decoder run-times did not match |
| CE-3.5 | Kwai |  |
| CE-3.6 | Kwai |  |

# Complexity analysis and memory requirements

## CE-1

| Test | Complexity | Memory requirement |
| --- | --- | --- |
| CE-1.1 | 2 conditions, 1 shifts, 1 adds per Rice call 32/16/16 per 4x4 TU | 4 storages, 32 bits in ROM/RAM. |
| CE-1.2.a | 1 condition, 2 shifts, 1 add, 1 MSB find. 16/32/16/16 per 4x4 TU, respectively. | No memory increase. |
| CE-1.2.b | 10 conditions, 31 shifts, 12 adds, 16 MSB finds per 4x4 TU. | 12 bits of storage for history counters. |
| CE-1.3 | Use floorLog2 calculation to derive rice parameter. 1 condition, 1 mult (1shift+1add), 2 adds, floorLog2 (1 clz, 1 add) per Rice call | No memory increase. |
| CE-1.4 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (per coeff): 3 conditionals, 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditional, 1 fixed shift, 1 add For updating associated parameters (per coeff): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 80 conditionals, 48 variable shifts, 80 fixed shifts, 160 adds | Eight stores each comprising 1*16 bit, 2 * 32 bit, 2*8 bit unsigned values |
| CE-1.5 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (4 per block): 3 conditionals, 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditional, 1 fixed shift, 1 add For updating associated parameters (4 per block): 6 adds, 3 fixed shifts, 2 variable shifts For selecting a parameter set (per coeff): Total per 4x4 TU: 44 conditionals, 40 variable shifts, 28 fixed shifts, 64 adds | Eight stores, each comprising a single 16 bit unsigned value. |

## CE-2

| Test | Complexity | Memory requirement |
| --- | --- | --- |
| CE-2.1 | 1 condition and 1 add per slice. | No memory increase. |
| CE-2.2 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (per coeff): 3 conditionals, 3 adds, 1 fixed shift For updating associated parameters (per coeff): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 48 conditionals, 48 variable shifts, 64 fixed shifts, 160 adds | Two stores each comprising 1*16 bit, 2 * 32 bit, 2*8 bit unsigned values |
| CE-2.3 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (4 per block): 3 conditionals, 3 adds, 1 fixed shift For updating associated parameters (1 per block): 6 adds, 3 fixed shifts, 2 variable shifts For selecting a parameter set (per coeff): Total per 4x4 TU: 12 conditionals, 18 variable shifts, 7 fixed shifts, 34 adds | Two stores, each comprising a single 16 bit unsigned value. |

## CE-3

| Test | Complexity | Memory requirement |
| --- | --- | --- |
| CE-3.1 | Use floorLog2 calculation to derive rice parameter. 1 condition, 1 mult (1shift+1add), 2 adds, floorLog2 (1 clz, 1 add) per Rice call | No memory increase. |
| CE-3.2 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (per coeff): 3 conditionals (max), 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditional, 1 fixed shift, 1 add For updating associated parameters (per coeff): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 80 conditionals, 48 variable shifts, 80 fixed shifts, 160 adds | Eight stores each comprising 1*16 bit, 2 * 32 bit, 2*8 bit unsigned values |
| CE-3.3 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (4 per block): 3 conditionals (max), 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditional, 1 fixed shift, 1 add For updating associated parameters (4 per block): 6 adds, 3 fixed shifts, 2 variable shifts For selecting a parameter set (per coeff): Total per 4x4 TU: 44 conditionals, 40 variable shifts, 28 fixed shifts, 64 adds | Eight stores, each comprising a single 16 bit unsigned value. |
| CE-3.4 | 1 condition, 2 shift, 1 add, 1 MSB find. 16/32/16/16 per 4x4 TU. | No memory increase. |
| CE-3.5 | Number of operations per 4x4 TU , condition/shift/adds/MSB find: CE-1.2.a: 16/32/16/16 History update: 1/1/2/1 History usage: 0/1/20/0 Total number: 17/34/38/17 | 12 bits of storage for history counters. |
| CE-3.6 | CE-1.3: … History update: 1/1/2/1 History usage: 0/1/20/0 | 12 bits of storage for history counters. |

# Input contributions for CE

## CE contributions

- JVET-U0050, “CE-1.3 and CE-3.1: Rice parameter derivation for high bit-depth coding”, T. Hashimoto, T. Ikai (Sharp)

- JVET-U0057, “CE-1.4, CE-1.5, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

- JVET-U0058, “CE-3.2: Combination of CE-1.5 and CE-2.1”, A. Browne, S. Keating, K. Sharman (Sony)

- JVET-U0059, “CE-3.3: Combination of CE-1.5 and CE-2.3”, A. Browne, S. Keating, K. Sharman (Sony)

- JVET-U0064, “CE-1.1 and CE-1.2: On the Rice parameter derivation for high bit-depth coding”, D.Rusanovskyy, L. P. Van, M. Karczewicz (Qualcomm)

- JVET-U0065, “CE-3.4: Combination of CE-1.2 and CE-2.1”, D.Rusanovskyy, L. P. Van, M. Karczewicz (Qualcomm)

- JVET-U0066, “CE-3.5 and CE-3.6: Combination of CE-1.2, CE-1.4/1.5 and CE-1.3”, D.Rusanovskyy, L. P. Van, M. Karczewicz (Qualcomm)

- JVET-U0075, “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang (Kwai Inc.)