| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21th Meeting, by teleconference, 6– 15 Jan. 2021 | Document: JVET-U0023_r1 |
| --- | --- |

| Title: | EE Summary Report: Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H. Wang | Email: | elena.alshina@huawei.com, shanl@tencent.com, hongtaow@qti.qualcomm.com, yue.li@bytedance.com, ruling.lrl@alibaba-inc.com, mazhan@nju.edu.cn, chenwei06@kwai.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document summarizes Exploration Experiments (EEs) have been performed between JVET-T and JVET-U meetings in order to get better understanding of neural network-based video coding technologies, analyze and verify their performance and complexity aspects.

# Introduction

In the 20th JVET meeting it was decided [1] to setup an exploration experiment on NN-based video coding to learn how to assess NN technology, which is the major purpose of the EE activity. The important goal of Exploration Experiment is to exercise test conditions and complexity assessment methodology.

Anchor for this EE supposed to be VTM10.0 [1]. Reason for this is relatively short meeting circle and later for start EE code work after it release of VTM11.0.

Proponent were encouraged to use Results Reporting Template for performance and complexity assessment methodology [2]. Most of proponents follow this recommendation. None of the proponents reported MS-SSIM. Mostly because HDRTools SW, recommended for MS-SSIM computation was not very convenient (reconstructed YUV storage is needed). VTM11.0 comes with MS-SSIM computation integrated, so in a future this metric computation should be easier. One proponent reported VMAF (and it was observed that PSNR gain corresponds in some cases in drop in VMAF metric).

5 EE participants studied CNN based filters as addition to VVC. All used float-point implementation, 32bits float representation of NN parameters. PyTorch or Tensorflow packages needed additionally to VTM SW. Most of proponents provided detailed instructions for SW usage (which far less trivial than just running VTM). Originally build instruction were provided for Linux only (later updated for Windows).

All CNN filters proponents used results reporting template and reported key parameters for complexity assessment.

It looks fair to compare run time between different variants tested by same proponent, but run-time comparison between 2 different proponents looks tricky at current stage, since NN implementations and levels of optimization vary in wide range.

Among CNN filters proposals compression gain up to 5% was demonstrated. In all cases this gain comes with significant decoder complexity (decoder run time increment is 33 … 579).

One EE test can be seen as combination of super-resolution and enchantment filters with almost any base codec (both HEVC and VVC as base codec were tested by proponents). NN VC test conditions do not suit well to assessment such kind of technologies (for getting comparable bit-rate proponents used QP > 42). How to asses technologies of this kind need to be further discussed during meeting.

Due to the short meeting circle several important aspects (such as limited precision of computations inside NN) study was not conducted.

# Summary of experiments in numbers

High level summary of complexity aspects and compression performance demonstrated in EE tests is summarised in Tables below. More details can be found in excel sheet attached to this report.

Using performed experiments following questions regarding CNN-based in-loop filters were be answered and useful information obtained.

- Regular convolution gives ~0.3% compression gain compared to DSC (Depth wise Separable convolution), but leads to decoder run time increment 1.25, also the number of NN parameters by ~ 4. Thanks to Tencent for this study in JVET-U0060.

- SE (squeeze-and-excitation) block contributed ~1% compression gain for models trained with SSIM as part of loss-function. Inclusion SSIM as part of loss-function during training results in PSNR drop (>1% in RA cfg), but shows gain in VMAF metric. Visual quality needs to be studied to explain this phenomenon. Thanks to Tencent for this information in JVET-U0074.

- Nearly 5% compression gain was demonstrated by Qualcomm in JVET-U0094. Default number of hidden convolution layers is 12. If reduced to 6 then performance degrades roughly 0.6…0.8% but decoder run time is reduced by half. Residual scale in NN filter contributes ~0.5% compression gain, w/o decoder run-time implication. Increment number of models (from 4 to 12) with content adaptive selection between those actually leads to performance degradation 0.1…0.4% (possible reasons include signalling overhead, adaptivity introduced by using QP as input, etc.). Same NN based filter placed prior to SAO performs ~1% better compared after ALF. Many thanks to Qualcomm for this extensive study in JVET-U0094.

- 2 ways if Luma and Chroma sizes equalization (Chroma up-sampling and Luma split to 4 planes) have been compared in JVET-U0101. Chroma up-sampling appears to perform ~1% better than Luma split. But in terms of decoding run time (also memory usage) Luma split is roughly twice simpler. With 10 residual blocks 3.4% compression gain in RA cfg can be achieved by NN based filter. With twice more residual blocks (20) complexity doubles and 0.7...0.8% additional compression gain can be achieved. Thanks to Kwai for conducting this study in JVET-U0101.

Table 1. Summary of CNN-based in-loop filters experiments.

|  |  |  |  |  |  |  | Random Access |  |  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposal | Anchor, VTM, GOP size | Total Parameter Num. | Total Conv. Layers | Position | Variations |  | Y-PSNR | U-PSNR | V-PSNR | EncT, | DecT, | Y-PSNR | U-PSNR | V-PSNR | EncT, | DecT, |
| JVET-U0060 | 10.0 | 2.2E+04 | 13 | prior SAO | DSC |  | -1.0% | -3.8% | -3.8% | 1.1 | 35 | -1.3% | -2.8% | -3.6% | 1.1 | 33 |
| (JVET-T0057) | 16 | 7.7E+04 | 13 |  | Regular Conv |  | -1.3% | -5.8% | -5.9% | 1.1 | 44 | -1.6% | -4.5% | -5.4% | 1.1 | 41 |
| JVET-U0074 | 10.0 | 1.3E+05 | 45 | prior SAO | SE block | 0.8 SSIM; 0.2 L1 | -0.8% | -7.9% | -5.9% | 1.4 | 243 | -2.6% | -10.1% | -9.2% | 1.4 | 303 |
| (JVET-T0069) | 16 | 1.3E+05 | 45 |  | SE block | 0.0 SSIM; 1.0 L1 | -2.2% | -10.4% | -8.6% | 1.4 | 288 | -2.9% | -9.3% | -9.1% | 1.4 | 319 |
|  |  | 1.3E+05 | 45 |  | no | 0.8 SSIM; 0.2 L1 | 0.1% | -3.7% | -1.7% | 1.4 | 215 | -1.4% | -5.7% | -5.7% | 1.4 | 267 |
|  |  | 1.3E+05 | 45 |  | no | 0.0 SSIM; 1.0 L1 | -2.0% | -8.3% | -7.6% | 1.4 | 246 | -2.5% | -7.3% | -7.7% | 1.3 | 218 |
| JVET-U0094 | 10.0 | 4.0E+06 | 14 | prior SAO | N=12, | X=4, res. scale | -5.3% | -13.1% | -12.9% | 1.4 | 149 | -4.8% | -10.1% | -10.6% | 1.3 | 85 |
| (JVET-T0079) | 32 | 2.0E+06 | 8 |  | N=6, | X=4, res. scale | -4.6% | -11.8% | -11.6% | 1.2 | 78 | -4.2% | -8.8% | -9.9% | 1.1 | 45 |
|  |  | 4.0E+06 | 14 |  | N=12, | X=4, NO res. scale | -4.8% | -11.7% | -11.9% | 1.4 | 148 | -4.6% | -9.4% | -10.4% | 1.2 | 85 |
|  |  | 1.2E+07 | 14 |  | N=12, | X=12, res. scale | -4.9% | -12.7% | -12.6% | 2.1 | 148 | -4.4% | -9.9% | -10.7% | 1.6 | 85 |
|  |  | 1.2E+07 | 14 | a.ALF | N=12, | X=12, res. scale | -4.0% | -12.1% | -12.3% | 2.1 | 148 | -3.3% | -8.3% | -9.3% | 1.6 | 85 |
| JVET-U0101 | 10.0 | 1.6E+06 | 40 | prior SAO | Ch.Up | ResBlocks 20 | -4.1% | -18.5% | -17.8% | 1.5 | 410 | -5.3% | -16.7% | -17.7% | 1.5 | 404 |
| (JVET-T0094) | 32 | 8.9E+05 | 21 |  | Ch.Up | ResBlocks 10 | -3.4% | -14.2% | -13.9% | 1.2 | 187 | -4.5% | -12.6% | -13.8% | 1.3 | 193 |
|  |  | 1.5E+06 | 40 |  | L.split | ResBlocks 20 | -3.3% | -15.9% | -15.1% | 1.3 | 212 | -4.2% | -14.1% | -14.8% | 1.3 | 224 |
| JVET-U0054 | 10.0 | 9.0E+06 | 66 | prior SAO | Ch.Up | ResBlocks 32 |  |  |  |  |  |  |  |  |  |  |
| (JVET-T0128) | 16 |  |  |  | (*)only classes BCD |  | -5.0% | -15.9% | -17.1% |  |  |  |  |  |  |  |

Useful information group gets out of last EE test is

- Compression gain demonstrated is 8.7% (if VVC is used as base codec) and 24.3% (if HEVC is used as base codec). Thanks to Nanjing University and OPPO for this information in JVET-U0096.

Changes relatively to VTM much more significant for this test, so only Low-delay-P results were provided, no run-time, no Chroma quality metrics.

Table 2. Summary experiment on Decomposition, Compression, Synthesis technology.

| Proposal | Anchor, VTM, GOP size | Total Parameter Num. | Total Conv. Layers | Variations |  | Y-PSNR | U-PSNR | V-PSNR | EncT,  | DecT,  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JVET-U0096 | 10.0 |  |  | vs HEVC |  | -24.3% |  |  | 0.2 |  |
| (JVET-T0125) | 16 |  |  | vs VVC (QP=32,37,42,47) |  | -8.7% |  |  |  |  |

Larger number of NN parameters (more memory for storage and higher computational complexity) results in higher compression gain (as shown on Fig. 1-3).

Fig. 1. Larger number of NN parameters results in higher compression gain.

Fig. 2. RA gain vs. number of conv. layers

Fig. 3. RA gain vs. decoder runtime increment

# List of EE tests

## EE1: NN-based filtering

### JVET-U0060 EE-1.1: A comparison of depthwise separable convolution and regular convolution with the JVET-T0057 neural network based in-loop filter  [C. Auyeung, W. Wang, W. Jiang, X. Li, S. Liu (Tencent)]

Description of the technology.

This test is based on JVET-T0057 AHG11: A case study to reduce computation of a neural network based in-loop filter by pruning [C. Auyeung, W. Wang, W. Jiang, X. Li, S. Liu (Tencent)

SW branch  https://vcgit.hhi.fraunhofer.de/jvet-t-ee1/VVCSoftware_VTM/-/tree/EE-JVET-T0057

SW branch owner Cheung Auyeung cauyeung@tencent.com

Observations from the code:

- Luma Chroma size adjustment  Chroma up-sampling.

- Variables dynamic range  float point.

- Position in pipeline: in-loop (before SAO).

- Number of residual blocks 4 (with 25% CNN parameters set to 0)

Questions recommended to be answered during EE test.

- What is difference in terms of complexity / performance between DCS and regular Convolution?

| Test | All Intra, cfg |  |  |  |  | Random Access, cfg |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc, | Dec, | Y | U | V | Enc, | Dec, |
| DCS | -1.3% | -2.8% | -3.6% | 1.1 | 33 | -1.0% | -3.8% | -3.8% | 1.1 | 35 |
| Regular convolution | -1.6% | -4.5% | -5.4% | 1.1 | 41 | -1.3% | -5.8% | -5.9% | 1.1 | 44 |

### JVET-U0074 EE: SSIM based CNN model for in-loop filtering [T. Ouyang, F. Liu, H. Zhu, Z. Chen (Wuhan Unvi.), X. Xu, S. Liu (Tencent)]

This test is based on JVET-T0069 AHG11: SSIM based CNN model for in-loop filtering [T. Ouyang, F. Liu, H. Zhu, Z. Chen (Wuhan Unvi.), X. Xu, S. Liu (Tencent)]

SW branch  https://vcgit.hhi.fraunhofer.de/jvet-t-ee1/VVCSoftware_VTM/-/tree/EE-JVET-T0069

SW branch owner Xiaozhong Xu xiaozhongxu@tencent.com

# 2 key aspects of this technology is SE (squeeze-and-excitation) block and weighted combination of SSIM and L1 in loss function (affects only training process).

Observations from the code:

- Luma Chroma size adjustment  Chroma up-sampling.

- Variables dynamic range  float point.

- Position in pipeline: in-loop (before SAO).

- Number of residual blocks 8

- Normalized QP map is auxiliary input

- Long skip connection

Questions recommended to be answered during EE test:

- What is the performance impact of using the “SE block”?

- What is the performance impact of SSIM based loss function?

Both questions have been answered in 4 tests conducted. Proponent used VTM11.0 as anchor (GOPsize=32, spatial-temporal filer). Additionally to PSNR proponent computed VMAF metric. In conducted tests we observe that PSNR and VMAF show different trends: BD-rate gain in PSNR is BD-rate loss in VMAF.

SE block contributes 1% PSNR gain in RA and 1.2% in All Intra (PSNR), performance impact of SE block in VMAF is under 1%.

Models trained with pure L1 (SSIM weight in loss function 0) provide 1.3…1.4% better performance in PSNR, but 3-4% drop in VMAF. This phenomenon is very interesting; visual quality should be carefully investigated.

Tests summary:

| Algorithm |  |  | All Intra, cfg |  |  |  |  |  | Random Access, cfg |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SE | SSIM | L1 | Y | U | V | VMAF | Enc, | Dec, | Y | U | V | VMAF | Enc, | Dec, |
| Y | 0.8 | 0.2 | -2.6% | -10% | -9% | -8% | 1.4 | 24 | -0.8% | -8% | -6% | -6% | 1.4 | 35 |
| Y | 0.0 | 1.0 | -2.9% | -9% | -9% | -4% | 1.4 | 32 | -2.2% | -10% | -9% | -3% | 1.4 | 29 |
| N | 0.8 | 0.2 | -1.4% | -6% | -6% | -7% | 1.4 | 37 | 0.1% | -4% | -2% | -6% | 1.4 | 22 |
| N | 0.0 | 1.0 | -2.5% | -7% | -8% | -4% | 1.3 | 22 | -2.0% | -8% | -8% | -3% | 1.4 | 25 |

### JVET-U0094  EE: Tests on Neural Network-based In-Loop Filter H. Wang, M. Karczewicz, J. Chen, A. M. Kotra (Qualcomm).

This test is based on JVET-T0079 AHG11: Neural Network-based In-Loop Filter [H. Wang, M. Karczewicz, J. Chen, A.M. Kotra (Qualcomm)]

SW branch  https://vcgit.hhi.fraunhofer.de/jvet-t-ee1/VVCSoftware_VTM/-/tree/EE-JVET-T0079

SW branch owner Hongtao Wang: hongtaow@qti.qualcomm.com

Key aspects of this technology is residual scaling.

Observations from the code:

- Luma Chroma size adjustment  Luma split to 4 planes.

- Variables dynamic range  float point.

- Position in pipeline  subject for study in this EE.

- 12 hidden convolution layers in total (variant with 6 was studied)

Questions recommended to be answered during EE test:

- What is performance impact of NN filter residual scaling?

- How gain depends on number N hidden layers?

- How gain depends on number X models supported by decoder?

- Is it possible to include QP in the model and reduce the number of models?

Tests summary:

| Test | All Intra, cfg |  |  |  |  | Random Access, cfg |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc, | Dec, | Y | U | V | Enc, | Dec, |
| N=12, X=4, res.scale, prior SAO | -4.8% | -10% | -11% | 1.3 | 85 | -5.3% | -13% | -13% | 1.4 | 149 |
| N=6, X=4, res.scale, prior SAO | -4.2% | -9% | -10% | 1.1 | 45 | -4.6% | -12% | -12% | 1.2 | 78 |
| N=12, X=4, NO res.scale, prior SAO | -4.6% | -9% | -10% | 1.2 | 85 | -4.8% | -12% | -12% | 1.4 | 148 |
| N=12, X=12, res.scale, prior SAO | -4.4% | -10% | -11% | 1.6 | 85 | -4.9% | -13% | -13% | 2.1 | 148 |
| N=12, X=12, res.scale, after ALF | -3.3% | -8% | -9% | 1.6 | 85 | -4.0% | -12% | -12% | 2.1 | 148 |

### Withdrawn

### This test is based on JVET-T0088 AHG11: Convolutional neural networks-based in-loop filter [Y. Li, L. Zhang, K. Zhang, Y. He, J. Xu (Bytedance)] was withdrawn from EE.

### JVET-U0101 EE-2.1.5: In-loop filtering based on neural network 
W. Chen, X. Xiu, Y.-W. Chen, H.-J. Jhu, C.-W. Kuo, X. Wang (Kwai).

This test is based on JVET-T0094 AHG11: In-loop filtering based on neural network [T.-C. Ma, W. Chen, X. Xiu, Y.-W. Chen, H.-J. Jhu, C.-W. Kuo, X. Wang (Kwai)]

SW branch  https://vcgit.hhi.fraunhofer.de/jvet-t-ee1/VVCSoftware_VTM/-/tree/EE-JVET-T0094

SW branch owner  Wei Chen chenwei06@kwai.com

Interesting aspect of this technology is Luma and Chroma size equalization prior to CNN filter block.

Observations from the code:

- Luma Chroma size adjustment   subject for study in this EE.

- Variables dynamic range  float point.

- Position in pipeline  in loop, prior to SAO.

- 20 residual blocks (10 as variant was tested)

Questions recommended to be answered during EE test:

- Performance impact of initial convolution based Chroma upsampling?

Question was answered by series of test with different methods of Luma & Chroma sizes adjustment.

Tests summary:

| Test | All Intra, cfg |  |  |  |  | Random Access, cfg |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc, | Dec, | Y | U | V | Enc, | Dec, |
| Chroma Up, 20 RB | -5.3% | -17% | -18% | 1.5 | 404 | -4.1% | -19% | -18% | 1.5 | 410 |
| Chroma Up, 10 RB | -4.5% | -13 | -14% | 1.3 | 193 | -3.4% | -14% | -14% | 1.2 | 187 |
| Luma Split, 20 RB | -4.2% | -14% | -15% | 1.3 | 224 | -3.3% | -16% | -15% | 1.3 | 212 |

### JVET-U0054 EE: Neural network based in-loop filtering 	 [Z. Wang, R.-L. Liao, C.Y. Ma, Y. Ye (Alibaba)]

This test is based on JVET-T0128 [DNNVC] Preliminary results of Neural Network Loop Filter	 [Z. Wang, R.-L. Liao, C.Y. Ma, Y. Ye (Alibaba)]

SW branch  https://vcgit.hhi.fraunhofer.de/jvet-t-ee1/VVCSoftware_VTM/-/tree/EE-JVET-T0128

SW branch owner  R.-L. Liao ruling.lrl@alibaba-inc.com

This technology demonstrated the highest BD-rate gain based on last meeting contribution, but also uses the largest number of residual blocks in CNN filter. Total number of convolution layers is 66.

Questions recommended to be answered during EE test:

- What steps are planned for a complexity reduction??

Just one variant was tested, no steps toward complexity reduction were taken. Test results in first uploaded version of contribution reported only for RA configuration, w/o 4K classes. Also run-time was not provided, This indicates that complexity of proposed method is rather high.

Tests summary:

| Test | All Intra, cfg |  |  |  |  | Random Access, cfg |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc, | Dec, | Y | U | V | Enc, | Dec, |
| 66 conv. layers |  |  |  |  |  | -5.0% | -16% | -17% |  |  |

## EE2: NN-based super resolution

### JVET-U0096 EE: Tests on Decomposition, Compression and Synthesis (DCS)-based Technology  [M. Lu, Z. Ma (Nanjing Univ.), L. Xu, D. Wang (OPPO)]

This test is based on JVET-T0125 [DNNVC] Decomposition, Compression, Synthesis (DCS)-based Framework: A New Exploration on Video Coding [Ming Lu, Zhan Ma (Nanjing University)]

SW branch  https://vcgit.hhi.fraunhofer.de/jvet-t-ee1/VVCSoftware_VTM/-/tree/EE-JVET-T0125

SW branch owner  Ma Zhan mazhan@nju.edu.cn

General Architecture is “Decomposition, Compression & Synthesis” (DCS)-based Video Coding, or DCS for short, is to first decompose the input video into respective spatial texture into respective spatial texture frames (STF) at its native spatial resolution that preserve the rich spatial details, and the other temporal motion frames (TMF) at a lower spatial resolution that retain the motion smoothness; then compress them together using HEVC or VVC video coder; and finally synthesize decoded STFs and TMFs for high-fidelity video reconstruction at the same resolution as its native in- put.

Learnt, resolution adaptive synthesis which includes Multi-frame motion compensation and Non-Local Texture Transfer Network are used for creation output video in original resolution.

Questions recommended to be answered during EE test:

- What is the performance impact at higher QPs?

- What is the performance impact of different prediction structures, LDP, LDB, RA?

- What is the performance impact of intra period?

- What is the performance impact of window size in motion compensation module?

- What rea result compared to anchor that uses RPR and upsampling filters of VVC multilayer profile.

- What is the impact of the number of input frames to MC-Net? Number of input frames 5, 3, 2…

Due to the limited time only some results are available. Technology was studied with HEVC and with VVC as base codec. Chroma PSNR was not provided. In excel reporting template only LPD test results are shown. NO run-time data provided.

Tests summary:

| Base codec | Random Access, cfg |  |  |  |  | Low-delay -P, cfg |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc, | Dec, | Y | U | V | Enc, | Dec, |
| HEVC |  |  |  |  |  | -24.3% |  |  |  |  |
| VVC |  |  |  |  |  | -8.7% |  |  |  |  |

# Training and test conditions

For training proponents used BVI-DVC (JVET-U0054, JVET-U-0101, JVET-U-0094), DIV2K (JVET-U0060, JVET-U0074), SJTU 4K (JVET-U0096).

# Conclusion

Results reporting template and complexity assessment methodology was used in this EE. It appears to provide useful information and recommended to further refined.

Several technical questions which can result in practical recommendation for NN technologies development were answered during this EE round. It is recommended to continue EE practice.

Different trends in PSNR and VMAF (gain in one metric, but drop in another) were demonstrated during this EE. Inclusion and active report of more than just PSNR is highly encouraged for future EE.

It is recommended to review all EE contributions for further details

# References

[1] BoG Report: Neural Network Technology, A. Segall, JVET-T0130.

[2] Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-T2006.