| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0046 |
| --- | --- |

| Title: | AHG4: Agenda and report of the AHG meeting on the HDR video verification test preparation on 2020-12-10 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Vittorio Baroncini | Tel: Email: | asegall@sharplabs.com wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | AHG |  |  |

_____________________________

# Agenda and Notes

The meeting was held online on Thurs, 2020-12-09, 17:00h UTC with the connection information provided via the JVET mailing list.

- Review of contributions JVET-U0042

- Discussion on following steps

| JVET-U0042 | m55696 | 2020-12-08 16:49:00 | 2020-12-10 12:32:04 | 2020-12-10 12:32:04 | Status Report on HDR Verification Test Preparation | M. Wien, V. Baroncini, A. Segall | JVET-U0042. |
| --- | --- | --- | --- | --- | --- | --- | --- |

JVET-U0042 Status Report on HDR Verification Test Preparation [M. Wien, V. Baroncini, A. Segall]

The document summarized the status of test sequence selection and rate point determination for the VVC verification test of the HDR-HLG and HDR-PQ categories.  For HDR-HLG, it was reported that a set of test sequences has been identified and visually inspected by JVET experts.  Rate points have been identified for three of the sequences.  During evaluation, a color artifact was reported to be identified in two additional test sequences and is currently being studied and corrected.  For HDR-PQ, a set of test candidates has been visually inspected by JVET experts.  One or two of the available sequences were determined to be suitable for further consideration.  Additional sources for sequences have been identified, and these sequences are currently under study.

During the presentation, it was reported that the HM encodings of the HLG content generated chroma artifacts at the lower bit-rates.  Initial investigation concluded that this was representative of the HM performance.  However, it was suggested that a reallocation of bits from the luma channel to the chroma channel would likely address the problem without significant impact on the luma coding performance.

The group discussed that the chroma QP control was changed between the HM and VTM, with the VTM using a custom luma-to-chroma QP mapping table.  It was unclear at the time of the discussion of the HM or VTM HLG configuration would result in a more aggressive allocation of bits to the chroma channel.

One participant suggested that visual evaluation could be performed by configuring the HM to approximately use the VTM chroma adaptation strategy.  It was agreed within the group that this evaluation would be performed.

The group also discussed the need for additional PQ content.  It was reported in JVET-U0042 that three different sources for test content had been identified.  The first was the “Sparks” sequences from the Netflix Open Source Content; the second was the test content described in ITU-T BT.2445-9, and the third was the “Tears of Steel” content available from the Blender project.

It was reported that the “Sparks” sequences were available to process at the time of the meeting.

It was also reported that the “Tears of Steel” content would require grading to generate an HDR version suitable for visual testing.  It was acknowledged that JVET did not have the expertise to perform the grading and that other input would be needed.  There was consensus to treat the “Tears of Steel” sequence as a longer-term activity.

There was additional discussion about the Driving POV and CosmosLaundromat sequences.  Both sequences were identified as having desirable characteristics for the HDR verification test.  However, it was identified that the Driving POV sequence had already been used in the SDR verification test.  And, that an HDR grade of the CosmosLaundromat sequence was only available at HD resolution.  The group concluded that these sequences would not be suitable for the verification test because of the stated issues.

AHG meeting was closed at approximately 17:55 UTC.

# List of AHG Meeting Participants

(in alphabetical order)

- Andrew Segall (Sharp - US) (Andrew Segall)

- Atsuro Ichigaya (NHK - JP)

- Benjamin Bross (HHI - DE) (Benjamin Bross)

- Christian Helmrich (HHI - DE)

- Edouard François (InterDigital - France)

- Fabrice Le Léannec (InterDigital - FR)

- Fangjun Pu (Dolby - US) (Fangjun Pu)

- Gary Sullivan

- Jens-Rainer Ohm (RWTH - DE) (Jens-Rainer Ohm)

- Kenneth Andersson (Ericsson - SE)

- Koohyar Minoo (IR-Koohyar Minoo)

- Lulin Chen (MediaTek - US)

- Mathias Wien (RWTH - DE)

- Olena Chubach (MediaTek Inc. - US)

- Peng Yin (Dolby - US) (Peng Yin (Dolby))

- Sean McCarthy (Dolby - US)

- Shimpei Nemoto (NHK - JP)

- Shunsuke Iwamura (NHK - JP)

- Vittorio Baroncini

- Walt Husak