| oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0053-v1 |
| --- | --- |

| Title: | AHG9/AHG11 Level information for super-resolution neural network |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Takuya Suzuki Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp corporation |  |  |

_____________________________

- Abstract

In this contribution, level information for a super-resolution post-filter NN (Neural Network) based SEI is proposed. In VVC, RPR (Reference Picture Re-sampling) has been introduced. As a result of some experiments to change the resolution of the whole sequence, in several 4K sequences, there are coding gains. In this experiment, the interpolation filter of VVC is used as the up-sampling filter. In addition, when a super-resolution post-filters using NN is introduced, it is shown that the visual quality is improved. For example, MPEG NNR which is discussed in SC29/WG4 can describe neural networks efficiently. At the previous meeting, JVT-T0092 proposed an SEI which sends the neural network parameters of super-resolution post-filter. In this contribution, an SEI which defines level information for the complexity of neural network processing is proposed.

Problem Statement

Experiments of half-resolution coding using RPR

It has been known that there are some cases that the coding efficiency of down-sampled coding is better than that of original resolution coding when high-resolution sequences are encoded in the condition of low-bit rate. Since recently super-resolution technologies using NN have been significantly improved [1], its possibility should be utilized.

Some experiments to change the resolution of the whole sequence have been conducted by RPR coding on VTM-11.0. As in Tables 1, several 4K sequences get good coding gains. In this experiment, the cases of ScalingRatioHor and ScalingRatioVer equal to 2.0 (1/2 resolution) is tested. The QP range of anchor is 32, 37, 42, 47 and that of RPR coding is 27, 32, 37, 42. PSNR between the original sequence and the decoded up-sampled (--UpscaledOutput = 2) sequence is calculated. At the previous meeting, there was a comment that if a temporal pre-filter were to be applied, there might not be the gain of RPR. At this time, the temporal pre-filter is applied on VTM-11.0 [2] in both anchor and RPR coding. The experimental for class A results show the coding gains of RPR as shown in Table 1.

Figure 1  Adaptive resolution coding

Table 1  BD gains (%) of ScalingRatioHor and ScalingRatioVer equal to 2.0 (QP 27,32,37, 42)

|  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over up-sampling filtering in Table 1 |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -9.44% | -3.07% | 2.49% | 55% | 29% |
|  | FoodMarket4 | -9.93% | 2.52% | 2.89% |  |  |
|  | Campfire | -13.09% | 15.79% | 7.71% |  |  |
| Class A2 4K | CatRobot1 | -0.34% | 22.92% | 29.23% | 48% | 29% |
|  | DaylightRoad2 | 4.40% | 3.05% | 3.55% |  |  |
|  | ParkRunning3 | -8.94% | 121.76% | 45.88% |  |  |

When super-resolution instead of up-sampling filtering is used, the performance has been improved as shown in Figure 2. Table 2 shows BD-gains by a super-resolution [4] compared to the up-sampling filter by RPR. This super-resolution is based on 12 layers of network based on simplified ESRGAN [5]. The loss function is the L1 norm and part of the BVI-DVC test set [6] is used as training sequences. Table 3 shows BD-gains by the super-resolution compared to VTM-11.0 (QP 32,37,42,47). Although the balance of gain between color components is not adjusted, the improvement of coding gains has been shown.

Figure 2  Applying super-resolution

Table 2  BD gains (%) by super-resolution (QP 27,32,37, 42)

|  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over up-sampling filtering in Table 1 |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -0.37% | -4.72% | -2.67% | 100% | 581% |
|  | FoodMarket4 | -0.98% | -0.62% | -2.36% |  |  |
|  | Campfire | -2.65% | -1.78% | -4.27% |  |  |
| Class A2 4K | CatRobot1 | -4.96% | -4.09% | -4.63% | 100% | 528% |
|  | DaylightRoad2 | -5.99% | -3.13% | -2.48% |  |  |
|  | ParkRunning3 | -2.44% | -4.07% | 4.11% |  |  |

Table 3  BD gains (%) of half-resolution coding with super-resolution

|  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over up-sampling filtering in Table 1 |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -9.78% | -7.64% | -0.21% | 55% | 170% |
|  | FoodMarket4 | -10.77% | 2.04% | 0.55% |  |  |
|  | Campfire | -15.34% | 13.60% | 3.29% |  |  |
| Class A2 4K | CatRobot1 | -4.63% | 18.21% | 23.62% | 48% | 154% |
|  | DaylightRoad2 | -0.57% | -0.03% | 1.09% |  |  |
|  | ParkRunning3 | -11.17% | 114.24% | 51.05% |  |  |

Coding of neural network parameters

At the previous meeting, JVT-T0092 [7] proposed a framework of super-resolution post-filter using NN. In that framework, an SEI which can be described network parameters was proposed. There was a comment the network parameters can be sent also on the system layer.

Figure 3  A framework of super-resolution NN

In this contribution, a super-resolution network using the above experiments is compressed by using NCTM-5.0 [8] which is the reference software of MPEG NNR [9]. Both approx_method and enc_method is set to baseline. Change quantizer parameters, a relationship between the performance of super-resolution and compression ratio have been investigated.

As shown in Figure 4, the performance of the super-resolution is not changed until the compression ratio is about 4. Since the original size of network parameters is 1946523 bytes, we can compress the network parameters to about 480kbIytes.

Figure 4  A relationship between the performance of super-resolution and compression ratio

In this framework, there was a problem that the complexity of the processing for super-resolution might be too larger on the decoder side. In the case of the video codec standard, the profile and the level have been defined in terms of decoding capability but there is no such information for SEI processing. Since the processing of the neural network has large computational complexity generally, we think it is necessary to indicate a certain criterion for the complexity.

Although there are many discussions about the complexity of the neural networks, in this contribution, we would like to propose the size of compressed neural network parameters as the complexity for the neural networks. This idea is based on the concept of Kolmogorov complexity. Kolmogorov complexity of an object is defined as the length of the shortest computer program that produces the object as output.

Proposal

In this contribution, we propose to use the byte size of compressed network parameters as a criterion for the computational complexity. Concretely, an SEI which can define level information based on the byte size of compressed neural network parameters is proposed.

| nnr_level_sei_message( payloadSize ) { | Descriptor |
| --- | --- |
| nnr_level_idc | u(8) |
| nnr_parameter_scale | u(8) |
| } |  |

nnr_level_idc specifies the level information for the neural network processing based on the maximum byte size of neural network representation in Table 4 and the variable NNR_level which defines the complexity of neural network processing is set.

nnr_parameter_scale indicates the scaling factor for level information which is used to adjust the bit-depth of image pixel or the chroma format or the operational precision of neural network.

Table 4  NNR level

| NNR_level | nnr_level_idc | Max_nnr_byte |
| --- | --- | --- |
| 1 | 16 | 32768 |
| 2 | 32 | 65536 |
| 2.1 | 35 | 74632 |
| 3 | 48 | 131072 |
| 3.1 | 51 | 149263 |
| 4 | 64 | 262144 |
| 4.1 | 67 | 298527 |
| 5 | 80 | 524288 |
| 5.1 | 83 | 597053 |
| 5.2 | 86 | 679917 |
| 6 | 96 | 1048576 |
| 6.1 | 99 | 1194106 |
| 6.2 | 102 | 1359835 |

Reference

[1]	W. Yang, X. Zhang, Y. Tian, W. Wang, J-H Xue and Q. Liao, “Deep Learning for Single Image Super-Resolution: A Brief Review”, IEEE Trans. Multimedia, vol.21, no.12, pp.3106-3121, Dec. 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, Joint Video Experts Team Document, JVET-T2010, teleconference, Oct. 2020.

[3]	S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien and D. Grois, “Methodology and reporting template for neural network coding tool testing”, Joint Video Experts Team Document, JVET-T0041, teleconference, Oct. 2020.

[4]	T. Suzuki, E. Sasaki, T. Chujoh, T. Ikai and H. Watanabe, “Coded Video Transmission using Super-Resolution and MPEG-NNR Deep Learning Metadata”, the 35th Picture Coding Symposium of Japan/the 25th Image Media Processing Symposium (PCSJ/IMPS 2020), Nov. 2020.

[5]	X. Wang, K. Yu, S. Wu, J. Gu, Y. Liu, C. Dong, Y. Qiao, and C. Change Loy, “ESRGAN: Enhanced super-resolution generative adversarial networks,” in Proceedings of the European Conference on Computer Vision (ECCV) workshops, 2018.

[6]	Di Ma, Fan Zhang and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

[7]	T. Chujoh, E. Sasaki and T. Ikai, “AHG9/AHG11: Neural network based super resolution SEI”, Joint Video Experts Team Document, JVET-T0092, teleconference, Oct. 2020.

[8]	“Test Model 5 of Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29/WG11 Documents, N 19500, Online, July 2020.

[9]	“Text of ISO/IEC DIS 15938-17 Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29 Documents, MDS19764_WG04_N00016, Online, Oct. 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).