| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0057 |
| --- | --- |

| Title: | CE-1.4, CE-1.5, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document reports results for CE-1.4, CE-1.5, CE-2.2 and CE-2.3 which are modifications to the method for the selection of Rice parameters for residual coding. These modifications are intended for consideration for high bit depth coding in VVC version 2. The modifications are based on those previously described in JVET-T0072[1].  CE-1.4 and CE-1.5 are modifications of regular residual coding (RRC) where CE-1.5 is a simplification of CE-1.4. CE-2.2 and CE-2.3 are modifications of transform skip residual coding (TSRC) where CE-2.3 is a simplification of CE-2.2. The aim of the modifications is to offer BD-rate gains for high bit depths when typically residual coefficient values become larger, whilst being backwardly compatible with the existing Rice coding techniques in VVC version 1(JVET-T2001)[2].

It is reported that average BD-rate differences (wPSNR for PQ, PSNR all others) with high bit depth CTC (JVET-T2018)[3] are as follows:

CE-1.4:

- PQ: 	-0.94%/-0.89%/-0.91% (AI), -0.09%/-0.10%/-0.13% (LDB), -0.20%/-0.19%/-0.21% (RA)

- HLG:	-0.60%/-0.42%/-0.43% (AI), -0.08%/-0.08%/-0.07% (LDB), -0.09%/-0.08%/-0.07% (RA)

- SVT-12:	-4.22%/-3.72%/-3.73% (AI), -0.98%/-0.96%/-0.96% (LDB), -1.15%/-1.01%/-1.02% (RA)

- SVT-16:	-27.78%/-25.68%/-25.68% (AI), -17.78%/-16.96%/-16.98% (LDB), -16.86%/-16.97%/-16.92% (RA)

CE-1.5:

- PQ:		-0.92%/-0.85%/-0.85% (AI), -0.09%/-0.10%/-0.12% (LDB), -0.19%/-0.17%/-0.20% (RA)

- HLG:	-0.63%/-0.47%/-0.46% (AI), -0.09%/-0.07%/-0.07% (LDB), -0.10%/-0.07%/-0.07% (RA)

- SVT-12: -4.26%/-3.74%/-3.75% (AI), -1.05%/-1.02%/-1.02% (LDB), -1.22%/-1.07%,-1.07% (RA)

- SVT-16:	-27.67%/-25.41%/-25.44% (AI), -17.75%/-16.87%/-16.89% (LDB), -16.91%/-16.95%/-16.90% (RA)

CE-2.2

- PQ:		-0.18%/-0.20%/-0.19% (AI), -0.02%/-0.03%/-0.06% (LDB), -0.05%/-0.04%/-0.05% (RA)

- HLG:	-0.07%/-0.05%/-0.05% (AI),  0.00%/-0.01%/-0.01% (LDB).  0.00%/ 0.00%/ 0.00% (RA)

- SVT-12:	-1.48%/-1.61%/-1.56% (AI), -0.37%/-0.36%/-0.34% (LDB), -0.45%/-0.39%/-0.37% (RA)

- SVT-16:	-22.50%/-21.54%/-21.50% (AI), -13.82%/-13.22%/-13.22% (LDB), -12.90%, -13.51%, -13.44% (RA)

CE-2.3

- PQ:		-0.15%/-0.14%/-0.13% (AI), -0.01%/-0.01%/-0.01% (LDB), -0.02%/-0.02%/-0.02% (RA)

- HLG:	-0.17%/-0.12%/-0.17% (AI),  0.00%/-0.01%/-0.01% (LDB),  0.00%/-0.01%/ 0.00% (RA)

- SVT-12:	-1.24%/-1.37%/-1.32% (AI), -0.28%/-0.24%/-0.23% (LDB), -0.30%/-0.23%/-0.23% (RA)

- SVT-16:	-16.24%/-15.24%/-15.20% (AI), -9.01%/-8.49%/-8.51% (LDB), -7.90%/-8.51%/-8.46% (RA)

# Introduction

In VVC version 1[3], the range of Rice parameters that may be used for coefficients during regular residual coding is limited to the range of 0 to 3 (inclusive). This range has been found to be suitable for both 8 and 10 bit data; however, as bit depths increase and QPs become lower, the size of each coded coefficient becomes larger. For high bit depth content at QPs below 0 it has previously been shown that VVC coding efficiency is below that of HEVC[4].

Previous work[5] has described approaches that increase the coding efficiency for high bit depths and low QPs to match or exceed HEVC.  This proposal is based on the implementation of one of these approaches, “Adding a Rice-based shift to existing VVC”[1].

For the purposes of this core experiment, the implementation has been split into separate components for RRC (CE-1.4 and CE-1.5) and TSRC (CE-2.2 and CE-2.3) to allow for comparison with other techniques proposed within the CE.

This modification described results in gains for 12 and 16-bit content. The modification provides backwards compatibility with VVC version 1 as the existing Rice coding method can be considered as a subset of the modification.

Version 2 of this document updates the draft text changes and adds a complexity section.

# Algorithm description

The algorithm impacts entropy coding and in particular the selection of Rice parameters for the Golomb-Rice coding of residual coefficients. The algorithm for each experiment (CE-1.4, CE-1.5, CE-2.2 and CE-2.3) can be considered as having four components as follows:

- A coefficient size estimation function which is backed by a series of counters and an index function to select which counter is used for each coefficient.

- An update function for the selected counter.

- A modification to the current algorithm for computing the clipped sum of adjacent nearby coefficients.

- A modification to the current algorithm for selecting Rice parameters.

## Coefficient size estimation

The technique requires an estimation of the log2 size of coefficients being coded. This is derived in a similar manner to Rice parameter estimation in HEVC (RExt) using a series of counters. For this proposal the counters are mapped to a one (CE-2.2 and CE-2.3) or two (CE-1.4 and CE-1.5) dimensional array, with the first (or only) dimension being the component type. For CE-1.4 and CE-1.5, the second dimension of the array is an index function coded as shown in Table 1 giving a total of 8 counters. This is implemented in a lookup table using the scan order. When coding losslessly, VVC uses RRC rather than TSRC and therefore the second array dimension requires an element for transform-skip.

| Index | Description | 16x16 transform block |
| --- | --- | --- |
| 0 | Transform-skip |  |
| 1 (Red) | DC coefficient in top left sub-TB |  |
| 2 (Blue) | Coefficients within distance of 3 from DC coefficient (inclusive) |  |
| 3 (Yellow) | All other coefficients |  |

Table 1. Index function derivation

## Counter update function

Each counter is set to 0 at the beginning of every slice.  The counters are handled in the same way as CABAC contexts, thus allowing the values to be rewound during the encoder search. This also provides support for wave-front parallel coding as each wave-front copies the counters before starting.

A counter is updated when a coefficient is coded as follows:

if (abs(value - cutoff) > 0) {   						// cutoff is the amount of the coefficient coded with flags

int idx = lookup(isTS, scanPos);    				// index function as defined in Table 1

int &count = riceCount[componentType][idx];	// riceCount is the array of counters, reset to 0 for each slice

if (abs(value) >= upperThreshold) {  			// value larger than expected range, initially 3

count += increment;    					// increment = 4

} else if ((abs(value) << 1) < lowerThreshold) {	// value smaller than expected range, initially 1

count = max(0, count - decrement);			// decrement = 1, clipped to be non-negative

}

}

For CE-1.4 and CE-2.2, a counter may be updated for every coefficient that is coded. For CE-1.5 and CE-2.3, only coefficients on the leading diagonal of the block of coefficients can cause a counter to be updated.

The value of rice provides an estimate of the log2 size of the average non-zero coefficient.  Mismatched increments and decrements have been found to be beneficial as slightly over-estimating the Rice parameter seems to offer the best BD-rate gains.

As soon as the update is done (CE-1.4) or between sub-blocks has been coded (CE-1.5), the upperThreshold and lowerThreshold are recomputed as below:

int rice = count >> log2Divisor;				// log2Divisor = 4

upperThreshold = 3 << rice;

lowerThreshold = 1<< rice;

## Modification to clipped sum of coefficients (CE-1.4, CE-1.5, CE-2.2)

For regular residual coding (RRC), VVC (section 9.3.3.2) defines locSumAbs as the sum of the absolute values of 5 previously coded coefficients below and right of the coefficient, less an offset which relates to the proportion of the coefficient that is coded with flags, and then clips this value to the range 0 to 31. This technique works well if clipping is only occasionally used, but as average coefficient values increase the number of times clipping is required increases.

In the modification described here, a shift right is introduced as below (modification in green).  The purpose of this shift is to reduce the magnitude of the sum and thus reduce the amount of clipping. The size of the shift is described in the following section.

locSumAbs = Clip3( 0, 31, Max( locSumAbs − baseLevel * 5, 0  ) >> shift )

For transform skip residual coding (TSRC) VTM currently does not use a clipped sum. The modification for CE-2.2 introduces a clipped sum for TSRC which is identical to that described for RRC but uses 5 coefficients above and left of the coefficient being coded.

## Rice parameter selection (CE-1.4 and CE-1.5)

In VTM, RRC selects the Rice parameter by using the value returned by the template function described in section 2.3 as an index into a lookup table which gives the Rice parameter.

In the proposed modification one of two shift values can be used, one when a coefficient is coded using a parity flag and one when a coefficient is coded without a parity flag. Both of these values are based on the value of count derived as described in section 2.2.

shiftWithoutParity = std::min(12, std::max(0, ((count + rounding) >> log2Divisor) – 2)); // rounding = 8

shiftWithParity = std::max(0, shiftWithoutParity – 1);

The shift is clipped to a maximum of 12 to support an extended table of Rice coding costs specified for dependent quantisation. This now supports a range of Rice parameters from 0 to 15 rather than 0 to 3 as at present in VTM 11.0.

The computation of these two shift values is done at the same time as the values of upperThreshold and lowerThreshold described in section 2.2, i.e. after each update of count (CE-1.4) or after each sub-block is coded (CE-1.5).

The selection of the Rice parameter can then be described using the pseudo code below:

int idx = cctx.templateAbsSum(scanPos, absCoeff, baseLevel, shift);

// compute template fn using sum shifted right by shift modifier

int rice = g_auiGoRiceParsCoeff[idx] + shift;		// add shift modifier onto result from lookup table

Thus the selection of the Rice parameter may be seen as a modified version of the existing VVC technique rather than a new technique.  This modification ensures that when estimated coefficient values are larger and a more appropriate range of Rice parameters is selectable. Unlike simply swapping to a technique based purely on size estimation, this modification introduces a hybrid technique which attempts to extend the operating point of the existing VVC technique.

## Rice parameter selection (CE-2.2)

For CE-2.2 Rice parameter selection is achieved in the same way as CE-1.4, but the template sum is changed to five upper left coefficients rather than five lower right coefficients as described in section 2.3. This is different from VVC version 1, but can be disabled for standard bit depths of 10 and below.

## Rice parameter selection (CE-2.3)

For CE-2.3 Rice parameter selection is simplified from CE-2.2 and can be considered an extension of the existing VVC method (which always uses a Rice parameter of 1) as follows:

int rice = 1 + shift;		// shift is either shiftWithParity or shiftWithoutParity depending on flag usage

// during coefficient coding

## 8 and 10 bit operation

For CE-1.4, CE-1.5 and CE-2.3, coding that exactly matches VVC version 1 can be achieved if the derivation of the shift values shiftWithoutParity and shiftWithParity is constrained to only occur when the bit depth is greater than 10, and they are set to 0 otherwise.

# Results

## CE-1.4

### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.88% | -0.82% | -0.83% | -0.89% | -0.83% | -0.83% | 105% | 100% |
| PQ422 | -0.99% | -0.97% | -0.99% | -1.00% | -0.98% | -1.00% | 103% | 100% |
| Overall | -0.94% | -0.89% | -0.91% | -0.95% | -0.90% | -0.92% | 104% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.08% | -0.09% | -0.13% | -0.09% | -0.10% | -0.12% | 103% | 100% |
| PQ422 | -0.10% | -0.11% | -0.13% | -0.10% | -0.10% | -0.12% | 102% | 102% |
| Overall | -0.09% | -0.10% | -0.13% | -0.10% | -0.10% | -0.12% | 103% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.19% | -0.18% | -0.19% | -0.20% | -0.18% | -0.19% | 103% | 101% |
| PQ422 | -0.20% | -0.20% | -0.23% | -0.21% | -0.20% | -0.23% | 103% | 103% |
| Overall | -0.20% | -0.19% | -0.21% | -0.20% | -0.19% | -0.21% | 103% | 102% |
| Overall PQ | -0.41% | -0.39% | -0.42% | -0.42% | -0.40% | -0.42% | 103% | 101% |

### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.58% | -0.42% | -0.43% | 102% | 101% |
| HLG422 | -0.63% | -0.48% | -0.48% | 104% | 101% |
| Overall | -0.60% | -0.45% | -0.45% | 103% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.07% | -0.07% | -0.06% | 103% | 100% |
| HLG422 | -0.10% | -0.09% | -0.08% | 103% | 103% |
| Overall | -0.08% | -0.08% | -0.07% | 103% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.07% | -0.07% | 102% | 100% |
| HLG422 | -0.10% | -0.08% | -0.08% | 103% | 101% |
| Overall | -0.09% | -0.08% | -0.07% | 103% | 100% |
| Overall HLG | -0.26% | -0.20% | -0.20% | 103% | 101% |

### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -27.78% | -25.68% | -25.68% | 107% | 105% |
| SVT12 | -4.22% | -3.72% | -3.73% | 107% | 108% |
| Overall | -16.00% | -14.70% | -14.71% | 107% | 106% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -17.78% | -16.96% | -16.98% | 111% | 110% |
| SVT12 | -0.98% | -0.96% | -0.96% | 104% | 108% |
| Overall | -9.38% | -8.96% | -8.97% | 107% | 109% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.86% | -16.97% | -16.92% | 110% | 108% |
| SVT12 | -1.15% | -1.01% | -1.02% | 105% | 107% |
| Overall | -9.00% | -8.99% | -8.97% | 107% | 108% |
| Overall RGB | -11.46% | -10.88% | -10.88% | 107% | 108% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0 rc1 | CE-1.4 |  | VTM11.0 rc1 | CE-1.4 |  | VTM11.0 rc1 | CE-1.4 |  |
| PQ444 | 2.4 | 2.5 | -4.46% | 3.0 | 3.1 | -1.50% | 3.0 | 3.1 | -1.40% |
| PQ422 | 2.2 | 2.3 | -5.60% | 2.8 | 2.8 | -1.76% | 2.8 | 2.9 | -1.62% |
| Overall | 2.3 | 2.4 | -5.03% | 2.9 | 2.9 | -1.63% | 2.9 | 3.0 | -1.51% |
| Enc Time[%] | 99% |  |  | 116% |  |  | 117% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  | 109% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0 rc1 | CE-1.4 |  | VTM11.0 rc1 | CE-1.4 |  | VTM11.0 rc1 | CE-1.4 |  |
| HLG444 | 1.9 | 2.0 | -3.81% | 2.1 | 2.1 | -1.61% | 2.1 | 2.1 | -1.58% |
| HLG422 | 1.8 | 1.8 | -4.65% | 2.0 | 2.0 | -1.50% | 2.0 | 2.0 | -1.41% |
| Overall | 1.8 | 1.9 | -4.23% | 2.0 | 2.1 | -1.56% | 2.1 | 2.1 | -1.49% |
| Enc Time[%] | 98% |  |  | 105% |  |  | 107% |  |  |
| Dec Time[%] | 117% |  |  | 110% |  |  | 115% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0 rc1 | CE-1.4 |  | VTM11.0 rc1 | CE-1.4 |  | VTM11.0 rc1 | CE-1.4 |  |
| SVT16 | 0.7 | 1.2 | -42.17% | 0.7 | 1.2 | -44.02% | 0.7 | 1.2 | -44.08% |
| SVT12 | 1.0 | 1.3 | -23.38% | 1.0 | 1.3 | -21.91% | 1.0 | 1.3 | -21.62% |
| Overall | 0.8 | 1.2 | -32.78% | 0.8 | 1.3 | -32.97% | 0.8 | 1.3 | -32.85% |
| Enc Time[%] | 116% |  |  | 126% |  |  | 119% |  |  |
| Dec Time[%] | 110% |  |  | 93% |  |  | 93% |  |  |

## CE-1.5

### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.87% | -0.77% | -0.75% | -0.88% | -0.78% | -0.75% | 103% | 100% |
| PQ422 | -0.98% | -0.92% | -0.95% | -0.99% | -0.93% | -0.95% | 101% | 100% |
| Overall | -0.92% | -0.85% | -0.85% | -0.93% | -0.85% | -0.85% | 102% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.10% | -0.09% | -0.13% | -0.10% | -0.09% | -0.12% | 101% | 100% |
| PQ422 | -0.08% | -0.12% | -0.12% | -0.09% | -0.10% | -0.10% | 101% | 101% |
| Overall | -0.09% | -0.10% | -0.12% | -0.09% | -0.10% | -0.11% | 101% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.18% | -0.17% | -0.19% | -0.18% | -0.17% | -0.18% | 102% | 101% |
| PQ422 | -0.19% | -0.18% | -0.21% | -0.20% | -0.18% | -0.20% | 102% | 102% |
| Overall | -0.19% | -0.17% | -0.20% | -0.19% | -0.17% | -0.19% | 102% | 101% |
| Overall PQ | -0.40% | -0.38% | -0.39% | -0.41% | -0.37% | -0.38% | 102% | 100% |

### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.60% | -0.44% | -0.43% | 103% | 102% |
| HLG422 | -0.66% | -0.49% | -0.49% | 102% | 101% |
| Overall | -0.63% | -0.47% | -0.46% | 102% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.06% | -0.07% | 102% | 101% |
| HLG422 | -0.10% | -0.08% | -0.08% | 101% | 101% |
| Overall | -0.09% | -0.07% | -0.07% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.07% | -0.06% | 101% | 101% |
| HLG422 | -0.11% | -0.08% | -0.08% | 101% | 101% |
| Overall | -0.10% | -0.07% | -0.07% | 101% | 101% |
| Overall HLG | -0.27% | -0.20% | -0.20% | 102% | 101% |

### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -27.67% | -25.41% | -25.44% | 109% | 111% |
| SVT12 | -4.26% | -3.74% | -3.75% | 105% | 108% |
| Overall | -15.96% | -14.57% | -14.60% | 107% | 110% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -17.75% | -16.87% | -16.89% | 105% | 109% |
| SVT12 | -1.05% | -1.02% | -1.02% | 102% | 108% |
| Overall | -9.40% | -8.95% | -8.96% | 104% | 108% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.91% | -16.95% | -16.90% | 106% | 108% |
| SVT12 | -1.22% | -1.07% | -1.07% | 101% | 106% |
| Overall | -9.06% | -9.01% | -8.98% | 103% | 107% |
| Overall RGB | -11.48% | -10.84% | -10.85% | 105% | 108% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0 rc1 | CE-1.5 |  | VTM11.0 rc1 | CE-1.5 |  | VTM11.0 rc1 | CE-1.5 |  |
| PQ444 | 2.4 | 2.5 | -4.56% | 3.0 | 3.1 | -1.65% | 3.0 | 3.1 | -1.55% |
| PQ422 | 2.2 | 2.3 | -5.70% | 2.8 | 2.8 | -1.93% | 2.8 | 2.9 | -1.79% |
| Overall | 2.3 | 2.4 | -5.13% | 2.9 | 3.0 | -1.79% | 2.9 | 3.0 | -1.67% |
| Enc Time[%] | 98% |  |  | 115% |  |  | 118% |  |  |
| Dec Time[%] | 112% |  |  | 108% |  |  | 110% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0 rc1 | CE-1.5 |  | VTM11.0 rc1 | CE-1.5 |  | VTM11.0 rc1 | CE-1.5 |  |
| HLG444 | 1.9 | 2.0 | -4.07% | 2.1 | 2.1 | -1.84% | 2.1 | 2.1 | -1.80% |
| HLG422 | 1.8 | 1.8 | -4.93% | 2.0 | 2.0 | -1.80% | 2.0 | 2.0 | -1.70% |
| Overall | 1.8 | 1.9 | -4.50% | 2.0 | 2.1 | -1.82% | 2.1 | 2.1 | -1.75% |
| Enc Time[%] | 98% |  |  | 104% |  |  | 108% |  |  |
| Dec Time[%] | 119% |  |  | 113% |  |  | 117% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0 rc1 | CE-1.5 |  | VTM11.0 rc1 | CE-1.5 |  | VTM11.0 rc1 | CE-1.5 |  |
| SVT16 | 0.7 | 1.2 | -42.24% | 0.7 | 1.2 | -44.08% | 0.7 | 1.2 | -44.14% |
| SVT12 | 1.0 | 1.3 | -23.57% | 1.0 | 1.3 | -22.04% | 1.0 | 1.3 | -21.76% |
| Overall | 0.8 | 1.2 | -32.91% | 0.8 | 1.3 | -33.06% | 0.8 | 1.3 | -32.95% |
| Enc Time[%] | 112% |  |  | 124% |  |  | 118% |  |  |
| Dec Time[%] | 108% |  |  | 92% |  |  | 93% |  |  |

## CE-2.2

### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.16% | -0.15% | -0.14% | -0.16% | -0.16% | -0.14% | 102% | 102% |
| PQ422 | -0.21% | -0.25% | -0.24% | -0.22% | -0.25% | -0.25% | 102% | 102% |
| Overall | -0.18% | -0.20% | -0.19% | -0.19% | -0.21% | -0.19% | 102% | 102% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.03% | -0.08% | -0.03% | -0.03% | -0.06% | 102% | 101% |
| PQ422 | -0.02% | -0.02% | -0.04% | -0.03% | -0.02% | -0.03% | 102% | 101% |
| Overall | -0.02% | -0.03% | -0.06% | -0.03% | -0.02% | -0.05% | 102% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.03% | -0.04% | -0.05% | -0.03% | -0.04% | 102% | 101% |
| PQ422 | -0.06% | -0.05% | -0.07% | -0.06% | -0.05% | -0.06% | 102% | 101% |
| Overall | -0.05% | -0.04% | -0.05% | -0.05% | -0.04% | -0.05% | 102% | 101% |
| Overall PQ | -0.08% | -0.09% | -0.10% | -0.09% | -0.09% | -0.10% | 102% | 101% |

### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.07% | -0.04% | -0.04% | 101% | 100% |
| HLG422 | -0.07% | -0.05% | -0.06% | 101% | 101% |
| Overall | -0.07% | -0.05% | -0.05% | 101% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | -0.01% | 0.00% | 102% | 101% |
| HLG422 | 0.00% | 0.00% | -0.01% | 102% | 100% |
| Overall | 0.00% | -0.01% | -0.01% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| HLG422 | 0.00% | 0.00% | -0.01% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall HLG | -0.02% | -0.02% | -0.02% | 102% | 101% |

### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -22.50% | -21.54% | -21.50% | 108% | 103% |
| SVT12 | -1.48% | -1.61% | -1.56% | 103% | 104% |
| Overall | -11.99% | -11.58% | -11.53% | 106% | 104% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -13.82% | -13.22% | -13.22% | 100% | 107% |
| SVT12 | -0.37% | -0.36% | -0.34% | 101% | 101% |
| Overall | -7.10% | -6.79% | -6.78% | 101% | 104% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -12.90% | -13.51% | -13.44% | 101% | 105% |
| SVT12 | -0.45% | -0.39% | -0.37% | 101% | 101% |
| Overall | -6.67% | -6.95% | -6.90% | 101% | 103% |
| Overall RGB | -8.59% | -8.44% | -8.40% | 103% | 103% |

### Lossless

Lossless results are not relevant for CE-2.2 as TSRC is not used for lossless coding in VVC.

## CE-2.3

### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.14% | -0.13% | -0.11% | -0.14% | -0.13% | -0.11% | 102% | 101% |
| PQ422 | -0.16% | -0.16% | -0.15% | -0.17% | -0.17% | -0.15% | 101% | 100% |
| Overall | -0.15% | -0.14% | -0.13% | -0.15% | -0.15% | -0.13% | 101% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | 0.00% | -0.03% | -0.02% | -0.01% | -0.02% | 100% | 100% |
| PQ422 | -0.01% | -0.01% | 0.01% | -0.02% | -0.01% | 0.01% | 100% | 100% |
| Overall | -0.01% | -0.01% | -0.01% | -0.02% | -0.01% | -0.01% | 100% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.01% | -0.03% | -0.03% | -0.02% | -0.02% | 101% | 100% |
| PQ422 | -0.02% | -0.02% | -0.01% | -0.03% | -0.02% | -0.01% | 101% | 101% |
| Overall | -0.02% | -0.02% | -0.02% | -0.03% | -0.02% | -0.02% | 101% | 100% |
| Overall PQ | -0.06% | -0.06% | -0.05% | -0.07% | -0.06% | -0.05% | 101% | 100% |

### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.15% | -0.10% | -0.10% | 101% | 100% |
| HLG422 | -0.18% | -0.13% | -0.13% | 99% | 100% |
| Overall | -0.17% | -0.12% | -0.12% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.01% | 0.00% | 101% | 100% |
| HLG422 | 0.01% | -0.01% | -0.01% | 101% | 100% |
| Overall | 0.00% | -0.01% | -0.01% | 101% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| HLG422 | 0.00% | -0.01% | -0.01% | 101% | 100% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Overall HLG | -0.06% | -0.04% | -0.04% | 100% | 100% |

### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.24% | -15.24% | -15.20% | 105% | 97% |
| SVT12 | -1.24% | -1.37% | -1.32% | 101% | 101% |
| Overall | -8.74% | -8.30% | -8.26% | 103% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -9.01% | -8.49% | -8.51% | 98% | 98% |
| SVT12 | -0.28% | -0.24% | -0.23% | 101% | 101% |
| Overall | -4.65% | -4.37% | -4.37% | 100% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -7.90% | -8.51% | -8.46% | 99% | 97% |
| SVT12 | -0.30% | -0.23% | -0.23% | 101% | 100% |
| Overall | -4.10% | -4.37% | -4.34% | 100% | 99% |
| Overall RGB | -5.83% | -5.68% | -5.66% | 101% | 99% |

### Lossless

Lossless results are not relevant for CE-2.3 as TSRC is not used for lossless coding in VVC.

# Discussion of Results

All four CE experiments described here achieve savings over VTM 11.0rc1 for all types of content tested, with larger gains for all intra simulations and SVT (RGB) content. For TSRC (CE-2.2 and CE-2.3) the gains are limited for content other than full range RGB content.

Looking in more detail at the BD-rate graphs for CE-1.5 below it can be seen that the gain increases as quantisation decreases and PSNR increases. It can also be seen that unlike VTM, the BD-rate curve maintains a constant gradient as PSNR increases which matches the expected outcome[4].

# Draft Text

## CE-1.4

#### 7.3.11.11	Residual coding syntax

| … |  |
| --- | --- |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] |  |
| sbType = (chType << 2) + (transform_skip_flag[ xC ][ yC ][ chType ] ? 0 : xC = 0 && yC = 0 ? 1 : xC < 4 && yC < 4 ? 2 : 3) |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) { |  |
| riceShift = riceShiftWithParity |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] += 2 * abs_remainder[ n ] |  |
| if( AbsLevel[xC][yC] - 4 >= upperThreshold[ sbType ] ) |  |
| StatCoeff[ sbType ]+=4 |  |
| else if( 2 * ( AbsLevel[ xC ][ yC ] - 4 ) < ( 1 << lowerThreshold [sbType] )) { |  |
| if( StatCoeff[ sbType ] > 0 ) |  |
| StatCoeff[ sbType ]− − |  |
| } |  |
| lowerThreshold[ sbType ] = 1 << (StatCoeff[ sbType ] >> 4) |  |
| upperThreshold[ sbType ] = 3 << (StatCoeff[ sbType ] >> 4) |  |
| if (BitDepth > 10) { |  |
| RiceShiftNoParity[ sbType ] = Min(12, Max(0, (StatCoeff[ sbType ] + 8) >> 4 - 2)) |  |
| RiceShiftWithParity[ sbType ] = Max(0, RiceShiftNoParity[ sbType ] – 1) |  |
| } |  |
| } |  |
| } |  |
| AbsLevel[ xC ][ yC ] += AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| sbType = (chType << 2) + (transform_skip_flag[ xC ][ yC ][ chType ] ? 0 : xC = 0 && yC = 0 ? 1 : xC < 4 && yC < 4 ? 2 : 3) |  |
| if( sb_coded_flag[ xS ][ yS ] ) { |  |
| riceShift = riceShiftNoParity[sbType] |  |
| dec_abs_level[ n ] | ae(v) |
| } |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| if( AbsLevel[xC][yC] >= upperThreshold[ sbType ] ) |  |
| StatCoeff[ sbType ]+=4 |  |
| else if( 2 * AbsLevel[ xC ][ yC ] ) < ( 1 << lowerThreshold[sbType])) |  |
| if( StatCoeff[ sbType ] > 0 |  |
| StatCoeff[ sbType ]− |  |
| } |  |
| lowerThreshold[ sbType ] = 1 << (StatCoeff[ sbType ] >> 4) |  |
| upperThreshold[ sbType ] = 3 << (StatCoeff[ sbType ] >> 4) |
| if (BitDepth > 10) { |  |
| RiceShiftNoParity[ sbType ] = Min(12, Max(0, (StatCoeff[ sbType ] + 8) >> 4 -2)) |  |
| RiceShiftWithParity[ sbType ] = Max(0, RiceShiftNoParity[ sbType ] – 1) |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| if( sh_dep_quant_used_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |

…

## 9.3	CABAC parsing process for slice data

### 9.3.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in subclause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in subclause 7.3.11.2 and one or more of the following conditions are true:

–	The CTU is the first CTU in a slice.

–	The CTU is the first CTU in a tile.

–	The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

The storage process for context variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for context variables as specified in subclause 9.3.2.3 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as outputs.

The storage process for rice coding variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for rice coding variables is specified in subclause 9.3.2.8 is invoked with StatCoeffWpp as output.

When sps_palette_enabled_flag is equal to 1, the storage process for palette predictor is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2 and the decoding process of the last CU in the CTU in subclause 8.1.2, sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for palette predictor as speficied in subclause 9.3.2.6 is invoked.

…

### 9.3.2	Initialization process

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

Figure 12 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a slice or tile, the following applies: the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, if sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 12) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(1509)

–	The derivation process for neighbouring block availability as specified in subclause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the following applies:

–	The synchronization process for context variables as specified in subclause 9.3.2.4 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as inputs.

–	When sps_palette_enabled_flag is equal to 1, the synchronization process for palette predictor as specified in subclause 9.3.2.7 is invoked.

–	The synchronization process for rice coding as specified in subclause 9.3.2.9 is invoked.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

…

9.3.2.8	Storage process for Rice coding

This process stores the values of the arrays StatCoeff, RiceShiftWithParity, RiceShiftNoParity, LowerThreshold and UpperThreshold in the arrays StatCoeffWpp, RiceShiftWithParityWpp, RiceShiftNoParityWpp, LowerThresholdWpp and UpperThresholdWpp as follows:

for( idx = 0; idx < 8; idx++ ) {

StatCoeffWpp[ idx ] = StatCoeff[ idx ]

RiceShiftWithParityWpp[ idx ] = RiceShiftWithParity[ idx ]

RiceShiftNoParityWpp[ idx ] = RiceShiftNoParity[ idx ]

LowerThresholdWpp[ idx ] = LowerThreshold[ idx ]

UpperThresholdWpp[ idx ] = UpperThreshold[ idx ]

}

9.3.2.9	Synchronization process for Rice coding

This process synchronizes the values of the arrays PredictorPaletteSize and PredictorPaletteEntries in the arrays TablePaletteSizeWpp and TablePaletteEntriesWpp as follows:

for( idx = 0; idx < 8; idx++ ) {

StatCoeff[ idx ] = StatCoeffWpp[ idx ]

RiceShiftWithParity[ idx ] = RiceShiftWithParityWpp[ idx ]

RiceShiftNoParity[ idx ] = RiceShiftNoParityWpp[ idx ]

LowerThreshold[ idx ] = LowerThresholdWpp[ idx ]

UpperThreshold[ idx ] = UpperThresholdWpp[ idx ]

}

9.3.2.10	Initialization process for rice coding

The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 7, inclusive.

The variables RiceShiftWithParity[ k ] are set equal to 0, for k in the range 0 to 7, inclusive.

The variables RiceShiftNoParity[ k ] are set equal to 0, for k in the range 0 to 7, inclusive.

The variables LowerThreshold[ k ] are set equal to 1, for k in the range 0 to 7, inclusive.

The variables UpperThreshold[ k ] are set equal to 3, for k in the range 0 to 7, inclusive.

…

#### 9.3.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the rice shift used in the rice calculation, riceShift, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process:

locSumAbs = 0
if( xC < ( 1  <<  log2TbWidth ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC + 1 ][ yC ]
	if( xC < ( 1  <<  log2TbWidth ) − 2 )
		locSumAbs  +=  AbsLevel[ xC + 2 ][ yC ]
	if( yC < ( 1  <<  log2TbHeight ) − 1 )
		locSumAbs  +=  AbsLevel[ xC + 1 ][ yC + 1 ]	(1517)
}
if( yC < ( 1  <<  log2TbHeight ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC + 1 ]
	if( yC < ( 1  <<  log2TbHeight ) − 2 )
		locSumAbs  +=  AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, Max( locSumAbs − baseLevel * 5, 0 ) >> riceShift )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as the addition of the value of riceShift and the value specified in Table 128.

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ? 1 : 2 )  <<  cRiceParam	(1518)

Table 128 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

…

## CE-1.5

#### 7.3.11.11	Residual coding syntax

| … |  |
| --- | --- |
| for(sbType = chType << 2; sbType < chType << 2 + 4; sbType ++) { |  |
| lowerThreshold[ sbType ] = 1 << (StatCoeff[ sbType ] >> 4) |  |
| upperThreshold[ sbType ] = 3 << (StatCoeff[ sbType ] >> 4) |
| if (BitDepth > 10) { |  |
| RiceShiftNoParity[sbType] = Min(12, Max(0, (StatCoeff[ sbType ] + 8) >> 4 - 2)) |  |
| RiceShiftWithParity[sbType] = Max(0, RiceShiftNoParity[sbType] – 1) |  |
| } |  |
| } |  |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| sbType = chType << 2 + (transform_skip_flag[ xC ][ yC ][ chType ] ? 0 : xC = 0 && yC = 0 ? 1 : xC < 4 && yC < 4 ? 2 : 3) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) { |  |
| riceShift = riceShiftWithParity |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] += 2 * abs_remainder[ n ] |  |
| if (xC == yC) { |  |
| if( AbsLevel[xC][yC] - 4 >= upperThreshold[ sbType ] ) |  |
| StatCoeff[ sbType ]+=4 |  |
| else if( 2 * ( AbsLevel[ xC ][ yC ] - 4 ) < ( 1 << lowerThreshold [sbType] )) { |  |
| if( StatCoeff[ sbType ] > 0 ) |  |
| StatCoeff[ sbType ]− − |  |
| } |  |
| } |  |
| } |  |
| } |  |
| AbsLevel[ xC ][ yC ] += AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| sbType = chType << 2 + (transform_skip_flag[ xC ][ yC ][ chType ] ? 0 : xC = 0 && yC = 0 ? 1 : xC < 4 && yC < 4 ? 2 : 3) |  |
| if( sb_coded_flag[ xS ][ yS ] ) { |  |
| riceShift = riceShiftNoParity[sbType] |  |
| dec_abs_level[ n ] | ae(v) |
| } |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| if( xC == yC ) { |  |
| if( AbsLevel[xC][yC] >= upperThreshold[ sbType ] ) |  |
| StatCoeff[ sbType ]+=4 |  |
| else if( 2 * AbsLevel[ xC ][ yC ] ) < ( 1 << lowerThreshold[sbType])) |  |
| if( StatCoeff[ sbType ] > 0) |  |
| StatCoeff[ sbType ]− |  |
| } |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| if( sh_dep_quant_used_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| … |  |

…

## 9.3	CABAC parsing process for slice data

### 9.3.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in subclause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in subclause 7.3.11.2 and one or more of the following conditions are true:

–	The CTU is the first CTU in a slice.

–	The CTU is the first CTU in a tile.

–	The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

The storage process for context variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for context variables as specified in subclause 9.3.2.3 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as outputs.

The storage process for rice coding variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for rice coding variables is specified in subclause 9.3.2.8 is invoked with StatCoeffWpp as output.

When sps_palette_enabled_flag is equal to 1, the storage process for palette predictor is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2 and the decoding process of the last CU in the CTU in subclause 8.1.2, sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for palette predictor as speficied in subclause 9.3.2.6 is invoked.

…

### 9.3.2	Initialization process

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

Figure 12 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a slice or tile, the following applies: the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, if sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 12) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(1509)

–	The derivation process for neighbouring block availability as specified in subclause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the following applies:

–	The synchronization process for context variables as specified in subclause 9.3.2.4 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as inputs.

–	When sps_palette_enabled_flag is equal to 1, the synchronization process for palette predictor as specified in subclause 9.3.2.7 is invoked.

–	The synchronization process for rice coding as specified in subclause 9.3.2.9 is invoked.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

…

9.3.2.8	Storage process for rice coding

This process stores the values of the arrays StatCoeff, RiceShiftWithParity, RiceShiftNoParity, LowerThreshold and UpperThreshold in the arrays StatCoeffWpp, RiceShiftWithParityWpp, RiceShiftNoParityWpp, LowerThresholdWpp and UpperThresholdWpp as follows:

for( idx = 0; idx < 8; idx++ ) {

StatCoeffWpp[ idx ] = StatCoeff[ idx ]

}

9.3.2.9	Synchronization process for rice coding

This process synchronizes the values of the arrays PredictorPaletteSize and PredictorPaletteEntries in the arrays TablePaletteSizeWpp and TablePaletteEntriesWpp as follows:

for( idx = 0; idx < 8; idx++ ) {

StatCoeff[ idx ] = StatCoeffWpp[ idx ]

}

9.3.2.10	Initialization process for rice coding

The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 7, inclusive.

…

#### 9.3.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the rice shift used in the rice calculation, riceShift, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process:

locSumAbs = 0
if( xC < ( 1  <<  log2TbWidth ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC + 1 ][ yC ]
	if( xC < ( 1  <<  log2TbWidth ) − 2 )
		locSumAbs  +=  AbsLevel[ xC + 2 ][ yC ]
	if( yC < ( 1  <<  log2TbHeight ) − 1 )
		locSumAbs  +=  AbsLevel[ xC + 1 ][ yC + 1 ]	(1517)
}
if( yC < ( 1  <<  log2TbHeight ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC + 1 ]
	if( yC < ( 1  <<  log2TbHeight ) − 2 )
		locSumAbs  +=  AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, Max( locSumAbs − baseLevel * 5, 0 ) >> riceShift )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as the addition of the value of riceShift and the value specified in Table 128.

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ? 1 : 2 )  <<  cRiceParam	(1518)

Table 128 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

…

## CE-2.2

#### 7.3.11.11	Residual coding syntax

…

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && sb_coded_flag[ xS ][ yS ] ) ) { |  |  |
| riceShift = n > lastScanPosPass2 ? RiceShiftNoParity[ chType ] : RiceShiftWithParity[ chType ] |  |  |
| abs_remainder[ n ] | ae(v) |  |
| } |  |  |
| if( n <= lastScanPosPass2 ) { |  |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |  |
| offset = AbsLevelPass2[ xC ][ yC ] |  |  |
| } else if(n <= lastScanPosPass1 ) { |  |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |  |
| offset = AbsLevelPass2[ xC ][ yC ] |  |  |
| } else { /* bypass */ |  |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |  |
| offset = 0 |  |  |
| if( abs_remainder[ n ] ) |  |  |
| coeff_sign_flag[ n ] | ae(v) |  |
| } |  |  |
| if( AbsLevel[ xC ][ yC ] - offset > 0 ) { |  |
| if( (AbsLevel[xC][yC] – offset) >= upperThreshold ) |  |
| StatCoeff[ chType ]+=4 |  |
| else if( 2 * (AbsLevel[xC][yC] – offset) < lowerThreshold ) { |  |
| if( StatCoeff[ chType ] > 0 ) |  |
| StatCoeff[ chType ]− − |  |
| } |  |
| } |  |
| lowerThreshold[ chType ] = 1 << (StatCoeff[ chType ] >> 4) |  |
| upperThreshold[ chType ] = 3 << (StatCoeff[ chType ] >> 4) |
| if (BitDepth > 10) { |  |
| RiceShiftNoParity[ chType ] = Min(12, Max(0, (StatCoeff[ chType ] + 8) >> 4 - 2)) |  |
| RiceShiftWithParity[ chType ] = Max(0, RiceShiftNoParity[ chType ] – 1) |  |
| } |  |
| if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { |  |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] : 0 |  |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] : 0 |  |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |  |
| AbsLevel[ xC ][ yC ]− − |  |  |
| } |  |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |  |
| } |  |  |

…

## 9.3	CABAC parsing process for slice data

### 9.3.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in subclause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in subclause 7.3.11.2 and one or more of the following conditions are true:

–	The CTU is the first CTU in a slice.

–	The CTU is the first CTU in a tile.

–	The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

The storage process for context variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for context variables as specified in subclause 9.3.2.3 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as outputs.

The storage process for rice coding variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for rice coding variables is specified in subclause 9.3.2.8 is invoked with StatCoeffWpp as output.

When sps_palette_enabled_flag is equal to 1, the storage process for palette predictor is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2 and the decoding process of the last CU in the CTU in subclause 8.1.2, sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for palette predictor as speficied in subclause 9.3.2.6 is invoked.

…

### 9.3.2	Initialization process

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

Figure 12 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a slice or tile, the following applies: the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, if sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 12) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(1509)

–	The derivation process for neighbouring block availability as specified in subclause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the following applies:

–	The synchronization process for context variables as specified in subclause 9.3.2.4 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as inputs.

–	When sps_palette_enabled_flag is equal to 1, the synchronization process for palette predictor as specified in subclause 9.3.2.7 is invoked.

–	The synchronization process for rice coding as specified in subclause 9.3.2.9 is invoked.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

…

9.3.2.8	Storage process for rice coding

This process stores the values of the arrays StatCoeff, RiceShiftWithParity, RiceShiftNoParity, LowerThreshold and UpperThreshold in the arrays StatCoeffWpp, RiceShiftWithParityWpp, RiceShiftNoParityWpp, LowerThresholdWpp and UpperThresholdWpp as follows:

for( idx = 0; idx < 2; idx++ ) {

StatCoeffWpp[ idx ] = StatCoeff[ idx ]

RiceShiftWithParityWpp[ idx ] = RiceShiftWithParity[ idx ]

RiceShiftNoParityWpp[ idx ] = RiceShiftNoParity[ idx ]

LowerThresholdWpp[ idx ] = LowerThreshold[ idx ]

UpperThresholdWpp[ idx ] = UpperThreshold[ idx ]

}

9.3.2.9	Synchronization process for rice coding

This process synchronizes the values of the arrays PredictorPaletteSize and PredictorPaletteEntries in the arrays TablePaletteSizeWpp and TablePaletteEntriesWpp as follows:

for( idx = 0; idx < 2; idx++ ) {

StatCoeff[ idx ] = StatCoeff[ idx ]

RiceShiftWithParity[ idx ] = RiceShiftWithParityWpp[ idx ]

RiceShiftNoParity[ idx ] = RiceShiftNoParityWpp[ idx ]

LowerThreshold[ idx ] = LowerThresholdWpp[ idx ]

UpperThreshold[ idx ] = UpperThresholdWpp[ idx ]

}

9.3.2.10	Initialization process for rice coding

The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 1, inclusive.

The variables RiceShiftWithParity[ k ] are set equal to 0, for k in the range 0 to 1, inclusive.

The variables RiceShiftNoParity[ k ] are set equal to 0, for k in the range 0 to 1, inclusive.

The variables LowerThreshold[ k ] are set equal to 1, for k in the range 0 to 1, inclusive.

The variables UpperThreshold[ k ] are set equal to 3, for k in the range 0 to 1, inclusive.

…

#### 9.3.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the rice shift used in the rice calculation, riceShift, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

The variable initRiceValue is derived as follows:

initRiceValue = StatCoeff[ chType ] >> 4

The variable shift is derived as follows:

shift = Max(0, initRiceValue – 2)

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process:

If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0:

locSumAbs = 0
if( xC < ( 1  <<  log2TbWidth ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC + 1 ][ yC ]
	if( xC < ( 1  <<  log2TbWidth ) − 2 )
		locSumAbs  +=  AbsLevel[ xC + 2 ][ yC ]
	if( yC < ( 1  <<  log2TbHeight ) − 1 )
		locSumAbs  +=  AbsLevel[ xC + 1 ][ yC + 1 ]	(1517)
}
if( yC < ( 1  <<  log2TbHeight ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC + 1 ]
	if( yC < ( 1  <<  log2TbHeight ) − 2 )
		locSumAbs  +=  AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, Max( locSumAbs − baseLevel * 5, 0 ) >> riceShift )

Otherwise,

locSumAbs = 0
if( xC > 0 ) {
	locSumAbs  +=  AbsLevel[ xC - 1 ][ yC ]
	if( xC > 1 )
		locSumAbs  +=  AbsLevel[ xC - 2 ][ yC ]
	if( yC > 0 )
		locSumAbs  +=  AbsLevel[ xC - 1 ][ yC - 1 ]	(1517)
}
if( yC > 0 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC - 1 ]
	if( yC > 1 )
		locSumAbs  +=  AbsLevel[ xC ][ yC - 2 ]
} 
locSumAbs = Clip3( 0, 31, (locSumAbs − baseLevel * 5) >> riceShift )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as the addition of the value of riceShift and the value specified in Table 128.

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ? 1 : 2 )  <<  cRiceParam	(1518)

Table 128 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

…

#### 9.3.3.11	Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The variables lastAbsRemainder and lastRiceParam are derived as follows:

–	If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0.

–	Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this subclause.

The rice parameter cRiceParam is derived as follows:

–	If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0, the Rice parameter cRiceParam is set equal to 1. the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in subclause 9.3.3.2 with the variable baseLevel set equal to 0, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

–	Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in subclause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

…

## CE-2.3

#### 7.3.11.11	Residual coding syntax

…

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| lowerThreshold = 1 << (StatCoeff[ chType ] >> 4) |  |  |
| upperThreshold = 3 << (StatCoeff[ chType ] >> 4) |
| if (BitDepth > 10) { |  |
| RiceShiftNoParity = Min(12, Max(0, (StatCoeff[ chType ] + 8) >> 4 - 2)) |  |
| RiceShiftWithParity = Max(0, RiceShiftNoParity – 1) |  |
| } |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && sb_coded_flag[ xS ][ yS ] ) ) { |  |  |
| riceShift = n > lastScanPosPass2 ? RiceShiftNoParity : RiceShiftWithParity |  |  |
| abs_remainder[ n ] | ae(v) |  |
| } |  |  |
| if( n <= lastScanPosPass2 ) { |  |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |  |
| offset = AbsLevelPass2[ xC ][ yC ] |  |  |
| } else if(n <= lastScanPosPass1 ) { |  |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |  |
| offset = AbsLevelPass2[ xC ][ yC ] |  |  |
| } else { /* bypass */ |  |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |  |
| offset = 0 |  |  |
| if( abs_remainder[ n ] ) |  |  |
| coeff_sign_flag[ n ] | ae(v) |  |
| } |  |  |
| if ( xC == yC) { |  |  |
| if( AbsLevel[ xC ][ yC ] - offset > 0 ) { |  |
| if( (AbsLevel[xC][yC] – offset) >= upperThreshold ) |  |
| StatCoeff[ chType ]+=4 |  |
| else if( 2 * (AbsLevel[xC][yC] – offset) < lowerThreshold ) { |  |
| if( StatCoeff[ chType ] > 0 ) |  |
| StatCoeff[ chType ]− − |  |
| } |  |
| } |  |
| } |  |
| if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { |  |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] : 0 |  |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] : 0 |  |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |  |
| AbsLevel[ xC ][ yC ]− − |  |  |
| } |  |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

…

## 9.3	CABAC parsing process for slice data

### 9.3.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in subclause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in subclause 7.3.11.2 and one or more of the following conditions are true:

–	The CTU is the first CTU in a slice.

–	The CTU is the first CTU in a tile.

–	The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

The storage process for context variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for context variables as specified in subclause 9.3.2.3 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as outputs.

The storage process for rice coding variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for rice coding variables is specified in subclause 9.3.2.8 is invoked with StatCoeffWpp as output.

When sps_palette_enabled_flag is equal to 1, the storage process for palette predictor is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2 and the decoding process of the last CU in the CTU in subclause 8.1.2, sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for palette predictor as speficied in subclause 9.3.2.6 is invoked.

…

### 9.3.2	Initialization process

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

Figure 12 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a slice or tile, the following applies: the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, if sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 12) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(1509)

–	The derivation process for neighbouring block availability as specified in subclause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the following applies:

–	The synchronization process for context variables as specified in subclause 9.3.2.4 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as inputs.

–	When sps_palette_enabled_flag is equal to 1, the synchronization process for palette predictor as specified in subclause 9.3.2.7 is invoked.

–	The synchronization process for rice coding as specified in subclause 9.3.2.9 is invoked.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the initialization process for rice coding variables is invoked as specified in subclause 9.3.2.10, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

…

9.3.2.8	Storage process for rice coding

This process stores the values of the arrays StatCoeff, RiceShiftWithParity, RiceShiftNoParity, LowerThreshold and UpperThreshold in the arrays StatCoeffWpp, RiceShiftWithParityWpp, RiceShiftNoParityWpp, LowerThresholdWpp and UpperThresholdWpp as follows:

for( idx = 0; idx < 2; idx++ ) {

StatCoeffWpp[ idx ] = StatCoeff[ idx ]

}

9.3.2.9	Synchronization process for rice coding

This process synchronizes the values of the arrays PredictorPaletteSize and PredictorPaletteEntries in the arrays TablePaletteSizeWpp and TablePaletteEntriesWpp as follows:

for( idx = 0; idx < 2; idx++ ) {

StatCoeff[ idx ] = StatCoeffWpp[ idx ]

}

9.3.2.10	Initialization process for rice coding

The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 1, inclusive.

…

#### 9.3.3.11	Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), , the rice shift used in the rice calculation, riceShift  the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The variables lastAbsRemainder and lastRiceParam are derived as follows:

–	If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0.

–	Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this subclause.

The rice parameter cRiceParam is derived as follows:

–	If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0, the Rice parameter cRiceParam is set equal to the sum of riceShift and 1.

–	Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in subclause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

…

# Estimates of Complexity

This section attempts to give an estimate of the additional complexity of CE-1.5 (the simplification of CE-1.4) for both hardware and software. The additional complexity can be separated into four sections: coding a value, updating a counter, selecting a parameter set and updating a parameter set as follows:

## Coding a value (every coefficient)

| Additional complexity | Estimated hardware |
| --- | --- |
| 1 * shift (variable size) 1 * add | 1 * N-way multiplex 1 * N-bit adder |

## Updating a counter (maximum of 4 times in a 4x4 block)

| Additional complexity | Estimated hardware |
| --- | --- |
| 3 * conditionals, 2 * add 1 * shift (fixed size) | 1 * multiplex 2 * subtractor 1 * half adder 1 * half subtractor 1 * clip to zero |

## Selecting a parameter set (every coefficient)

| Additional complexity | Estimated hardware |
| --- | --- |
| 2 * conditional 1 * add | 1 * AND gate 1 * OR gate 1 * 5-input OR gate and inverter |

## Updating a parameter set (maximum of 3 times per 4x4 block)

| Additional complexity | Estimated hardware |
| --- | --- |
| 6 * adds 3 * shift (fixed size) 2 * shift (variable size) | 3 * add 1 * subtractor 2 * clip to zero 1 * clip to constant 2 * shift (variable size) |

# Conclusions

This proposal has described a series of experiments (CE-1.4, CE-1.5, CE-2.2 and CE-2.3) based on modifications to the method for the selection of Rice parameters for residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains when residual coefficient values become larger but do not affect version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. Two experiments, CE-1.4 and CE-1.5, modify RRC, whilst the other two, CE-2.2 and CE-2.3 target TSRC, with the second experiment in each category being a simplification of the first.

It is proposed that the modifications included in the described experiments are considered alongside other proposals in the CE for adoption into VVC version 2 and VTM.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “AHG12: Rice parameter selection for high bit depths”, JVET-T0072, A. Browne, S. Keating and K. Sharman

[2] “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, B. Bross, J. Chen, S. Liu and Y-K. Wang

[3] “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu

[4] “High bit depth coding”, JVET-R0351, A. Browne, S. Keating and K. Sharman

[5] “On operation beyond 10-bit”, JVET-S0243, A. Browne, S. Keating and K. Sharman