| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0058 |
| --- | --- |

| Title: | CE-3.2: Combination of CE-1.5 and CE-2.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document reports results for CE-3.2 that combines the regular residual coding modifications within CE-1.5 (JVET-U0057)[1] with the transform skip residual coding modifications in CE-2.1 (JVET-U0075)[2]. Both modifications change the method for the selection of Rice parameters for residual coding and are intended for consideration for high bit depth coding in VVC version 2. The aim of the modifications is to offer BD-rate gains for high bit depths when typically residual coefficient values become larger, whilst being backwardly compatible with the existing Rice coding techniques in VVC version 1(JVET-T2001)[3].

It is reported that average BD-rate differences (wPSNR for PQ, PSNR all others) with high bit depth CTC (JVET-T2018)[4] are as follows:

- PQ: 	-0.92%/-0.81%/-0.82% (AI), -0.07%/-0.09%/-0.12% (LDB), -0.18%/-0.17%/-0.19% (RA)

- HLG:	-0.63%/-0.47%/-0.46% (AI), -0.09%/-0.07%/-0.07% (LDB), -0.09%/-0.07%/-0.06% (RA)

- SVT-12:	-4.22%/-3.79%/-3.77% (AI), -1.09%/-1.04%/-1.03 % (LDB), -1.26%/-1.08%/-1.08% (RA)

- SVT-16:	-26.86%/-24.99%/-24.97% (AI), -17.76%/-16.88%/-16.89% (LDB), -16.83%/-16.91%/-16.85% (RA)

# Introduction

In VVC version 1[3], the range of Rice parameters that may be used for coefficients during regular residual coding is limited to the range of 0 to 3 (inclusive). This range has been found to be suitable for both 8 and 10 bit data; however, as bit depths increase and QPs become lower, the size of each coded coefficient becomes larger.

CE-1.5 represents a modification to the Rice parameter selection method in RRC, whilst CE-2.1 represents a modification to the Rice parameter selection method in TSRC. The aim of combining both modifications is to produce a more comprehensive method for Rice parameter selection for high bit depth coding in VVC version 2.

Version 2 of this document updated the results for HLG RA.

# Results

## PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.87% | -0.76% | -0.75% | -0.88% | -0.77% | -0.76% | 103% | 99% |
| PQ422 | -0.96% | -0.86% | -0.90% | -0.97% | -0.88% | -0.91% | 102% | 100% |
| Overall | -0.92% | -0.81% | -0.82% | -0.93% | -0.82% | -0.83% | 103% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.08% | -0.12% | -0.09% | -0.08% | -0.11% | 102% | 104% |
| PQ422 | -0.07% | -0.10% | -0.11% | -0.09% | -0.09% | -0.10% | 100% | 105% |
| Overall | -0.07% | -0.09% | -0.12% | -0.09% | -0.08% | -0.10% | 101% | 104% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.17% | -0.16% | -0.18% | -0.18% | -0.16% | -0.18% | 102% | 105% |
| PQ422 | -0.19% | -0.18% | -0.20% | -0.19% | -0.18% | -0.20% | 102% | 103% |
| Overall | -0.18% | -0.17% | -0.19% | -0.19% | -0.17% | -0.19% | 102% | 104% |
| Overall PQ | -0.39% | -0.36% | -0.38% | -0.40% | -0.36% | -0.38% | 102% | 103% |

## HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.61% | -0.43% | -0.42% | 101% | 100% |
| HLG422 | -0.66% | -0.50% | -0.51% | 102% | 102% |
| Overall | -0.63% | -0.47% | -0.46% | 102% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.07% | -0.07% | 102% | 103% |
| HLG422 | -0.10% | -0.07% | -0.07% | 103% | 99% |
| Overall | -0.09% | -0.07% | -0.07% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.07% | -0.06% | 102% | 99% |
| HLG422 | -0.11% | -0.08% | -0.07% | 102% | 102% |
| Overall | -0.10% | -0.07% | -0.07% | 102% | 100% |
| Overall HLG | -0.27% | -0.20% | -0.20% | 102% | 101% |

## SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -26.86% | -24.99% | -24.97% | 106% | 104% |
| SVT12 | -4.22% | -3.79% | -3.77% | 104% | 108% |
| Overall | -15.54% | -14.39% | -14.37% | 105% | 106% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -17.76% | -16.88% | -16.89% | 106% | 108% |
| SVT12 | -1.09% | -1.04% | -1.03% | 101% | 107% |
| Overall | -9.43% | -8.96% | -8.96% | 103% | 107% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.83% | -16.91% | -16.85% | 106% | 105% |
| SVT12 | -1.26% | -1.08% | -1.08% | 101% | 106% |
| Overall | -9.04% | -9.00% | -8.96% | 104% | 106% |
| Overall RGB | -11.34% | -10.78% | -10.77% | 104% | 106% |

## Lossless

As lossless coding uses RRC for coding transform skip residuals, please refer to the results for CE-1.5.

# Draft Text

For draft text changes please refer to JVET-U0057 (CE-1.5) and JVET-U0075 (CE-2.1)

# Conclusions

This proposal has described an experiment that combines two modifications, CE-1.5 and CE-2.1, to the method for the selection of Rice parameters for residual coding. The modifications offer BD-rate gains when residual coefficient values become larger but do not affect version 1 operating points.  It is suggested that the combination of modifications extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths.

It is proposed that the combination of modifications is considered alongside other proposals in the CE for adoption into VVC version 2 and VTM.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “CE-1.4, CE-1.5, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, JVET-U0057, A. Browne, S. Keating and K. Sharman

[2] “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-U0075, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang

[3] “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, B. Bross, J. Chen, S. Liu and Y-K. Wang

[4] “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu