| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0063-v2 |
| --- | --- |

| Title: | A constraint of max transform size for high bit depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo Masaru Ikeda Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-70-7598-7407 Kenji.Kondo@sony.com Masaru.Ikeda@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to restrict max transform size for high bit depth. This constraint helps to reduce memory size in hardware architecture. To know the impact of coding efficiency, experiments to restrict transform size 64 is carried out. For AI test case, it is reported that the simulation results are {0.02%, 0.08%, 0.09%} for PQ contents, {0.05%, -0.03%, 0.03%} for HLG contents and {0.00%, 0.00%, 0.00%} for SVT RGB contents.

Introduction

In the last meeting, it was discussed about potential extensions of VVC. One of them was high bit depth coding. Generally, if we encode contents with high bit depth, lower QPs will be used to keep high accuracy of input. Thus, in a contribution JVET-T2018 [1], the common test condition (CTC) for high bit depth mandates to use low QPs as below.

- 12-bit: 12, 7, 2, -3, -8 and -13

- 16-bit: -8, -13, -18, -23, -28 and -33

On the other hand, in a hardware architecture, the max size of transform is important factor because a memory is needed as corresponding to the transform size. Consequently, the max transform size affects LSI cost. And it’s desirable for LSI manufacturing cost if the max transform size is restricted smaller.

In a specification of VVC version-1 [2], the max transform size is 64-point. And, the 64-point transformation process uses a technique so called zero-out. This zero-out can reduce a computational cost, but it forces high frequency coefficients to zero. However, it is not desirable for high bit depth and low QP because quantization step is small at low QP, and it often make high frequency coefficients non-zero.

Since above situations, for complexity reduction for high bit depth coding, this contribution proposes to restrict max transform size.

Experiment

To investigate an impact of coding efficiency, we tested to prohibit a 64-point transformation by setting Log2MaxTbSize to 5 in VTM software (VTM-11.0). In this test, a common test condition for high bit depth [1] was followed, and additionally contents are added that is used in a core experiment, on entropy coding for high bit depth and high bit rate coding [3].

Experimental results

The test results of BD rate are shown in Table 1, Table 2 and Table 3 for PQ contents, HLG contents and SVT RGB contents. As shown in the results, the coding performance loss is very small. In CTC condition for high bit depth, low QP sets are used. In this condition, it’s expected that a large transform as 64-point doesn’t improve coding efficiency so much. Consequently, even without 64-point transform, the coding loss is very small.

For SVT RGB contents, the all of BD rate are zero. This is because the anchor uses Log2MaxTbSize to 5 in RGB contents.

Table 1: Test results for PQ contents.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.04% | 0.05% | 0.01% | 0.04% | 0.04% | 99% | 100% |
| PQ422 | 0.02% | 0.11% | 0.14% | 0.02% | 0.09% | 0.11% | 100% | 101% |
| Overall | 0.02% | 0.08% | 0.09% | 0.01% | 0.06% | 0.07% | 99% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.02% | -0.01% | 0.01% | 0.01% | -0.01% | 102% | 100% |
| PQ422 | 0.01% | 0.01% | 0.03% | 0.01% | 0.01% | 0.03% | 101% | 100% |
| Overall | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.01% | -0.01% | 0.00% | 0.01% | -0.01% | 101% | 100% |
| PQ422 | 0.01% | 0.02% | 0.01% | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Overall | 0.01% | 0.02% | 0.00% | 0.00% | 0.01% | 0.00% | 101% | 100% |
| Overall PQ | 0.01% | 0.04% | 0.03% | 0.01% | 0.03% | 0.03% | 101% | 100% |

Table 2: Test results for HLG contents.

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.07% | -0.06% | 0.02% | 101% | 101% |
| HLG422 | 0.03% | -0.01% | 0.04% | 99% | 100% |
| Overall | 0.05% | -0.03% | 0.03% | 100% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | 0.04% | 0.05% | 101% | 101% |
| HLG422 | -0.02% | 0.05% | 0.01% | 102% | 100% |
| Overall | -0.03% | 0.04% | 0.03% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | 0.07% | 0.07% | 101% | 100% |
| HLG422 | -0.01% |  | 0.04% |  | 0.01% |  | 101% | 101% |  |
| Overall | -0.02% |  | 0.06% |  | 0.04% |  | 101% | 101% |  |
| Overall HLG | 0.00% |  | 0.02% |  | 0.03% |  | 101% | 100% |  |

Table 3: Test results for SVT RGB contents.

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 100% | 100% |

Conclusion

This contribution introduced that a usage of large transform increases hardware cost. And, for high bit depth contents, low QP sets are generally used to keep high accuracy bit depth. When low QPs are used, large transform is not so necessary to get coding performance. To show the impact of coding performance, the test results to restrict transform size 64 was reported, and the coding efficiency losses were negligible. With the evidence, this contribution proposes to restrict the transform size 64 for high bit depth contents.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- Adrian Browne, Tomohiro Ikai, D. Rusanovskyy, Xiaoyu Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, Oct 2020

- Benjamin Bross, Jianle Chen, Shan Liu, Ye-Kui Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001-v2, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, Oct 2020

- Adrian Browne, Tomonori Hashimoto, Hong-Jheng Jhu, Dmytro Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-T2022, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, Oct 2020