| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0071-v2 |
| --- | --- |

| Title: | Performance of a VVC Software Decoder on Mobile Platform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yiming Li Jian Lou Shan Liu Yu Chen Yushan Zheng Sijia Chen Bin Zhu | Tel: Email: | marcli@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

An independent VVC software decoder implemented by Tencent was demonstrated in [1]. Based on this version, an optimized implementation based on Arm Neon technology is presented in this contribution. Currently, this VVC software decoder can support the real-time decoding on the mobile platform. The performance is provided on an Apple A14 processor (iPhone 12pro) by decoding the 8bit CTC bitstreams generated by VTM-11.0 encoder. It is reported that on the single thread, the average decoding speed is 53.0 fps (RA)/ 50.3 fps (LB) for full HD (1080p) bitstreams; 81.5 fps (RA)/ 88.4 fps (LB) for full HD (1080p) SCC bitstreams. When multithreading is enabled, an average of 31.6 fps (RA) can be achieved when decoding 4K bitstreams. When decoding 2K bitstreams, an average of 129.0 fps (RA)/ 102.1 fps (LB) can be achieved for HD (1080p) bitstreams; an average of 180.0 fps (RA)/ 168.0 fps (LB) can be achieved for full HD (1080p) SCC bitstreams. Test results based on different mobile platforms are also provided.

Introduction

Upon the completion of VVC standard, the implementability of such a new codec format remains a challenge to its wide deployment in the industry, especially on the mobile platform. As an early stage attempt, an independent VVC software decoder has been developed by Tencent, trying to demonstrate the capability of efficient decoding VVC bitstreams. The performance is provided by decoding the 8bit CTC bitstreams generated by VTM encoder, i.e., the bitstreams are generated following JVET-T2010 by setting “InternalBitDepth” to 8. The tests are conducted on different devices, including iPhone 12pro (Apple A14), iPhone 11pro (Apple A13), iPhone Xs Max (Apple A12), and Vivo IQOO3 (Snapdragon 865).

Results

Per-sequence Results on IOS platform

Table I below shows the results of decoding 4K bitstreams (generated with VTM version 11.0, RA configuration). The average acceleration factor over VTM decoder is 6X (11.49/1.82) with single thread setting. For showing the performance in different platforms, the results in A13 and A12 processer are also provided.

Table I: decoding speed comparison among VTM and Tencent O266 decoders, 4K CTC streams (RA)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 1 [fps] (A13) | O266 Thread 1 [fps] (A12) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tango | 22 | 294 | 16.57 | 1.90 | 10.51 | 8.07 | 6.71 |
|  | 27 | 294 | 6.74 | 2.10 | 12.07 | 9.16 | 7.75 |
|  | 32 | 294 | 3.43 | 2.18 | 12.96 | 9.83 | 8.30 |
|  | 37 | 294 | 1.93 | 2.49 | 13.92 | 10.61 | 8.96 |
| FoodMarket | 22 | 300 | 14.66 | 1.85 | 10.63 | 8.04 | 6.75 |
|  | 27 | 300 | 7.16 | 1.75 | 11.50 | 8.74 | 7.31 |
|  | 32 | 300 | 3.66 | 2.15 | 12.33 | 9.39 | 7.83 |
|  | 37 | 300 | 1.97 | 2.67 | 14.15 | 10.94 | 9.08 |
| Campfire | 22 | 300 | 42.33 | 1.65 | 7.99 | 6.50 | 5.19 |
|  | 27 | 300 | 12.44 | 2.15 | 11.21 | 9.10 | 7.57 |
|  | 32 | 300 | 6.54 | 2.51 | 13.12 | 10.80 | 9.00 |
|  | 37 | 300 | 3.47 | 2.78 | 14.66 | 12.18 | 10.19 |
| CatRobot | 22 | 300 | 17.45 | 1.79 | 10.28 | 7.89 | 6.46 |
|  | 27 | 300 | 7.85 | 1.92 | 11.55 | 8.85 | 7.40 |
|  | 32 | 300 | 3.91 | 2.16 | 12.38 | 9.53 | 7.96 |
|  | 37 | 300 | 2.12 | 3.13 | 15.83 | 12.32 | 10.21 |
| DaylightRoad | 22 | 300 | 22.73 | 1.68 | 9.50 | 7.45 | 6.17 |
|  | 27 | 300 | 7.89 | 1.87 | 11.30 | 8.81 | 7.41 |
|  | 32 | 300 | 3.78 | 2.22 | 12.29 | 9.29 | 7.87 |
|  | 37 | 300 | 1.97 | 2.33 | 13.41 | 10.30 | 8.51 |
| ParkRunning | 22 | 300 | 95.98 | 1.23 | 6.26 | 5.09 | 4.18 |
|  | 27 | 300 | 37.91 | 1.47 | 7.96 | 6.31 | 4.88 |
|  | 32 | 300 | 16.59 | 1.52 | 9.18 | 7.12 | 5.90 |
|  | 37 | 300 | 7.47 | 1.74 | 10.73 | 8.43 | 6.87 |
| Average |  |  |  | 1.82 | 11.49 | 8.95 | 7.44 |

Table II and III below show the results of decoding 2K bitstreams (generated with VTM version 11.0, RA configuration). For screen content bitstream generation, the SCC CE test condition is used. It is shown that decoding speed can meet the real-time requirement which is higher than 30fps with single thread. At the similar bitrate, it was also observed that decoder can achieve higher performance(fps) while decoding SCC sequences compared with non-SCC ones.

Table II: decoding speed comparison among VTM and Tencent O266 decoders, 2K CTC streams (RA)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 1 [fps] (A13) | O266 Thread 1 [fps] (A12) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MarketPlace | 22 | 600 | 11.49 | 7.05 | 39.19 | 30.27 | 23.99 |
|  | 27 | 600 | 4.68 | 8.30 | 45.42 | 35.02 | 27.60 |
|  | 32 | 600 | 2.10 | 9.04 | 50.69 | 38.98 | 30.61 |
|  | 37 | 600 | 0.96 | 11.50 | 60.83 | 47.30 | 37.27 |
| RitualDance | 22 | 600 | 8.96 | 7.25 | 42.46 | 32.65 | 26.24 |
|  | 27 | 600 | 4.47 | 8.43 | 49.95 | 38.26 | 28.46 |
|  | 32 | 600 | 2.34 | 9.69 | 57.82 | 44.31 | 35.64 |
|  | 37 | 600 | 1.25 | 11.30 | 66.97 | 51.65 | 41.19 |
| Cactus | 22 | 500 | 8.98 | 7.87 | 45.08 | 34.92 | 25.86 |
|  | 27 | 500 | 3.50 | 9.25 | 56.80 | 43.84 | 33.69 |
|  | 32 | 500 | 1.68 | 11.14 | 67.54 | 52.30 | 39.15 |
|  | 37 | 500 | 0.85 | 12.80 | 73.91 | 57.91 | 52.48 |
| BasketballDrive | 22 | 500 | 10.39 | 7.31 | 39.66 | 30.48 | 23.19 |
|  | 27 | 500 | 4.17 | 7.30 | 47.40 | 36.20 | 28.39 |
|  | 32 | 500 | 2.00 | 9.14 | 53.14 | 40.50 | 31.17 |
|  | 37 | 500 | 1.03 | 9.55 | 53.11 | 40.82 | 34.96 |
| BQTerrace | 22 | 600 | 17.90 | 6.51 | 34.10 | 26.61 | 20.71 |
|  | 27 | 600 | 3.49 | 9.23 | 51.32 | 39.39 | 31.18 |
|  | 32 | 600 | 1.44 | 10.34 | 58.48 | 44.62 | 35.69 |
|  | 37 | 600 | 0.70 | 11.80 | 65.68 | 50.38 | 40.63 |
| Average |  |  |  | 9.24 | 52.98 | 40.82 | 32.40 |

Table III: decoding speed comparison among VTM and Tencent O266 decoders, 2K SCC streams (RA)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 1 [fps] (A13) | O266 Thread 1 [fps] (A12) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ArenaOfValor | 22 | 600 | 11.66 | 7.63 | 41.98 | 32.22 | 25.44 |
|  | 27 | 600 | 5.05 | 8.73 | 52.65 | 39.62 | 31.69 |
|  | 32 | 600 | 2.40 | 9.88 | 61.35 | 48.81 | 39.13 |
|  | 37 | 600 | 1.21 | 12.67 | 76.40 | 60.18 | 48.05 |
| FlyingGraphics | 22 | 300 | 23.10 | 6.71 | 32.81 | 26.14 | 20.30 |
|  | 27 | 300 | 12.17 | 7.52 | 39.72 | 31.00 | 24.12 |
|  | 32 | 300 | 6.99 | 8.17 | 45.79 | 34.56 | 28.01 |
|  | 37 | 300 | 4.03 | 8.83 | 51.70 | 38.50 | 30.34 |
| Desktop | 22 | 600 | 1.98 | 13.58 | 94.71 | 68.50 | 56.15 |
|  | 27 | 600 | 1.47 | 17.50 | 106.87 | 78.59 | 62.89 |
|  | 32 | 600 | 1.15 | 21.14 | 124.94 | 91.82 | 77.22 |
|  | 37 | 600 | 0.92 | 23.66 | 137.67 | 100.49 | 82.61 |
| Console | 22 | 600 | 3.64 | 11.98 | 83.09 | 59.81 | 49.59 |
|  | 27 | 600 | 2.70 | 11.86 | 88.71 | 63.53 | 52.27 |
|  | 32 | 600 | 2.03 | 12.50 | 91.85 | 65.54 | 54.28 |
|  | 37 | 600 | 1.54 | 15.40 | 105.85 | 76.64 | 63.00 |
| ChineseEditing | 22 | 600 | 3.89 | 12.01 | 78.10 | 56.64 | 47.64 |
|  | 27 | 600 | 2.86 | 14.00 | 87.81 | 63.77 | 52.50 |
|  | 32 | 600 | 2.08 | 18.56 | 108.61 | 79.97 | 65.00 |
|  | 37 | 600 | 1.49 | 20.78 | 120.16 | 88.20 | 71.59 |
| Average |  |  |  | 13.16 | 81.54 | 60.23 | 49.09 |

For Low-Delay B configuration (LB), the experiment results are shown in Table IV and Table V. The average decoding speed is similar to RA configuration.

Table IV: decoding speed comparison among VTM and Tencent O266 decoders, 2K CTC streams (LB)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 1 [fps] (A13) | O266 Thread 1 [fps] (A12) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MarketPlace | 22 | 600 | 14.96 | 6.12 | 34.43 | 28.97 | 21.58 |
|  | 27 | 600 | 5.20 | 6.96 | 41.75 | 35.11 | 26.57 |
|  | 32 | 600 | 2.20 | 8.02 | 49.78 | 41.69 | 31.75 |
|  | 37 | 600 | 0.96 | 9.54 | 59.62 | 48.70 | 37.64 |
| RitualDance | 22 | 600 | 9.92 | 5.87 | 39.25 | 31.41 | 25.94 |
|  | 27 | 600 | 4.69 | 7.15 | 44.69 | 37.34 | 27.83 |
|  | 32 | 600 | 2.40 | 8.4 | 27.18 | 44.38 | 33.72 |
|  | 37 | 600 | 1.28 | 10.22 | 65.18 | 52.37 | 41.11 |
| Cactus | 22 | 500 | 14.53 | 6.46 | 37.04 | 31.37 | 22.80 |
|  | 27 | 500 | 3.80 | 7.91 | 53.68 | 44.42 | 33.51 |
|  | 32 | 500 | 1.68 | 9.56 | 67.67 | 55.58 | 42.08 |
|  | 37 | 500 | 0.82 | 11.31 | 79.31 | 66.03 | 50.39 |
| BasketballDrive | 22 | 500 | 15.14 | 5.97 | 32.70 | 27.79 | 20.18 |
|  | 27 | 500 | 4.72 | 6.99 | 43.97 | 36.08 | 26.77 |
|  | 32 | 500 | 2.17 | 7.97 | 51.65 | 41.46 | 31.06 |
|  | 37 | 500 | 1.10 | 9.37 | 58.92 | 48.40 | 36.44 |
| BQTerrace | 22 | 600 | 37.98 | 5.44 | 25.87 | 21.40 | 17.49 |
|  | 27 | 600 | 4.35 | 8.25 | 52.54 | 43.16 | 32.47 |
|  | 32 | 600 | 1.29 | 9.82 | 65.22 | 52.86 | 40.16 |
|  | 37 | 600 | 0.53 | 11.64 | 76.36 | 61.14 | 46.99 |
| Average |  |  |  | 8.15 | 50.34 | 42.48 | 32.32 |

Table V: decoding speed comparison among VTM and Tencent O266 decoders, 2K SCC streams (LB)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 1 [fps] (A13) | O266 Thread 1 [fps] (A12) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ArenaOfValor | 22 | 600 | 12.29 | 5.55 | 40.36 | 31.49 | 23.29 |
|  | 27 | 600 | 4.69 | 8.16 | 52.24 | 40.31 | 30.05 |
|  | 32 | 600 | 2.06 | 8.07 | 64.13 | 49.06 | 36.65 |
|  | 37 | 600 | 1.00 | 12.66 | 78.04 | 59.48 | 44.73 |
| FlyingGraphics | 22 | 300 | 25.70 | 6.36 | 33.43 | 25.99 | 19.11 |
|  | 27 | 300 | 12.53 | 6.13 | 41.63 | 32.10 | 23.64 |
|  | 32 | 300 | 7.04 | 7.84 | 47.49 | 36.40 | 27.42 |
|  | 37 | 300 | 4.08 | 8.57 | 53.90 | 41.23 | 31.33 |
| Desktop | 22 | 600 | 1.09 | 10.16 | 93.55 | 69.83 | 54.00 |
|  | 27 | 600 | 0.83 | 16.51 | 112.76 | 85.47 | 66.59 |
|  | 32 | 600 | 0.69 | 25.72 | 152.79 | 119.86 | 90.21 |
|  | 37 | 600 | 0.58 | 32.65 | 173.61 | 138.54 | 102.35 |
| Console | 22 | 600 | 3.29 | 5.68 | 78.26 | 58.83 | 46.55 |
|  | 27 | 600 | 2.48 | 10.77 | 83.64 | 63.04 | 48.73 |
|  | 32 | 600 | 1.87 | 13.35 | 90.13 | 68.18 | 53.16 |
|  | 37 | 600 | 1.44 | 14.29 | 97.66 | 74.29 | 62.51 |
| ChineseEditing | 22 | 600 | 2.33 | 11.75 | 84.06 | 62.69 | 49.91 |
|  | 27 | 600 | 1.71 | 14.36 | 100.93 | 76.36 | 58.59 |
|  | 32 | 600 | 1.21 | 23.67 | 136.21 | 105.88 | 79.91 |
|  | 37 | 600 | 0.83 | 27.86 | 153.69 | 120.60 | 100.45 |
| Average |  |  |  | 13.51 | 88.43 | 67.98 | 52.46 |

The multithreading results are shown from Table VI to Table XI. On A14 platform, the core number is 6, thus we provide the results of 4K bitstream on thread-1, thread-2, and thread-6 for showing the multi-core decoding performance. From the results, some 4K stream can achieve 30fps in A14 platform. The average acceleration factor over VTM decoder is 17X (31.55/1.82).

Table VI: decoding speed comparison among VTM and Tencent O266 decoders, 4K CTC streams (RA)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 2 [fps] (A14) | O266 Thread 6 [fps] (A14) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tango | 22 | 294 | 16.57 | 1.90 | 10.51 | 17.26 | 28.69 |
|  | 27 | 294 | 6.74 | 2.10 | 12.07 | 19.95 | 32.73 |
|  | 32 | 294 | 3.43 | 2.18 | 12.96 | 21.37 | 35.25 |
|  | 37 | 294 | 1.93 | 2.49 | 13.92 | 23.03 | 37.59 |
| FoodMarket | 22 | 300 | 14.66 | 1.85 | 10.63 | 17.57 | 28.81 |
|  | 27 | 300 | 7.16 | 1.75 | 11.50 | 18.99 | 31.29 |
|  | 32 | 300 | 3.66 | 2.15 | 12.33 | 20.39 | 33.48 |
|  | 37 | 300 | 1.97 | 2.67 | 14.15 | 23.53 | 38.52 |
| Campfire | 22 | 300 | 42.33 | 1.65 | 7.99 | 13.77 | 23.09 |
|  | 27 | 300 | 12.44 | 2.15 | 11.21 | 19.16 | 32.34 |
|  | 32 | 300 | 6.54 | 2.51 | 13.12 | 22.36 | 37.63 |
|  | 37 | 300 | 3.47 | 2.78 | 14.66 | 25.08 | 41.93 |
| CatRobot | 22 | 300 | 17.45 | 1.79 | 10.28 | 17.04 | 27.80 |
|  | 27 | 300 | 7.85 | 1.92 | 11.55 | 19.21 | 31.57 |
|  | 32 | 300 | 3.91 | 2.16 | 12.38 | 20.61 | 33.70 |
|  | 37 | 300 | 2.12 | 3.13 | 15.83 | 26.83 | 42.81 |
| DaylightRoad | 22 | 300 | 22.73 | 1.68 | 9.50 | 15.80 | 25.70 |
|  | 27 | 300 | 7.89 | 1.87 | 11.30 | 18.83 | 30.67 |
|  | 32 | 300 | 3.78 | 2.22 | 12.29 | 20.27 | 33.26 |
|  | 37 | 300 | 1.97 | 2.33 | 13.41 | 22.38 | 36.41 |
| ParkRunning | 22 | 300 | 95.98 | 1.23 | 6.26 | 10.61 | 17.52 |
|  | 27 | 300 | 37.91 | 1.47 | 7.96 | 13.16 | 21.83 |
|  | 32 | 300 | 16.59 | 1.52 | 9.18 | 14.99 | 25.21 |
|  | 37 | 300 | 7.47 | 1.74 | 10.73 | 17.77 | 29.28 |
| Average |  |  |  | 1.82 | 11.49 | 19.17 | 31.55 |

Table VII: decoding speed comparison among VTM and Tencent O266 decoders, 2K CTC streams (RA)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 2 [fps] (A14) | O266 Thread 6 [fps] (A14) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MarketPlace | 22 | 600 | 11.49 | 7.05 | 39.19 | 61.97 | 98.42 |
|  | 27 | 600 | 4.68 | 8.30 | 45.42 | 71.08 | 112.70 |
|  | 32 | 600 | 2.1 | 9.04 | 50.69 | 82.29 | 124.22 |
|  | 37 | 600 | 0.96 | 11.50 | 60.83 | 100.93 | 144.14 |
| RitualDance | 22 | 600 | 8.96 | 7.25 | 42.46 | 68.26 | 107.86 |
|  | 27 | 600 | 4.47 | 8.43 | 49.95 | 78.47 | 123.60 |
|  | 32 | 600 | 2.34 | 9.69 | 57.82 | 94.49 | 139.98 |
|  | 37 | 600 | 1.25 | 11.30 | 66.97 | 111.35 | 158.70 |
| Cactus | 22 | 500 | 8.98 | 7.87 | 45.08 | 68.47 | 113.40 |
|  | 27 | 500 | 3.5 | 9.25 | 56.80 | 89.49 | 138.83 |
|  | 32 | 500 | 1.68 | 11.14 | 67.54 | 104.27 | 160.20 |
|  | 37 | 500 | 0.85 | 12.80 | 73.91 | 141.21 | 171.40 |
| BasketballDrive | 22 | 500 | 10.39 | 7.31 | 39.66 | 61.51 | 101.89 |
|  | 27 | 500 | 4.17 | 7.30 | 47.40 | 73.61 | 118.63 |
|  | 32 | 500 | 2 | 9.14 | 53.14 | 84.80 | 130.99 |
|  | 37 | 500 | 1.03 | 9.55 | 53.11 | 92.69 | 130.62 |
| BQTerrace | 22 | 600 | 17.9 | 6.51 | 34.10 | 54.60 | 86.96 |
|  | 27 | 600 | 3.49 | 9.23 | 51.32 | 80.83 | 124.28 |
|  | 32 | 600 | 1.44 | 10.34 | 58.48 | 94.37 | 138.98 |
|  | 37 | 600 | 0.7 | 11.80 | 65.68 | 106.62 | 154.99 |
| Average |  |  |  | 9.24 | 52.98 | 86.07 | 129.04 |

Table VIII: decoding speed comparison among VTM and Tencent O266 decoders, 2K SCC streams (RA)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 2 [fps] (A14) | O266 Thread 6 [fps] (A14) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ArenaOfValor | 22 | 600 | 11.66 | 7.63 | 41.98 | 63.70 | 95.90 |
|  | 27 | 600 | 5.05 | 8.73 | 52.65 | 78.72 | 118.00 |
|  | 32 | 600 | 2.4 | 9.88 | 61.35 | 88.36 | 135.90 |
|  | 37 | 600 | 1.21 | 12.67 | 76.40 | 113.38 | 167.41 |
| FlyingGraphics | 22 | 300 | 23.1 | 6.71 | 32.81 | 55.94 | 90.45 |
|  | 27 | 300 | 12.17 | 7.52 | 39.72 | 59.69 | 88.07 |
|  | 32 | 300 | 6.99 | 8.17 | 45.79 | 75.07 | 112.64 |
|  | 37 | 300 | 4.03 | 8.83 | 51.70 | 80.60 | 125.71 |
| Desktop | 22 | 600 | 1.98 | 13.58 | 94.71 | 144.18 | 207.82 |
|  | 27 | 600 | 1.47 | 17.50 | 106.87 | 167.40 | 233.11 |
|  | 32 | 600 | 1.15 | 21.14 | 124.94 | 198.26 | 270.21 |
|  | 37 | 600 | 0.92 | 23.66 | 137.67 | 216.78 | 294.51 |
| Console | 22 | 600 | 3.64 | 11.98 | 83.09 | 127.88 | 184.67 |
|  | 27 | 600 | 2.7 | 11.86 | 88.71 | 136.15 | 198.16 |
|  | 32 | 600 | 2.03 | 12.50 | 91.85 | 138.84 | 201.19 |
|  | 37 | 600 | 1.54 | 15.40 | 105.85 | 163.06 | 228.49 |
| ChineseEditing | 22 | 600 | 3.89 | 12.01 | 78.10 | 118.86 | 170.68 |
|  | 27 | 600 | 2.86 | 14.00 | 87.81 | 132.67 | 192.47 |
|  | 32 | 600 | 2.08 | 18.56 | 108.61 | 168.54 | 229.02 |
|  | 37 | 600 | 1.49 | 20.78 | 120.16 | 188.87 | 255.60 |
| Average |  |  |  | 13.16 | 81.54 | 125.85 | 180.00 |

Table IX: decoding speed comparison among VTM and Tencent O266 decoders, 2K CTC streams (LB)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 2 [fps] (A14) | O266 Thread 6 [fps] (A14) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MarketPlace | 22 | 600 | 14.96 | 6.12 | 34.43 | 56.14 | 74.88 |
|  | 27 | 600 | 5.20 | 6.96 | 41.75 | 62.15 | 82.68 |
|  | 32 | 600 | 2.20 | 8.02 | 49.78 | 71.06 | 94.03 |
|  | 37 | 600 | 0.96 | 9.54 | 59.62 | 103.23 | 127.36 |
| RitualDance | 22 | 600 | 9.92 | 5.87 | 39.25 | 64.86 | 84.72 |
|  | 27 | 600 | 4.69 | 7.15 | 44.69 | 68.18 | 87.68 |
|  | 32 | 600 | 2.40 | 8.4 | 27.18 | 77.31 | 99.80 |
|  | 37 | 600 | 1.28 | 10.22 | 65.18 | 109.31 | 125.52 |
| Cactus | 22 | 500 | 14.53 | 6.46 | 37.04 | 58.15 | 75.64 |
|  | 27 | 500 | 3.80 | 7.91 | 53.68 | 87.77 | 107.32 |
|  | 32 | 500 | 1.68 | 9.56 | 67.67 | 117.84 | 138.47 |
|  | 37 | 500 | 0.82 | 11.31 | 79.31 | 128.83 | 153.56 |
| BasketballDrive | 22 | 500 | 15.14 | 5.97 | 32.70 | 49.96 | 68.25 |
|  | 27 | 500 | 4.72 | 6.99 | 43.97 | 64.67 | 91.73 |
|  | 32 | 500 | 2.17 | 7.97 | 51.65 | 83.19 | 108.65 |
|  | 37 | 500 | 1.10 | 9.37 | 58.92 | 87.34 | 110.13 |
| BQTerrace | 22 | 600 | 37.98 | 5.44 | 25.87 | 42.58 | 59.62 |
|  | 27 | 600 | 4.35 | 8.25 | 52.54 | 74.17 | 97.80 |
|  | 32 | 600 | 1.29 | 9.82 | 65.22 | 88.65 | 115.72 |
|  | 37 | 600 | 0.53 | 11.64 | 76.36 | 108.24 | 138.99 |
| Average |  |  |  | 8.15 | 50.34 | 80.18 | 102.13 |

Table X: decoding speed comparison among VTM and Tencent O266 decoders, 2K SCC streams (LB)

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] (A14) | O266 Thread 2 [fps] (A14) | O266 Thread 6 [fps] (A14) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ArenaOfValor | 22 | 600 | 12.29 | 5.55 | 40.36 | 63.83 | 82.54 |
|  | 27 | 600 | 4.69 | 8.16 | 52.24 | 83.52 | 103.73 |
|  | 32 | 600 | 2.06 | 8.07 | 64.13 | 102.16 | 124.38 |
|  | 37 | 600 | 1.00 | 12.66 | 78.04 | 124.87 | 149.25 |
| FlyingGraphics | 22 | 300 | 25.70 | 6.36 | 33.43 | 55.65 | 72.27 |
|  | 27 | 300 | 12.53 | 6.13 | 41.63 | 68.81 | 87.11 |
|  | 32 | 300 | 7.04 | 7.84 | 47.49 | 78.19 | 97.37 |
|  | 37 | 300 | 4.08 | 8.57 | 53.90 | 88.08 | 109.37 |
| Desktop | 22 | 600 | 1.09 | 10.16 | 93.55 | 149.14 | 170.70 |
|  | 27 | 600 | 0.83 | 16.51 | 112.76 | 182.54 | 206.11 |
|  | 32 | 600 | 0.69 | 25.72 | 152.79 | 258.73 | 291.26 |
|  | 37 | 600 | 0.58 | 32.65 | 173.61 | 297.91 | 337.08 |
| Console | 22 | 600 | 3.29 | 5.68 | 78.26 | 124.22 | 145.60 |
|  | 27 | 600 | 2.48 | 10.77 | 83.64 | 132.86 | 153.73 |
|  | 32 | 600 | 1.87 | 13.35 | 90.13 | 143.95 | 165.52 |
|  | 37 | 600 | 1.44 | 14.29 | 97.66 | 156.94 | 180.40 |
| ChineseEditing | 22 | 600 | 2.33 | 11.75 | 84.06 | 132.28 | 151.59 |
|  | 27 | 600 | 1.71 | 14.36 | 100.93 | 161.99 | 184.90 |
|  | 32 | 600 | 1.21 | 23.67 | 136.21 | 226.24 | 256.74 |
|  | 37 | 600 | 0.83 | 27.86 | 153.69 | 259.29 | 289.44 |
| Average |  |  |  | 13.51 | 88.43 | 144.56 | 167.95 |

Per-class Summary Results

The per-class average results are shown in Table XI. Besides the results on IOS platform, an Android device “Vivo IQOO3” with Snapdragon 865 SoC is used for providing the performance on Android platform. Considering the core number for A14 is 6, and Snapdragon 865 has 8 cores. Thus, “thread-full” in Table XI means 6 or 8 separately. From the results, both Android and IOS can support real-time decoding in class-B when multithreading is enabled.

Table XI: Per-class average decoding speed on iOS/Android Devices

|  | Class | VTM [fps] (A14) | O266 Thread 1 [fps] | O266 Thread 2 [fps] | O266 Thread-Full [fps] |
| --- | --- | --- | --- | --- | --- |
| iOS (iPhone12pro)-6Core | ClassA-RA | 2 | 11 | 19 | 32 |
|  | ClassB-RA | 9 | 53 | 86 | 129 |
|  | ClassB-LB | 8 | 50 | 80 | 102 |
|  | ClassSCC-1080p-RA | 13 | 82 | 126 | 180 |
|  | ClassSCC-1080p-LB | 14 | 88 | 145 | 168 |
| Android (Vivo IQOO Snapdragon 865)-8Core | ClassA-RA | / | 6 | 10 | 20 |
|  | ClassB-RA | / | 26 | 38 | 75 |
|  | ClassSCC-1080p-RA | / | 41 | 60 | 104 |

The Speedup ratio is shown in Table XII, where the multithreading performance, the acceleration ratio when compared with VTM, and acceleration ratio of NEON are all shown in this Table.

Table XII: Speedup ratio (Multithreading, acceleration ratio vs VTM, acceleration ratio of NEON)

| Class | VTM [fps] | O266 Thread 1 No-Neon[fps] (A14) | T-2 Speedup vs T-1 | T-6 Speedup vs T-1 | Thread1 Speedup vs VTM | Thread1 Neon Speedup |
| --- | --- | --- | --- | --- | --- | --- |
| ClassA-RA | 2 | 3 | 1.69 | 2.87 | 6.04 | 3.53 |
| ClassB-RA | 9 | 14 | 1.62 | 2.44 | 5.73 | 3.78 |
| ClassB-LB | 8 | 13 | 1.59 | 2.03 | 6.18 | 3.73 |
| ClassSCC-1080p-RA | 13 | 26 | 1.54 | 2.21 | 6.20 | 3.08 |
| ClassSCC-1080p-LB | 14 | 28 | 1.63 | 1.90 | 6.55 | 3.15 |
| Avg |  |  | 1.62 | 2.29 | 6.14 | 3.45 |

Player

A video player in IOS platform is implemented based on the presented VVC decoding capability and can display VVC bitstreams in real-time.

Conclusion

An example VVC software decoder is demonstrated in this contribution. Based on the first version, the capability in mobile platform is supported. With multithreading, the decoder can decode 2K bitstreams in real-time with a large margin and achieve an average speed of 31.55 fps when decoding4K bitstreams.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Reference

[1]. B. Zhu, S. Liu, X. Xu, X. Zhang, C. Gu, L. Wang, W. Feng, Performance of a VVC software decoder. JVET-T0095.