| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0072v3 |
| --- | --- |

| Title: | AHG8: Tool Off Tests for High Bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Steve Keating Kenji Kondo | Tel: Email: | steve.keating@sony.com kenji.kondo@sony.com |
| Source: | Sony |  |  |

_____________________________

# Abstract

This contribution reports the test results of various tools at high bit depths / rates. The reported gains and run times may help in considering disabling tools for future high bit depth / high bit rate simulations.  It is also proposed that LMCS be disabled for PQ content in the CTC due to its BD-rate loss.

# Introduction

At the previous meeting, AHG13 reported tool off test results for the Version 1 4:2:0 CTC JVET-M1010 as well as for non-4:2:0 colour format contents [1]. This contribution reports tool off test results of various tools at high bit depths / rates according to the JVET-T2018 CTC [2].

# Tests

A selection of VTM tools was tested at either AI, LDB or RA; tests were conducted on VTM-10.2 software with VTM configuration by switching off or on specific tool(s) either in configuration files or in macros. Only 4:4:4 results are reported for PQ/HLG for LDB and RA, only 12-bit results are reported for SVT for LDB and RA, 4:4:4 and 4:2:2 PQ/HLG and 12/16-bit SVT results are reported for AI.

The tested tools are listed in the tables below.

## Tools included in VTM (Tool off test vs VTM Anchor)

Table 1 List of adoptions included in VTM (Tool off test (unless specified) vs VTM anchor)

| Tool Name | Acronym | Document reference(s) | AI | LDB | RA |
| --- | --- | --- | --- | --- | --- |
| Cross-component linear model | LMChroma | JVET-O1124, JVET-Q0500, JVET-Q0194, | X |  |  |
| Adaptive colour transform | ACT | JVET-P0517, JVET-Q0820, JVET-Q0353, JVET-Q0512, | X |  |  |
| Multiple transform set | MTS | JVET-O0294, JVET-O0474, JVET-O0541, JVET-Q0055, JVET-Q0516, | X |  |  |
| Joint coding of chrominance residuals | JCbCr | JVET-N0054 | X |  |  |
| Matrix based intra prediction | MIP | JVET-P0054, JVET-P0199, JVET-P0803, JVET-Q0446 | X |  |  |
| Intra sub-partitioning | ISP | JVET-O0106, JVET-O0341, JVET-O0502 | X |  |  |
| Low frequency non-separable transform * | LFNST | JVET-P1026, JVET-P0350, JVET-Q0784, | X |  |  |
| Adaptive loop filter Cross component adaptive loop filter | ALF_CCALF | JVET-P0162, JVET-P0164, JVET-P0505, JVET-P0554, JVET-P0665, JVET-P1038, JVET-Q0249, JVET-Q0150, JVET-Q0495, JVET-Q0795; JVET-Q0795 | X | X | X |
| Luma mapping with chroma scaling ** | LMCS | JVET-P0254, JVET-P0371 | X |  |  |
| Multi-reference line prediction | MRL | JVET-P0418 | X |  |  |
| Dependent quantization | DepQuant | JVET-P0170, JVET-Q0433, | X | X | X |
| Chroma separate tree | DualITree | JVET-P0063, JVET-P0406 | X | X | X |
| Adaptive motion vector resolution | IMV | JVET-O0057 |  | X | X |
| Combined intra/inter prediction | CIIP | JVET-O0108, JVET-O0681 |  | X | X |
| Bi-predictive with CU weights | BCW | JVET-P0280 |  | X | X |
| Merge with MVD | MMVD | JVET-P1023 |  | X | X |
| Geometric partition mode | Geo | JVET-Q0806 |  | X | X |
| Sub-block transform | SBT | JVET-P1026 |  | X | X |
| Affine motion model Prediction refinement using optical flow | Affine-PROF | JVET-O0070; JVET-P0409, JVET-P0057, JVET-P0154, JVET-P0491, JVET-P0653 |  |  | X |
| Symmetric motion vector difference | SMVD | JVET-O0284, JVET-O0414, JVET-O0567, JVET-O0572 |  |  | X |
| Bi-directional optical flow | BIO | JVET-P0091, JVET-P0519, JVET-P1023, JVET-Q0128, |  |  | X |
| Decoder motion vector refinement | DMVR | JVET-O0297, JVET-O0590, JVET-O0634 |  |  | X |

* Tool on test as LFNST is disabled in CTC

** Tool on test for SVT content as LMCS is disabled for SVT in CTC.

# Results

The results of the tests are summarized in Table 3-5 below. The attached spreadsheet provides some additional data. Scatter plots (Figs. 1-9) are also provided for the tested tools in each configuration, comparing PSNR-Y,U,V or PSNR-G,B,R based BD-rate on the Y-axis vs.  Enc runtime ratio.

Table 3. Simulation results in all intra configuration (AI). (VTM anchor)

|  |  | AI HDR PQ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | psnrY | psnrU | psnrV | EncT | DecT |
| LMChroma | PQ444 | 1.74% | 2.90% | 2.39% | 90% | 100% |
|  | PQ422 | 1.25% | 1.81% | 1.73% | 96% | 99% |
|  | Overall | 1.49% | 2.36% | 2.06% | 93% | 100% |
| MTS | PQ444 | 1.28% | 0.43% | 0.43% | 77% | 99% |
|  | PQ422 | 1.31% | 0.52% | 0.50% | 64% | 99% |
|  | Overall | 1.30% | 0.47% | 0.46% | 70% | 99% |
| JCbCr | PQ444 | 0.82% | 0.14% | 0.06% | 89% | 100% |
|  | PQ422 | 0.67% | -0.09% | -0.17% | 94% | 101% |
|  | Overall | 0.75% | 0.03% | -0.05% | 92% | 101% |
| MIP | PQ444 | 0.17% | 0.08% | 0.10% | 90% | 101% |
|  | PQ422 | 0.21% | 0.12% | 0.11% | 85% | 101% |
|  | Overall | 0.19% | 0.10% | 0.10% | 87% | 101% |
| ISP | PQ444 | 0.02% | 0.04% | 0.05% | 96% | 98% |
|  | PQ422 | 0.03% | 0.04% | 0.04% | 95% | 99% |
|  | Overall | 0.03% | 0.04% | 0.04% | 95% | 99% |
| LFNST * | PQ444 | -0.33% | -0.02% | 0.03% | 93% | 100% |
|  | PQ422 | -0.15% | -0.14% | 0.12% | 96% | 100% |
|  | Overall | -0.24% | -0.08% | 0.07% | 94% | 100% |
| ALF_CCALF | PQ444 | 0.10% | 0.39% | 0.80% | 97% | 91% |
|  | PQ422 | 0.10% | 0.37% | 0.70% | 96% | 91% |
|  | Overall | 0.10% | 0.38% | 0.75% | 97% | 91% |
| LMCS ** | PQ444 | -5.08% | -4.32% | -4.74% | 92% | 94% |
|  | PQ422 | -4.69% | -3.86% | -4.19% | 95% | 94% |
|  | Overall | -4.88% | -4.09% | -4.46% | 93% | 94% |
| MRL | PQ444 | 0.20% | 0.08% | 0.09% | 99% | 100% |
|  | PQ422 | 0.21% | 0.11% | 0.11% | 100% | 101% |
|  | Overall | 0.21% | 0.09% | 0.10% | 100% | 101% |
| DepQuant | PQ444 | -0.24% | 2.85% | 2.60% | 76% | 100% |
|  | PQ422 | 0.31% | 3.38% | 3.42% | 80% | 101% |
|  | Overall | 0.04% | 3.11% | 3.01% | 78% | 101% |
| DualITree | PQ444 | 1.02% | 1.08% | 1.07% | 80% | 89% |
|  | PQ422 | 0.63% | 0.48% | 0.54% | 102% | 94% |
|  | Overall | 0.82% | 0.78% | 0.80% | 91% | 91% |

*Tool on test

**A bug was reported with LMCS in VTM10.2, the test has been repeated with VTM11.0rc1 but the results were unchanged.

|  |  | AI HDR HLG |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | psnrY | psnrU | psnrV | EncT | DecT |
| LMChroma | HLG444 | 1.52% | 1.65% | 1.92% | 90% | 101% |
|  | HLG422 | 0.48% | 0.56% | 0.62% | 95% | 99% |
|  | Overall | 1.00% | 1.10% | 1.27% | 93% | 100% |
| MTS | HLG444 | 1.77% | 0.50% | 0.50% | 77% | 99% |
|  | HLG422 | 1.72% | 0.58% | 0.57% | 66% | 98% |
|  | Overall | 1.75% | 0.54% | 0.54% | 72% | 99% |
| JCbCr | HLG444 | 0.02% | 0.00% | -0.01% | 87% | 100% |
|  | HLG422 | 0.02% | -0.01% | -0.02% | 92% | 99% |
|  | Overall | 0.02% | -0.01% | -0.01% | 89% | 100% |
| MIP | HLG444 | 0.19% | 0.09% | 0.09% | 90% | 101% |
|  | HLG422 | 0.19% | 0.11% | 0.11% | 85% | 100% |
|  | Overall | 0.19% | 0.10% | 0.10% | 88% | 100% |
| ISP | HLG444 | 0.02% | 0.01% | 0.01% | 96% | 99% |
|  | HLG422 | 0.02% | 0.01% | 0.01% | 93% | 98% |
|  | Overall | 0.02% | 0.01% | 0.01% | 95% | 99% |
| LFNST * | HLG444 | -0.54% | 0.06% | 0.08% | 89% | 100% |
|  | HLG422 | -0.43% | 0.09% | 0.12% | 91% | 100% |
|  | Overall | -0.48% | 0.08% | 0.10% | 90% | 100% |
| ALF_CCALF | HLG444 | 0.24% | 1.57% | 2.15% | 98% | 91% |
|  | HLG422 | 0.24% | 0.85% | 1.12% | 98% | 92% |
|  | Overall | 0.24% | 1.21% | 1.63% | 98% | 91% |
| LMCS | HLG444 | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | HLG422 | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| MRL | HLG444 | 0.10% | 0.03% | 0.03% | 99% | 100% |
|  | HLG422 | 0.10% | 0.04% | 0.04% | 99% | 99% |
|  | Overall | 0.10% | 0.04% | 0.03% | 99% | 100% |
| DepQuant | HLG444 | 0.25% | 4.71% | 4.82% | 76% | 102% |
|  | HLG422 | 0.98% | 4.93% | 5.21% | 79% | 105% |
|  | Overall | 0.61% | 4.82% | 5.01% | 77% | 104% |
| DualITree | HLG444 | 2.73% | 0.63% | 0.82% | 71% | 91% |
|  | HLG422 | 1.17% | 0.75% | 0.78% | 93% | 100% |
|  | Overall | 1.95% | 0.69% | 0.80% | 82% | 95% |

|  |  | AI SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | psnrG | psnrB | psnrR | EncT | DecT |
| LMChroma | SVT16 | -0.37% | 2.06% | 2.11% | 95% | 101% |
|  | SVT12 | -0.16% | 1.73% | 1.74% | 96% | 100% |
|  | Overall | -0.27% | 1.89% | 1.92% | 95% | 101% |
| ACT ** | SVT16 | 1.26% | -0.42% | -0.46% | 72% | 119% |
|  | SVT12 | 0.77% | -0.08% | -0.08% | 65% | 115% |
|  | Overall | 1.02% | -0.25% | -0.27% | 68% | 117% |
| MTS | SVT16 | 1.04% | 0.57% | 0.56% | 66% | 98% |
|  | SVT12 | 0.98% | 0.28% | 0.28% | 66% | 99% |
|  | Overall | 1.01% | 0.43% | 0.42% | 66% | 98% |
| JCbCr | SVT16 | 0.00% | 0.00% | 0.00% | 88% | 100% |
|  | SVT12 | -0.01% | 0.00% | 0.00% | 88% | 100% |
|  | Overall | -0.01% | 0.00% | 0.00% | 88% | 100% |
| MIP | SVT16 | 0.58% | 0.30% | 0.30% | 79% | 99% |
|  | SVT12 | 0.46% | 0.17% | 0.16% | 80% | 100% |
|  | Overall | 0.52% | 0.23% | 0.23% | 79% | 100% |
| ISP | SVT16 | 0.00% | 0.04% | 0.05% | 97% | 100% |
|  | SVT12 | 0.00% | 0.01% | 0.01% | 97% | 99% |
|  | Overall | 0.00% | 0.03% | 0.03% | 97% | 100% |
| LFNST * | SVT16 | -0.51% | -0.19% | -0.19% | 68% | 100% |
|  | SVT12 | -0.57% | 0.12% | 0.14% | 69% | 99% |
|  | Overall | -0.54% | -0.03% | -0.02% | 68% | 99% |
| ALF_CCALF | SVT16 | 0.00% | 0.00% | 0.00% | 98% | 99% |
|  | SVT12 | 0.30% | 0.00% | 0.00% | 99% | 99% |
|  | Overall | 0.15% | 0.00% | 0.00% | 98% | 99% |
| LMCS * | SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | SVT12 | -1.66% | -0.02% | -0.02% | 105% | 100% |
|  | Overall | -0.83% | -0.01% | -0.01% | 103% | 100% |
| MRL | SVT16 | 0.04% | 0.03% | 0.03% | 116% | 100% |
|  | SVT12 | -0.10% | -0.02% | -0.02% | 108% | 100% |
|  | Overall | -0.03% | 0.01% | 0.01% | 112% | 100% |
| DepQuant | SVT16 | 2.04% | 2.45% | 2.52% | 94% | 98% |
|  | SVT12 | 1.97% | 2.75% | 2.81% | 85% | 99% |
|  | Overall | 2.00% | 2.60% | 2.67% | 89% | 98% |
| DualITree | SVT16 | 0.00% | 0.00% | 0.00% | 116% | 100% |
|  | SVT12 | -0.12% | -0.03% | -0.03% | 108% | 99% |
|  | Overall | -0.06% | -0.02% | -0.02% | 112% | 99% |

* Tool on test

** Note that enabling ACT requires DualITree to be disabled.

Table 4. Simulation results in low delay B configuration (LDB). (VTM anchor)

|  | LDB HDR PQ444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.11% | 0.08% | 0.09% | 88% | 99% |
| CIIP | 0.33% | 0.54% | 0.59% | 96% | 100% |
| BCW | 0.06% | 0.03% | 0.01% | 94% | 101% |
| MMVD | 0.21% | 0.01% | 0.00% | 93% | 99% |
| Geo | 0.12% | 0.15% | 0.10% | 94% | 100% |
| SBT | -0.02% | 0.24% | 0.12% | 96% | 100% |
| Affine-PROF | 0.00% | -0.03% | 0.00% | 90% | 98% |
| ALF_CCALF | 1.52% | 3.11% | 3.36% | 97% | 84% |
| DepQuant | 2.38% | 3.24% | 2.94% | 89% | 101% |
| DualITree | 0.05% | 0.06% | 0.05% | 99% | 100% |

|  | LDB HDR HLG444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.06% | 0.28% | 0.28% | 88% | 101% |
| CIIP | 0.24% | 0.19% | 0.16% | 93% | 100% |
| BCW | 0.03% | 0.10% | 0.10% | 90% | 99% |
| MMVD | -0.05% | -0.01% | -0.01% | 95% | 100% |
| Geo | 0.02% | 0.01% | 0.01% | 94% | 100% |
| SBT | 0.00% | 0.00% | 0.00% | 97% | 100% |
| Affine-PROF | 0.00% | 0.05% | 0.07% | 89% | 97% |
| ALF_CCALF | 2.25% | 6.14% | 7.37% | 99% | 82% |
| DepQuant | 3.61% | 4.96% | 5.22% | 84% | 100% |
| DualITree | 0.21% | 0.05% | 0.06% | 97% | 99% |

|  | LDB SVT RGB12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| IMV | -0.01% | 0.00% | 0.00% | 92% | 98% |
| CIIP | 0.07% | 0.03% | 0.03% | 103% | 98% |
| BCW | 0.00% | 0.01% | 0.01% | 93% | 101% |
| MMVD | 0.07% | 0.09% | 0.09% | 103% | 99% |
| Geo | 0.04% | 0.05% | 0.05% | 96% | 100% |
| SBT | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Affine-PROF | -0.01% | 0.03% | 0.03% | 100% | 98% |
| ALF_CCALF | 0.38% | 0.00% | 0.00% | 98% | 98% |
| DepQuant | 2.46% | 3.13% | 3.22% | 87% | 96% |
| DualITree | 0.00% | 0.00% | 0.00% | 106% | 99% |

Table 5. Simulation results in random access configuration (RA). (VTM anchor)

|  | RA HDR PQ444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.10% | 0.03% | 0.06% | 95% | 109% |
| CIIP | 0.26% | 0.44% | 0.49% | 96% | 99% |
| BCW | 0.03% | 0.04% | 0.06% | 96% | 100% |
| MMVD | 0.01% | -0.19% | -0.20% | 92% | 99% |
| SMVD | 0.08% | 0.01% | -0.01% | 97% | 100% |
| Geo | 0.04% | 0.08% | 0.09% | 97% | 100% |
| BIO | 0.02% | 0.03% | 0.03% | 98% | 99% |
| SBT | -0.11% | 0.45% | 0.16% | 96% | 100% |
| DMVR | 0.02% | 0.03% | 0.02% | 100% | 99% |
| Affine-PROF | 0.07% | -0.06% | -0.24% | 93% | 98% |
| ALF_CCALF | 1.26% | 2.74% | 3.10% | 97% | 84% |
| DepQuant | 2.55% | 2.60% | 2.50% | 90% | 100% |
| DualITree | 0.07% | 0.07% | 0.11% | 99% | 98% |

|  | RA HDR HLG444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.12% | 0.47% | 0.49% | 86% | 101% |
| CIIP | 0.13% | 0.10% | 0.09% | 96% | 100% |
| BCW | 0.04% | 0.13% | 0.15% | 93% | 102% |
| MMVD | 0.02% | 0.03% | 0.04% | 92% | 100% |
| SMVD | 0.06% | 0.07% | 0.07% | 95% | 99% |
| Geo | 0.01% | 0.01% | 0.01% | 95% | 99% |
| BIO | -0.01% | -0.02% | -0.01% | 96% | 98% |
| SBT | 0.01% | -0.01% | 0.00% | 97% | 100% |
| DMVR | 0.00% | -0.01% | 0.00% | 98% | 98% |
| Affine-PROF | 0.01% | 0.03% | 0.04% | 91% | 99% |
| ALF_CCALF | 2.25% | 6.14% | 7.37% | 99% | 82% |
| DepQuant | 3.69% | 5.26% | 5.51% | 81% | 101% |
| DualITree | 0.13% | 0.02% | 0.03% | 98% | 101% |

|  | RA SVT RGB12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| IMV | 0.01% | 0.03% | 0.03% | 96% | 111% |
| CIIP | 0.04% | 0.04% | 0.04% | 98% | 100% |
| BCW | 0.01% | 0.00% | 0.00% | 95% | 100% |
| MMVD | 0.05% | 0.08% | 0.08% | 97% | 101% |
| SMVD | 0.02% | 0.01% | 0.01% | 99% | 106% |
| Geo | 0.02% | 0.01% | 0.01% | 98% | 100% |
| BIO | -0.05% | 0.01% | 0.01% | 99% | 99% |
| SBT | 0.00% | 0.00% | 0.00% | 99% | 100% |
| DMVR | -0.03% | 0.02% | 0.02% | 100% | 99% |
| Affine-PROF | 0.01% | 0.03% | 0.03% | 99% | 99% |
| ALF_CCALF | 0.43% | 0.00% | 0.00% | 99% | 97% |
| DepQuant | 2.41% | 3.11% | 3.21% | 84% | 97% |
| DualITree | 0.00% | 0.00% | 0.00% | 106% | 99% |

Figure 1. PQ PSNR-Y,U,V vs encoding runtime ratio of VTM (AI) (VTM anchor)

Figure 2. HLG PSNR-Y,U,V vs encoding runtime ratio of VTM (AI) (VTM anchor)

Figure 3. SVT PSNR-G,B,R vs encoding runtime ratio of VTM (AI) (VTM anchor)

Figure 4. PQ PSNR-Y,U,V vs encoding runtime ratio of VTM (LDB) (VTM anchor)

Figure 5. HLG PSNR-Y,U,V vs encoding runtime ratio of VTM (LDB) (VTM anchor)

Figure 6. SVT PSNR-G,B,R vs encoding runtime ratio of VTM (LDB) (VTM anchor)

Figure 7. PQ PSNR-Y,U,V vs encoding runtime ratio of VTM (RA) (VTM anchor)

Figure 8. HLG PSNR-Y,U,V vs encoding runtime ratio of VTM (RA) (VTM anchor)

Figure 9. SVT PSNR-G,B,R vs encoding runtime ratio of VTM (RA) (VTM anchor)

## Multiple Tool-off test

As an example of the effect of disabling more than one tool a test was run with the following tools disabled:  BCW, MMVD, SMVD, Geo, BIO, DMVR, Affine, ISP and LFNST.  These tools were chosen as each having less than 0.1% gain for all image classes in RA configuration.

|  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM10.2 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.08% | 0.51% | 0.35% | 0.11% | 0.36% | 0.25% | 70% | 96% |
| PQ422 | 0.39% | 0.80% | 0.83% | 0.40% | 0.66% | 0.69% | 68% | 95% |
| Overall | 0.23% | 0.65% | 0.59% | 0.25% | 0.51% | 0.47% | 69% | 95% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM10.2 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.03% | 0.18% | 0.22% | 69% | 95% |
| HLG422 | 0.06% | 0.30% | 0.39% | 69% | 95% |
| Overall | 0.05% | 0.24% | 0.31% | 69% | 95% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM10.2 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.07% | 0.13% | 0.14% | 85% | 97% |
| SVT12 | -0.06% | 0.16% | 0.16% | 89% | 97% |
| Overall | -0.06% | 0.15% | 0.15% | 87% | 97% |

Similar run-time savings are seen in LDB configuration although the loss is larger:

|  |  |  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM10.2 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.39% | 0.60% | 0.37% | 0.41% | 0.49% | 0.31% | 70% | 99% |
| PQ422 | 0.59% | 0.76% | 0.60% | 0.60% | 0.68% | 0.52% | 67% | 98% |
| Overall | 0.49% | 0.68% | 0.48% | 0.51% | 0.59% | 0.41% | 68% | 99% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM10.2 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.16% | 0.21% | 0.24% | 70% | 97% |
| HLG422 | 0.11% | 0.27% | 0.28% | 69% | 97% |
| Overall | 0.14% | 0.24% | 0.26% | 70% | 97% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM10.2 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.17% | 0.29% | 0.29% | 83% | 99% |
| SVT12 | 0.11% | 0.20% | 0.20% | 86% | 99% |
| Overall | 0.14% | 0.25% | 0.25% | 84% | 99% |

# Recommendations

As run times of simulations have significantly increased for the high bit depth/rate CTC it should be considered whether disabling a number of tools would be justified to reduce simulation times for future tests.  Possible interaction between tools should also be considered if multiple tools are to be disabled.  It is also proposed that, due to its BD-rate loss, LMCS should be disabled for PQ content (as it is for SVT content) in the CTC.

# Reference

[1] W-J. Chien, .J Boyce, W. Chen, Y-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y-W. Huang, H. Jang, R-L. Liao, S. Liu, “JVET AHG report: Tool reporting procedure (AHG13)”, JVET-T2013, by teleconference, 7–16 Oct 2020

[2] A. Browne, T. Ikai, D. Rusanovskyy, X. Xu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, by teleconference, 7–16 Oct 2020