| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0075 |
| --- | --- |

| Title: | CE-2.1: Slice based Rice parameter selection for transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Wei Chen, Che-Wei Kuo, Xianglin Wang | Email: | {jhuhong-jheng, xiaoyuxiu, yiwenchen, chenwei06, cheweikuo, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 2-1 [1]. In this test, slice based Rice parameter selection for transform skip residual coding is tested. It is proposed to explicitly signal the Rice parameter for each slice to indicate the Rice parameter for the binary codewords of abs_remainder. The proposed method is implemented and evaluated with VTM11.0rc1[2] by following CE test conditions. Results are reported as below:

SVT RGB:

16-bit with QP (-33, -28, -23, -18, -13, -8): -22.24% AI, -13.99% LB

12-bit with QP (-13, -8, -3, 2, 7, 12): -1.44% AI, -0.38% LB

HDR HLG:

12-bit with QP (-13, -8, -3, 2, 7, 12): -0.10% AI, 0.00% LB

HDR PQ:

12-bit with QP (-13, -8, -3, 2, 7, 12): -0.12% AI, 0.01% LB

TGM RGB:

12-bit with QP (-8, -3, 2, 7, 12): -4.71% AI

10-bit with QP (-8, -3, 2, 7, 12): -4.15% AI

# Introduction

Above 12-bit an issue with the Golomb-Rice coding is reported as the cause of a significant reduction in performance from VVC. In residual coding of a transform skip block, the remainder of the absolute levels are binarized using fixed Rice parameter “1”. But it is observed that different coding bit-depths use different Rice parameters obtains higher coding performance. In this proposal, a slice based Rice parameter is proposed for the transform skip residual coding for higher coding efficiency especially for high bit depth conditions.

# Proposal

In the proposed method, one control flag is signaled in sequence parameter set to indicate the signaling of Rice parameter for the transform skip blocks is enabled or disabled. When the control flag is signaled as enabled, one syntax element is further signaled for each transform skip slice to indicate the Rice parameter of that slice. When the control flag is signaled as disabled (e.g. set equal to “0”), no further syntax element is signaled at lower level to indicate the Rice parameter for the transform skip slice and a default Rice parameter (e.g. 1) is used for all the transform skip slice.

In the proposed method, the Rice parameter is coded by ue(v), unsigned integer 0-th order Exp-Golomb-coded syntax element with the left bit first.

In the encoder, the used initial Rice parameter for each slice is caculated as followings: rice = Clip3( 1, 8, (19-QP)/6 ). The Rice parameter is adjusted based on previous slice results. All encoding bits spent on Rice encoding parameters from 1 to 8 will be recorded for the next slice reference. If the QP in previous slice is the same as the QP in current slice, the Rice parameter which has overall minimal encoding bits on previous slice is used for current slice. If not, previous encoding bits of the initial Rice parameter is multiplied by 0.9 first and then the Rice parameter which has overall minimal encoding bits on previous slice is used for current slice.

# Simulation Results

## CE CTC

The proposed method is implemented based on the VTM-11.0rc1 reference software [2] and tested using the test condition and evaluation criteria that are used in CE [1] are applied.

Table 1: Results of Slice based Rice Parameters for SVT RGB

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -22.24% | -21.35% | -21.30% | 101% | 92% |
| SVT12 | -1.44% | -1.61% | -1.56% | 100% | 100% |
| Overall | -11.84% | -11.48% | -11.43% | 101% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -13.99% | -13.39% | -13.39% | 100% | 99% |
| SVT12 | -0.38% | -0.42% | -0.39% | 99% | 100% |
| Overall | -7.19% | -6.91% | -6.89% | 99% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -13.07% | -13.65% | -13.57% | 101% | 98% |
| SVT12 | -0.44% | -0.43% | -0.40% | 98% | 101% |
| Overall | -6.75% | -7.04% | -6.99% | 100% | 100% |
| Overall RGB | -8.59% | -8.48% | -8.44% | 100% | 98% |

Table 2: Results of Slice based Rice Parameters for HDR HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.10% | -0.04% | -0.03% | 100% | 100% |
| HLG422 | -0.10% | -0.07% | -0.07% | 100% | 100% |
| Overall | -0.10% | -0.05% | -0.05% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | -0.01% | 98% | 99% |
| HLG422 | 0.00% | -0.02% | -0.01% | 99% | 101% |
| Overall | 0.00% | -0.01% | -0.01% | 99% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| HLG422 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall HLG | -0.03% | -0.02% | -0.02% | 100% | 100% |

Table 3: Results of Slice based Rice Parameters forHDR PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.12% | -0.09% | -0.08% | -0.12% | -0.10% | -0.10% | 100% | 98% |
| PQ422 | -0.12% | -0.08% | -0.08% | -0.13% | -0.09% | -0.10% | 97% | 96% |
| Overall | -0.12% | -0.09% | -0.08% | -0.12% | -0.10% | -0.10% | 99% | 97% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.00% | 0.01% | 0.01% | 0.00% | 0.02% | 99% | 99% |
| PQ422 | 0.01% | -0.01% | 0.01% | 0.01% | 0.00% | 0.01% | 98% | 100% |
| Overall | 0.01% | -0.01% | 0.01% | 0.01% | 0.00% | 0.01% | 98% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | -0.01% | 0.00% | -0.01% | -0.01% | -0.01% | 99% | 100% |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | -0.01% | 0.00% | -0.01% | -0.01% | -0.01% | 99% | 100% |
| Overall PQ | -0.04% | -0.03% | -0.03% | -0.04% | -0.04% | -0.03% | 99% | 99% |

## Additional Results

The proposed method is implemented based on the VTM-11.0rc1 reference software [2] and tested using TGM RGB 4:4:4 sequences defined in [4], QP (-8, -3, 2, 7, 12) and 100 frames. In the testing, Intra block copy (IBC), BDPCM, Hash-based motion search, palette mode (PLT), and Adaptive colour transform (ACT) were enabled in the RGB 4:4:4 test condition defined in [4] with RExt__HIGH_BIT_DEPTH_SUPPORT = 1. CE-1.4 is also tested under the same setting to compare the performance between TSRC method and RRC method.

Table 4: Results for InternalBitDepth=12

| TGM 1080p | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | EncT | DecT |
| CE-2.1 | -4.71% | -4.51% | -4.54% | 100% | 97% |
| CE-1.4 | -0.54% | -0.56% | -0.54% | 107% | 97% |

Table 5: Results for InternalBitDepth=10

| TGM 1080p | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | EncT | DecT |
| CE-2.1 | -4.15% | -3.98% | -4.05% | 98% | 96% |
| CE-1.4 | -0.37% | -0.40% | -0.40% | 104% | 101% |

# Working Draft Text

The changes to the VVC Draft [5] are highlighted below.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_virtual_boundaries_enabled_flag | u(1) |
| … |  |
| } |  |

sps_ts_residual_coding_rice_present_in_sh_flag equal to 1 specifies that sh_ts_residual_coding_rice_idx could be present in SH syntax structures referring to the SPS. sps_ts_residual_coding_rice_present_in_sh_flag equal to 0 specifies that sh_ts_residual_coding_rice_idx is not present in SH syntax structures referring to the SPS. When sps_ts_residual_coding_rice_present_in_sh_flag is not present, the value of sps_ts_residual_coding_rice_present_in_sh_flag is inferred to be equal to 0.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if((!sh_ts_residual_coding_disabled_flag) && sps_ts_residual_coding_rice_enabled _flag ) |  |
| sh_ts_residual_coding_rice_idx | ue(v) |
| … |  |
| } |  |

sh_ts_residual_coding_rice_idx plus 1, when present, specifies the rice parameter used for the residual_ts_coding( ) syntax structure. When sh_ts_residual_coding_rice_idx is not present, the value of sh_ts_residual_coding_rice_idx is inferred to be equal to 1.

### 9.3.3.11	Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The variables lastAbsRemainder and lastRiceParam are derived as follows:

–	If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0.

–	Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause.

The rice parameter cRiceParam is derived as follows:

–	If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0, the Rice parameter cRiceParam is set equal to sh_ts_residual_coding_rice_idx1.

–	Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

…

# Conclusions

- A slice based Rice parameter for transform skip is proposed in this CE test, which has no negligible complexity to the decoder but provides significant BD-rate saving for high bit depth conditions, especially for TGM sequences and high bit depth conditions. It is recommended the proposed method be adopted into VVC version 2.

# References

- A. Browne, T. Hashimoto, H.-J. Jhu and D. Rusanovskyy, “ CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-T2022, 20th meeting, by teleconference, 7 – 16 Oct. 2020.

- VTM-11.0rc1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-11.0rc1.

- A. Browne, T. Ikai, D. Rusanovskyy and X. Xiu, “ Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, 20th meeting, by teleconference, 7 – 16 Oct. 2020.

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-Q2013, Brussels, BE, 7–17 January 2020.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “ Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, 20th meeting, by teleconference, 7 – 16 Oct. 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).