| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0076 |
| --- | --- |

| Title: | AHG2/AHG3: Proposal to remove some RPL constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antti Hallapuro Miska Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution asserts that there is a problem in RPL constraints in VVC specification. The constraints state that an RPL shall not have entries that represent (sub)pictures that precede previous IRAP (sub)picture in decoding or output order. The contribution alleges that these constraints prevent many prediction possibilities when IRAP and non-IRAP subpictures are mixed in a picture and can lead to coding efficiency of the bitstream being lower than it could be. The contribution proposes to remove the constraints.

# Introduction

Mixed NAL unit types is a feature in VVC that can be used for subpicture stream switching when a bitstream contains multiple subpictures. This feature is useful, for example, in 360 degrees video streaming where quality of different parts of the videos could be dynamically adjusted based on viewing orientation.

In 360 degrees use case, where the video is divided into several subpictures, subpictures covering the viewport should have high quality while off-viewport subpictures can have lower quality. When viewing orientation changes, some of the low-quality subpictures should be switched to higher quality and some of the high-quality subpictures to lower quality so that the best quality is always in the area being viewed.

Switching is performed at IRAP subpictures, individually for each subpicture. As a result, there can be both IRAP subpictures and non-IRAP subpictures in a coded picture. Mixing of different types of VCL NAL units is illustrated in Figure 1. In this example, there are two subpicture bitstreams and one of the them is switched to a different quality starting at CRA subpicture.

An important aspect of mixing of VCL NAL unit types in that all subpicture bitstreams should maintain the same DPB content in terms of POC values. That is, RPL syntax of each subpicture bitstream should define the same reference frames even if some of the reference frames are not used for prediction.

Figure 1- Mixing NAL unit types within a picture

# Problem statement

VVC specification contain the following constraints related to RPL:

- When the current picture follows an IRAP picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index in both decoding and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture follows an IRAP picture having the same value of nuh_layer_id and all the leading pictures, if any, associated with that IRAP picture in both decoding order and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index and all the leading subpictures, if any, associated with that IRAP subpicture in both decoding and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

Going back to the example in Figure 1, it can be seen that the third and the fourth constraints effectively prevent prediction from picture N to picture N+4 because RPL of the top subpicture N+4 cannot contain reference picture N and therefore RPL of the bottom subpicture N+4 cannot contain it either.

Analyzing third and fourth constraints further, it can be concluded that the mention of leading pictures in these constraints is redundant. That is because leading pictures always precede associated IRAP picture in output order as per definition.

Ignoring the redundant mention of leading pictures, the only difference between the first and the third constraint is that the first constraint has “active entry” instead of just “entry”. The same applies to the second and the fourth constraints. The third and the fourth constraints are just more strict versions of the first and the second constraints and thus render the first two constraints redundant.

Since inactive entries in RPL do not affect the decoding process, it seems that constraining only the active entries as in constraints 1 and 2 is sufficient and allows more flexibility in prediction structures for mixing subpictures of different VCL NAL unit types.

# Proposed solution

It is proposed that the third and the fourth constraint in the previous paragraph be removed from the specification. The specification text for the constraints would look as follows:

- When the current picture follows an IRAP picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index in both decoding and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture follows an IRAP picture having the same value of nuh_layer_id and all the leading pictures, if any, associated with that IRAP picture in both decoding order and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index and all the leading subpictures, if any, associated with that IRAP subpicture in both decoding and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

Removing these constraints enables more reference frames to be used in a mixed NAL unit types bitstreams where IRAP and non-IRAP subpictures are mixed in the same picture. This can lead to better coding efficiency in the relevant use cases.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).