| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0081_v2 |
| --- | --- |

| Title: | AHG10: ALF filter optimization with filter strength target |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jacob Ström | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The VTM encoder currently has the possibility to control the amount of ALF filtering by changing the filter strength parameters. The current VTM behavior is that the optimal filters are calculated and are then scaled by the filter strength parameters (decreasing the magnitude) as part of the quantization. That approach can, together with disabling pre-defined filters and omitting the refinement step of the quantized coefficients, reduce the filter strength for ALF according to the filter strength parameters. This proposal suggests including the filter strength target parameter in the ALF filter optimization. This approach favors filters that have a reduced strength according to the filter strength target in all stages of VTM’s filter optimization, i.e., during merging, refinement of quantized coefficients and when using pre-defined filter coefficients, but does not guarantee a reduction of filter strength according to the target parameter. Separate parameters for luma and chroma ALF are also suggested.

BDR impact with filter strength target parameters of 0.875 for both luma and chroma:

SDR CTC: AI/RA/LDB: 0.04%/-0.03%/-0.06%

HDR CTC RA:  -0,20%	-0,06%, -0,05%, -0,03% (deltaE, psnrL, wpsnrY, psnrY)

BDR impact with filter strength target parameter of 0.875 only for luma:

SDR CTC: AI/RA/LDB:  0.04%/-0.02%/-0.09%

HDR CTC RA:  -0,03%	-0,08%, -0,08%, -0,04% (deltaE, psnrL, wpsnrY, psnrY)

We also provide results for the current approach in VTM for the case of ALFStrength equal to 0.875:

SDR CTC: AI/RA/LDB: 0.10, 0.07%, 0.08%

It is claimed that the proposed approach favor filters according to the filter strength target and that in some cases it can improve the subjective quality of VTM for inter predictive coding.

# Introduction

The amount of filtering with ALF can be controlled by filter strength parameters in VTM [1]. This contribution adds further flexibility to the ALF encoder such that the filter optimization is given a filter strength target.

# Proposal

## Incorporation of filter strength target in filter optimization

Instead of applying the scaling of the filter coefficients as part of the quantization of the filter coefficients as in JVET-T0064, here the autocorrelations and the cross-correlations are scaled by the inverse of the filter strength target parameter. This can be viewed as magnifying the input to the filter optimization such that filters with strengths according to the filter strength target are favored. This also enables the use of VTM’s merging, refinement of quantized coefficients and pre-defined filter coefficients with a favouring of filters according to the target filter strength.

## Addition of a strength parameter for chroma ALF

To enable control of filter strength separately for luma and chroma ALF a separate parameter for chroma is proposed.

# Results

The objective performance (BDR) for the proposal is shown below in comparison to VTM-11.0 [2] using SDR CTC [3] and HDR CTC [4]. The objective results show that by favoring filters with reduced strength BDR can be improved for RA and LDB. Encoding time is measured on simulations run on a cluster with machines with varying capability and is not reliable. Decoding time is measured on simulations on same machine.

## Strength Target 0.875 for both luma and chroma

|  | All Intra VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,03% | 0,18% | 0,07% | 89% | 100% |
| Class A2 | 0,05% | 0,03% | -0,02% | 94% | 102% |
| Class B | 0,04% | 0,03% | 0,06% | 92% | 100% |
| Class C | 0,04% | 0,03% | 0,08% | 97% | 100% |
| Class E | 0,06% | 0,06% | 0,08% | 91% | 100% |
| Overall | 0,04% | 0,06% | 0,06% | 93% | 100% |
| Class D | 0,01% | 0,05% | 0,10% | 90% | 97% |
| Class F | 0,06% | 0,02% | 0,05% | 100% | 101% |

|  | Random access VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,13% | -0,04% | 92% | 100% |
| Class A2 | -0,02% | -0,10% | -0,09% | 101% | 100% |
| Class B | -0,05% | -0,14% | -0,01% | 93% | 101% |
| Class C | -0,05% | -0,21% | -0,08% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,15% | -0,05% | 97% | 100% |
| Class D | 0,06% | -0,07% | 0,03% | 102% | 101% |
| Class F | -0,03% | -0,12% | -0,14% | 102% | 103% |

|  | Low delay B Over VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,05% | -0,21% | -0,07% | 99% | 100% |
| Class C | -0,09% | -0,21% | -0,58% | 85% | 101% |
| Class E | -0,01% | -0,45% | 0,11% | 94% | 101% |
| Overall | -0,06% | -0,27% | -0,19% | 93% | 101% |
| Class D | 0,03% | 0,11% | 0,13% | 93% | 99% |
| Class F | -0,26% | -0,45% | 0,01% | 94% | 101% |

|  | Low delay P Over VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | -0,08% | 0,05% | 95% | 101% |
| Class C | -0,07% | -0,24% | 0,17% | 90% | 100% |
| Class E | -0,19% | 0,28% | -0,15% | 96% | 96% |
| Overall | -0,09% | -0,05% | 0,04% | 94% | 100% |
| Class D | -0,17% | -0,72% | -0,31% | 99% | 98% |
| Class F | -0,16% | -0,54% | -0,23% | 91% | 101% |

|  | Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,01% | 0,09% | 0,04% | 0,03% | 0,06% | 0,06% | 0,10% | 0,12% | 98% | 100% |
| Class H2 |  |  |  |  |  | 0,05% | 0,02% | 0,02% | 99% | 100% |
| Overall | -0,01% | 0,09% | 0,04% | 0,03% | 0,06% | 0,06% | 0,07% | 0,08% | 98% | 100% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,20% | -0,06% | -0,05% | -0,32% | -0,27% | -0,03% | -0,27% | -0,15% | 97% | 102% |
| Class H2 |  |  |  |  |  | -0,03% | 0,03% | 0,05% | 104% | 100% |
| Overall | -0,20% | -0,06% | -0,05% | -0,32% | -0,27% | -0,03% | -0,16% | -0,08% | 100% | 101% |

## Strength Target 0.875 for luma

|  | All Intra VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | 0,00% | 0,00% | 105% | 99% |
| Class A2 | 0,05% | 0,00% | 0,00% | 105% | 101% |
| Class B | 0,04% | 0,00% | 0,00% | 108% | 99% |
| Class C | 0,04% | 0,00% | 0,00% | 108% | 101% |
| Class E | 0,06% | 0,00% | 0,00% | 104% | 97% |
| Overall | 0,04% | 0,00% | 0,00% | 106% | 99% |
| Class D | 0,01% | 0,03% | 0,03% | 99% | 103% |
| Class F | 0,06% | 0,00% | 0,00% | 108% | 100% |

|  | Random access VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,03% | 0,00% | 103% | 101% |
| Class A2 | -0,03% | 0,06% | 0,05% | 105% | 99% |
| Class B | -0,03% | -0,05% | 0,09% | 106% | 99% |
| Class C | -0,04% | -0,10% | 0,13% | 111% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | -0,04% | 0,07% | 107% | 100% |
| Class D | 0,00% | 0,30% | 0,19% | 111% | 100% |
| Class F | -0,05% | 0,07% | -0,04% | 111% | 102% |

|  | Low delay B Over VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | 0,04% | 0,27% | 83% | 100% |
| Class C | -0,12% | -0,02% | 0,01% | 78% | 100% |
| Class E | -0,15% | -0,42% | 0,54% | 77% | 99% |
| Overall | -0,09% | -0,09% | 0,25% | 79% | 100% |
| Class D | -0,04% | 0,36% | -0,05% | 75% | 97% |
| Class F | -0,23% | -0,28% | 0,37% | 82% | 98% |

|  | Low delay P Over VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,06% | 0,08% | 0,28% | 97% | 100% |
| Class C | -0,07% | 0,20% | 0,54% | 91% | 100% |
| Class E | -0,07% | -0,36% | 0,56% | 97% | 102% |
| Overall | -0,06% | 0,01% | 0,44% | 95% | 100% |
| Class D | -0,12% | -0,53% | -0,15% | 99% | 97% |
| Class F | -0,08% | 0,18% | 0,32% | 90% | 102% |

|  | Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,05% | 0,08% | 0,04% | 0,00% | 0,00% | 0,06% | 0,00% | 0,00% | 99% | 100% |
| Class H2 |  |  |  |  |  | 0,05% | 0,00% | 0,00% | 101% | 99% |
| Overall | 0,05% | 0,08% | 0,04% | 0,00% | 0,00% | 0,06% | 0,00% | 0,00% | 100% | 100% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,03% | -0,08% | -0,08% | -0,10% | 0,00% | -0,05% | -0,13% | 0,00% | 94% | 100% |
| Class H2 |  |  |  |  |  | -0,04% | 0,02% | -0,04% | 101% | 102% |
| Overall | -0,03% | -0,08% | -0,08% | -0,10% | 0,00% | -0,04% | -0,08% | -0,01% | 96% | 101% |

## ALFStrength = 0.875

|  | All Intra VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | 0,18% | 0,21% | 73% | 101% |
| Class A2 | 0,10% | 0,08% | 0,01% | 74% | 101% |
| Class B | 0,09% | 0,04% | 0,08% | 82% | 100% |
| Class C | 0,11% | -0,01% | 0,04% | 83% | 102% |
| Class E | 0,17% | 0,04% | 0,05% | 82% | 98% |
| Overall | 0,10% | 0,06% | 0,08% | 79% | 100% |
| Class D | 0,05% | -0,03% | 0,02% | 82% | 98% |
| Class F | 0,16% | 0,00% | 0,05% | 88% | 102% |

|  | Random access VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,06% | 0,20% | 0,03% | 80% | 101% |
| Class A2 | 0,09% | -0,07% | -0,14% | 82% | 102% |
| Class B | 0,11% | -0,05% | 0,09% | 76% | 100% |
| Class C | 0,01% | -0,13% | 0,00% | 76% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,07% | -0,02% | 0,01% | 78% | 101% |
| Class D | 0,12% | -0,08% | -0,10% | 80% | 101% |
| Class F | 0,12% | -0,05% | -0,16% | 79% | 100% |

|  | Low delay B Over VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,14% | 0,01% | 0,70% | 82% | 99% |
| Class C | 0,01% | 0,05% | -0,27% | 80% | 100% |
| Class E | 0,04% | -0,42% | 0,38% | 74% | 98% |
| Overall | 0,08% | -0,08% | 0,29% | 79% | 99% |
| Class D | 0,04% | 0,42% | -0,04% | 81% | 102% |
| Class F | -0,03% | -0,05% | 0,53% | 83% | 101% |

|  | Low delay P Over VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,16% | 0,01% | 0,38% | 95% | 100% |
| Class C | 0,01% | 0,08% | -0,03% | 94% | 101% |
| Class E | 0,08% | -0,70% | 0,50% | 98% | 101% |
| Overall | 0,09% | -0,14% | 0,27% | 95% | 101% |
| Class D | 0,00% | -0,37% | -0,71% | 98% | 103% |
| Class F | 0,04% | -0,21% | -0,25% | 88% | 95% |

# Conclusions

It has been shown that filter optimization which favors filters with a reduced strength can provide a small objective performance benefit compared to when filters with full strength are used. We propose to include filter strength target parameter for ALF filter optimization to VTM and that parameters are separately for luma and chroma ALF.

# References

[1]  K. Andersson,and J. Ström, “Addition of ALF filter strength control to VTM”, JVET-T0064, Meeting by Teleconference, October 2020.

[2] VTM-11.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-11.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-T2010, Meeting by Teleconference, October 2020

[4] A. Segall, E. François, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, Meeting by Teleconference, October 2020.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).