| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0092-v1 |
| --- | --- |

| Title: | AHG9: Allocation of SEI message payload type for MPEG-I MIV/V3C carriage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

It is proposed that a new v3c_metadata( )  SEI message payload type be allocated in the VVC, HEVC, and AVC specifications, with the SEI message payload to be defined by in the ISO/IEC 23090-10 V3C version 2 specification, for carriage of V3C/MIV metadata within a VVC, HEVC, or AVC bitstream.

A similar approach was used for the definition of the HEVC/AVC green metadata SEI message, in which the message payload was defined outside of the HEVC and AVC specifications, with the payload type allocated within the HEVC and AVC specs.

At the time of preparation of the -v1 version of this document, the related MPEG contribution m55800 had not yet been reviewed by the MIV or V-PCC groups in MPEG Video and MPEG 3DG.

# Motivation

The motivation for a new V3C metadata SEI message specified in the video codec (VVC, HEVC, AVC) specifications is to enable an alternative method of carriage of V3C/MIV content to more easily leverage legacy distribution and playback tools, for the subset of V3C bitstreams which can meet some restrictions on the flexibilities supported in the V3C specification. In particular, use of the proposed SEI message imposes a requirement that all V3C video sub-bitstreams be temporally aligned, e.g. coded in the same order.

When V3C frame packing is used, (as defined in the draft V3C version 2 spec in MDS19639_WG07_N00003 [1])  a single layer video can be used to carry geometry, attributes, and occupancy video within a packed video sub-bitstream. The V3C unit payloads with non-video sub-bitstream V3C unit types can be carried within an SEI message, enabling carriage of V3C data using legacy video streaming systems.  The proposed SEI message can also be used without the use of packing, with multi-layer bitstreams, with the SEI message associated with the layer it is present in, based on its nuh_layer_id value.

Related MPEG WG4 contributions m55799 [2] and m55800 [3] are attached to this contribution, because some JVET participants may not have access to MPEG documents.

MPEG WG4 m55799 “Carriage of V3C/MIV data in VVC/HEVC/AVC SEI message” proposes that V3C version 2 define a V3C metadata SEI message carried in the video codec, and contains proposed syntax and semantics and additional motivation for creation of such an SEI message.

MPEG WG4 m55800 “Immersive video playback of HEVC bitstream on Intel GPU” describes a demonstration on an Intel Xe Max GPU of a proof of concept immersive video player which modifies the open source VLC player [4] to add a filter to render immersive video content which contains the proposed V3C metadata SEI message and frame packed video within an HEVC bitstream.

# VVC specification change

D.2.1 General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| … |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else if( payloadType = = 205 ) /* Specified in ISO/IEC 23090-10 */ |  |
| v3c_metadata( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| … |  |

# References

[1] m55799 “Carriage of V3C/MIV data in VVC/HEVC/AVC SEI message”

[2] m55800 “Immersive video playback of HEVC bitstream on Intel GPU”

[3] MDS19639_WG07_N00003 “Text of ISO/IEC CD 23090-5 Visual Volumetric Video-based Coding and Video-based Point Cloud Compression 2nd Edition”

[4] https://www.videolan.org/vlc/index.html

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).