INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 4

MPEG VIDEO CODING

ISO/IEC JTC 1/SC 29/WG 4 m 55799

January 2021, Online

Title: 	Carriage of V3C/MIV data in VVC/HEVC/AVC SEI message

Source: 	Jill Boyce, Basel Salahieh (Intel)

Abstract

An alternative method of carriage of V3C/MIV data is proposed, using a video codec (VVC/HEVC/AVC) SEI message. The use of this proposed carriage method places some restrictions on the flexibilities supported in the V3C specification. In particular, it imposes a requirement that all video sub-bitstreams be temporally aligned, e.g. coded in the same order.

When V3C frame packing is used a single video can be used to carry geometry, attributes, and occupancy within the packed video sub-bitstream. The V3C unit payloads with non video sub-bitstream V3C unit types can be carried within an SEI message, enabling carriage of V3C data using legacy video streaming systems.

The proposed SEI message can also be used without the use of packing, with multi-layer bitstreams.

# Introduction

It is proposed to define a video codec SEI message, with a payload type allocated in the VVC, HEVC, and AVC specifications, for carriage of the V3C units that do not contain video sub-bitstreams, e.g. for vuh_unit_type values other than V3C_OVD, V3C_GVD, V3C_AVD, V3C_PVD. The V3C_OVD, V3C_GVD, V3C_AVD, V3C_PVD will be referred to as “video V3C units”, and the other units, including V3C_VPS, V3C_AD, and V3C_CAD, will be referred to as “non-video” V3C units. When packing is used (vuh_unit_type = V3C_PVD), geometry, attributes, and occupancy can be carried within a single video.

It is proposed that a new v3c_metadata( )  SEI message payload type would be allocated in the VVC, HEVC, and AVC specifications, and that the v3c_metadata( ) SEI message payload be defined in the V3C version 2 base specification, or alternatively in the MIV specification.  A similar approach was used for the definition of the HEVC/AVC green metadata SEI message, in which the message payload was defined outside of the HEVC and AVC specifications, with the payload type allocated within the HEVC and AVC specs.

This proposed v3c_metadata( ) SEI message differs from the existing SEI messages in V3C and MIV, because the v3c_metadata( ) SEI messages are carried within the video codec bitstream as a video codec SEI message, rather than in a V3C NAL unit. Any V3C SEI messages that are present would be carried within the video codec’s v3c_metadata( ) SEI messages, as part of the atlas data.

As an alternative to the Systems Carriage of V3C draft spec in N19441, when this v3c_metadata( ) SEI message is used, V3C bitstreams can be represented by an HEVC (or VVC/AVC) bitstream, and can utilize the systems standards defined for carriage of HEVC (or VVC/AVC), such as ISOBMFF in ISO/IEC 14496-15, which enables the use of legacy video streaming systems, which is especially useful when V3C packing is used, such that only a single video stream is used.

Related contribution M56061 proposes an optional extension of AVC/HEVC/VVC file formats to indicate the presence of V3C data within the systems layer.

Related contribution M55800 illustrates an example implementation of MIV streaming playback using the proposed SEI message and frame packing in an HEVC bitstream.

Related contribution JVET-U0092 seeks allocation of SEI message payload type for MPEG-I MIV/V3C carriage.

In the following figure, the top shows the existing sample stream format for a bitstream containing multiple sequences, in which video and non-video V3C units are contained within the sample stream. The bottom shows an HEVC bitstream containing the proposed v3c_metadata( ) SEI message, which itself contains the non-video V3C units.

Figure 1. Comparison between V3C/MIV bitstream and HEVC bitstream with V3C metadata SEI.

# Background

Relevant sections of the V3C version 2 spec from WG07N0003 are copied below.

Table 2 : V3C unit types

| vuh_unit_type | Identifier | V3C unit type | Description |
| --- | --- | --- | --- |
| 0 | V3C_VPS | V3C parameter set | V3C level parameters |
| 1 | V3C_AD | Atlas data | Atlas information |
| 2 | V3C_OVD | Occupancy video data | Occupancy information |
| 3 | V3C_GVD | Geometry video data | Geometry information |
| 4 | V3C_AVD | Attribute video data | Attribute information |
| 5 | V3C_PVD | Packed video data | Packing information |
| 6 | V3C_CAD | Common atlas data | Information that is common for atlases in CVS. Specified in ISO/IEC 23090-12 |
| 7…31 | V3C_RSVD | Reserved | - |

#### 8.3.2.1 General V3C unit syntax

| v3c_unit( numBytesInV3CUnit ) { | Descriptor |
| --- | --- |
| v3c_unit_header( ) |  |
| v3c_unit_payload( numBytesInV3CUnit – 4 ) |  |
| } |  |

#### 8.3.2.3 V3C unit payload syntax

| v3c_unit_payload( numBytesInV3CPayload ) { | Descriptor |
| --- | --- |
| if( vuh_unit_type == V3C_VPS ) |  |
| v3c_parameter_set( numBytesInV3CPayload ) |  |
| else if( vuh_unit_type == V3C_AD ¦¦ vuh_unit_type == V3C_CAD ) |  |
| atlas_sub_bitstream( numBytesInV3CPayload ) |  |
| else if( vuh_unit_type == V3C_OVD ¦¦ vuh_unit_type == V3C_GVD ¦¦ vuh_unit_type == V3C_AVD ¦¦ vuh_unit_type == V3C_PVD ) |  |
| video_sub_bitstream( numBytesInV3CPayload ) |  |
| } |  |

# Proposed SEI message details

The proposed v3c_metatadata( ) video codec SEI message uses syntax structures defined in the V3C specification, including the v3c_unit_header( ). However, it does not directly use the v3c_unit( ) syntax structure in the V3C specification. Instead the SEI message syntax provides a variation on the v3c_unit( ), in order to avoid calling the v3c_unit_payload( ) syntax structure for video V3C units, which includes the video sub-bitstream, video_sub_bitstream( ), within it. Otherwise the SEI message contains the syntax elements and syntax structures to represent the same information present in the v3c_unit( ).  Other V3C syntax structures as defined in the V3C specification are used unchanged, including the v3c_parameter_set( ) and atlas_sub_bitstream( ) syntax structures.

Multiple unit types can be represented within the same SEI message, each with a unit header, v3c_unit_header( ). A syntax element, v3cm_unit_payload_size, is included in the SEI message to indicate the size in bytes of the payload size of the unit payload, for each occurrence of the non-videoV3C units, e.g. the V3C_VPS, V3C_AD, or V3C_CAD types. For each occurrence of those types the same syntax structures are called as would be the same as would be used in the v3c_unit_payload( ), which for the V3C_VPS unit type is the parameter set payload, v3c_parameter_set( ), and for V3C_AD or V3C_CAD unit types, is atlas_sub_bitstream( ).

For the video V3C unit types, here is no need to include a payload size or include the video sub-bitstream payload within the SEI message, because the video sub-bitstream payload itself is not contained within the SEI message.

In AVC, HEVC, and VVC, SEI messages are contained within an access unit, which for single layer video bitstreams contain one coded picture. In multi-layer bitstreams, such as those supported by HEVC multi-layer design in Annex F or in VVC’s multi-layer Main 10 profile, an access unit can have multiple layers, each layer of which may contain at most one coded picture for the access unit. Carriage of MIV with the HEVC multi-layer extension was originally proposed by Nokia in [1].

The contents of an individual v3c_metadata( ) SEI message, including the v3c_unit_header( ) and the associated V3C payload data, will have the same association with the coded picture in the same access unit (or access unit layer) as would have been the case if the v3c_unit_header( ) and the coded picture of the video sub-bitstream were contained within the same V3C unit.

When the V3C_PVD unit type is used, representing a packed frame, and there is a single video sub-bitstream, the entire V3C bitstream can be represented in a single layer video bitstream with the v3c_metadata( ) SEI message(s) embedded within it to represent the V3C metadata which describes the parameter sets and atlas data.

For each video sequence, which begins with a random access point, it is expected that an SEI message will be present in the first IRAP access unit that includes a V3C_VPS unit type for a parameter set. It is also expected that a V3C_AD unit type will also be represented, containing atlas data in the first IRAP picture.

For other coded pictures in the sequence, it is optional for V3C_AD unit type for atlas data to be present for MIV, which similarly does not require atlas data be present in all access units, with previously coded atlas data having persistence.

Use of V3C packing inherently requires that the coding order of geometry, occupancy, and attributes be temporally aligned for all pictures. The use of the proposed v3c_metadata( ) SEI message also imposes a requirement of temporal alignment of the atlas data coding order with the video sub-bitstreams, so can only be used for use cases which do not require additional flexibility.

When multiple video sub-bitstreams corresponding to V3C_GVD, V3C_OVD, V3C_AVD, and V3C_PVD types are present, they can be coded as multiple layers within a single multi-layer bitstream. Use of multi-layer HEVC or VVC implies a requirement that the coding order of the layers be temporally aligned, similar to when packing is used. For the multi-layer case, the V3C_VPS unit, V3C_AD, and V3C_CAD, when present, will be carried in an SEI message in the base layer of the video codec bitstream, e.g. the layer with NUH layer ID equal to 0. The V3C units containing other V3C types (V3C_GVD, V3C_OVD, V3C_AVD, or V3C_PVD) would be associated with the layer in which it is present. So, for example, an SEI message containing the nal_unit_header( ) representing type V3C_GVD would be included in the same layer as the coded video sub-bistream containing the geometry video coded pictures.

## Syntax

| v3c_metadata( payloadSize ) { | Descriptor |
| --- | --- |
| i = 0 |  |
| do { |  |
| v3c_unit_header( ) |  |
| i += 4 |  |
| if( vuh_unit_type == V3C_VPS ¦¦ vuh_unit_type == V3C_AD ¦ ¦ vuh_unit_type = = V3C_CAD ) { |  |
| v3cm_unit_payload_size | u(16) |
| if( vuh_unit_type == V3C_VPS ) |  |
| v3c_parameter_set( v3cm_unit_payload_size ) |  |
| else if( vuh_unit_type == V3C_AD ¦ ¦ vuh_unit_type = = V3C_CAD ) |  |
| atlas_sub_bitstream( v3cm_unit_payload_size ) |  |
| i += v3cm_unit_payload_size |  |
| } |  |
| } while ( i < payloadSize) |  |
| } |  |

## Semantics

The v3c_metadata( ) SEI message is used for carriage of V3C bitstreams within a video codec bitstream.

v3cm_unit_payload_size specifies the size in bytes of the numBytesInV3CPayload argument to the v3c_parameter_set( numBytesInV3CPayload) or atlas_sub_bitstream( ) syntax structures.

# Recommendations

We recommend the following:

- Request JVET to allocate an SEI message payload type for v3c_metadata( ) in VVC, HEVC, and AVC

- Define v3c_metadata( ) syntax and semantics in V3C version 2 specification, or alternatively in the MIV specification

No change is recommended to the Systems draft design for Carriage of V3C in N19441, because it provides support for a wider range of use cases than the proposed SEI message can support.

# References

[1] Vinod Kumar Malamal Vadakital, Kimmo Roimela, Lauri Ilola, Jaakko Keränen, Miska Hannuksela, Lukasz Kondrad, M50814 “Immersive Video bitstream format using multi-layered extension of HEVC,” Oct 2019.

[2 ] N19441, “Potential improvement for ISO/IEC 23090-10 Carriage of Visual Volumetric Video-based Coding Data”