| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0097-r1 |
| --- | --- |

| Title: | GDR Software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | seungwook.hong@nokia.com limin.2.wang@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution includes GDR software updated on VTM11.0. VVC has been finalized, and the GDR software based upon VTM11.0 is also stabilized. It is therefore proposed to integrate the GDR software into the VTM source code package. The GDR software offers the following setups:

- Encoding input video sequences into GDR bitstreams with the following features:

  - Flexible GDR period configured through parameter set by user,

  - The first picture can be either IDR or GDR picture (configurable),

  - SPS/PPS/APS are signaled at each GDR picture,

  - Similar number of bits per picture, implying the delay is as low as one frame interval,

  - Progressive intra refresh over a GDR period with even distribution of the forced intra areas over pictures within the GDR period using virtual boundary syntax in picture header,

  - Necessary (encoding) constraints on coding tools to prevent the leaks,

  - Exact match at the recovery point (or leak-free).

  - MD5 can be turned on or off for GDR pictures and recovering pictures (configurable),

  - ph_pic_output_flag may be signaled for GDR pictures and recovering pictures.

- Decoding the leak-free GDR bitstream using the VTM decoder.

  - If MD5 is off for GDR pictures and recovering pictures, decoder will ignore hash check for those pictures and only check the hashes for non GDR/recovering pictures.

# Introduction

GDR software was adopted as a branch of VTM at the 20th JVET meeting. Since then, the GDR software has been updated to VMT11.0 with some improvements.

# Software

This contribution includes the source code for the GDR software and proposes adding the software in the VTM source code package. In this GDR software,

- GDR period can be flexible. For example, a GDR period can cover all the pictures of a full second.

- The first picture can be either IDR or GDR picture, and SPS/PPS/APS will be signaled at each GDR picture.

- MD5 can be turned on or off for GDR pictures and recovering pictures. If MD5 is off for GDR pictures and recovering pictures, decoder will ignore hash check for those pictures and only check the hashes for non GDR/recovering pictures.

- ph_pic_output_flag is signaled for GDR pictures and recovering pictures. Decoder can output GDR pictures and recovering pictures according to VVC spec and the associated ph_pic_output_flag.

- Vertical progressive intra refresh over a GDR period is implemented. The width of vertical intra refresh strip per picture can be as small as 8 pixels. For GDR period equal to one second, the width of vertical intra refresh strip is calculated based on the frame rate and the picture’s width.

- The boundary of clean area and dirty area is specified by virtual boundary syntax in picture header.

- CUs in clean area will not use any coding information in dirty areas of the current picture and the reference pictures to prevent leaks and achieve exact match at the recovery point.

- For CUs in clean area, coding modes involving any coding information in dirty area are considered as invalid.  Mode selection for CUs in clean area is performed based upon both the coding costs and the validities of possible coding modes.

- Decoder also provides an optional functionality for leak testing.

# Simulation Results

In evaluating the performance of GDR software, simulations were conducted over JVET sequences under low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with a few necessary changes. A sample config file for GDR is provided, where

- The first picture is coded as a GDR picture with ph_recovery_poc_cnt = 0, and then GDR pictures with ph_recovery_poc_cnt = frame rate are inserted every two seconds,

- DecodingRefreshType is 3,

- GOPSize is 1.

- Each picture refers up to 4 immediately preceding pictures in decoding order,

- IntraQPOffset and QPoffset are set equal to 0.

- SEIDecodedPictureHash is set to 1. MD5 is off for GDR pictures and recovering pictures.

For the comparison purpose, simulations of low-delay B anchor are also performed, where the first picture is coded as IDR, and then, CRA (or intra) pictures are inserted periodically, which are sync with GDR pictures.

Table 1 shows the comparison results of the GDR software over the anchor in terms of BD-rate. The overall loss is 2.32% in terms of luma BD-rate.

Table 1. GDR over VTM-11.0 Anchor in BD-rate

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Anchor (VTM11.0) on Linux |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.27% | 4.75% | 7.20% | 101% | 99% |
| Class C | 1.69% | 4.90% | 5.23% | 100% | 100% |
| Class E | 4.91% | 11.98% | 11.86% | 104% | 96% |
| Overall | 2.32% | 6.61% | 7.71% | 101% | 99% |
| Class D | 2.24% | 6.94% | 7.02% | 99% | 101% |
| Class F | 4.14% | 7.70% | 8.30% | 109% | 97% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).