| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0099-v2 |
| --- | --- |

| Title: | AHG11: Neural Network-based Super Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Kevin Reuzé Jianle Chen Hongtao Wang Marta Karczewicz Jingya Li | Tel: Email: | akotra@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution studied the performance of applying a Neural-Network based super-resolution used as upsampling filter in the context of VVC RPR. Prior to encoding, a given picture is downsampled by a factor of 2x using the inbuilt RPR mechanism of VTM11. PSNR of the coded frame is computed by calculating the MSE between the original picture and the up-sampled version of the decoded picture. The upsampled picture is generated by the Neural Network-based up-sampling filter instead of the existing VTM up-sampling filter.

On average, for Class A1 sequences, Luma BD-Rate gains of 5.74% and 8.79% for AI and RA configurations respectively were observed.

# Introduction

The contribution JVET-T0092 [2] asserts that few Class A sequences show considerable BD-Rate gain by down-sampling the complete sequence before encoding by using RPR coding for low-bitrate scenarios. The Anchor (VTM-10) QPs used in JVET-T0092 were 32, 37, 42, 47. This contribution further evaluates the BD-Rate gain improvements by using a Neural Network based up-sampling filter in the place of the existing VTM upsampling/interpolation filter. Moreover, the Anchor (VTM-11.0) QPs used in this proposal are 22, 27, 32, 37, 42 as per the CTC of Neural Network based video coding [5].

Prior to encoding, a given picture is down-sampled by a factor of 2x (ScalingRatioHor and ScalingRatioVer are both set equal to 2) using the inbuilt RPR mechanism of VTM-11.0 [1]. PSNR of the decoded picture is computed by calculating the MSE between the original picture and the up-sampled version of the decoded picture. The upsampled decoded picture is generated by a Neural Network-based upsampling filter instead of the existing VTM upsampling/interpolation filter.

# Proposal

The Network architecture used in the proposed up-sampling filter is a simplified (baseline, single-scale) version of EDSR [3]. The number of network parameter is around one million parameters.

# Training details

BVI_DVC dataset [4] is used as the training dataset. BVI_DVC dataset consists a total of 200 sequences in 4 different spatial resolutions. Each sample in the training data is basically a tuple of (decoded, original (2x resolution)).

Two versions of the training were tried. The training data used in “Training 1” uses the BVI_DVC  encoded with VTM directly as input, whereas the training data used in “Training 2” uses the BVI_DVC data downsampled by 2x using VTM downsampling filter and then encoded using VTM. Moreover “Training 1” uses QPs 22, 27, 32, 37 to generate training data whereas “Training 2” uses QPs 20, 25, 30, 35 to generate training data.

Two models are trained to handle different QP ranges. Other details of inference and training are as shown in the tables below.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | XEON_E5_2690v4 256 GB Memory No GPU |
|  | SW environment: | SUSE Linux Enterprise Server 11 |
|  | Framework: | Pytorch C++ library |
|  | Total Conv. Layers | 16 |
|  | Total FC Layers | 0 |
|  | Total Parameter Number | 1,369,859 per model. (times 2 models) = 2,739,718 |
|  | Parameter Precision | 32(F) |
|  | Memory Parameter (MB) | 10 MB |
| Optional | Preprocessing: | Upsample YUV 4:2:0 to YUV 4:4:4 before feeding to the network, Chroma up-sampling is done by repeating in vertical direction and averaging in horizontal direction. The output from network is converted from YUV 4:4:4 to YUV4:2:0 by down sampling the Chroma (averaging in both horizontal and vertical direction) |
| Network Information in Training Stage |  |  |  |
| Mandatory |  | HW environment: | CPU: Intel(R) Xeon(R) Gold 6138 CPU @ 2.00GHz, 754GB Memory GPU: NVIDIA Geforce gtx 1080 ti 11gb |
|  |  | SW environment: | Ubuntu 18.04.4 LTS CUDA Version:10.1, Driver Version: 418.87.00 |
|  |  | Framework: | Pytorch 1.7.0 |
|  |  | Epoch: | 300 Epochs for “Training 1” 126 Epochs for “Training 2” |
|  |  | Batch size: | 16 |
|  |  | Training time: | 36h for “Training 1”, “16h for Training 2” |
|  |  | Training data information: | BVI-DVC |
|  |  | Configurations for generating compressed training data (if different to VTM CTC): | For “Training 1”: QP values used were: 22, 27, 32, 37 For “Training 2”: QP values used were: 20, 25, 30, 35 Model1, uses the two low QP values as input Model2, uses the two high QP values as input Model selection during inference uses the “Sequence level QP” |
| Optional |  |  |  |
|  |  | Patch size | 48 x 48 (for reconstructed), 96 x 96 (for original) |
|  |  | Learning rate: | 1e-4 |
|  |  | Optimizer: | Adam |
|  |  | Loss function: | Weighted (8:1:1) L1 loss |
|  |  | Preprocessing: | Upsample YUV 4:2:0 to YUV 4:4:4 before feeding to the network, Chroma up-sampling is done by repeating in vertical direction and averaging in horizontal direction. The output from network is converted from YUV 4:4:4 to YUV4:2:0 by down sampling the Chroma (averaging in both horizontal and vertical direction) |
|  |  | Other information: |  |

# Simulation results

The proposed Neural-Network based upsampling filter is integrated on top of VTM-11.0.

The Anchor used in the simulation is VTM-11.0 and the QP values used for the Anchor are 22, 27, 32, 37 and 42. For the test, the QP values used are 17, 22, 27, 32 and 37 respectively. MCTF filter is enabled both in the Anchor and the Test. The GOP size used is 32 both in the Anchor and the Test.

Table 2 and 3 below shows the result of applying the super resolution scheme using the existing VTM up-sampling/interpolation filter for Random access and All Intra configurations respectively.

Table 2. Results of applying super resolution scheme using existing VTM up-sampling filter for RA

|  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 (QP 22,27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K |  | Tango2 |  | -4.41% |  | 0.19% |  | 10.52% |  | 50% |  | 33% |  |
|  |  | FoodMarket4 |  | -0.57% |  | 10.37% |  | 9.06% |  |  |  |  |  |
|  |  | Campfire |  | -2.56% |  | 95.98% |  | 35.38% |  |  |  |  |  |
| Class A2 4K |  | CatRobot1 |  | 24.07% |  | 44.74% |  | 68.09% |  | 40% |  | 30% |  |
|  | DaylightRoad2 | 42.46% | 18.66% | 41.19% |  |  |
|  |  | ParkRunning3 |  | -0.06% |  | 297.20% |  | 146.30% |  |  |  |  |  |

Table 3. Results of applying super resolution scheme using existing VTM up-sampling filter for AI

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 (QP 22,27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K |  | Tango2 |  | -2.73% |  | -4.22% |  | 4.95% |  | 68% |  | 36% |  |
|  | FoodMarket4 | 1.41% | 6.63% | 7.16% |  |  |
|  | Campfire | 8.75% | 784.57% | 46.39% |  |  |
| Class A2 4K | CatRobot1 | 7.16% | 23.13% | 36.15% | 54% | 38% |
|  |  | DaylightRoad2 |  | 19.26% |  | -1.14% |  | 17.41% |  |  |  |  |  |
|  | ParkRunning3 | -2.45% | 147.69% | 78.17% |  |  |

Table 4 and 5 below shows the result of applying the super resolution scheme using the Neural-Network based up-sampling filter trained using “Training 1” scheme for RA and AI configurations, respectively.

Table 4. Results of applying super resolution scheme using Neural-Network based up-sampling filter    for RA (“Training 1” approach (trained for 300 Epochs))

|  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 (QP 22,27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K |  | Tango2 |  | -7.48% |  | -11.02% |  | -1.39% |  | 92% |  | 32% |  |
|  |  | FoodMarket4 |  | -4.03% |  | 7.27% |  | 5.77% |  |  |  |  |  |
|  |  | Campfire |  | -12.68% |  | 79.71% |  | -3.04% |  |  |  |  |  |
| Class A2 4K | CatRobot1 | 11.86% | 21.67% | 32.83% | 99% | 37% |
|  | DaylightRoad2 | 25.36% | 5.73% | 21.00% |  |  |
|  | ParkRunning3 | -4.87% | 194.75% | 89.13% |  |  |

Table 5. Results of applying super resolution scheme using Neural-Network based up-sampling filter    for AI (“Training 1” approach (trained for 300 Epochs))

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 (QP 22,27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -6.49% | -9.62% | -7.78% | 147% | 36% |
|  | FoodMarket4 | -3.35% | 4.74% | 3.14% |  |  |
|  | Campfire | -3.63% | 293.00% | -3.72% |  |  |
| Class A2 4K | CatRobot1 | -2.59% | 6.64% | 11.29% | 99% | 37% |
|  |  | DaylightRoad2 |  | 5.47% |  | -10.53% |  | 0.18% |  |  |  |  |  |
|  | ParkRunning3 | -7.39% | 100.34% | 46.43% |  |  |

Table 6 and 7 below shows the result of applying the super resolution scheme using the Neural-Network based up-sampling filter trained using “Training 2” scheme for RA and AI configurations respectively.

Table 6. Results of applying super resolution scheme using Neural-Network based up-sampling filter   for RA (“Training 2” approach (trained for 126 Epochs))

|  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 (QP 22,27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K |  | Tango2 |  | -7.71% |  | -9.67% |  | 4.70% |  | 90% |  | 31% |  |
|  | FoodMarket4 | -5.20% | 4.88% | 4.81% |  |  |
|  |  | Campfire |  | -13.44% |  | 85.12% |  | 6.07% |  |  |  |  |  |
| Class A2 4K |  | CatRobot1 |  | 7.85% |  | 23.33% |  | 40.51% |  | 74% |  | 31% |  |
|  |  | DaylightRoad2 |  | 21.33% |  | 5.31% |  | 23.25 |  |  |  |  |  |
|  | ParkRunning3 | -7.44% | 197.45% | 93.99% |  |  |

Table 7. Results of applying super resolution scheme using Neural-Network based up-sampling filter   for AI (“Training 2” approach (trained for 126 Epochs))

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 (QP 22,27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K |  | Tango2 |  | -6.74% |  | -10.56% |  | -0.37% |  | 148% |  | 36% |  |
|  | FoodMarket4 | -3.96% | 3.00% | 3.20% |  |  |
|  | Campfire | -6.53% | 302.39% | 3.27% |  |  |
| Class A2 4K |  | CatRobot1 |  | -4.22% |  | 7.44% |  | 16.21% |  | 92% |  | 37% |  |
|  |  | DaylightRoad2 |  | 2.83% |  | -11.59% |  | 0.74% |  |  |  |  |  |
|  | ParkRunning3 | -10.69% | 97.93% | 48.40% |  |  |

# Conclusion

This contribution studied the performance of applying a Neural-Network based super-resolution used as upsampling filter in the context of VVC RPR. Prior to encoding, a given picture is downsampled by a factor of 2x using the inbuilt RPR mechanism of VTM11. PSNR of the coded frame is computed by calculating the MSE between the original frame and the up-sampled version of the coded frame. The upsampled frame is generated by the Neural Network-based up-sampling filter instead of the existing VTM up-sampling filter.

On average, for Class A1 sequences, Luma BD-Rate gains of 5.74% and 8.73% for AI and RA configurations respectively are observed.

In future work, the Neural-Network based upsampling filter will be tested by integrating it into the Encoder RD loop, wherein, the decision to code a given picture at a downsampled resolution is taken by comparing its RD cost with the RD cost of coding picture at the normal resolution. This way the super-resolution scheme is asserted to also have a benefit for other sequences including low resolution sequences (Class B etc.)

# References

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- AHG9/AHG11: Neural network based super resolution SEI, JVET-T0092, T.Chujoh, E. Sasaki, T. Ikai

- B. Lim, S. Son, H. Kim, S. Nah and K. M. Lee, "Enhanced Deep Residual Networks for Single Image Super-Resolution," 2017 IEEE Conference on Computer Vision and Pattern Recognition Workshops (CVPRW), Honolulu, HI, 2017, pp. 1132-1140, doi: 10.1109/CVPRW.2017.151.

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-T2006, S. Liu, A. Segall, E. Alshina, R.-L. Liao

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).