| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0102 |
| --- | --- |

| Title: | AHG11: Variable rate end-to-end image compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin, Fangdong Chen, Li Wang (Hikvision). | Email: | linchaoyi@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

In order to yield compressed images with different quality, most existing end-to-end (E2E) image compression models require training separate networks for different compression rates. This contribution proposes to use conditional convolution for image compression, which can adjust the compression rates by input parameter λ with only one model. We test this conditional convolution on existing E2E image compression model.  Experimental results that the proposed model is comparable with the BPG 4:4:4 while it performs a little worse than mbt2018 model

Introduction

The auto-encoder based image compression frameworks optimize the networks by minimizing the rate-distortion function with the Lagrange multipliers. The Lagrange multiplier λ is treated as a hyper-parameter for a trade-off between the rate and the distortion. This implies that it is necessary to train and deploy separate networks for different rates. However, this is impractical when a broad range of rates is desired. Recently, some works [1][2] focus on designing a variable rate E2E image compression model. In this contribution, a variable rate image compression model is proposed. The proposed model can achieve a broad range of rates with a variable input parameter λ.

Network architecture

The proposed image compression model is based on the mean-scale image compression model (mbt2018) [3]. The overall framework is shown in Figure 1. Compared with the original mbt2018 model, the proposed model utilizes the condition convolution to replace the regular convolution in the analysis transform and the synthesis transform. Thus, both the image to be compressed and the lambda are the input of the proposed model. In the hyperprior network, the regular convolution is utilized.

The conditional convolution, which conditioned on the Lagrange multiplier λ, is utilized to replace the regular convolution for adjusting the rates. The architecture for conditional convolution is shown in Figure 2. Let x be the input feature map, the output y can be denoted as:

y=convx×s(λ)

where s(λ) is a channel-wise scaling factor.

Figure 1: The proposed variable rates image compression framework. The Cond_conv denotes the conditional convolution.

Figure 2: The network architecture for conditional convolution. The Conv represents the regular convolution, the kernel size is 5, the number of filters is 192, and the s denotes the stride. The Fully connected (m, n) denotes the fully connected operation with input dimension m and output dimension n.

Training

The whole image compression model is optimized by:

L=i=0N-1λi×d(xi,xi)+R

where d denotes the distortion metric mean squared error (MSE), R denotes the rates of the latent, and λi denotes the λ value chosen for each patch. For each patch, the λi is picked randomly from a pre-determined finite λ set. The λ set used in this experiment is:

λ=[0.001,  0.002,  0.005,  0.008,  0.01,  0.014,  0.017,  0.02,  0.024,  0.027,  0.03,  0.034,  0.037,  0.05,  0.054,  0.057,  0.06,  0.064,  0.067,  0.07]

The ImageNet dataset [4] is used in training. The 256×256 patches are extracted at random locations without resizing images and the batch size is set to 8. The iteration steps are set to 3,500,000 (3.5M). The Adam optimizer is used for optimization. The detailed information are summarized in Table 1.

Table 1 Training information

| Training information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type: | 1x Telsa-V100-32G |
|  | CPU Type: | Intel(R) Xeon(R) Gold 6132 CPU @ 2.60GHz |
|  | Framework: | Tensorflow 1.14.0 |
|  | Batch size: | 8 |
|  | Training time: | about 120 hours |
|  | Total Parameter Number： | 13.2M |
|  | Parameter Precision (Bits): | 32 (F) |
|  | Training set: | ImageNet |
|  | Iterations: | 3.5M steps |
| Optional | Learning rate: | 10-4 for the first 1M iterations and 10-5 for the next 2.5M iterations. |
|  | Patch size: | 256x256x3 |
|  | Optimizer: | ADAM |

Inference

In inference stage, given an image, we can adjust the λ parameter to achieve the desired rate. Detailed inference information is summarized in below Table 2.

Table 2 Inference information

| Inference information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type: | 1x Telsa-V100-32G |
|  | CPU Type: | Intel(R) Xeon(R) Gold 6132 CPU @ 2.60GHz |
|  | Framework: | Tensorflow 1.14.0 |
|  | Number of GPUs per Task: | 1 |
|  | Parameter Precision (Bits): | 32 (F) |
|  | Total Conv. Layers: | 18 |
| Optional | Total FC Layers: | 16 |
|  | Batch size: | 1 |
|  | Patch size: | frame |

Experimental Results

The original mbt2019 model and BPG are used for comparison. Figure 3 presents the coding performance comparison RD curves on Kodak dataset [5]. It can be seen that the proposed model can cover a wide range of rates (0.1 bpp – 1.2 bpp) by adjusting the λ parameter. The RD curve of the proposed model is almost as well as that of the BPG 4:4:4. Compared with the original mbt2018 model, the proposed model performs a little worse.

Figure 3: Coding performance comparison in Kodak dataset.

The encoding time and decoding time are presented in Table 3. In our source code, the range_coder [6] is implemented for entropy coding and entropy decoding. The inference is performed on CPU. It can be seen that the proposed model costs more time compared with the original mbt2018 model in both encoding and decoding stages. It is not surprising since the conditional convolution is applied on the decoder, which increases the complexity for decoding. In the future work, it is desired to remove the conditional convolution in decoder.

Table 3 Encoding time and decoding time in Kodak dataset.

|  | Proposed model (sec) | mbt2018 model (sec) |
| --- | --- | --- |
| Encoding time (sec) | 26.37 | 20.39 |
| Decoding time (sec) | 25.72 | 20.16 |

Conclusion

In this work, the conditional convolution is proposed for image compression model to enable the rate adaptation by adjusting input parameter λ. The proposed model can achieve a wide range of rates while keep its compression performance comparable. Besides, the conditional convolution can be used in any other image compression model theoretically. Further study will be focused on integrating the conditional convolution on other image compression models to test the generalization ability and remove the conditional convolution in the decoding process.

Reference

- Yoojin Choi, Mostafa El-Khamy, Jungwon Lee, ‘Variable Rate Deep Image CompressionWith a Conditional Autoencoder’, Proceedings of the IEEE/CVF International Conference on Computer Vision (ICCV), 2019, pp. 3146-3154

- Fei Yang, Luis Herranz, Joost van de Weijer, José A. Iglesias Guitián, Antonio López, Mikhail Mozerov, “Variable Rate Deep Image Compression With Modulated Autoencoder”, arXiv: 1912.05526.

- D. Minnen, J. Ballé, and G. Toderici, ‘Joint Autoregressive and Hierarchical Priors for Learned Image Compression’, arXiv:1809.02736.

- Olga Russakovsky, Jia Deng, Hao Su, Jonathan Krause, Sanjeev Satheesh, Sean Ma, Zhiheng Huang, Andrej Karpathy, Aditya Khosla, Michael Bernstein, et al. ImageNet large scale visual recognition challenge. International Journal ofComputer Vision, 115(3):211–252, 2015.

- Eastman Kodak. Kodak lossless true color image suite (PhotoCD PCD0992). http://r0k.us/graphics/kodak, 1993

- https://github.com/lucastheis/rangecoder

Patent rights declaration(s)

Hikvision may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).