| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0115-v2 |
| --- | --- |

| Title: | AHG11: Neural Network-based In-Loop Filter Performance with No Deblocking Filtering stage |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Jianle Chen Anand Meher Kotra Marta Karczewicz | Email: | hongtaow@qti.qualcomm.com cjianle@qti.qualcomm.com akotra@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a Neural Network-based in-loop filter (NN filter). The NN model design is based on the subtest 1 of JVET-U0094. Additionally, 8 lines of neighboring pixels to each side of the current CTU are included into the input reconstructed samples to the NN model. The deblocking boundary strength information of VVC is also used as additional NN model input information but the actual deblocking filtering process is bypassed. Simulation results reportedly show 6.95%, 13.92 % and 14.97% BD rate saving for RA and 6.13%, 11.87%, 13.46% BD rate saving for AI, for Y, Cb and Cr components respectively.

# Introduction

Convolutional Neural Networks (CNN)-based methods recently showed lot of potential in artifact removal and objective quality improvement. A Neural Network based In-Loop Filter proposed as an additional in-loop filter stage is being tested in the EE contribution: JVET-U0094 [1]. In this contribution, modifications on top of subtest 1 of [1] that use neighboring samples of the current CTU and the deblocking boundary strength information as input to the NN model are studied.

# Proposed NN filter

Figure 1 Proposed network architecture

As shown in Figure 1, the network structure used in this contribution is the same as that in subtest 1 of [1], which contains 14 convolutional layers in total, including N=12 hidden layers with 96 channels in each layer. Each hidden layer consists of a 3x3 convolutional layer and a Leaky RELU. There are about one million model parameters in each model.

Compared to [1], the input size of the NN filtering process is increased from 128x128 to 144x144 to include 8 neighboring samples to each side of the current CTU. The luma samples are interleaved into four 72x72 sized blocks before being used as inputs of the filtering process as proposed in [4]. While the output size stays the same (64x64x6), corresponding to the 4 luma blocks and 2 chroma blocks.

Additionally, the boundary strength information generated by the de-blocking filter module of VVC is used as additional input planes to the network.

## Training process

Same as done in [1], the BVI-DVC dataset [2] was used in the training of models. The raw images are compressed with AI and RA configurations, and training examples are generated from the reconstructed frames. The QPs used for training are 20, 25, 30, 35, 40, 45. To be aligned with the modified network structure, 144x144 luma and 72x72 chroma blocks, as well as the boundary strength (BS) information are extracted to create the training set.

The detailed training and inference information can be found in Table 1 and Table 2.

Table 1 Network Information in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | TF v2.2.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~130 |
|  | Batch size: | 13Kx64 |
|  | Training time: | 89h |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72x10 |
|  | Learning rate: | 8.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

Table 2 Network Information in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | TF v2.2.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1,010,406 x 4Models |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 3.85 x 4 models |
|  | MAC (Giga) | 1.99e-4 Giga/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 14 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 72x72x10 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage | 1.90MB (processing unit size 128x128) |
|  | Other information: |  |

## Implementation in VTM software

The filtering process of the proposed method was same as done in [1]. The proposed NN filter is incorporated into VTM-10.0 as in-loop filter prior to SAO. While the boundary strength calculation of VTM software is used to create the corresponding additional input planes to the NN filter, the actual de-blocking filtering is not applied. CTU level switching (on/off) flags are signaled to indicated whether a block is NN filtered or not. Scaling of NN filter residues is also applied. As done subtest 1 of [1], 4 models are trained to cover different QP ranges and picture types, and the model selection is done at picture level.

# Experimental results

The common test conditions defined in [3] are used to evaluate the proposed NN filter. The results are summarized in Table 3. It was observed that 6.95%, 13.92 % and 14.97% BD rate saving for RA and 6.13%, 11.87%, 13.46% BD rate saving for AI, for Y, Cb and Cr components respectively are achieved.

Table 3 Simulation results of the proposed method

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -7.10% | -8.89% | -13.28% | 145% | 13252% | 0% |
| Class A2 | -7.46% | -15.28% | -13.78% | 140% | 11771% | 1% |
| Class B | -6.63% | -14.69% | -15.24% | 145% | 14161% | 0% |
| Class C | -6.88% | -15.73% | -16.80% | 130% | 12292% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -6.95% | -13.92% | -14.97% | 140% | 12969% | 0% |
| Class D | -7.97% | -17.08% | -18.64% | 131% | 13743% | 1% |
| Class F | -3.48% | -10.73% | -10.16% | 170% | 16351% | 0% |
| Class H | -6.47% | -9.98% | -13.24% | 180% | 16645% | 0% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -5.78% | -9.14% | -12.10% | 154% | 9600% | 0% |
| Class A2 | -5.65% | -12.03% | -10.75% | 136% | 8291% | 0% |
| Class B | -5.58% | -11.60% | -13.58% | 125% | 7038% | 0% |
| Class C | -6.05% | -13.31% | -15.69% | 118% | 5970% | 0% |
| Class E | -7.95% | -12.99% | -14.39% | 133% | 8634% | 0% |
| Overall | -6.13% | -11.87% | -13.46% | 131% | 7507% | 0% |
| Class D | -6.27% | -12.75% | -16.56% | 117% | 6718% | 0% |
| Class F | -4.26% | -10.40% | -10.56% | 114% | 6859% | 0% |
| Class H | -6.32% | -8.99% | -10.41% | 138% | 7044% | 0% |

# Reference

- EE: Tests on Neural Network-based In-Loop Filter, JVET-U0094, Hongtao Wang, Marta Karczewicz, Jianle Chen, Anand Meher Kotra

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-T2006, S. Liu, A. Segall, E. Alshina, R.-L. Liao

- AHG9: Convolutional neural network loop filter, JVET-M0159, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).