| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0133 |
| --- | --- |

| Title: | BoG report on CE complexity analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a report of activities from the BoG on CE complexity analysis. The BoG held meetings at the following times during the 21st JVET meeting:

- January 8th 13:00 – 15:00

- January 8th 15:20 – 16:40

# Mandates

The BoG was established with the following mandates:

- to perform more analysis on the impact of different proposals in terms of introducing dependencies between blocks, and possible impact on parallel operation, pipelining and throughput.

- to also include proposals for non-CE and CE-related contributions from section 5.1.3 of the meeting report in this analysis.

# Principles

It was suggested that a fixed shift does not count as an operation in these assessments and can be considered “free”

# Discussion of Proposals

This document presents assessment on the number of operations for proposed modification to the Rice Derivation process (clause 9.3.3.2) for CE1.1, CE1.2.a and CE1.3. These tests feature no inter-block dependencies.

## CE-1.1

This test features no inter-block dependencies.

This test features slice-level signaling (1 parameter of up to 2 bits)

| Components | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| Values RiceT are initialized as following: RiceT = {{32, 52}, {32 << sh_shift_rice, 52 << sh_shift_rice}} |  |  | This is slice level. No additional process in TU |
| Variable shift is derived as following for idx set equal to 0, if baseLevel is equal to 0, and set equal to 1, otherwise |  |  | Idx dependency on baseLevel is TU level. |
| shift = (localSumAbs < RiceT[0][idx]) ? 0 : (localSumAbs < RiceT[1][idx]) ? sh_shift_rice : (sh_shift_rice << 1) locSumAbs = locSumAbs >> shift | 1 cond., 1 shift | 2 | Parallel comparator can be used. |
| Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. | 2 condition. | 2 | baseLevel is TU level defined and can be integrated with clipping thresholds. |
| cRiceParam is then is refined as: cRiceParam = cRiceParam + shift | 1 add | 1 |  |
| Total per Rice call | 3 condition, 1 shift, 1 add. | 5 |  |

Total per Rice call (in addition to VTM) = 3

One participant queried the type of parallel comparator that would be used

The proponent stated that the parallel comparator could be completed in 1 cycle with 2 threshold comparisons

Another participant stated they would consider the parallel comparator as 1 cycle.

One participant suggested that operations from the draft specification should be taken literally, whilst one suggested that we should be considering the implementation not the specification.

## CE-1.2a

This test features no inter-block dependencies.

This test features no slice-level signaling

| Components | Operations Per Rice Call | Num Ops | Comments |  |
| --- | --- | --- | --- | --- |
| Variable shift is derived as following: scalledCoef = (localSumAbs * sh_scale_rice) >> 3 | 1 shift | 1 | Allowed multipliers are 1 or 2, with slice level switching. Mult. and shift integrated in single operation. |
| shift = (scalledCoef == 0) ? 0 : Floor( Log2( scalledCoef) ) | 1 MSB find | 1 | Condition is covered by MSB find. |
| locSumAbs = locSumAbs >> shift | 1 shift | 1 |  |
| Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. | 2 condition | 2 | Same as in CE1.1 |
| cRiceParam is then is refined as: cRiceParam = cRiceParam + shift | 1 add | 1 |  |
| Total per Rice call | 2 cond., 2 shift, 1 add, 1 MBS find | 6 |  |

Total per Rice call (in addition to VTM) = 4

## CE-1.2b

Slice-level signalling combined with on/off control

| CE-1.2.b |  | Num Ops per TU | Comments |
| --- | --- | --- | --- |
| CE1.2 num. of ops (clause 9.3.3.2) |  | 64 | Assessed in CE1.2.a |
| History maintenance, once per TU StatCoeff[ cIdx ] = (StatCoeff[ cIdx ] + Floor( Log2(abs_remainder[ n ] ) ) + 2) >> 1 | 1 condition, 1 shift, 2 add, 1 MSB find | 3 |  |
| Replacement of Rice derivation of CE1.2 . | CE1.2 Rice -2 cond. -1 shift - 1 MSB -1 shift -2 cond. -1 add | -8 ops |  |
| Total per 4x4 TU: |  | 59 |  |
| Memory requirements |  | 12 bits of storage |  |
| History characteristics |  |  |  |
| Dependencies | No cross-sub-block dependencies. No cross-CTU dependencies. Cross-TU dependency within CTU. |  |  |
| History update rate | 1 per TU |  |  |
| History fetching usage rate | 1 per TU |  |  |

Proponent confirmed that history updated once per TU

## CE-1.3

This test features no inter-block dependencies.

This test features no slice-level signalling

| Components | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| if ( riceAdaptationFlag ) locSumAbs0 = locSumAbs locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) |  |  | Condition is just a TU level not a coeff level locSumAbs − baseLevel * 5 can be saved. |
| - If riceAdaptationFlag is equal to 0, given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. - Otherwise (riceAdaptationFlag is equal to 1), the following applies: - Depending on baseLevel, the variables a and b are derived as follows: - If baseLevel is not equal to 0, a is set equal to 4 and b is set equal to 0. - Otherwise (baseLevel is equal to 0), a is set equal to 9 and b is equal set to 20. |  |  | This is just a TU level not a coeff level |
| - Given the variable locSumAbs0, the Rice parameter cRiceParam is derived as follows: cRiceParam = Max(0, floorLog2(a * locSumAbs0 + b) – 5) | 1 shift (+1 add) // a * locSumAbs0, this can be 1 cycle 1 add // + b 1 MSB find 1 add // - 5 1 condition // Max, Saturation operation | 5 | 9*x needs 2 sequential operations 1 shift – “for free”, 1 add 1 add 1 MSB 1 add 1 cond. |
| Total per Rice call | 1 shift, 3 add, 1 MSB find, 1 condition | 5 |  |

Total per Rice call (in addition to VTM) = 3

One proponent suggested that saving versus existing spec should not be taken into account (leaving total at 5) if savings only for HBD coding

The proponent suggested that the shift and add of the multiply operation could be implemented in 1 cycle.

One participant suggested that this was 2 sequential operations requiring two cycles.

One participant suggested that the shift came “for free” and hence shouldn’t be counted

The proponent asked for the total be adjusted to 5.

One participant suggested that the range of rice parameters will have an effect on complexity.

One participant asked what the theoretical range of rice parameters.

It was suggested that theoretically a * locSumAbs0 could require 27 bits in the 16 bit extended precision

cRiceParam is 5 less than this so could be 22

Further work: assess the maximum possible value of cRiceParam (or equivalent) for all testsOne participant asked if the formula can be used for regular VVC coding

One participant suggested it might be possible (with clipping)

One participant suggested there were differences between results the formula and the existing look up table.

Further work: Table of values for rice parameter derivation for CE-1.3 and VVC

| locSumAbs | VVC | CE-1.3 |
| --- | --- | --- |
| 24 | 3 | ? |

## CE-1.5

This section presents an assessment of the number of operations for the proposed modification to the Rice Derivation process (clause 9.3.3.2) and an analysis of inter-block dependencies using the proposed modification.

### Rice derivation process

| Components | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| locSumAbs = Min(31, Max( locSumAbs − baseLevel * 5, 0 ) >> riceShift ) | 2 cond., 1 shift | 3 | baseLevel is TU level defined and can be integrated with clipping thresholds. |
| Given the variable locSumAbs, the Rice parameter cRiceParam is derived as the addition of the value of riceShift and the value specified in Table 128. | 1 add | 1 |  |
| Total per Rice call | 2 condition, 1 shift, 1 add | 4 |

Total per Rice call (in addition to VTM) = 2

In addition a number of instructions are required off-line (in parallel) from the main decoding loop. This has been previously assessed to be 101 per 4x4 if implemented as per spec.

### Analysis of inter-block dependencies

In CE-1.5 (and CE-2.3) the value of a counter is not used to calculate a parameter (such as valueWithParity) until the next 4x4 is coded.

The updating of counters and parameter sets (the values of valueWithParity, valueNoParity etc.) has been designed to be done asynchronously from the main decoding loop (see figure 1). The updated parameter set values are only needed when the next block is partially coded. Thus the critical decoding path is unaffected by these updates (see figure 1)

Figure 1. Parallel counter and parameter set update

To solve any issues with inter-block dependency, it is necessary to show the parallel update can be completed before the first output parameter (valueNoParity or valueWithParity) is required for the next block.

Counters are only updated for coefficients on the leading diagonal of the TU. Thus only sub-TUs on the leading diagonal lead to updates (figure 2). The update for the top left (DC) sub-TU uses different counters and parameter sets from all other sub-TUs.

| Upd | X | X | X |
| --- | --- | --- | --- |
| X | Upd | X | X |
| X | X | Upd | X |
| X | X | X | Upd |

Figure 2. Sub-TUs in a 16x16 TU that required updates

Analysis shows:

- If 4x4s in two consecutive TUs are coded

  - No issues – time for update

- If a 4x8 TU is coded

  - No issue – different counters and parameter sets are used in the different 4x4s

- If 8x8 TU is coded

  - No issue – CABAC context bins cannot run out in first sub-TU. Thus second sub-TU will utilise both passes 1 and 2 of coefficient coding. There is time for update whilst processing pass 1 (flags) which does not require the rice parameter to be known.

- Larger than 8x8 TU

  - The Worst case is coding a 4x4 after a 4x4 on the leading diagonal when all CABAC context coded bins for the TU have already been used

    - 1st and 2nd pass not used

    - All coefficients coded using 3rd (bypass) pass

In this worst case situation the update must be completed within the time taken to code:

- sign bits (at least 1)

- significant group flag

- the summation of the 5 neighbouring coefficients for the bottom left coefficient in the sub-TU

- the unary component of the bypass bin (at least 1 bin).

Only one counter update is required in this time, as previous counter updates in the 4x4 can be handled in parallel to coefficient coding in the 4x4. This is due to the design of CE-1.5 (and CE-2.3) only updating counters for coefficients on the leading diagonal as shown below (figure 3):

| Upd | X | X | X |
| --- | --- | --- | --- |
| X | Upd | X | X |
| X | X | Upd | X |
| X | X | X | Upd |

Figure 3. Coefficients in a sub-TU that require updates

Thus, at least 3 coefficients are coded between counter updates – allowing for time for the update to complete.

The update required to complete is the computation of nextValueNoParity.

Previously the following values for the sub-TU will have been calculated:

int baseValue = count >> LOG2_DIVISOR;

lowerLimit = 1 << baseValue;

upperLimit = 3 << baseValue;

The update function can be written as:

if (value > 0) {

if (value >= upperLimit) {

count += INCREMENT_COUNT;

} else if ((value << 1) < lowerLimit) {

count = std::max(0, count - DECREMENT_COUNT);

}

}

nextValueNoParity = std::min(12, std::max(0, ((count + ROUNDING) >> LOG2_DIVISOR) - 2));

However given INCREMENT_COUNT is 4, DECREMENT_COUNT is 1, ROUNDING is 8 and LOG2_DIVISOR is 4, the update process can be simplified to:

if (value >= upperLimit) {

nextValueNoParity = std::min(12, std::max(0, (count - 20) >> 4));

count += INCREMENT_COUNT;

} else if ((value << 1) < lowerLimit) (&& value > 0)) {

nextValueNoParity = std::min(12, std::max(0, (count - 25) >> 4));

count = std::max(0, count - DECREMENT_COUNT);

} else {

//       nextValueNoParity = nextValueNoParity;

}

Thus the update requires 1 subtraction, 1 shift and 3 conditionals, which can be completed in the available time.

Conclusion

By allowing the counters and parameters required for the implementation of CE-1.5 to be updated in parallel from coefficient decoding it is possible to minimize the operations added to the critical path of the decoder. CE-1.5 adds a maximum of 2 operations per coefficient or 32 per 4x4 TU.

One participant requested a table of all additional off-line operations The proponent stated that previously the number stated was 133.

The proponent stated that alternative implementations could reduce this by the use of look of tables of approximately 160 bits.

4x4 block 3 different maps of 2 bits 96 bits, 8x2, 16x1, 1 map

Further work: count of extra off-line operations for 4x4 133-32=101

## CE-3.5

| CE-3.5 |  | Num Ops per TU | Comments |
| --- | --- | --- | --- |
| CE1.2 num. of ops (clause 9.3.3.2) |  | 64 | Assessed in CE1.2.a |
| History maintenance, once per TU StatCoeff[ cIdx ] = (StatCoeff[ cIdx ] + Floor( Log2(abs_remainder[ n ] ) ) + 2) >> 1 | 1 condition, 1 shift, 2 add, 1 MSB find | 3 |  |
| History usage per 4x4 TU else locSumAbs += HistValue | 1shift, 19 adds | 20 |  |
| Total per 4x4 TU: |  | 87 |  |
| Memory requirements |  | 12 bits of storage |  |
| History characteristics |  |  |  |
| Dependencies | No cross-sub-block dependencies. No cross-CTU dependencies. Cross-TU dependency within CTU. |  |  |
| History update rate | 1 per TU |  |  |
| History fetching usage rate | 1 per TU |  |  |

## CE-3.6

| CE-3.6 |  | Num Ops per TU | Comments |
| --- | --- | --- | --- |
| CE1.3 num. of ops (clause 9.3.3.2) |  | 64 | Assessed in CE1.3 |
| History maintenance, once per TU StatCoeff[ cIdx ] = (StatCoeff[ cIdx ] + Floor( Log2(abs_remainder[ n ] ) ) + 2) >> 1 | 1 condition, 1 shift, 2 add, 1 MBS find | 3 |  |
| History usage per 4x4 TU else locSumAbs += HistValue | 1shift, 19 adds | 20 |  |
| Total per 4x4 TU: |  | 87 |  |
| Memory requirements |  | 12 bits of storage |  |
| History characteristics |  |  |  |
| Dependencies | Cross-TU dependency, reset at CTU No cross-sub-block dependencies. |  |  |
| History update rate | 1 per TU |  |  |
| History fetching usage rate | 1 per TU |  |  |

## CE-2.1

This test does not have inter-block dependencies.

This test has Slice-level signaling and requires up to 7 bits (Max Rice: 8).

| Components | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0, the Rice parameter cRiceParam is set equal to sh_ts_residual_coding_rice_idx. |  |  | This is slice level. No additional process in TU |
| Total per Rice call | 0 | 0 |  |

# Other proposals

## JVET-U0051

| Components | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| Coeff loop: currRem = (AbsLevel[ xC ][ yC ] + 3 * currRem + 2 ) >> 2 | 3 add (+1 shift) 1 shift | 3 | Appears as 5 operations, but proponent claims can be done in 4. Multiplication of 3 * currRem can be broken into shift and add. Proponent claims this can be done in 1 cycle. |
| if (n= = firstPosMode0) firstRem = (AbsLevel[ xC ][ yC ] + 3 * firstRem + 2 ) >> 2 | 1 cond 2 add (+1 shift) 1 shift | (5) | It’s off line operation (see comments below) |
| if ( riceAdaptationFlag ) locSumAbs0 = locSumAbs locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) |  |  | Condition is just a TU level not a coeff level locSumAbs − baseLevel * 5 can be saved. |
| - If riceAdaptationFlag is equal to 0, given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. - Otherwise (riceAdaptationFlag is equal to 1), the following applies: - Depending on baseLevel, the variables a and b are derived as follows: - If baseLevel is not equal to 0, a is set equal to 1 and b is set equal to 12. - Otherwise (baseLevel is equal to 0), a is set equal to 6 and b is equal set to 13. |  |  | This is just a TU level not a coeff level |
| - Given the variable locSumAbs0, the Rice parameter cRiceParam is derived as follows: cRiceParam = Max(0, floorLog2(a * locSumAbs0 + b * currRem) – 5) | 1add (2shift) 1 add 1 MSB find 1 add 1 condition // Max, Saturation operation | 5 | (1<<2)+(1<<1) 1 add 1 add 1 MSB 1 add 1 cond. |
| Total per Rice call |  | 9 | Critical loop |

It is assessed at least 5 operations are required per 4x4 TU

It was suggested that the number of operations for “cRiceParam = Max(0, floorLog2(a * locSumAbs0 + b * currRem) – 5)” be checked.

The calculation of currRem introduces an additional parse dependency with a subblock – the previous component must be completely decoded before the next may be processed. This potentially adds a delay to the decoding process.

Proponent states this approach is already used in HEVC RExt

The offline nature of the second component was queried. It was suggested that this component may introduce a sub-block parse dependency. One participant asked for more time to study.

Further work: Add history / memory requirements

## JVET-U0062

Further work: provide table of operations before next meeting