| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0139v2 |
| --- | --- |

| Title: | BoG report on high bit rate / high bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a report of activities from the BoG on high bit rate / high bit depth coding. The BoG held meetings at the following times during the 21st JVET meeting:

- January 13th 13:10 – 15:15

- January 14th 15:20 – 17:15

The BoG recommends:

- Downscale 4K HLG content in CTC to 2K (all content in CTC will be HD)

- Include JVET-U0103 in VTM 12.0 (SIMD changes for HBD)

- Including tests at a higher QP range in the  high bit-depth CTC for consumer 12-bit

  - QPs 22-37

  - Using tool settings from 10-bit HDR CTC (JVET-T2011) with exception of InternalBitDepth=12

  - Using 4:2:0 versions of the 12bit HLG and PQ sequences used in high bit depth CTC (JVET-T2018)

  - All intra and random access simulations only

- The following changes for existing tests at QPs of 12 to -33 in the high bit depth CTC

  - Disable LMCS for HLG, PQ and SVT

  - Disable BCW, MMVD, SMVD, Geo, BIO, DMVR, Affine, ISP, but encourage testing of tool-on configurations in the CE / other contributions.

  - Include FireEater sequence for AI only (previously in CE but not CTC)

  - Use GOP 16 structure to allow comparison with HEVC.

  - Distribute HM RExt anchor with the CTC template.

  - SVT-16 simulations optional for the additional range of QP=12 to QP=-3

- The following with regards to the CE

  - Use VTM 12.0 with CE-1.1 for RRC and CE-2.1 for TSRC and SIMD modifications as codebase and anchor.

  - For CE 1 tests

    - CE-1.1 is replaced by the proposed test, and CE-2.1 is used for TSRC.

  - For CE-2 tests

    - CE-2.1 is replaced by the proposed test, and CE-1.1 is used for RRC.

  - Lossless results are encouraged

  - For CE-2, screen content testing using TGM 1080 and TGM 720 from the screen content CTC (with InternalBitDepth set to 12) is mandatory, for other CE tests it is optional

  - Use timetable outlined in section 4.2 of this document

- Asking for contributions of new 12/16 bit RGB sequences

- Requesting 12 bit screen content to include in HBD CTC and opinions on suitable QPs for this content

The BoG also identified the following volunteers:

- Downscale HLG content from 4K to 2K (D. Rusanovskyy / A. Browne to crosscheck)

- Convert 2K HLG from 4:4:4 to 4:2:2 and 4:2:0 (D.Rusanovskyy / A. Browne to crosscheck)

- Crosscheck conversion of 12 bit PQ from half float to 4:2:0 (A. Browne)

- Update CTC document (M. Sarwer)

# Mandates

The BoG was established with the following mandates:

- to propose the plan for the next round of the CE on entropy coding for high bit depths.

- identify which of the current proposals should be further investigated.

- discuss modifications of test conditions

  - sequences

  - QP ranges

  - anchor for 12/16 bit

  - additional HEVC RExt anchor

# Contributions

JVET-U0138, “Information on VVC coding efficiency in high bit depth coding”, T. Ikai (Sharp)

There was no time to review this contribution.

# Common Test Conditions

## Meeting 13th January

Greater range of QPs – including higher QPs normally associated with consumer applications

- Using modified HDR CTC settings

  - What QPs?

    - 12-27

  - 420 / 422?

  - Sequences from HBD CTC?

One expert considered testing at such high QPs already handled by previous experiments.

One experts QPs at 10 – 25 to enable simpler comparison between 10 and 12 bit

Another expert backs the range of 10-25 to enable evaluation of other 12 bit tools

One expert suggests existing range 22-37 for comparison with 10 bit HDR CTC and potentially suitable for premium consumer level applications

One expert suggests the decision should depend on the expected bit rate increase and this is a good way of thinking about how much QPs can be adjusted

One expert suggests that the bit rate in a 12 bit consumer profile/use-case will be close to that for a 10 bit profile

Note: need to create 420 versions of HBD CTC sequences

One expert suggested only including All Intra and Random Access – not Low Delay B/P

One expert would like to include some screen content sequences – this could be optional

The lack of 12 bit screen content is an issue that needs to be resolved

Recommendation: Request 12 bit screen content to include in HBD CTC and opinions on suitable QPs for this content

Replace 4K HLG sequences with HD HLG sequences to reduce simulation times (need HDRTools for conversion)

Recommendation: use 2K/HD HLG sequences.

Alternatives to SVT 12/16 bit RGB sequences

Recommendation: ask for contributions of new 12/16 bit RGB sequences

Disable tools (not for 10-25 420 QP tests) above? (JVET-U0072 tool off) (JVET-U0103 SIMD)

- LMCS for PQ / HLG

- BCW

- MMVD

- SMVD

- Geo

- BIO

- DMVR

- Affine

- ISP

- LFNST (already disabled)

One expert suggested keeping the tools enabled to allow comparison once CE is complete

Several experts suggested disabling tools was useful for encode run time reduction

Recommendation: consider disabling tools above.

VTM 12.0 anticipated 3 weeks after the meeting – end first week Feb

Recommendation: to include JVET-U0103 in VTM 12.0

Recommendation: include FireEater sequence in high bit depth CTC

## Meeting 14th January

For all simulations in the CTC (both -33 to 12 and 22 to 37)

Recommendation: Replace existing 4K HLG sequences in JVET-T2018 with down-scaled 2K HLG sequences

Volunteer / help required: downscale HLG content from 4K to 2K (D.Rusanovskyy / A, Browne to cross-check)

For high QP (consumer) 12 bit

Recommendation: Include tests at a higher QP range in the  high bit-depth CTC for consumer 12-bit

QPs 22-37

Using tool settings from 10-bit HDR CTC (JVET-T2011) with exception of InternalBitDepth=12

Using 4:2:0 versions of the 12bit HLG and PQ sequences used in high bit depth CTC (JVET-T2018)

All intra and random access simulations only

Volunteer required: convert PQ from half float to 4:2:0 (already done / A, Browne to cross check )

Volunteer required: convert 2K HLG from 4:4:4 to 4:2:0 (D.Rusanovskyy / A, Browne to cross check )

For low QP (professional) 12 and 16 bit

Volunteer required: convert HLG from 4:4:4 to 4:2:2 (D.Rusanovskyy / A, Browne to cross check )

Recommendation: Disable LMCS for HLG, PQ and SVT from QP=12 to QP=-33

Recommendation: Disable BCW, MMVD, SMVD, Geo, BIO, DMVR, Affine, ISP for QPs 12 to -33 in high bit depth CTC (JVET-T2018), but encourage testing of tool-on configurations in the CE / other contributions

Recommendation: include FireEater sequence in high bit depth CTC as mandatory for AI only for QPs 12 to -33

Volunteer: Update CTC document (M. Sarwer)

One expert suggested only using SVT for 16 bit (not 12 bit) but with a greater QP range (QP=-33 to 12)

One expert suggested keeping SVT-12 as it is a valid use case

Recommendation: Several experts making SVT-16 optional for to QP=12 to QP=-3

One expert suggest a secondary anchor using HM RExt (16.22 or later) for low QP study in the CTC - Recommend

Recommendation: use GOP 16 structure for low QPs to allow comparison with HEVC

Several experts suggest the HM RExt anchor can be prepared by AHG and distributed as part of template

Recommendation: distribute HM RExt anchor with CTC template

# CE

## Meeting 13th January

Experiments

CE-1 – RRC

CE-1.1

CE-1.2a

CE-1.2b

CE-1.3

CE-1.5

CE-3.5

CE-3.6

JVET-U0062

One expert suggested not having a combination phase in this CE – combinations of techniques can be CE-related or non-CE contributions

One expert suggested that the agenda for the CE is not clear – how to evaluate the combination of transform coefficient clipping and rice coding modifications

CE-2 – TSRC?

CE-2.1

CE-2.3

One expert suggested combinations of 1 technique from either CE-1 and CE-2 with CE-3 would be useful as a combination not further combinations of multiple CE-1 / CE-2 tests -this is better as a CE-related / non-CE contribution

One expert suggested that any combinations only be allowed in the CE if defined in the initial CE drafting phase.

CE-3 – Transform coefficient range clipping (JVET-U0052)

Tested clipping to 20 – have other values been checked e.g. 18, 19, 20, 21, 22 for both 12 and 16 bit cases

Code base

Recommendation: VTM 12.0 with SIMD modifications

Anchors

VTM with rice coding tool for assessing 16 bit SVT

One expert suggested CE-2.1 for TSRC

One expert suggested highest performance for 16 bit configuration

Several experts suggested using simplest would be appropriate

Several expert suggested CE-1.1 for RRC

Recommendation: use CE-1.1 for RRC and CE-2.1 for TSRC as anchor testing in CE (all proponents of CE-1 and CE-2 content)

One expert emphasized that tests for CE-1.1 and CE-2.1 would still be required against VTM

One expert suggested full agreed anchor for VTM still required

## Meeting 14th January

Timetable (what is deadline for next meeting)

T1: 2021-January-29:	   			Version of CE description with final descriptions for tests CE-1, CE-2 and CE-3 uploaded. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to marked as "withdrawn".

T2: VTM12.0+1week:				Integration of CE-1.1 and CE-2.1 into a separate CE branch of the VTM (CE anchor) is completed

T3: VTM12.0+2 week (not later than February 19) :

Integration of all CE-1/CE-2/CE-3 tools into a separate CE branch of the VTM is completed and initial study by cross-checkers begins.

T4: T3 + 2 weeks: 	 				Final description of combined tests is provided.

T5: T3 + 2 weeks: 					Final version of CE-1/CE-2/CE-3 software and full results are provided, final cross-check begins.

T6: T3 + 4 weeks: 					Final version of combination software and full results are provided, final cross-check of combination begins.

T7: 2021-April-09: 			   		CE contribution documents including specification text and complete test results are uploaded to the JVET document repository.

Recommendation: All CE code to be based on a code base of VTM 12.0 + CE-1.1 + CE-2.1. For CE 1 tests, CE-1.1 is replaced by the proposed test, and CE-2.1 is used for TSRC. For CE-2 test CE-2.1 is replaced by the proposed test, and CE-1.1 is used for RRC

Recommendation: Lossless results are encouraged

Recommendation: For CE-2, screen content testing using TGM 1080 and TGM 720 from the screen content CTC (with InternalBitDepth set to 12) is mandatory, for other CE tests it is optional