| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0047 |
| --- | --- |

| Title: | CE-3.1 and CE-3.2: Transform coefficients range extension for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This document reports results for CE-3.1 and CE-3.2, an extended coefficient precision for high bit-depth coding. The proposal uses bit-depth dependent coefficient range to bitDepth+6 in CE-3.1 / bitdepth + 5 in CE-3.2 and it also clips its maximum range to 20.

The experimental results of bdrate gain are reported in the condition where the anchor uses extended precision = 0 in 12bit case and 1 in 16 bit case, test uses extended precision = 1 in both 12bit and 16bit cases.

CE-3.1 (include extended_precision_processing_flag, Min(BitDepth + 6, 20) ):

Low QP

- 1.23 %, 0.47 % and 0.75 % in bdrateY in PQ, HLG and SVT in AI

- 0.67 %, 0.62 % and 0.49 % in bdrateY in PQ, HLG and SVT in LDB

- 0.60 %, 0.50 % and 0.33 % in bdrateY in PQ, HLG and SVT in RA

Normal QP

- 0.00 % and 0.00 % in bdrateY in H1 and H2 in AI

- - 0.01 % and 0.01 % in bdrateY in H1 and H2 in RA

CE-3.2 (no extended_precision_processing_flag, Max(Min(BitDepth + 5, 20),15) ):

Low QP

- 1.12 %, 0.43 % and 0.68 % in bdrateY in PQ, HLG and SVT in AI

- 0.64 %, 0.58 % and 0.44 % in bdrateY in PQ, HLG and SVT in LDB

- 0.57 %, 0.47 % and 0.29 % in bdrateY in PQ, HLG and SVT in RA

Normal QP

- 0.01 % and - 0.01 % in bdrateY in H1 and H2 in AI

- - 0.02 % and - 0.01 % in bdrateY in H1 and H2 in RA

It is also reported that the additional bdrate gain by clipping range in combination with other CEs proposals.

CE-3.1

Low QP

In AI

- (-6.82%	-6.39%	-6.37% in psnrG, psnrG and psnrG in SVT16 over vtm12.0)

- -0.69%	-0.45%	-0.45% in psnrG, psnrG and psnrG in SVT16 over cebase

- -0.20%	-0.21%	-0.21% in psnrG, psnrG and psnrG in SVT16 over CE1.4

- -0.18%	-0.22%	-0.22% in psnrG, psnrG and psnrG in SVT16 over CE1.5

- -0.17%	-0.22%	-0.22% in psnrG, psnrG and psnrG in SVT16 over CE1.6

- -0.81%	-0.51%	-0.53% in psnrG, psnrG and psnrG in SVT16 over CE1.7

In LDB

- (-2.98%	-2.91%	-2.88% in psnrG, psnrG and psnrG in SVT16 over vtm12.0)

- -0.20%	-0.23%	-0.22% in psnrG, psnrG and psnrG in SVT16 over cebase

- -0.09%	-0.07%	-0.07% in psnrG, psnrG and psnrG in SVT16 over CE1.4

- -0.10%	-0.07%	-0.07% in psnrG, psnrG and psnrG in SVT16 over CE1.5

- -0.10%	-0.07%	-0.07% in psnrG, psnrG and psnrG in SVT16 over CE1.6

- -0.27%	-0.22%	-0.20% in psnrG, psnrG and psnrG in SVT16 over CE1.7

In RA

- (-2.60%	-2.88%	-2.83% in psnrG, psnrG and psnrG in SVT16 over vtm12.0)

- -0.05%	-0.32%	-0.30% in psnrG, psnrG and psnrG in SVT16 over cebase

- -0.01%	-0.13%	-0.13%in psnrG, psnrG and psnrG in SVT16 over CE1.4

- -0.01%	-0.13%	-0.13% in psnrG, psnrG and psnrG in SVT16 over CE1.5

- 0.01%	-0.13%	-0.13% in psnrG, psnrG and psnrG in SVT16 over CE1.6

- -0.08%	-0.38%	-0.35% in psnrG, psnrG and psnrG in SVT16 over CE1.7

It is argued that CE-3.1 has more gain while CE-3.2 would reduce verification cost because there’s no two versions in more than 10 bit case.

In 8 bit and 10 bit case, there are no change compared to VVC version 1 because the proposal disables the extended range when bit-depth is equal to or less than 10 bit.

Introduction

In the current VVC design, the extended precision (extended coefficient range) has not been specified.

However, the VTM software supports the extended precision with an enabling macro named JVET_R0351_HIGH_BIT_DEPTH_ENABLED.

When extended precision is enabled the following changes are applied.

- Scaling shift value (bdShift) is reduced while transform shift value is increased depending on bitdepth

- Coefficient range (i.e. clipping value) is increased depending on bitdepth

- residual coding in EG(k) is changed to extend the suffix length to enable the enlarged coefficient signaling. Specifically, truncSuffixLen is set equal to Log2TransformRange

- To keep the maximum length of Truncated Rice (TR) binarization and EG(k) binarization being equal to 32, the maxPreExtLen in EG(k) is also set equal to 26 - Log2TransformRange

In addition to the above, the following changes are applied.

For CE-3.1:

It is derives ExtendedPrecisionFlag as follow to prevent 10 bit case changes.

ExtendedPrecisionFlag = extended_precision_processing_flag && bitDepth > 10.

It is proposed to derive Log2TransformRange, bdShift1 (bdShift in 8.7.3	Scaling process for transform coefficients), bdShift2 (bdShift in 8.7.4	Transformation process for scaled transform coefficients), truncSuffixLen and maxPreExtLen as follows.

If ExtendedPrecisionFlag is equal to 0.

- Log2TransformRange = 15

- bdShift1 = BitDepth + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) − 5 + sh_dep_quant_used_flag

- bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? ( 20 − BitDepth ) : ( 21 − BitDepth )

- truncSuffixLen = 15

- maxPreExtLen = 11

Otherwise (ExtendedPrecisionFlag is equal to 1)

- Log2TransformRange = ExtendedPrecisionFlag ? Max( 15, Min(20, BitDepthY + 6) ) : 15

- bdShift1 = BitDepthY + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) + 10 − log2TransformRange+ sh_dep_quant_used_flag

- bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth

- truncSuffixLen = Log2TransformRange

- maxPreExtLen = 32 – 6 – truncSuffixLen = 26 – Log2TransformRange

For CE-3.2:

It is derives without ExtendedPrecisionFlag, and there is no change compared to VVC version 1 when bit-depth is equal to or less than 10 bit.

- Log2TransformRange = Max( 15, Min(20, BitDepthY + 5 ) )

- bdShift1 = BitDepthY + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) + 10 – log2TransformRange+ sh_dep_quant_used_flag

- bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth

- truncSuffixLen = Log2TransformRange

- maxPreExtLen = 32 – 6 – truncSuffixLen = 26 – Log2TransformRange

Combination test

At the last meeting, it was suggested to investigate the relationship between transform precision and entropy coding modifications·in CE tests.

We did some combination tests and compared them with different anchors to see the performance, the results are as follows (16bit test in Low QP case):

Test : CE-3.1 + other CE tests, Log2TransformRange = 20

Anchor : Log2TransformRange = 15 in vtm12.0 and 22 in other CE tests

| SVT RGB |  | AI |  |  |
| --- | --- | --- | --- | --- |
|  |  | psnrG | psnrB | psnrR |
| SVT16 | Over vtm12.0 | -6.82% | -6.39% | -6.37% |
| SVT16 | Over cebase | -0.69% | -0.45% | -0.45% |
| SVT16 | Over CE1.4 | -0.20% | -0.21% | -0.21% |
| SVT16 | Over CE1.5 | -0.18% | -0.22% | -0.22% |
| SVT16 | Over CE1.6 | -0.17% | -0.22% | -0.22% |
| SVT16 | Over CE1.7 | -0.81% | -0.51% | -0.53% |
| SVT16 | Over CE1.8 |  |  |  |
|  |  | LDB |  |  |
|  |  | psnrG | psnrB | psnrR |
| SVT16 | Over vtm12.0 | -2.98% | -2.91% | -2.88% |
| SVT16 | Over cebase | -0.20% | -0.23% | -0.22% |
| SVT16 | Over CE1.4 | -0.09% | -0.07% | -0.07% |
| SVT16 | Over CE1.5 | -0.10% | -0.07% | -0.07% |
| SVT16 | Over CE1.6 | -0.10% | -0.07% | -0.07% |
| SVT16 | Over CE1.7 | -0.27% | -0.22% | -0.20% |
| SVT16 | Over CE1.8 |  |  |  |
|  |  | RA |  |  |
|  |  | psnrG | psnrB | psnrR |
| SVT16 | Over vtm12.0 | -2.60% | -2.88% | -2.83% |
| SVT16 | Over cebase | -0.05% | -0.32% | -0.30% |
| SVT16 | Over CE1.4 | -0.01% | -0.13% | -0.13% |
| SVT16 | Over CE1.5 | -0.01% | -0.13% | -0.13% |
| SVT16 | Over CE1.6 | 0.01% | -0.13% | -0.13% |
| SVT16 | Over CE1.7 | -0.08% | -0.38% | -0.35% |
| SVT16 | Over CE1.8 |  |  |  |

We can see that combining clipping transform precision extended method and entropy coding modifications have an additional coding performance and can fix the problem of maxPreExtLen becomes smaller in 16bit case.

Background of transform coefficient range and maxPreExtLen

At the last meeting, JVET-U0052 mentioned that to keep the maximum length of Truncated Rice (TR) binarization and EG(k) binarization being equal to 32, the maxPreExtLen is set equal to 26 - Log2TransformRange. Thus maxPreExtLen becomes smaller as Log2TransformRange increases as sown Table 1.

- maxPreExtLen = 32 – 6 – truncSuffixLen = 26 – Log2TransformRange

Table 1. Example of TR and EG(k) binarization length in the 16 bit-depth case.

|  | TR | EG(k) |  | Sum |
| --- | --- | --- | --- | --- |
| Log2TransformRange | length of prefixVal | maxPreExtLen | truncSuffixLen (=Log2TransformRange) |  |
| 15 | 6 | 11 | 15 | 32 |
| BitDepth+3 | 6 | 7 | 19 | 32 |
| BitDepth+4 | 6 | 6 | 20 | 32 |
| BitDepth+5 | 6 | 5 | 21 | 32 |
| BitDepth+6 | 6 | 4 | 22 | 32 |
| BitDepth+7 | 6 | 3 | 23 | 32 |

To alleviate this problem, we proposes to clip the coefficient range up to 20 to keep maxPreExtLen equal to 6.

|  | TR | EG(k) |  | Sum |
| --- | --- | --- | --- | --- |
| Log2TransformRange | length of prefixVal | maxPreExtLen | truncSuffixLen |  |
| 20 = Min(BitDepth+6, 20) | 6 | 6 | 20 | 32 |

Experiment (CE-3.1 and CE-3.2 results over cebase anchor)

Low QP

CE-3.1

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0_cebase |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.33% | -1.94% | -2.09% | -1.33% | -1.97% | -2.11% | 101% | 94% |
| PQ422 | -1.13% | -1.44% | -1.56% | -1.12% | -1.44% | -1.55% | 101% | 94% |
| Overall | -1.23% | -1.69% | -1.82% | -1.23% | -1.70% | -1.83% | 101% | 94% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.70% | -0.74% | -0.81% | -0.71% | -0.74% | -0.81% | 101% | 98% |
| PQ422 | -0.64% | -0.57% | -0.64% | -0.64% | -0.57% | -0.63% | 100% | 98% |
| Overall | -0.67% | -0.65% | -0.72% | -0.68% | -0.66% | -0.72% | 100% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.62% | -0.72% | -0.77% | -0.62% | -0.71% | -0.77% | 101% | 99% |
| PQ422 | -0.59% | -0.54% | -0.62% | -0.58% | -0.54% | -0.61% | 100% | 98% |
| Overall | -0.60% | -0.63% | -0.69% | -0.60% | -0.62% | -0.69% | 100% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.51% | -0.88% | -0.91% | 111% | 93% |
| HLG422 | -0.43% | -0.78% | -0.80% | 104% | 94% |
| Overall | -0.47% | -0.83% | -0.86% | 108% | 93% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.67% | -0.93% | -0.97% | 102% | 95% |
| HLG422 | -0.57% | -0.67% | -0.72% | 101% | 95% |
| Overall | -0.62% | -0.80% | -0.84% | 102% | 95% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.55% | -0.83% | -0.87% | 101% | 97% |
| HLG422 | -0.45% | -0.63% | -0.69% | 102% | 97% |
| Overall | -0.50% | -0.73% | -0.78% | 102% | 97% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.69% | -0.45% | -0.45% | 107% | 98% |
| SVT12 | -0.80% | -1.25% | -1.27% | 95% | 87% |
| Overall | -0.75% | -0.85% | -0.86% | 101% | 93% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.20% | -0.23% | -0.22% | 103% | 100% |
| SVT12 | -0.79% | -0.65% | -0.66% | 94% | 93% |
| Overall | -0.49% | -0.44% | -0.44% | 98% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.05% | -0.32% | -0.30% | 102% | 99% |
| SVT12 | -0.61% | -0.64% | -0.65% | 94% | 93% |
| Overall | -0.33% | -0.48% | -0.48% | 98% | 96% |

CE-3.2

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0_cebase |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.21% | -1.77% | -1.91% | -1.21% | -1.80% | -1.92% | 101% | 95% |
| PQ422 | -1.03% | -1.33% | -1.43% | -1.03% | -1.33% | -1.43% | 102% | 95% |
| Overall | -1.12% | -1.55% | -1.67% | -1.12% | -1.57% | -1.67% | 101% | 95% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.68% | -0.71% | -0.77% | -0.69% | -0.70% | -0.78% | 99% | 98% |
| PQ422 | -0.61% | -0.52% | -0.60% | -0.60% | -0.52% | -0.59% | 100% | 98% |
| Overall | -0.64% | -0.61% | -0.68% | -0.64% | -0.61% | -0.68% | 100% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.59% | -0.67% | -0.72% | -0.59% | -0.66% | -0.72% | 100% | 93% |
| PQ422 | -0.55% | -0.51% | -0.59% | -0.54% | -0.51% | -0.59% | 100% | 94% |
| Overall | -0.57% | -0.59% | -0.66% | -0.56% | -0.58% | -0.65% | 100% | 94% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.45% | -0.81% | -0.83% | 110% | 94% |
| HLG422 | -0.40% | -0.72% | -0.75% | 104% | 94% |
| Overall | -0.43% | -0.76% | -0.79% | 107% | 94% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.62% | -0.85% | -0.90% | 102% | 96% |
| HLG422 | -0.54% | -0.63% | -0.66% | 101% | 96% |
| Overall | -0.58% | -0.74% | -0.78% | 101% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.51% | -0.76% | -0.82% | 101% | 89% |
| HLG422 | -0.42% | -0.57% | -0.65% | 102% | 91% |
| Overall | -0.47% | -0.67% | -0.73% | 101% | 90% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.69% | -0.45% | -0.45% | 107% | 98% |
| SVT12 | -0.67% | -1.04% | -1.06% | 95% | 90% |
| Overall | -0.68% | -0.74% | -0.76% | 101% | 94% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.20% | -0.23% | -0.22% | 103% | 100% |
| SVT12 | -0.69% | -0.57% | -0.58% | 100% | 93% |
| Overall | -0.44% | -0.40% | -0.40% | 101% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.05% | -0.32% | -0.30% | 102% | 89% |
| SVT12 | -0.53% | -0.57% | -0.58% | 99% | 83% |
| Overall | -0.29% | -0.44% | -0.44% | 100% | 86% |

Normal QP

CE-3.1

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over cebase |  |  |  |  |  |  |  |
|  | wPSNR |  |  | PSNR |  |  |  |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | -0.25% | 0.01% | 0.01% | -0.29% | 0.08% | 101% | 100% |
| Class H2 |  |  |  | -0.01% | 0.07% | 0.13% | 107% | 100% |
| Overall | 0.00% | -0.25% | 0.01% | 0.00% | -0.11% | 0.11% | 104% | 100% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over cebase |  |  |  |  |  |  |  |
|  | wPSNR |  |  | PSNR |  |  |  |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.03% | -0.02% | 0.00% | 0.01% | -0.01% | 102% | 100% |
| Class H2 |  |  |  | 0.00% | -0.09% | -0.08% | 102% | 100% |
| Overall | 0.00% | 0.03% | -0.02% | 0.00% | -0.04% | -0.05% | 102% | 100% |

CE-3.2

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over cebase |  |  |  |  |  |  |  |
|  | wPSNR |  |  | PSNR |  |  |  |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.01% | -0.41% | -0.02% | 0.02% | -0.47% | -0.22% | 101% | 98% |
| Class H2 |  |  |  | 0.01% | -0.13% | 0.20% | 107% | 101% |
| Overall | 0.01% | -0.41% | -0.02% | 0.01% | -0.30% | -0.01% | 104% | 99% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over cebase |  |  |  |  |  |  |  |
|  | wPSNR |  |  | PSNR |  |  |  |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.01% | 0.14% | -0.02% | -0.01% | 0.15% | -0.07% | 102% | 100% |
| Class H2 |  |  |  | 0.00% | -0.03% | 0.04% | 102% | 100% |
| Overall | -0.01% | 0.14% | -0.02% | -0.01% | 0.06% | -0.02% | 102% | 100% |

Conclusions

It is proposed to adopt a clipping value 20 for the coefficient range in high-bit depth case into the next VVC specification and VTM.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

[2] T. Zhou, T. Ikai, “AHG8: Transform coefficients range extension for high bit-depth coding”, JVET-U0052

[3] A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-U2022

- Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).