| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20-28th Apr. 2021 | Document: JVET-V0050v3 |
| --- | --- |

| Title: | CE-1.5, CE-1.6, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document describes CE-1.5, CE-1.6, CE-2.2 and CE-2.3 from the JVET U meeting which are modifications to the method for the selection of Rice parameters for residual coding. These modifications are intended for consideration for high bit depth coding in VVC version 2. The modifications are based on those previously described in JVET-U0057[1] and JVET-T0072[2].  CE-1.5 and CE-1.6 are modifications of regular residual coding (RRC) which differ only in the regularity of parameter updates. CE-2.2 and CE-2.3 are modifications of transform skip residual coding (TSRC). The aim of the modifications is to offer BD-rate gains for high bit depths when typically residual coefficient values become larger, whilst being backwardly compatible with the existing Rice coding techniques in VVC version 1(JVET-T2001)[3].

Results are reported with respect to an anchor of VTM12.0 augmented with the rice coding modifications of CE-1.1 and CE-2.1 using the high bit depth CTC (JVET-T2018)[3].

# Introduction

In VVC version 1[3], the range of Rice parameters that may be used for coefficients during regular residual coding is limited to the range of 0 to 3 (inclusive). This range has been found to be suitable for both 8 and 10 bit data; however, as bit depths increase and QPs become lower, the size of each coded coefficient becomes larger. For high bit depth content at QPs below 0 it has previously been shown that VVC coding efficiency is below that of HEVC[5].

Previous work[6] has described approaches that increase the coding efficiency for high bit depths and low QPs to match or exceed HEVC.  This proposal is based on the implementation of one of these approaches, “Adding a Rice-based shift to existing VVC”[2] and updated in the CE of the U meeting[1].

For the purposes of this core experiment, the implementation has been split into separate components for RRC (CE-1.5 and CE-1.6) and TSRC (CE-2.2 and CE-2.3) to allow for comparison with other techniques proposed within the CE.

The modification described here results in gains for 12 and 16-bit content. The modification provides backwards compatibility with VVC version 1 as the existing Rice coding method can be considered as a subset of the modification and is controlled by a parameter sent in the SPS.

# Algorithm description

The algorithm impacts entropy coding and in particular the selection of Rice parameters for the Golomb-Rice coding of residual coefficients. The algorithm for each experiment (CE-1.5, CE-1.6, CE-2.2 and CE-2.3) can be considered as having a series of components as follows:

- A coefficient size estimation function which is backed by a series of counters and an index function to select which counter is used for each coefficient.

- An update function for the selected counter.

- A modification to the current algorithm for computing the clipped sum of adjacent nearby coefficients (CE-1.5 and CE-1.6 only)

- A modification to the current algorithm for selecting Rice parameters.

## Coefficient size estimation

The technique requires an estimation of the log2 size of coefficients being coded. This is derived in a similar manner to Rice parameter estimation in HEVC (RExt) using a series of counters. For this proposal the counters are mapped to a one (CE-2.2 and CE-2.3) or two (CE-1.5 and CE-1.6) dimensional array, with the first (or only) dimension being the component type. For CE-1.5 and CE-1.6, the second dimension of the array is an index function coded as shown in Table 1 giving a total of 4 counters per component type. The counter selection for CE-1.5 and CE-1.6 can either be implemented as a piece of logic (as described in the draft text changes) or by using a lookup table (as implemented in the VTM modification). When coding losslessly, VVC uses RRC rather than TSRC and therefore the second array dimension for CE-1.5 and CE-1.6 requires an element for transform-skip.

| Index | Description | 16x16 transform block |
| --- | --- | --- |
| 0 | Transform-skip |  |
| 1 (Red) | DC coefficient in top left sub-TB |  |
| 2 (Blue) | Coefficients within distance of 3 from DC coefficient (inclusive) |  |
| 3 (Yellow) | All other coefficients |  |

Table 1. Index function derivation

## Counter update function

Each counter is set to a default value computed from the slice QP at the beginning of every slice.  The counters are handled in the same way as CABAC contexts, thus allowing the values to be rewound during the encoder search. This also provides support for wave-front parallel coding as each wave-front copies the counters before starting.

A counter is updated each time a coefficient on the leading diagonal of the coefficient block is coded using Golomb-Rice as follows:

if (abs(value - cutoff) > 0) {   						// cutoff is the amount of the coefficient coded with flags

int idx = lookup(isTS, scanPos);    				// index function as defined in Table 1

int &count = riceCount[componentType][idx];	// riceCount is the array of counters, reset  for each slice

if (abs(value) >= upperThreshold) {  			// value larger than expected range, initially 3

count += increment;    					// increment = 4

} else if ((abs(value) << 1) < lowerThreshold) {	// value smaller than expected range, initially 1

count = max(0, count - decrement);			// decrement = 1, clipped to be non-negative

}

}

The value of rice provides an estimate of the log2 size of the average non-zero coefficient.  Mismatched increments and decrements have been found to be beneficial as slightly over-estimating the Rice parameter seems to offer the best BD-rate gains.

The thresholds used in the update process are updated between coding subblocks (CE1.5 and CE2.2) or blocks (CE1.6 and CE2.3) as below:

int rice = count >> log2Divisor;				// log2Divisor = 4

upperThreshold = 3 << rice;

lowerThreshold = 1<< rice;

## Modification to clipped sum of coefficients (CE-1.5, CE-1.6)

For regular residual coding (RRC), VVC (section 9.3.3.2) defines locSumAbs as the sum of the absolute values of 5 previously coded coefficients below and right of the coefficient, less an offset which relates to the proportion of the coefficient that is coded with flags, and then clips this value to the range 0 to 31. This technique works well if clipping is only occasionally used, but as average coefficient values increase the number of times clipping is required increases.

In the modification described here, a shift right is introduced as below (modification in green).  The purpose of this shift is to reduce the magnitude of the sum and thus reduce the amount of clipping. The size of the shift is described in the following section.

locSumAbs = Clip3( 0, 31, Max( locSumAbs − baseLevel * 5, 0  ) >> shift )

## Rice parameter selection

In VVC Version 1, RRC selects the Rice parameter by using the value returned by the template function described in section 2.3 as an index into a lookup table which gives the Rice parameter.

In the proposed modification one of two shift values can be used, one when a coefficient is coded using a parity flag and one when a coefficient is coded without a parity flag. Both of these values are based on the value of count derived as described in section 2.2.

shiftWithoutParity = std::min(12, std::max(0, ((count + rounding) >> log2Divisor) – 2)); // rounding = 8

shiftWithParity = std::max(0, shiftWithoutParity – 1);

The shift is clipped to a maximum of 12 to support an extended table of Rice coding costs specified for dependent quantisation.

The computation of the two shift values is done at the same time as the values of upperThreshold and lowerThreshold described in section 2.2, i.e. between coding sub blocks (CE-1.5 and CE-2.2) or between coding blocks (CE-1.6 and CE-2.3).

For regular residual coding (CE-1.5 and CE-1.6), the selection of the Rice parameter can then be described using the pseudo code below:

int idx = cctx.templateAbsSum(scanPos, absCoeff, baseLevel, shift);

// compute template fn using sum shifted right by shift modifier

int rice = g_auiGoRiceParsCoeff[idx] + shift;		// add shift modifier onto result from lookup table

For transform skip residual coding (CE-2.2 and CE-2.3) Rice parameter selection can be described as follows:

int rice = 1 + shift;		// shift is either shiftWithParity or shiftWithoutParity depending on usage

// of the parity flag during coefficient coding

Thus for both regular and transform skip residual coding, the selection of the Rice parameter may be seen as a modified version of the existing VVC techniques rather than a new technique.  This modifications ensure that when estimated coefficient values are larger a more appropriate range of Rice parameters is selectable. Unlike simply swapping to a technique based purely on size estimation, this modification introduces a hybrid technique which attempts to extend the operating point of the existing VVC technique.

## SPS control flag

A flag has been added to the SPS to allow the effect of the modification to be disabled. When this occurs operation returns to that specified in VVC version 1.

# Results

All results reported here are compared with anchors generated using the CE anchor (VTM12.0 with rice coding modifications from CE-1.1 and CE-2.1) and the high bit depth, high bit rate and high frame rate CTC[4].

## CE-1.5

### Low QP range

#### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.23% | -0.12% | -0.13% | -0.25% | -0.13% | -0.13% | 101% | 101% |
| PQ422 | -0.22% | -0.12% | -0.13% | -0.25% | -0.13% | -0.14% | 102% | 100% |
| Overall | -0.22% | -0.12% | -0.13% | -0.25% | -0.13% | -0.13% | 101% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.03% | -0.04% | -0.06% | -0.04% | -0.04% | -0.06% | 101% | 101% |
| PQ422 | -0.01% | -0.01% | -0.06% | -0.01% | -0.02% | -0.06% | 100% | 101% |
| Overall | -0.02% | -0.03% | -0.06% | -0.03% | -0.03% | -0.06% | 101% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.03% | -0.04% | -0.05% | -0.02% | -0.04% | 101% | 100% |
| PQ422 | -0.03% | -0.03% | -0.05% | -0.03% | -0.04% | -0.04% | 100% | 100% |
| Overall | -0.04% | -0.03% | -0.04% | -0.04% | -0.03% | -0.04% | 101% | 100% |

#### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.42% | -0.22% | -0.24% | 103% | 102% |
| HLG422 | -0.41% | -0.23% | -0.23% | 103% | 103% |
| Overall | -0.42% | -0.23% | -0.23% | 103% | 103% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | -0.02% | -0.03% | 100% | 100% |
| HLG422 | -0.02% | -0.01% | 0.01% | 100% | 101% |
| Overall | -0.03% | -0.01% | -0.01% | 100% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.04% | -0.04% | 101% | 102% |
| HLG422 | -0.03% | -0.02% | -0.02% | 101% | 101% |
| Overall | -0.04% | -0.03% | -0.03% | 101% | 101% |

#### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.80% | -2.02% | -2.00% | 109% | 128% |
| SVT12 | -1.60% | -1.24% | -1.24% | 104% | 109% |
| Overall | -2.20% | -1.63% | -1.62% | 107% | 118% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.30% | -1.35% | -1.34% | 117% | 119% |
| SVT12 | -0.69% | -0.58% | -0.58% | 101% | 107% |
| Overall | -1.00% | -0.96% | -0.96% | 109% | 113% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.24% | -1.24% | -1.23% | 114% | 117% |
| SVT12 | -0.67% | -0.51% | -0.52% | 100% | 106% |
| Overall | -0.95% | -0.88% | -0.87% | 107% | 112% |

### Standard QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.25% | -0.01% | -0.03% | 0.12% | 0.20% | -0.03% | 0.04% | 0.17% | 101% | 101% |
| Class H2 |  |  |  |  |  | 0.02% | 0.10% | 0.14% | 102% | 102% |
| Overall | 0.25% | -0.01% | -0.03% | 0.12% | 0.20% | -0.01% | 0.07% | 0.16% | 101% | 101% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.01% | 0.00% | 0.00% | -0.05% | -0.06% | 0.00% | -0.05% | -0.03% | 98% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.10% | 98% | 99% |
| Overall | -0.01% | 0.00% | 0.00% | -0.05% | -0.06% | 0.00% | -0.03% | 0.03% | 98% | 99% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.5 |  | VTM_U_CE | CE-1.5 |  | VTM_U_CE | CE-1.5 |  |
| PQ444 | 2.5 | 2.6 | -1.77% | 3.0 | 3.1 | -2.29% | 3.0 | 3.1 | -2.22% |
| PQ422 | 2.3 | 2.4 | -1.86% | 2.8 | 2.8 | -2.30% | 2.8 | 2.9 | -2.22% |
| Overall | 2.4 | 2.5 | -1.81% | 2.9 | 2.9 | -2.29% | 2.9 | 3.0 | -2.22% |
| Enc Time[%] | 102% |  |  | 105% |  |  | 107% |  |  |
| Dec Time[%] | 116% |  |  | 105% |  |  | 106% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.5 |  | VTM_U_CE | CE-1.5 |  | VTM_U_CE | CE-1.5 |  |
| HLG444 | 1.7 | 1.8 | -3.13% | 1.9 | 1.9 | -3.49% | 1.9 | 1.9 | -3.42% |
| HLG422 | 1.6 | 1.7 | -2.79% | 1.9 | 1.9 | -3.23% | 1.9 | 1.9 | -3.16% |
| Overall | 1.7 | 1.7 | -2.96% | 1.9 | 1.9 | -3.36% | 1.9 | 1.9 | -3.29% |
| Enc Time[%] | 96% |  |  | 133% |  |  | 139% |  |  |
| Dec Time[%] | 130% |  |  | 115% |  |  | 117% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.5 |  | VTM_U_CE | CE-1.5 |  | VTM_U_CE | CE-1.5 |  |
| SVT16 | 1.2 | 1.2 | -1.46% | 1.2 | 1.2 | -0.40% | 1.2 | 1.2 | -0.39% |
| SVT12 | 1.2 | 1.3 | -2.91% | 1.3 | 1.3 | -1.56% | 1.3 | 1.3 | -1.49% |
| Overall | 1.2 | 1.2 | -2.19% | 1.2 | 1.3 | -0.98% | 1.2 | 1.3 | -0.94% |
| Enc Time[%] | 111% |  |  | 180% |  |  | 173% |  |  |
| Dec Time[%] | 144% |  |  | 121% |  |  | 118% |  |  |

## CE-1.6

### Low QP range

#### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.22% | -0.12% | -0.12% | -0.25% | -0.12% | -0.13% | 102% | 101% |
| PQ422 | -0.21% | -0.11% | -0.12% | -0.24% | -0.12% | -0.12% | 102% | 100% |
| Overall | -0.22% | -0.11% | -0.12% | -0.25% | -0.12% | -0.12% | 102% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.05% | -0.04% | -0.05% | -0.05% | -0.04% | -0.06% | 101% | 101% |
| PQ422 | -0.01% | -0.03% | -0.06% | -0.01% | -0.03% | -0.06% | 101% | 101% |
| Overall | -0.03% | -0.04% | -0.06% | -0.03% | -0.04% | -0.06% | 101% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.02% | -0.04% | -0.04% | -0.02% | -0.04% | 100% | 99% |
| PQ422 | -0.03% | -0.03% | -0.04% | -0.03% | -0.03% | -0.04% | 100% | 99% |
| Overall | -0.03% | -0.03% | -0.04% | -0.03% | -0.03% | -0.04% | 100% | 99% |

#### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.42% | -0.21% | -0.22% | 103% | 101% |
| HLG422 | -0.40% | -0.22% | -0.23% | 102% | 102% |
| Overall | -0.41% | -0.22% | -0.22% | 103% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | -0.02% | -0.02% | 101% | 101% |
| HLG422 | -0.03% | -0.02% | 0.01% | 101% | 101% |
| Overall | -0.04% | -0.02% | -0.01% | 101% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.02% | -0.03% | 101% | 101% |
| HLG422 | -0.04% | -0.02% | -0.02% | 101% | 101% |
| Overall | -0.04% | -0.02% | -0.02% | 101% | 101% |

#### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.78% | -2.00% | -1.98% | 111% | 128% |
| SVT12 | -1.58% | -1.21% | -1.22% | 105% | 109% |
| Overall | -2.18% | -1.61% | -1.60% | 108% | 119% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.31% | -1.34% | -1.32% | 117% | 117% |
| SVT12 | -0.68% | -0.57% | -0.56% | 101% | 105% |
| Overall | -0.99% | -0.95% | -0.94% | 109% | 111% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.24% | -1.22% | -1.21% | 114% | 116% |
| SVT12 | -0.66% | -0.50% | -0.50% | 101% | 106% |
| Overall | -0.95% | -0.86% | -0.86% | 107% | 111% |

### Standard QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.25% | -0.01% | -0.02% | 0.12% | 0.21% | -0.03% | 0.04% | 0.17% | 101% | 101% |
| Class H2 |  |  |  |  |  | 0.02% | 0.10% | 0.14% | 102% | 102% |
| Overall | 0.25% | -0.01% | -0.02% | 0.12% | 0.21% | -0.01% | 0.07% | 0.16% | 101% | 101% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.01% | 0.00% | 0.00% | -0.05% | -0.06% | 0.00% | -0.05% | -0.03% | 98% | 100% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.10% | 98% | 99% |
| Overall | -0.01% | 0.00% | 0.00% | -0.05% | -0.06% | 0.00% | -0.03% | 0.03% | 98% | 99% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.6 |  | VTM_U_CE | CE-1.6 |  | VTM_U_CE | CE-1.6 |  |
| PQ444 | 2.5 | 2.6 | -1.77% | 3.0 | 3.1 | -2.30% | 3.0 | 3.1 | -2.23% |
| PQ422 | 2.3 | 2.4 | -1.86% | 2.8 | 2.8 | -2.30% | 2.8 | 2.9 | -2.22% |
| Overall | 2.4 | 2.5 | -1.82% | 2.9 | 2.9 | -2.30% | 2.9 | 3.0 | -2.23% |
| Enc Time[%] | 102% |  |  | 106% |  |  | 107% |  |  |
| Dec Time[%] | 116% |  |  | 107% |  |  | 105% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.6 |  | VTM_U_CE | CE-1.6 |  | VTM_U_CE | CE-1.6 |  |
| HLG444 | 1.7 | 1.8 | -3.14% | 1.9 | 1.9 | -3.51% | 1.9 | 1.9 | -3.44% |
| HLG422 | 1.6 | 1.7 | -2.80% | 1.9 | 1.9 | -3.25% | 1.9 | 1.9 | -3.18% |
| Overall | 1.7 | 1.7 | -2.97% | 1.9 | 1.9 | -3.38% | 1.9 | 1.9 | -3.31% |
| Enc Time[%] | 95% |  |  | 130% |  |  | 137% |  |  |
| Dec Time[%] | 129% |  |  | 115% |  |  | 118% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.6 |  | VTM_U_CE | CE-1.6 |  | VTM_U_CE | CE-1.6 |  |
| SVT16 | 1.2 | 1.2 | -1.47% | 1.2 | 1.2 | -0.43% | 1.2 | 1.2 | -0.42% |
| SVT12 | 1.2 | 1.3 | -2.92% | 1.3 | 1.3 | -1.61% | 1.3 | 1.3 | -1.53% |
| Overall | 1.2 | 1.2 | -2.19% | 1.2 | 1.3 | -1.02% | 1.2 | 1.3 | -0.97% |
| Enc Time[%] | 110% |  |  | 175% |  |  | 168% |  |  |
| Dec Time[%] | 143% |  |  | 122% |  |  | 118% |  |  |

## CE-2.2

### Low QP range

#### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.01% | -0.01% | -0.02% | -0.02% | -0.01% | 98% | 99% |
| PQ422 | -0.03% | -0.04% | -0.05% | -0.02% | -0.04% | -0.05% | 99% | 99% |
| Overall | -0.02% | -0.02% | -0.03% | -0.02% | -0.03% | -0.03% | 98% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | -0.03% | -0.04% | 0.00% | -0.03% | -0.04% | 99% | 98% |
| PQ422 | 0.03% | 0.00% | -0.02% | 0.03% | 0.00% | -0.01% | 96% | 96% |
| Overall | 0.01% | -0.02% | -0.03% | 0.01% | -0.02% | -0.03% | 97% | 97% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | -0.01% | -0.01% | 0.02% | -0.01% | -0.01% | 102% | 102% |
| PQ422 | 0.02% | 0.01% | -0.03% | 0.02% | 0.01% | -0.02% | 100% | 101% |
| Overall | 0.02% | 0.00% | -0.02% | 0.02% | 0.00% | -0.02% | 101% | 102% |

#### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.07% | -0.09% | -0.09% | 99% | 99% |
| HLG422 | -0.07% | -0.09% | -0.09% | 99% | 100% |
| Overall | -0.07% | -0.09% | -0.09% | 99% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.02% | -0.01% | 101% | 101% |
| HLG422 | 0.02% | -0.01% | 0.00% | 100% | 101% |
| Overall | 0.01% | 0.00% | -0.01% | 101% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.00% | 0.00% | 101% | 102% |
| HLG422 | 0.01% | 0.00% | -0.01% | 99% | 101% |
| Overall | 0.01% | 0.00% | -0.01% | 100% | 101% |

#### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.26% | -0.17% | -0.16% | 99% | 101% |
| SVT12 | -0.13% | -0.14% | -0.13% | 100% | 100% |
| Overall | -0.20% | -0.16% | -0.15% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.14% | -0.14% | -0.12% | 102% | 105% |
| SVT12 | -0.11% | -0.07% | -0.08% | 101% | 105% |
| Overall | -0.13% | -0.11% | -0.10% | 102% | 105% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.13% | -0.16% | -0.15% | 101% | 104% |
| SVT12 | -0.10% | -0.08% | -0.09% | 99% | 103% |
| Overall | -0.12% | -0.12% | -0.12% | 100% | 103% |

### Standard QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.03% | -0.02% | -0.02% | -0.02% | -0.04% | -0.02% | -0.03% | -0.03% | 101% | 101% |
| Class H2 |  |  |  |  |  | -0.01% | -0.03% | 0.07% | 102% | 102% |
| Overall | -0.03% | -0.02% | -0.02% | -0.02% | -0.04% | -0.02% | -0.03% | 0.02% | 101% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 98% |

### Lossless

Lossless results are not relevant for CE-2.2 as TSRC is not used for lossless coding in VVC.

### TGM Content

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.12% | -0.14% | -0.12% | 102% | 101% |
| RGB 4:4:4 - TGM 720p | -0.58% | -0.30% | -0.29% | 102% | 101% |
| Overall | -0.35% | -0.22% | -0.21% | 102% | 101% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.69% | -0.73% | -0.73% | 101% | 101% |
| RGB 4:4:4 - TGM 720p | -2.35% | -2.88% | -3.43% | 101% | 102% |
| Overall | -1.52% | -1.81% | -2.08% | 101% | 101% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.25% | -0.24% | -0.23% | 100% | 100% |
| RGB 4:4:4 - TGM 720p | -0.80% | -1.09% | -1.08% | 100% | 100% |
| Overall | -0.52% | -0.67% | -0.66% | 100% | 100% |

## CE-2.3

### Low QP range

#### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.02% | -0.01% | -0.01% | -0.02% | -0.01% | 98% | 98% |
| PQ422 | -0.02% | -0.03% | -0.04% | -0.02% | -0.04% | -0.05% | 98% | 98% |
| Overall | -0.02% | -0.03% | -0.03% | -0.02% | -0.03% | -0.03% | 98% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | -0.04% | -0.04% | 0.00% | -0.04% | -0.04% | 99% | 97% |
| PQ422 | 0.02% | -0.01% | -0.03% | 0.02% | -0.01% | -0.02% | 96% | 95% |
| Overall | 0.01% | -0.03% | -0.03% | 0.01% | -0.02% | -0.03% | 98% | 96% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | -0.01% | -0.01% | 0.02% | -0.01% | -0.01% | 102% | 102% |
| PQ422 | 0.01% | 0.01% | -0.03% | 0.02% | 0.01% | -0.02% | 100% | 101% |
| Overall | 0.02% | 0.00% | -0.02% | 0.02% | 0.00% | -0.02% | 101% | 102% |

#### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.06% | -0.09% | -0.09% | 98% | 98% |
| HLG422 | -0.06% | -0.09% | -0.08% | 99% | 98% |
| Overall | -0.06% | -0.09% | -0.09% | 98% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.00% | 101% | 100% |
| HLG422 | 0.02% | -0.01% | -0.01% | 98% | 98% |
| Overall | 0.01% | 0.00% | -0.01% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.01% | 0.00% | 100% | 100% |
| HLG422 | 0.02% | 0.00% | -0.01% | 100% | 100% |
| Overall | 0.02% | 0.00% | 0.00% | 100% | 100% |

#### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.24% | -0.15% | -0.14% | 100% | 100% |
| SVT12 | -0.12% | -0.12% | -0.12% | 101% | 100% |
| Overall | -0.18% | -0.14% | -0.13% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.10% | -0.08% | -0.07% | 101% | 103% |
| SVT12 | -0.09% | -0.05% | -0.06% | 101% | 104% |
| Overall | -0.10% | -0.06% | -0.07% | 101% | 103% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.10% | -0.10% | -0.10% | 100% | 103% |
| SVT12 | -0.08% | -0.05% | -0.07% | 99% | 102% |
| Overall | -0.09% | -0.08% | -0.08% | 100% | 103% |

### Standard QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.03% | -0.02% | -0.02% | -0.02% | -0.03% | -0.02% | -0.03% | -0.03% | 101% | 101% |
| Class H2 |  |  |  |  |  | -0.01% | -0.03% | 0.07% | 101% | 102% |
| Overall | -0.03% | -0.02% | -0.02% | -0.02% | -0.03% | -0.02% | -0.03% | 0.02% | 101% | 101% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 99% |

### Lossless

Lossless results are not relevant for CE-2.3 as TSRC is not used for lossless coding in VVC.

### TGM Content

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.08% | -0.10% | -0.09% | 103% | 100% |
| RGB 4:4:4 - TGM 720p | -0.60% | -0.25% | -0.45% | 102% | 102% |
| Overall | -0.34% | -0.17% | -0.27% | 102% | 101% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.73% | -0.74% | -0.74% | 100% | 101% |
| RGB 4:4:4 - TGM 720p | -1.64% | -2.11% | -2.18% | 100% | 101% |
| Overall | -1.18% | -1.42% | -1.46% | 100% | 101% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.27% | -0.23% | -0.22% | 100% | 100% |
| RGB 4:4:4 - TGM 720p | -0.04% | -2.08% | -0.39% | 100% | 101% |
| Overall | -0.15% | -1.16% | -0.31% | 100% | 100% |

# Estimates of Complexity

# CE-1.5

| Components |  | Number of additional operations per TU | Comments |
| --- | --- | --- | --- |
| Rice evaluation: | 1 shift, 1 add | 32 |  |
| Counter selection: riceIndex = transform_skip_flag[ x0 ][ y0 ][ cIdx] ? 0 : i == 0 ? log2SbW & 3 : 4 | 2 conditional | 2 | Selection of LUT for sub-TU |
| Counter update: Once per coefficient: if (xC == yC) { Four times per sub-TU: if( AbsLevel[xC][yC] - 4 >= upperThreshold[ RiceLUT[riceIndex ][n&15]] ) StatCoeff[ chType ][RiceLUT[riceIndex][n&15]]+=4 else if( 2 * ( AbsLevel[ xC ][ yC ] - 4) < lowerThreshold [RiceLUT[riceIndex][n&15]])) { if( StatCoeff[ chType ][RiceLUT[riceIndex][n&15]] > 0 ) StatCoeff[ chType ][RiceLUT[riceIndex][n&15]]− − } } | 1 condition 3 condition, 2 add | 16 20 |  |
| Parameter update per TU Three times per TU lowerThreshold[riceIndex] = 1 << (StatCoeff[chType][riceIndex] >> 4) upperThreshold[riceIndex] = 3 << (StatCoeff[chType][riceIndex] >> 4) if (sps_adaptive_rice_coding_enabled_flag) { RiceShiftNoParity[riceIndex] = Min(12, Max(0, (StatCoeff[chType][riceIndex] + 8) >> 4 – 2)) RiceShiftWithParity[riceIndex] = Max(0, RiceIndexNoParity[riceIndex] – 1) } | 1 shift 1 shift 1 conditional (only required once) 2 conditional, 2 add 1 add, 1 conditional | 3 3 1 12 6 |  |
| Total increase per 4x4 TU (critical path): |  | 32 |  |
| Total increase per 4x4 TU |  | 95 |  |
| Memory requirements |  | 8 * 10 bits of storage for counters 6 * 16 * 2 bits of storage (ROM) for LUT |  |
| History characteristics |  |  |  |
| Dependencies | Cross sub-block dependency Cross TU dependency Cross CTU dependency. |  |  |
| History update rate | 4 per sub-TU |  |  |
| Parameter update rate | 1 per sub-TU |  |  |

# CE-1.6

| Components |  | Number of additional operations per TU | Comments |
| --- | --- | --- | --- |
| Rice evaluation: | 1 shift, 1 add | 32 |  |
| Counter selection: riceIndex = transform_skip_flag[ x0 ][ y0 ][ cIdx] ? 0 : i == 0 ? log2SbW & 3 : 4 | 2 conditional | 2 | Selection of LUT for sub-TU |
| Counter update: Once per coefficient: if (xC == yC) { Four times per sub-TU: if( AbsLevel[xC][yC] - 4 >= upperThreshold[ RiceLUT[riceIndex ][n&15]] ) StatCoeff[ chType ][RiceLUT[riceIndex][n&15]]+=4 else if( 2 * ( AbsLevel[ xC ][ yC ] - 4) < lowerThreshold [RiceLUT[riceIndex][n&15]])) { if( StatCoeff[ chType ][RiceLUT[riceIndex][n&15]] > 0 ) StatCoeff[ chType ][RiceLUT[riceIndex][n&15]]− − } } | 1 condition 3 condition, 2 add | 16 20 |  |
| Parameter update per TU Three times per TU lowerThreshold[riceIndex] = 1 << (StatCoeff[chType][riceIndex] >> 4) upperThreshold[riceIndex] = 3 << (StatCoeff[chType][riceIndex] >> 4) if (sps_adaptive_rice_coding_enabled_flag) { RiceShiftNoParity[riceIndex] = Min(12, Max(0, (StatCoeff[chType][riceIndex] + 8) >> 4 – 2)) RiceShiftWithParity[riceIndex] = Max(0, RiceIndexNoParity[riceIndex] – 1) } | 1 shift 1 shift 1 conditional (only required once) 2 conditional, 2 add 1 add, 1 conditional | 3 3 1 12 6 |  |
| Total increase per 4x4 TU (critical path): |  | 32 |  |
| Total increase per 4x4 TU |  | 95 |  |
| Memory requirements |  | 8 * 10 bits of storage for counters 6 * 16 * 2 bits of storage (ROM) for LUT |  |
| History characteristics |  |  |  |
| Dependencies | No cross sub-block dependency Cross TU dependency Cross CTU dependency. |  |  |
| History update rate | 4 per sub-TU |  |  |
| Parameter update rate | 1 per sub-TU |  |  |

# CE-2.2

| Components |  | Number of additional operations per TU | Comments |
| --- | --- | --- | --- |
| Rice evaluation: | 1 add | 16 |  |
| Counter update: Once per coefficient: if (xC == yC) { Four times per sub-TU: if( AbsLevel[xC][yC] - offset >= upperThreshold ) StatCoeff[ chType ]+=4 else if( 2 * ( AbsLevel[ xC ][ yC ] - offset) < lowerThreshold )) { if( StatCoeff[ chType ] > 0 ) StatCoeff[ chType ] − − } } | 1 condition 3 condition, 2 add | 16 20 |  |
| Parameter update per TU Once per TU lowerThreshold = 1 << (StatCoeff[chType] >> 4) upperThreshold = 3 << (StatCoeff[chType] >> 4) if (sps_adaptive_rice_coding_enabled_flag) { RiceShiftNoParity = Min(12, Max(0, (StatCoeff[chType] + 8) >> 4 – 2)) RiceShiftWithParity = Max(0, RiceIndexNoParity – 1) } | 1 shift 1 shift 1 conditional 2 conditional, 2 add 1 add, 1 conditional | 1 1 1 4 2 |  |
| Total increase per 4x4 TU (critical path): |  | 16 |  |
| Total increase per 4x4 TU |  | 61 |  |
| Memory requirements |  | 2 * 10 bits of storage for counters |  |
| History characteristics |  |  |  |
| Dependencies | Cross sub-block dependency Cross TU dependency Cross CTU dependency. |  |  |
| History update rate | 4 per sub-TU |  |  |
| Parameter update rate | 1 per sub-TU |  |  |

# CE-2.3

| Components |  | Number of additional operations per TU | Comments |
| --- | --- | --- | --- |
| Rice evaluation: | 1 add | 16 |  |
| Counter update: Once per coefficient: if (xC == yC) { Four times per sub-TU: if( AbsLevel[xC][yC] - offset >= upperThreshold ) StatCoeff[ chType ]+=4 else if( 2 * ( AbsLevel[ xC ][ yC ] - offset) < lowerThreshold )) { if( StatCoeff[ chType ] > 0 ) StatCoeff[ chType ] − − } } | 1 condition 3 condition, 2 add | 16 20 |  |
| Parameter update per TU Once per TU lowerThreshold = 1 << (StatCoeff[chType] >> 4) upperThreshold = 3 << (StatCoeff[chType] >> 4) if (sps_adaptive_rice_coding_enabled_flag) { RiceShiftNoParity = Min(12, Max(0, (StatCoeff[chType] + 8) >> 4 – 2)) RiceShiftWithParity = Max(0, RiceIndexNoParity – 1) } | 1 shift 1 shift 1 conditional 2 conditional, 2 add 1 add, 1 conditional | 1 1 1 4 2 |  |
| Total increase per 4x4 TU (critical path): |  | 16 |  |
| Total increase per 4x4 TU |  | 61 |  |
| Memory requirements |  | 2 * 10 bits of storage for counters |  |
| History characteristics |  |  |  |
| Dependencies | No cross sub-block dependency Cross TU dependency Cross CTU dependency. |  |  |
| History update rate | 4 per sub-TU |  |  |
| Parameter update rate | 1 per sub-TU |  |  |

# Conclusions

This proposal has described a series of experiments (CE-1.5, CE-1.6, CE-2.2 and CE-2.3) based on modifications to the method for the selection of Rice parameters for residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains when residual coefficient values become larger but do not affect version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. Two experiments, CE-1.5 and CE-1.6, modify RRC, whilst the other two, CE-2.2 and CE-2.3 target TSRC, with the second experiment in each category being a simplification of the first by reducing the number of times the values used in the rice parameter evaluation are updated.

It is proposed that the modifications included in the described experiments are considered alongside other proposals in the CE for adoption into VVC version 2 and VTM.

# Patent rights declaration(s)

Sony Group Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “CE-1.4, CE-1.5, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, JVET-U0057, A. Browne, S. Keating and K. Sharman

[2] “AHG12: Rice parameter selection for high bit depths”, JVET-T0072, A. Browne, S. Keating and K. Sharman

[3] “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, B. Bross, J. Chen, S. Liu and Y-K. Wang

[4] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu

[5] “High bit depth coding”, JVET-R0351, A. Browne, S. Keating and K. Sharman

[6] “On operation beyond 10-bit”, JVET-S0243, A. Browne, S. Keating and K. Sharman