| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0052 |
| --- | --- |

| Title: | CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Marta Karczewicz Luong Pham Van Muhammed Coban | Email: | dmytror@qti.qualcomm.com martak@qti.qualcomm.com lphamvan@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution describes a modification to the Rice parameters selection method for regular residual coding (RRC) being tested in CE-1.1, CE-1.2 and CE-1.4 for purposes of High Bit Depth and High Bit Rate Coding. Simulation results are reported as per CE requirements in JVET-U2022. Summary of simulation results is provided in CE report, JVET-V0022.

# Introduction

In the current VVC [1], rice parameter for RRC is derived using look-up table with considering of coefficient value of the neighboring transform coefficients in a template. The template of the neighbouring coefficients is presented in Figure 1.

First, locSumAbs which is the sum of absolute of five available neighbouring coefficients in the template is calculated.

The locSumAbs is then normalized (subtraction and clip) as following:

locSumAbs = Clip3( 0, 31, locSumAbs - baseLevel * 5 )

The rice parameter is derived using this locSumAbs with look-up table (Table 1). With the current design, the rice parameter range is constrained from 0 to 3.

Figure 1 Neighbouring coefficients which are using in calculation of the localSumAbs of the current  coefficient

Table 1. Look-up table for Rice Parameter based on locSumAbs in current spec

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

When the bitdepth of the input video is increased, or the extended precision is enabled, or the quantization parameter is set very low, the range of the coefficients and therefore locSumAbs value may significantly increase. In such cases, the current VVC range of allowed rice parameter may not be sufficient. This limitation causes a coding performance drop of VVC in high bit-depth coding.

# CE-1.1

CE-1.1 test was proposed in JVET-U0065 and was selected as a part of codebase and anchor for Core Experiment study from the U meeting.

To extend the range of allowed Rice parameters, it was proposed that output of the template derivation locSumAbs is compared against a set of thresholds {Tx} and apply normalization right shift value {Rx} corresponding to a met thresold. Such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with a shift value if scaling process was taken a place for a given locSumAbs value.

Set of thresholds Tx and corresponding set of shfits Rx are defined such that following each value is larger value of the previous threshold and corresponding shift_rice value respectively.

Tx = {T0, T0*22 * R0, T0*24*R0,…}

Rx = {0, R0, 2*R0,…}

## CE Anchor method:

The following configuration of CE1.1 has been utilized in CE Anchor.

Rice parameter derivation for decoding syntax ellement dec_abs_level is conducted with T0 value was set equal to 32, for syntax ellement abs_remainder, T0 was set equal to 52. Tx arrays defined for both syntax ellemnets comprise a 2D array RiceT[2][]. Variable R0 is selected by encoder and signalled as a syntax element sh_shift_rice in the slice header and being derived at the encoder side as a function of input data bit depth.

The specification text for original CE1.1 is accordingly modified as follows:

7.3.7	Slice header syntax

| … |  |
| --- | --- |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_high_bit_depth ) |  |
| sh_shift_rice | ue(v) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |

sh_shift_rice specifies the shift value utilized for derivation of the Rice parameter, the value of sh_shift_rice shall be in the range of 0 to 4, inclusive. When sh_shift_rice is not present, it is inferred to be equal to 0.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following for idx set equal to 0, if baseLevel is equal to 0, and set equal to 1, otherwise: shift = (localSumAbs < RiceT[0][idx]) ? 0 : (localSumAbs < RiceT[1][idx]) ? sh_shift_rice : (sh_shift_rice << 1) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

## CE1.1 improvement:

CE1.1 method studying after U meeting was further improved. Following changes have been introduced comparing to the Anchor RRC method:

- Increase number of thresholds in Tx and shift parameters Rx from 2 to 4 and from 3 to 5, respectively.

- Unified Tx table for abs_remainder[ ] and dec_abs_level[ ], T0 is set equal to 32.

- Removal of sh_shift_rice signaling, value R0 is set equal to 2.

- baseLevel dependent offset derivation is modified.

Tx[4] = { 32,128, 512, 2048 }, Rx[5] = { 0, 2, 4, 6, 8}

baseLevelOffset = (bitDepth > 12) ? (isIntra() ? 5: 10) : (isIntra() ? 6: 20)
Extract for proposed VVCv2 specification change is shown below.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following for idx set equal to 0, if baseLevel is equal to 0, and set equal to 1, otherwise: shift = (localSumAbs < Tx[0]) ? Rx[0] : ( (localSumAbs < Tx[1]) ? Rx[1]: ((localSumAbs < Tx[2]) ? Rx[2]: ((localSumAbs < Tx[3]) ? Rx[3]: Rx[4]))). locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevelOffset) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

# CE-1.1 Improvement: Simulation results for HBD CTC

Simulation results have been conducted according to JVET-U2018. Comparative performance for improved CE1.1 test is shown below against CE Anchor.

Simulation results have been cross-checked by Alibaba, related contribution is TBD.

Table 1. Comparative performance of CE1.1 test vs. CE Anchor, CE CTC, Low QP setting.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | 0.00% | 0.01% | -0.05% | 0.00% | 0.01% | 101% | 99% |
| PQ422 | -0.04% | 0.01% | -0.01% | -0.05% | 0.01% | -0.01% | 101% | 100% |
| Overall | -0.04% | 0.00% | 0.00% | -0.05% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over Anchor CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.04% | -0.01% | -0.02% | -0.02% | -0.02% | 100% | 101% |
| PQ422 | 0.00% | -0.02% | -0.03% | 0.00% | -0.02% | -0.02% | 100% | 98% |
| Overall | -0.01% | -0.03% | -0.02% | -0.01% | -0.02% | -0.02% | 100% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over Anchor CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.04% | -0.02% | -0.01% | -0.03% | -0.02% | 102% | 98% |
| PQ422 | -0.01% | -0.02% | -0.04% | -0.01% | -0.02% | -0.03% | 101% | 98% |
| Overall | -0.01% | -0.03% | -0.03% | -0.01% | -0.03% | -0.02% | 101% | 98% |
| Overall PQ | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | 101% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | 0.07% | 0.06% | 100% | 99% |
| HLG422 | -0.03% | 0.07% | 0.07% | 101% | 101% |
| Overall | -0.04% | 0.07% | 0.07% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over Anchor CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | 0.00% | -0.01% | 101% | 99% |
| HLG422 | -0.02% | -0.02% | 0.02% | 101% | 97% |
| Overall | -0.03% | -0.01% | 0.00% | 101% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over Anchor CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | -0.01% | 101% | 102% |
| HLG422 | -0.01% | 0.00% | 0.00% | 101% | 99% |
| Overall | -0.01% | 0.00% | 0.00% | 101% | 100% |
| Overall HLG | -0.03% | 0.02% | 0.02% | 101% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.67% | -0.72% | -0.73% | 104% | 102% |
| SVT12 | -0.35% | -0.19% | -0.19% | 103% | 103% |
| Overall | -0.51% | -0.46% | -0.46% | 103% | 103% |
|  |  |  | LDB |  |  |
|  |  |  | Over Anchor CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.53% | -0.61% | -0.62% | 104% | 103% |
| SVT12 | -0.05% | -0.05% | -0.05% | 103% | 105% |
| Overall | -0.29% | -0.33% | -0.33% | 103% | 104% |
|  |  |  | RA |  |  |
|  |  |  | Over Anchor CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.50% | -0.50% | -0.52% | 106% | 102% |
| SVT12 | -0.06% | -0.06% | -0.05% | 103% | 103% |
| Overall | -0.28% | -0.28% | -0.29% | 105% | 102% |
| Overall RGB | -0.36% | -0.36% | -0.36% | 104% | 103% |

Table 2. Comparative performance of CE1.1 test vs. CE Anchor, CE CTC, Normal QP setting.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-1.1 Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.03% | 0.02% | 0.00% | -0.29% | -0.05% | 0.00% | -0.27% | -0.04% | 98% | 98% |
| Class H2 |  |  |  |  |  | 0.00% | -0.11% | -0.21% | 98% | 98% |
| Overall | -0.03% | 0.02% | 0.00% | -0.29% | -0.05% | 0.00% | -0.27% | -0.04% | 98% | 98% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | CE-1.1 Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.07% | -0.01% | -0.02% | 0.07% | -0.07% | -0.03% | 0.06% | -0.05% | 100% | 98% |
| Class H2 |  |  |  |  |  | -0.01% | -0.04% | 0.03% | 99% | 97% |

Table 3. Comparative performance of CE1.1 test vs. CE Anchor, CE CTC, lossless setting.

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.1 |  | AnchorCE | CE-1.1 |  | AnchorCE | CE-1.1 |  |
| PQ444 | 2.5 | 2.5 | -0.97% | 3.0 | 3.0 | -1.55% | 3.0 | 3.0 | -1.62% |
| PQ422 | 2.3 | 2.3 | -0.98% | 2.8 | 2.8 | -1.55% | 2.8 | 2.8 | -1.64% |
| Overall | 2.4 | 2.4 | -0.98% | 2.9 | 2.9 | -1.55% | 2.9 | 2.9 | -1.63% |
| Enc Time[%] | 96% |  |  | 84% |  |  | 83% |  |  |
| Dec Time[%] | 102% |  |  | 93% |  |  | 95% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.1 |  | AnchorCE | CE-1.1 |  | AnchorCE | CE-1.1 |  |
| HLG444 | 1.7 | 1.7 | -1.65% | 1.9 | 1.9 | -2.67% | 1.9 | 1.9 | -2.74% |
| HLG422 | 1.6 | 1.7 | -1.35% | 1.9 | 1.9 | -2.35% | 1.9 | 1.9 | -2.42% |
| Overall | 1.7 | 1.7 | -1.50% | 1.9 | 1.9 | -2.51% | 1.9 | 1.9 | -2.58% |
| Enc Time[%] | 109% |  |  | 68% |  |  | 69% |  |  |
| Dec Time[%] | 104% |  |  | 92% |  |  | 89% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.1 |  | AnchorCE | CE-1.1 |  | AnchorCE | CE-1.1 |  |
| SVT16 | 1.2 | 1.2 | 0.10% | 1.2 | 1.2 | 0.17% | 1.2 | 1.2 | 0.18% |
| SVT12 | 1.2 | 1.3 | -1.38% | 1.3 | 1.3 | -0.77% | 1.3 | 1.3 | -0.71% |
| Overall | 1.2 | 1.2 | -0.64% | 1.2 | 1.2 | -0.30% | 1.2 | 1.2 | -0.27% |
| Enc Time[%] | 94% |  |  | 91% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 101% |  |  | 109% |  |  |

# Test CE-1.2

Initial design tested in CE-1.2 was presented in the JVET-U0065. Similarly, to the Test CE-1.1, to extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value.

The difference with CE-1.1 is limmited the way, shift parameter for locSumAbs scaling and cRiceParam increment is derived. In CE-1.2, it is tested as follows, with syntax element sh_scale_rice taking values 1 or 2.

scalledCoef = (localSumAbs * sh_scale_rice) >> 3

shift = (scalledCoef == 0) ? 0 : Floor( Log2( scalledCoef) )

Utilized value sh_scale_rice is signalled as syntax element in the slice header and being derived at the encoder side as a function of QP values.

The specification text is accordingly modified as follows:

sps_high_bit_depth equal to 1 specifies that sh_rice_parameter syntax element could be present in slice headers to enable a high bitdepth mode for Rice parameter for the binarization of abs_remaining[ ] and dec_abs_level. sps_high_bit_depth equal to 0 specifies that sh_rice_parameter syntax element is not present in slice headers and high bitdepth mode for Rice parameter derivation cannot be enabled. When not present, the value of sps_high_bit_depth is inferred to be equal to 0.

7.3.7	Slice header syntax

| … |  |
| --- | --- |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_high_bit_depth ) |  |
| sh_scale_rice | ue(v) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |

sh_scale_rice specifies the scale value utilized for derivation of the Rice parameter, the value of sh_scale_rice shall be in the range of 1 to 2, inclusive. When sh_scale_rice is not present, it is inferred to be equal to 0.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following: scalledCoef = (localSumAbs * sh_scale_rice) >> 3 shift = (scalledCoef == 0) ? 0 : Floor( Log2( scalledCoef) ) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

# CE-1.1 Improvement: Simulation results for HBD CTC

Simulation results have been conducted according to JVET-U2018. Comparative performance for improved CE1.1 test is shown below against CE Anchor.

Simulation results have been cross-checked by Sharp, related contribution is TBD.

Table 4. Comparative performance of CE1.2 test vs. CE Anchor, CE CTC, Low QP setting.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.10% | -0.06% | -0.07% | -0.11% | -0.07% | -0.07% | 102% | 100% |
| PQ422 | -0.08% | -0.06% | -0.07% | -0.08% | -0.06% | -0.06% | 102% | 103% |
| Overall | -0.09% | -0.06% | -0.07% | -0.10% | -0.07% | -0.07% | 102% | 102% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.01% | 0.04% | 0.02% | 0.02% | 0.03% | 101% | 103% |
| PQ422 | 0.03% | 0.04% | 0.00% | 0.03% | 0.03% | 0.01% | 100% | 97% |
| Overall | 0.03% | 0.03% | 0.02% | 0.02% | 0.02% | 0.02% | 100% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | -0.02% | -0.02% | -0.01% | -0.02% | -0.01% | 104% | 102% |
| PQ422 | -0.01% | 0.01% | -0.02% | -0.01% | 0.00% | -0.01% | 103% | 104% |
| Overall | -0.01% | -0.01% | -0.02% | -0.01% | -0.01% | -0.01% | 104% | 103% |
| Overall PQ | -0.02% | -0.02% | -0.02% | -0.03% | -0.02% | -0.02% | 102% | 102% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.09% | -0.02% | -0.03% | 100% | 102% |
| HLG422 | -0.11% | -0.05% | -0.05% | 103% | 104% |
| Overall | -0.10% | -0.03% | -0.04% | 102% | 103% |
|  |  |  | LDB |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.04% | 0.06% | 103% | 101% |
| HLG422 | 0.01% | 0.06% | 0.05% | 103% | 101% |
| Overall | 0.02% | 0.05% | 0.06% | 103% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.02% | 0.03% | 103% | 104% |
| HLG422 | 0.02% | 0.03% | 0.03% | 102% | 99% |
| Overall | 0.01% | 0.03% | 0.03% | 102% | 101% |
| Overall HLG | -0.02% | 0.02% | 0.02% | 102% | 102% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.79% | -0.79% | -0.79% | 101% | 99% |
| SVT12 | -0.45% | -0.39% | -0.39% | 102% | 103% |
| Overall | -0.62% | -0.59% | -0.59% | 102% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.61% | -0.69% | -0.70% | 100% | 99% |
| SVT12 | -0.20% | -0.19% | -0.19% | 101% | 101% |
| Overall | -0.41% | -0.44% | -0.44% | 100% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.61% | -0.58% | -0.60% | 102% | 99% |
| SVT12 | -0.19% | -0.15% | -0.16% | 102% | 103% |
| Overall | -0.40% | -0.37% | -0.38% | 102% | 101% |
| Overall RGB | -0.48% | -0.46% | -0.47% | 101% | 101% |

Table 5. Comparative performance of CE1.2 test vs. CE Anchor, CE CTC, Normal QP setting.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.09% | 0.03% | 0.02% | -0.14% | -0.22% | 0.02% | -0.25% | -0.26% | 100% | 101% |
| Class H2 |  |  |  |  |  | -0.01% | 0.11% | -0.30% | 100% | 100% |
| Overall | 0.09% | 0.03% | 0.02% | -0.14% | -0.22% | 0.02% | -0.25% | -0.26% | 100% | 101% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.01% | -0.03% | -0.03% | 0.09% | -0.08% | -0.03% | 0.04% | -0.13% | 100% | 99% |
| Class H2 |  |  |  |  |  | -0.01% | -0.10% | 0.05% | 100% | 101% |
| Overall | 0.01% | -0.03% | -0.03% | 0.09% | -0.08% | -0.03% | 0.04% | -0.13% | 100% | 100% |

Table 6. Comparative performance of CE1.2 test vs. CE Anchor, CE CTC, lossless setting.

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.2 |  | AnchorCE | CE-1.2 |  | AnchorCE | CE-1.2 |  |
| PQ444 | 2.5 | 2.5 | -0.57% | 3.0 | 3.0 | -1.05% | 3.0 | 3.0 | -1.01% |
| PQ422 | 2.3 | 2.3 | -0.57% | 2.8 | 2.8 | -1.01% | 2.8 | 2.8 | -0.97% |
| Overall | 2.4 | 2.4 | -0.57% | 2.9 | 2.9 | -1.03% | 2.9 | 2.9 | -0.99% |
| Enc Time[%] | 95% |  |  | 112% |  |  | 111% |  |  |
| Dec Time[%] | 119% |  |  | 111% |  |  | 97% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.2 |  | AnchorCE | CE-1.2 |  | AnchorCE | CE-1.2 |  |
| HLG444 | 1.7 | 1.7 | -1.34% | 1.9 | 1.9 | -1.91% | 1.9 | 1.9 | -1.87% |
| HLG422 | 1.6 | 1.7 | -0.98% | 1.9 | 1.9 | -1.65% | 1.9 | 1.9 | -1.61% |
| Overall | 1.7 | 1.7 | -1.16% | 1.9 | 1.9 | -1.78% | 1.9 | 1.9 | -1.74% |
| Enc Time[%] | 101% |  |  | 107% |  |  | 123% |  |  |
| Dec Time[%] | 127% |  |  | 122% |  |  | 105% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.2 |  | AnchorCE | CE-1.2 |  | AnchorCE | CE-1.2 |  |
| SVT16 | 1.2 | 1.2 | 0.42% | 1.2 | 1.2 | 0.59% | 1.2 | 1.2 | 0.60% |
| SVT12 | 1.2 | 1.3 | -0.82% | 1.3 | 1.3 | -0.52% | 1.3 | 1.3 | -0.44% |
| Overall | 1.2 | 1.2 | -0.20% | 1.2 | 1.2 | 0.04% | 1.2 | 1.2 | 0.08% |
| Enc Time[%] | 91% |  |  | 112% |  |  | 115% |  |  |
| Dec Time[%] | 94% |  |  | 101% |  |  | 107% |  |  |

# Test CE-1.4

Initial design tested in CE-1.4 was presented in the JVET-U0066. In this test, the RRC derivation method of CE1.2 is being extended with a history of rice parameters utilization.

History maintenance:

A single counter per color component is utilized, StatCoeff[3] is being updated once per TU from the first, non-zero, Golomb-Rice coded transform coefficient (abs_remainder[ ] or dec_abs_level[ ]) through a process of exponential moving average.

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(abs_remainder[ ])) + 2 ) >> 1

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(dec_abs_level[ ])) ) >> 1

History counter StatCoeff[3] can be reset at the CTU level with a default value, or history can be propagated between different CTUs.

History usage:

History Rice parameter value is utilized in the template derivation, clause 9.3.3.2 Implementation of this method is shown VVC specification text extract for clause 9.3.3.2, with text highlighted with green is related to changes from CE-1.2 and yellow highlighted text associated with classification utilized in CE-1.4. Variable g_historyValue is derived prior to coding of current TU from history counter associated with current color component id.

histValue = 1 << StatCoeff[cIdx]

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] else locSumAbs += histValue if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) else locSumAbs += histValue } else locSumAbs += 2 * histValue if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] else locSumAbs += histValue } else locSumAbs += histValue Variable shift is derived as following: scalledCoef = (localSumAbs * ScaleRice) >> 3 shift = (scalledCoef == 0) ? 0 : Floor( Log2( scalledCoef) ) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

CE1.4-A: configuration with signaling and CTU-level reset.

History counter StatCoeff[cIdx] is being reset at start of each CTU with a default value sh_rice_init which is determined at the encoder side and signalled at the slice header.

The following encoder side algorithms is utilized at the encoder side:

- Collect StatCoeff[cIdx] statistics for each CTU at the encoder side.

- Derive default value for StatCoeff[cIdx] as average counter value of previously coded frame and signal it to the decoder at the slice header.

| if( sps_high_bit_depth ) { |  |
| --- | --- |
| sh_rice_init | ue(v) |
| } |  |

- Derive variable ScaleRice parameter of CE1.2 from default value sh_rice_init for StatCoeff[cIdx] at the decoder side.

CE1.4-B: configuration w/o signaling and cross-CTU history propagation.

Status of the history counters StatCoeff[cIdx] is being preserved between CTUs, and ScaleRice parameter of CE1.2 is being derived at the CTU level from the current status of the StatCoeff[] counter. Signaling of the sh_rice_init variable is removed.

| if( sps_high_bit_depth ) { |  |
| --- | --- |
| sh_rice_init | ue(v) |
| } |  |

riceTemp = ((StatCoeff[0]<<1) + StatCoeff[1] + StatCoeff[2])>>2;

ScaleRice = (riceTemp >= 4) ? 2 : 1;

# CE-1.4-A Simulation results for HBD CTC

Table 7. Comparative performance of CE1.4-A test vs. CE Anchor, CE CTC, Low QP setting.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.32% | -0.22% | -0.21% | -0.34% | -0.22% | -0.21% | 102% | 101% |
| PQ422 | -0.34% | -0.25% | -0.26% | -0.36% | -0.25% | -0.27% | 103% | 101% |
| Overall | -0.33% | -0.24% | -0.24% | -0.35% | -0.24% | -0.24% | 102% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.06% | -0.07% | -0.07% | -0.06% | -0.07% | 103% | 106% |
| PQ422 | -0.05% | -0.09% | -0.10% | -0.05% | -0.08% | -0.09% | 103% | 103% |
| Overall | -0.06% | -0.07% | -0.08% | -0.06% | -0.07% | -0.08% | 103% | 105% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.08% | -0.07% | -0.09% | -0.07% | -0.07% | -0.09% | 105% | 102% |
| PQ422 | -0.09% | -0.07% | -0.09% | -0.09% | -0.07% | -0.09% | 102% | 100% |
| Overall | -0.08% | -0.07% | -0.09% | -0.08% | -0.07% | -0.09% | 104% | 101% |
| Overall PQ | -0.16% | -0.13% | -0.14% | -0.16% | -0.13% | -0.14% | 103% | 102% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.48% | -0.28% | -0.29% | 103% | 101% |
| HLG422 | -0.52% | -0.33% | -0.34% | 103% | 101% |
| Overall | -0.50% | -0.30% | -0.32% | 103% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | 0.00% | 0.01% | 103% | 102% |
| HLG422 | -0.02% | -0.02% | 0.02% | 102% | 100% |
| Overall | -0.02% | -0.01% | 0.01% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | -0.01% | -0.02% | 103% | 104% |
| HLG422 | -0.04% | -0.01% | -0.01% | 103% | 99% |
| Overall | -0.04% | -0.01% | -0.01% | 103% | 101% |
| Overall HLG | -0.19% | -0.11% | -0.11% | 103% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.72% | -2.09% | -2.08% | 109% | 121% |
| SVT12 | -1.64% | -1.32% | -1.34% | 106% | 108% |
| Overall | -2.18% | -1.71% | -1.71% | 107% | 115% |
|  |  |  | LDB |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.51% | -1.58% | -1.57% | 117% | 115% |
| SVT12 | -0.71% | -0.63% | -0.63% | 105% | 111% |
| Overall | -1.11% | -1.10% | -1.10% | 111% | 113% |
|  |  |  | RA |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.43% | -1.46% | -1.45% | 121% | 119% |
| SVT12 | -0.66% | -0.53% | -0.54% | 105% | 108% |
| Overall | -1.04% | -0.99% | -0.99% | 113% | 114% |
| Overall RGB | -1.44% | -1.27% | -1.27% | 110% | 114% |

Table 8. Comparative performance of CE1.4-A test vs. CE Anchor, CE CTC, Normal QP setting.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.10% | -0.02% | -0.03% | -0.17% | -0.15% | -0.02% | -0.24% | -0.16% | 99% | 99% |
| Class H2 |  |  |  |  |  | 0.02% | -0.07% | 0.01% | 98% | 98% |
| Overall | 0.10% | -0.02% | -0.03% | -0.17% | -0.15% | -0.02% | -0.24% | -0.16% | 98% | 98% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.02% | -0.02% | -0.03% | 0.12% | -0.06% | -0.03% | 0.08% | -0.11% | 101% | 96% |
| Class H2 |  |  |  |  |  | -0.01% | -0.02% | 0.04% | 101% | 99% |
| Overall | 0.02% | -0.02% | -0.03% | 0.12% | -0.06% | -0.03% | 0.08% | -0.11% | 101% | 97% |

Table 9. Comparative performance of CE1.4-A test vs. CE Anchor, CE CTC, lossless setting.

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.4_A2 |  | AnchorCE | CE-1.4_A2 |  | AnchorCE | CE-1.4_A2 |  |
| PQ444 | 2.5 | 2.6 | -2.26% | 3.0 | 3.1 | -3.07% | 3.0 | 3.1 | -2.99% |
| PQ422 | 2.3 | 2.4 | -2.36% | 2.8 | 2.9 | -3.06% | 2.8 | 2.9 | -2.97% |
| Overall | 2.4 | 2.5 | -2.31% | 2.9 | 3.0 | -3.06% | 2.9 | 3.0 | -2.98% |
| Enc Time[%] | 108% |  |  | 124% |  |  | 128% |  |  |
| Dec Time[%] | 119% |  |  | 110% |  |  | 110% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.4_A2 |  | AnchorCE | CE-1.4_A2 |  | AnchorCE | CE-1.4_A2 |  |
| HLG444 | 1.7 | 1.8 | -3.54% | 1.9 | 2.0 | -4.40% | 1.9 | 2.0 | -4.32% |
| HLG422 | 1.6 | 1.7 | -3.44% | 1.9 | 1.9 | -4.21% | 1.9 | 1.9 | -4.12% |
| Overall | 1.7 | 1.7 | -3.49% | 1.9 | 1.9 | -4.30% | 1.9 | 2.0 | -4.22% |
| Enc Time[%] | 106% |  |  | 146% |  |  | 141% |  |  |
| Dec Time[%] | 151% |  |  | 132% |  |  | 118% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.4_A2 |  | AnchorCE | CE-1.4_A2 |  | AnchorCE | CE-1.4_A2 |  |
| SVT16 | 1.2 | 1.2 | -1.38% | 1.2 | 1.2 | -0.39% | 1.2 | 1.2 | -0.37% |
| SVT12 | 1.2 | 1.3 | -2.88% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.42% |
| Overall | 1.2 | 1.2 | -2.13% | 1.2 | 1.3 | -0.95% | 1.2 | 1.3 | -0.89% |
| Enc Time[%] | 110% |  |  | 221% |  |  | 216% |  |  |
| Dec Time[%] | 151% |  |  | 128% |  |  | 140% |  |  |

# CE-1.4-B Simulation results for HBD CTC

Table 10. Comparative performance of CE1.4-B test vs. CE Anchor, CE CTC, Low QP setting.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.32% | -0.22% | -0.22% | -0.35% | -0.22% | -0.23% | 103% | 101% |
| PQ422 | -0.33% | -0.25% | -0.27% | -0.35% | -0.24% | -0.27% | 103% | 103% |
| Overall | -0.33% | -0.23% | -0.25% | -0.35% | -0.23% | -0.25% | 103% | 102% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.06% | -0.08% | -0.09% | -0.06% | -0.07% | -0.08% | 104% | 110% |
| PQ422 | -0.07% | -0.08% | -0.09% | -0.07% | -0.08% | -0.08% | 104% | 105% |
| Overall | -0.06% | -0.08% | -0.09% | -0.06% | -0.08% | -0.08% | 104% | 108% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over AnchorCE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.08% | -0.09% | -0.09% | -0.08% | -0.09% | -0.09% | 104% | 102% |
| PQ422 | -0.09% | -0.07% | -0.10% | -0.09% | -0.08% | -0.09% | 102% | 100% |
| Overall | -0.08% | -0.08% | -0.09% | -0.08% | -0.08% | -0.09% | 103% | 101% |
| Overall PQ | -0.16% | -0.13% | -0.14% | -0.17% | -0.13% | -0.14% | 103% | 104% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.48% | -0.28% | -0.30% | 104% | 102% |
| HLG422 | -0.52% | -0.33% | -0.34% | 104% | 104% |
| Overall | -0.50% | -0.30% | -0.32% | 104% | 103% |
|  |  |  | LDB |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | 0.01% | 0.01% | 103% | 104% |
| HLG422 | -0.06% | -0.03% | -0.02% | 103% | 101% |
| Overall | -0.06% | -0.01% | 0.00% | 103% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.01% | -0.02% | 103% | 105% |
| HLG422 | -0.04% | -0.02% | -0.02% | 103% | 99% |
| Overall | -0.04% | -0.02% | -0.02% | 103% | 102% |
| Overall HLG | -0.20% | -0.11% | -0.11% | 103% | 102% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.70% | -2.05% | -2.04% | 109% | 124% |
| SVT12 | -1.64% | -1.32% | -1.33% | 107% | 108% |
| Overall | -2.17% | -1.69% | -1.69% | 108% | 116% |
|  |  |  | LDB |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.49% | -1.55% | -1.54% | 118% | 119% |
| SVT12 | -0.71% | -0.63% | -0.63% | 106% | 114% |
| Overall | -1.10% | -1.09% | -1.09% | 112% | 116% |
|  |  |  | RA |  |  |
|  |  |  | Over AnchorCE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.43% | -1.47% | -1.46% | 120% | 117% |
| SVT12 | -0.65% | -0.53% | -0.54% | 106% | 107% |
| Overall | -1.04% | -1.00% | -1.00% | 113% | 112% |
| Overall RGB | -1.44% | -1.26% | -1.26% | 111% | 115% |

Table 11. Comparative performance of CE1.4-B test vs. CE Anchor, CE CTC, Normal QP setting.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.01% | -0.03% | -0.05% | -0.29% | -0.25% | -0.05% | -0.38% | -0.25% | 98% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | -0.02% | -0.09% | 96% | 98% |
| Overall | -0.01% | -0.03% | -0.05% | -0.29% | -0.25% | -0.05% | -0.38% | -0.25% | 97% | 99% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over AnchorCE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | -0.02% | -0.02% | 0.06% | -0.07% | -0.03% | 0.01% | -0.06% | 101% | 100% |
| Class H2 |  |  |  |  |  | -0.01% | 0.00% | -0.04% | 100% | 97% |
| Overall | 0.00% | -0.02% | -0.02% | 0.06% | -0.07% | -0.03% | 0.01% | -0.06% | 101% | 98% |

Table 12. Comparative performance of CE1.4-B test vs. CE Anchor, CE CTC, lossless setting.

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.4_B2 |  | AnchorCE | CE-1.4_B2 |  | AnchorCE | CE-1.4_B2 |  |
| PQ444 | 2.5 | 2.6 | -1.97% | 3.0 | 3.1 | -2.86% | 3.0 | 3.1 | -2.79% |
| PQ422 | 2.3 | 2.4 | -2.10% | 2.8 | 2.9 | -2.87% | 2.8 | 2.9 | -2.79% |
| Overall | 2.4 | 2.5 | -2.03% | 2.9 | 3.0 | -2.87% | 2.9 | 3.0 | -2.79% |
| Enc Time[%] | 100% |  |  | 119% |  |  | 110% |  |  |
| Dec Time[%] | 122% |  |  | 108% |  |  | 104% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.4_B2 |  | AnchorCE | CE-1.4_B2 |  | AnchorCE | CE-1.4_B2 |  |
| HLG444 | 1.7 | 1.8 | -3.21% | 1.9 | 1.9 | -4.11% | 1.9 | 2.0 | -4.05% |
| HLG422 | 1.6 | 1.7 | -2.92% | 1.9 | 1.9 | -3.99% | 1.9 | 1.9 | -3.91% |
| Overall | 1.7 | 1.7 | -3.06% | 1.9 | 1.9 | -4.05% | 1.9 | 1.9 | -3.98% |
| Enc Time[%] | 101% |  |  | 128% |  |  | 145% |  |  |
| Dec Time[%] | 153% |  |  | 121% |  |  | 122% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AnchorCE | CE-1.4_B2 |  | AnchorCE | CE-1.4_B2 |  | AnchorCE | CE-1.4_B2 |  |
| SVT16 | 1.2 | 1.2 | -1.38% | 1.2 | 1.2 | -0.38% | 1.2 | 1.2 | -0.36% |
| SVT12 | 1.2 | 1.3 | -2.84% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.42% |
| Overall | 1.2 | 1.2 | -2.11% | 1.2 | 1.3 | -0.95% | 1.2 | 1.3 | -0.89% |
| Enc Time[%] | 103% |  |  | 225% |  |  | 221% |  |  |
| Dec Time[%] | 151% |  |  | 127% |  |  | 145% |  |  |

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[3]	JVET-U2018, "Common test conditions for high bit depth and high bit rate video coding", A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

[4]	JVET-U2022, "CE on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).