| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0053 |
| --- | --- |

| Title: | CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Marta Karczewicz Luong Pham Van Muhammed Coban | Email: | dmytror@qti.qualcomm.com martak@qti.qualcomm.com lphamvan@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution describes combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with transform coefficients clipping tests of CE-3.1/CE-3.2. Proposal targets High Bit Depth and High Bit Rate Coding configurations of VVC. Simulation results are reported as per CE requirements in JVET-U2022.

# Introduction

RRC tests CE1.1, CE1.2 and CE1.4 are being described in JVET-V0052. CE3.1 and CE3.1 tests on transform coefficient range extensions are being described in JVET-V0047. Current document describes combination of these tests: CE4.5, CE4.6 and CE4.7 and present simulation results for LowQP, NormalQp and lossless configurations of CE CTC, as described in JVET-U0022.

# CE-4.5

CE4.5 presents a direct combination RRC test CE1.1 with CE3.1 and CE3.2. No changes are introduced to either of these original tests. Complete set of simulation results for Low QP, Normal QP and lossless configurations are provided along this document.

# CE-4.6

CE4.6 presents a direct combination RRC test CE1.2 with CE3.1 and CE3.2. No changes are introduced to either of these original tests. Complete set of simulation results for Low QP, Normal QP and lossless configurations are provided along this document.

# CE-4.7

CE4.7 presents a direct combination RRC test CE1.4 with CE3.1 and CE3.2. No changes are introduced to either of these original tests. Complete set of simulation results for Low QP, Normal QP and lossless configurations are provided along this document.

# Simulation results

Complete simulation results are provided along this contribution. Summary of the results for given test is provide in tables below.

Table 1. Summary of simulation results for 12 bit data, CE CTC, Low QP configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE4.5-CE3.1 | -1.24% | -1.63% | -1.76% | -0.49% | -0.71% | -0.74% | -1.04% | -0.87% | -1.11% | -1.13% |
|  | CE4.5-CE3.2 | -1.14% | -1.51% | -1.62% | -0.45% | -0.66% | -0.69% | -0.95% | -0.83% | -1.00% | -1.03% |
|  | CE4.6-CE3.1 | -1.30% | -1.72% | -1.86% | -0.54% | -0.85% | -0.88% | -1.40% | -1.16% | -1.51% | -1.54% |
|  | CE4.6-CE3.2 | -1.20% | -1.59% | -1.71% | -0.51% | -0.80% | -0.82% | -1.26% | -1.07% | -1.35% | -1.37% |
|  | CE4.7-A-CE3.1 | -1.52% | -1.89% | -2.02% | -0.95% | -1.07% | -1.10% | -1.93% | -1.89% | -1.94% | -1.97% |
|  | CE4.7-A-CE3.2 | -1.42% | -1.75% | -1.87% | -0.91% | -1.02% | -1.05% | -1.89% | -1.88% | -1.88% | -1.90% |
|  | CE4.7-B-CE3.1 | -1.52% | -1.88% | -2.02% | -0.95% | -1.07% | -1.11% | -1.92% | -1.89% | -1.93% | -1.95% |
|  | CE4.7-B-CE3.2 | -1.42% | -1.74% | -1.88% | -0.91% | -1.02% | -1.05% | -1.88% | -1.88% | -1.87% | -1.89% |
| LDB | CE4.5-CE3.1 | -0.69% | -0.67% | -0.73% | -0.63% | -0.81% | -0.84% | -0.79% | -0.88% | -0.74% | -0.75% |
|  | CE4.5-CE3.2 | -0.65% | -0.63% | -0.69% | -0.59% | -0.76% | -0.80% | -0.70% | -0.78% | -0.65% | -0.66% |
|  | CE4.6-CE3.1 | -0.64% | -0.63% | -0.68% | -0.56% | -0.73% | -0.75% | -0.83% | -0.92% | -0.77% | -0.79% |
|  | CE4.6-CE3.2 | -0.62% | -0.59% | -0.65% | -0.53% | -0.69% | -0.72% | -0.76% | -0.84% | -0.71% | -0.72% |
|  | CE4.7-A-CE3.1 | -0.72% | -0.70% | -0.76% | -0.63% | -0.80% | -0.83% | -1.10% | -1.19% | -1.04% | -1.06% |
|  | CE4.7-A-CE3.2 | -0.69% | -0.67% | -0.74% | -0.59% | -0.74% | -0.79% | -1.06% | -1.15% | -1.01% | -1.02% |
|  | CE4.7-B-CE3.1 | -0.72% | -0.70% | -0.75% | -0.63% | -0.78% | -0.83% | -1.09% | -1.18% | -1.04% | -1.05% |
|  | CE4.7-B-CE3.2 | -0.69% | -0.67% | -0.74% | -0.59% | -0.74% | -0.77% | -1.06% | -1.14% | -1.01% | -1.02% |
| RA | CE4.5-CE3.1 | -0.61% | -0.64% | -0.71% | -0.51% | -0.73% | -0.80% | -0.73% | -0.71% | -0.74% | -0.75% |
|  | CE4.5-CE3.2 | -0.58% | -0.61% | -0.68% | -0.48% | -0.68% | -0.73% | -0.65% | -0.62% | -0.66% | -0.68% |
|  | CE4.6-CE3.1 | -0.60% | -0.63% | -0.69% | -0.47% | -0.69% | -0.74% | -0.76% | -0.75% | -0.76% | -0.78% |
|  | CE4.6-CE3.2 | -0.57% | -0.60% | -0.67% | -0.44% | -0.64% | -0.68% | -0.70% | -0.69% | -0.70% | -0.72% |
|  | CE4.7-A-CE3.1 | -0.67% | -0.70% | -0.77% | -0.53% | -0.73% | -0.78% | -0.98% | -1.01% | -0.96% | -0.98% |
|  | CE4.7-A-CE3.2 | -0.64% | -0.66% | -0.73% | -0.49% | -0.68% | -0.74% | -0.95% | -0.98% | -0.93% | -0.95% |
|  | CE4.7-B-CE3.1 | -0.68% | -0.69% | -0.76% | -0.53% | -0.73% | -0.79% | -0.98% | -1.01% | -0.96% | -0.98% |
|  | CE4.7-B-CE3.2 | -0.65% | -0.67% | -0.73% | -0.50% | -0.68% | -0.74% | -0.95% | -0.97% | -0.93% | -0.95% |

Table 2. Summary of simulation results for 16 bit data, CE CTC, Low QP configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE4.5-CE3.1 | -1.21% | -1.39% | -1.12% | -1.13% |
|  | CE4.5-CE3.2 | -1.21% | -1.39% | -1.12% | -1.13% |
|  | CE4.6-CE3.1 | -1.24% | -1.40% | -1.16% | -1.17% |
|  | CE4.6-CE3.2 | -1.24% | -1.40% | -1.16% | -1.17% |
|  | CE4.7-A-CE3.1 | -2.48% | -2.90% | -2.27% | -2.26% |
|  | CE4.7-A-CE3.2 | -2.48% | -2.90% | -2.27% | -2.26% |
|  | CE4.7-B-CE3.1 | -2.45% | -2.88% | -2.24% | -2.22% |
|  | CE4.7-B-CE3.2 | -2.45% | -2.88% | -2.24% | -2.22% |
| LDB | CE4.5-CE3.1 | -0.80% | -0.78% | -0.82% | -0.81% |
|  | CE4.5-CE3.2 | -0.80% | -0.78% | -0.82% | -0.81% |
|  | CE4.6-CE3.1 | -0.87% | -0.86% | -0.88% | -0.87% |
|  | CE4.6-CE3.2 | -0.87% | -0.86% | -0.88% | -0.87% |
|  | CE4.7-A-CE3.1 | -1.62% | -1.60% | -1.64% | -1.63% |
|  | CE4.7-A-CE3.2 | -1.62% | -1.60% | -1.64% | -1.63% |
|  | CE4.7-B-CE3.1 | -1.60% | -1.57% | -1.62% | -1.60% |
|  | CE4.7-B-CE3.2 | -1.60% | -1.57% | -1.62% | -1.60% |
| RA | CE4.5-CE3.1 | -0.77% | -0.59% | -0.86% | -0.85% |
|  | CE4.5-CE3.2 | -0.77% | -0.59% | -0.86% | -0.85% |
|  | CE4.6-CE3.1 | -0.85% | -0.68% | -0.94% | -0.93% |
|  | CE4.6-CE3.2 | -0.85% | -0.68% | -0.94% | -0.93% |
|  | CE4.7-A-CE3.1 | -1.52% | -1.41% | -1.58% | -1.57% |
|  | CE4.7-A-CE3.2 | -1.52% | -1.41% | -1.58% | -1.57% |
|  | CE4.7-B-CE3.1 | -1.53% | -1.41% | -1.59% | -1.58% |
|  | CE4.7-B-CE3.2 | -1.53% | -1.41% | -1.59% | -1.58% |

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[3]	JVET-U2018, "Common test conditions for high bit depth and high bit rate video coding", A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

[4]	JVET-U2022, "CE on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).