| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0058 |
| --- | --- |

| Title: | AHG9: On post-filter SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, Hamed R. Tavakoli | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes a post-filter nesting SEI message, which contains inter-related SEI messages jointly specifying a post-filtering operation.

It is envisioned that the nested SEI messages could include the following:

- An SEI message specifying the purpose for filtering, such as quality improvement or super-resolution.

  - This SEI message could also contain parameters specific to the purpose, such as an applicable scaling ratio range for a super-resolution filter.

- An SEI message specifying the filter itself. Such an SEI message could for example contain or reference (through a URI) an MPEG Neural Network Representation (NNR) bitstream.

This contribution leaves the design of the nested SEI messages to other contributions.

# Introduction

## Earlier contributions

JVET-U0091 [1] presented syntax and semantics for a neural-network-based post-filtering SEI message, which included the following aspects:

- Purpose for the post-filtering

| nn_postfilter_type_idc | Post filtering type |
| --- | --- |
| 0 | Visual quality improvement with single input picture |
| 1 | Visual quality improvement with multiple input pictures |
| 2 | Super resolution with single input picture |
| 3 | Super resolution with multiple input pictures |
| 4..15 | Reserved |

- Neural network topology and parameter storage formats, e.g. NNEF or ONNX

- Either a URI identifying the neural network topology or a coded representation of the topology

- Either a URI identifying the neural network parameter values or a coded representation of the parameter values

JVET-T0076 [2] proposed enabling the usage of ALF as a post-filter and presented additional sharpening and bokeh effect as use cases. The contribution pointed out that ALF post-filtering enables selective region-wise sharpening or blurring.

## Observations

We have the following observations based on the earlier contributions:

- Post-filtering can be performed for multiple purposes, including but not limited to those mentioned in JVET-U0091 and JVET-T0076.

- Different types of post-filters (e.g. ALF or a neural-network-based filter) can be used for the same purpose, such as generic visual quality improvement.

- There are different formats available for representing neural-network-based filters (e.g. NNEF, ONNX, MPEG NNR).

We also remark that a principle to avoid extending an existing SEI message has been followed in the design of SEI messages. In our understanding, code point values that have been previously reserved in an SEI message cannot be taken into use in a later version of the standard according to the agreed design principle. Consequently, assuming that VSEI v2 contains a post-filtering SEI message, introducing a new post-filtering type or purpose in a VSEI v3 would require a new post-filter SEI message, which would also duplicate many aspects of the post-filtering SEI message of VSEI v2.

As a conclusion, we think that the design of a monolithic post-filtering SEI message is not desirable but rather it would be better to split post-filtering information into several SEI messages to improve future extensibility and re-using the same post-filter formats for different post-filtering purposes.

# Proposal

## Overview

The contribution proposes a post-filter nesting SEI message, which contains inter-related SEI messages jointly specifying a post-filtering operation.

It is envisioned that the nested SEI messages could include the following:

- An SEI message specifying the purpose for filtering, such as quality improvement or super-resolution.

  - This SEI message could also contain parameters specific to the purpose, such as an applicable scaling ratio range for a super-resolution filter.

- An SEI message specifying the filter itself. Such an SEI message could for example contain or reference (through a URI) an MPEG Neural Network Representation (NNR) bitstream.

This contribution leaves the design of the nested SEI messages to other contributions.

## Proposed syntax

| post_filter_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pfn_id | ue(v) |
| pfn_cancel_flag | u(1) |
| if( !pfn_cancel_flag ) { |  |
| pfn_persistence_flag | u(1) |
| pfn_num_seis_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| pfn_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i <= pfn_num_seis_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |
| } |  |

## Proposed semantics

This SEI message contains one or more SEI messages that jointly specify post-filtering and are referred to as post-filter-nested SEI messages.

pfn_id contains an identifying number that may be used to identify the usage of the post-filter nesting SEI message. The value of pfn_id shall be in the range of 0 to 232 − 2, inclusive.

Values of pfn_id from 0 to 255, inclusive, and from 512 to 231 − 1, inclusive, may be used as determined by the application. Values of pfn_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of pfn_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

The post-filter nesting SEI message with a particular pfn_id value cancels the persistence of any previous post-filter nesting SEI message in output order with the same pfn_id value.

pfn_cancel_flag equal to 1 indicates that no post-filter operation associated with the pfn_id value is specified in this SEI message. pfn_cancel_flag equal to 0 indicates that a post-filter operation associated with the pfn_id value is specified by the post-filter-nested SEI messages contained in this SEI message.

pfn_persistence_flag specifies the persistence of the post-filter nesting SEI message for the current layer.

pfn_persistence_flag equal to 0 specifies that the post-filter nesting SEI message applies to the current decoded frame only.

pfn_persistence_flag equal to 1 specifies that the post-filter nesting SEI message applies to the current decoded picture and persists all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture that follows the current picture in output order and is associated with a post-filter nesting SEI message with the same pfn_id value as that in this SEI message is output.

pfn_num_seis_minus1 plus 1 specifies the number of post-filter-nested SEI messages. The value of pfn_num_seis_minus1 shall be in the range of 0 to 63, inclusive.

pfn_zero_bit shall be equal to 0.

# Examples

## Example 1

The following pseudo-code presents a hypothetical example of a post-filter nesting SEI message that contains:

- a purpose SEI message (e.g. indicating super-resolution) and

- an MPEG NNR filter SEI message that contains a neural network represented as an MPEG NNR bitstream.

| post_filter_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| post_filter_purpose_sei_message( ) |  |
| post_filter_mpeg_nnr( ) |  |
| } |  |

## Example 2

The following pseudo-code presents a hypothetical example of a post-filter nesting SEI message that contains:

- a purpose SEI message (e.g. indicating super-resolution) and

- two SEI messages specifying a neural network used for the filtering, one for indicating the topology and another for indicating the parameter values, both using NNEF.

The coded representation of the NNEF-related SEI messages could be similar to the respective sections of the neural-network-based post-filter SEI message proposed in JVET-U0091.

| post_filter_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| post_filter_purpose_sei_message( ) |  |
| post_filter_nnef_topology( ) |  |
| post_filter_nnef_parameters( ) |  |
| } |  |

## Example 3

The following pseudo-code presents a hypothetical example of a post-filter nesting SEI message that contains:

- a purpose SEI message specified in VSEI v3, indicating a purpose that was not envisioned or included in VSEI v2, and

- an MPEG NNR filter SEI message that contains a neural network represented as an MPEG NNR bitstream.

| post_filter_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| post_filter_appended_purpose_sei_message( ) |  |
| post_filter_mpeg_nnr( ) |  |
| } |  |

# References

- Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Stephan Wenger, Shan Liu, "AHG9/AHG11: SEI message for carriage of neural network information for post filtering", JVET-U0091, Jan 2021.

- Han Boon Teo, Hai Wei Sun, Chong Soon Lim, "AHG9: ALF control SEI message", JVET-T0076, Oct. 2020.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).