| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0061-v2 |
| --- | --- |

| Title: | AHG9: Display orientation information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Muhammed Coban, Marta Karczewicz Jill Boyce | Email: | yonghe@qti.qualcomm.com jill.boyce@intel.com |
| Source: | Qualcomm Inc., Intel |  |  |

_____________________________

# Abstract

This contribution proposes the inclusion of display orientation information in the VSEI specification. The display orientation SEI message informs the decoder of a transformation that is recommended to be applied to the cropped decoded picture prior to display.

# Introduction

Display orientation SEI message was supported in HEVC to indicate the flip or rotation operation of a decoded picture for display. The benefits of orientation information may also apply to VVC based applications.

The display orientation SEI message enhances the user viewing experience by aligning the orientation of capture device and display for correct rendering. Most user-generated content (UGC) is captured with mobile devices with orientation sensors that measure the orientation of the device relative to the axis of gravity, while the content may be presented at various viewing displays. The content may be captured in a portrait or landscape mode using front or back cameras, hence capturing video or picture that may be flipped or rotated. For proper viewing, the video should be inversely flipped or rotated accordingly before presentation to a viewer.

A coding efficiency difference was observed for some rotated sequences in the original picture orientation information proposal JCTVC-E280. A similar difference is also observed for VVC. The proposed display orientation SEI message may enable the encoder to achieve coding efficiency savings for some sequences when the flip and/or rotation is applied.

Table 1 and Table 2 show those sequences with more than 1% BD-rate difference when the flip or rotation is applied.

Table 1 AI-10 BD-rate (VTM-11.0)

| Class | Sequence | orientation | BD-rate (piecewise cubic) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V |
| Class A1 | Campfire | Horizontal flip | 2.88% | 8.90% | 24.85% |
|  |  | Rotation by 90 degrees | 1.97% | 4.08% | 17.83% |
| Class B | BQTerrace | Horizontal flip | -1.76% | -0.70% | 4.65% |
|  |  | Rotation by 90 degrees | -1.33% | 2.50% | 0.63% |
| Class C | BasketballDrill | Horizontal flip | 4.09% | 14.30% | 17.26% |
|  |  | Rotation by 90 degrees | 2.96% | 9.52% | 12.22% |
| Class D | BasketballPass | Horizontal flip | 1.27% | 3.61% | 3.29% |
|  |  | Rotation by 90 degrees | 1.40% | 3.83% | 3.03% |
| Class F | BasketballDrillText | Horizontal flip | 4.01% | 11.57% | 14.22% |
|  |  | Rotation by 90 degrees | 3.81% | 9.94% | 12.35% |
|  | ArenaOfValor | Horizontal flip | 1.11% | 7.44% | 6.26% |
|  |  | Rotation by 90 degrees | 1.22% | 6.57% | 6.01% |
|  | SlideEditing | Horizontal flip | 2.48% | 5.13% | 19.56% |
|  |  | Rotation by 90 degrees | 6.87% | 7.33% | 21.32% |

Table 2 RA-10 BD-rate (VTM-11.0)

| Class | Sequence | orientation | BD-rate (piecewise cubic) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V |
| Class A1 | Campfire | Horizontal flip | 1.98% | 3.44% | 11.22% |
|  |  | Rotation by 90 degrees | 0.43% | 1.35% | 6.46% |
| Class B | BQTerrace | Horizontal flip | -1.18% | 0.64% | 7.28% |
|  |  | Rotation by 90 degrees | -0.50% | 1.73% | 0.28% |
| Class C | BasketballDrill | Horizontal flip | 1.42% | 6.82% | -3.55% |
|  |  | Rotation by 90 degrees | 1.54% | 4.56% | 5.66% |
| Class D | BasketballPass | Horizontal flip | 0.45% | 5.53% | 4.16% |
|  |  | Rotation by 90 degrees | 1.40% | 3.83% | 3.03% |
| Class F | BasketballDrillText | Horizontal flip | 1.33% | 7.68% | 7.68% |
|  |  | Rotation by 90 degrees | 1.15% | 5.12% | 6.27% |
|  | ArenaOfValor | Horizontal flip | 0.40% | 5.45% | 4.31% |
|  |  | Rotation by 90 degrees | 0.63% | 4.34% | 3.81% |
|  | SlideEditing | Horizontal flip | 2.20% | 2.50% | 7.90% |
|  |  | Rotation by 90 degrees | 6.83% | 7.84% | 11.34% |

# Proposal

The proposed display orientation SEI message is provided below:

Table 3 Display orientation SEI message

| display_orientation( payloadSize ) { | Descriptor |
| --- | --- |
| dor_cancel_flag | u(1) |
| if( !dor_cancel_flag ) { |  |
| dor_persistence_flag | u(1) |
| dor_constituent_picture_matching_flag | u(1) |
| dor_transform_type | u(3) |
| dor_reserved_zero_2bits | u(2) |
| } |  |
| } |  |

The DOR SEI message provides information to inform the decoder of a transformation that is recommended to be applied to the cropped decoded picture prior to display.

dor_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous DOR SEI message in output order. dor_cancel_flag equal to 0 indicates that DOR information follows.

dor_persistence_flag specifies the persistence of the DOR SEI message for the current layer.

dor_persistence_flag equal to 0 specifies that the DOR SEI message applies to the current decoded picture only.

dor_persistence_flag equal to 1 specifies that the DOR SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a DOR SEI message is output that follows the current picture in output order.

If a frame packing arrangement SEI message with fp_arrangement_cancel_flag equal to 0, fp_arrangement_type equal to 3, 4, or 5, and fp_quincunx_sampling_flag equal to 0 is not present that applies to the current picture, the variable StereoFlag is set equal to 0. Otherwise StereoFlag is set equal to 1.

dor_constituent_picture_flag equal to 1 specifies that this SEI message applies individually to each constituent picture of the stereoscopic frame packing format indicated by the frame packing arrangement SEI message. dor_constituent_picture_flag equal to 0 specifies that this SEI message applies to the cropped decoded picture.

When a frame packing arrangement SEI message with fp_arrangement_cancel_flag equal to 0, fp_arrangement_type equal to 3, 4, or 5, and fp_quincunx_sampling_flag equal to 0 is not present that applies to the current picture, or a frame packing arrangement SEI message with fp_arrangement_type is equal to 5 applies to the current picture, the value of dor_constituent_picture_flag shall be equal to 0.

dor_transform_type specifies the rotation and mirroring to be applied to the picture. When por_transform_type specifies both rotation and mirroring, rotation applies before mirroring. The values of por_transform_type are specified in Table 3.

Table 3 dor_transform_type values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (anticlockwise) |
| 3 | rotation by 180 degrees (anticlockwise) before mirroring horizontally |
| 4 | rotation by 90 degrees (anticlockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (anticlockwise) |
| 6 | rotation by 270 degrees (anticlockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (anticlockwise) |

# dor_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of reserved_zero_2bits.

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Intel has no current or pending patent rights relating to the technology described in this contribution.