| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0062-v2 |
| --- | --- |

| Title: | AHG9: Picture quality metrics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the inclusion of picture quality metrics in the VSEI specification. The message provides a quality metric for each colour component of the current decoded picture.

# Introduction

Picture quality metrics are used to measure a vareity of picture distortions during the acquisition, processing, compression, storage, transmission and reproduction.

ISO/IEC 23001-10, carriage of timed metadata metrics of media in ISOBMFF, specifies the quality metrics of media such as PSNR, SSIM, MS-SSIM, VQM, PEVQ and MOS in file format.

The quality relevant quality ranking is specified in ISO/IEC 23090-2 OMAF to indicate the a relative quality order of quality ranking region. The value of the ranking may be derived from specific quality metrics.

ISO/IEC 23090-6, Immersive Media Metrics, also uses quality ranking to measure the viewport switching latency.

Some objective quality metrics such as PSNR and SSIM can only be obtained at the encoder side, a SEI message to carry such information is able to provide the relevant information to the application. In addition, such SEI message may also be used for the picture coding performance indication and verification, similar as picture HASH SEI message.

# Proposal

The proposed picture metrics SEI message is provided below:

Table 3 Picture quality metric SEI message

| Picture_quality_metrics( payloadSize ) { | Descriptor |
| --- | --- |
| pqm_metric_type | u(7) |
| pqm_single_component_flag | u(1) |
| for( cIdx = 0; cIdx < ( pqm_single_component_flag ? 1 : 3 ); cIdx++ ){ |  |
| if( pqm_metric_type = = 0 ) |  |
| pqm_psnr[ cIdx ] | u(16) |
| else if ( pqm_metric_type = = 1 ) |  |
| pqm_ssim[ cIdx ] | u(8) |
| else if ( pqm_metric_type == 2 ) |  |
| pqm_ms_ssim[ cIdx ] | u(8) |
| } |  |
| } |  |

This message provides quality metric for each colour component of the current decoded picture.

pqm_metric_type indicates the quality metric type associated with the component as specified in Table 5. The value of pqm_metric_type from 3 to 127 are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification.

Table 5 Interpretation of pqm_metric_type

| pqm_metric_type | metric |
| --- | --- |
| 0 | PNSR |
| 1 | SSIM |
| 2 | MS-SSIM |

pqm_single_component_flag equal to 1 specifies that the picture associated with the picture quality metrics SEI message contains a single colour component. pqm_ single_component_flag equal to 0 specifies that the picture associated with the picture quality metrics SEI message contains three colour components. The value of pqm_single_component_flag shall be equal to (ChromaFormatIdc = = 0).

pqm_psnr[ cIdx ]  specifies the value of PSNR metric. The corresponding PSNR value (in dB) of the colour component cIdx of the decoded picture is derived as follows (expressed in floating point):

PSNR = pqm_psnr[ cIdx ] / 100; with the exception of PSNR = infinity for pqm_psnr[ cIdx ] is equal to 0

pqm_ssim[ cIdx ]  specifies the value of SSIM. The corresponding SSIM of the colour component cIdx of the decoded picture is derived as follows (expressed in floating point):

SSIM = ( pqm_ssim[ cIdx ] – 127 ) / 128

pqm_ms_ssim[ cIdx ]  specifies the value of MS-SSIM. The corresponding MS-SSIM of the colour component cIdx of the decoded picture is derived as follows (expressed in floating point):

MS-SSIM = ( pqm_ms_ssim[ cIdx ] – 127 ) / 128

# Quality metrics definition

## Peak Signal to Noise Ratio (PSNR)

PSNR for encoded picture is defined based on mean square error (MSE) differences:

where I denotes component plane of the reference m×n picture, K denotes component plane of the reconstructed picture, and i,j denote indices enumerating all pixel locations.

The picture-level PSNR is defined as:

where MAXI  = 2B−1 where B is the number of bits per sample in pictures.

## Structural SIMilarity Index (SSIM)

SSIM index map for encoded picture is computed as follows:

where

x   	denotes the 8x8 window in the reference picture

y 	denotes the 8x8 window in the reconstructed picture

denotes the average sample value for pixels in x

denotes the average sample value for pixels in y

denotes the variance computed for pixel values in x

denotes the variance computed for pixel values in y;

denotes the covariance computed for pixel values in x and y;

and where

,

are constants computed using

, , and  L = 2B−1

where B is the number of bits per sample in reference picture.

This formula is applied using an 8x8 sliding window, and producing a map of SSIM index values for all pixel positions within a picture. The overall SSIM index is then computed as the average of index values in the SSIM map.

## Multi-Scale Structural SIMilarity index

MS-SSIM calculation procedure is described in Figure 1. Taking the reference and distorted image signals as the input, the system iteratively applies a low-pass filter and downsamples the filtered image by a factor of 2. The original scale is indexed by j=1 and the highest scale is indexed by j=M, for M-1 levels of iteration. Further details can be found in[1].

Figure 1 – MS-SSIM calculation procedure

Based on such M scales of processing, MS-SSIM for encoded picture is defined as follows:

,

where:

- is the contrast comparison at scale j (j=1,…M) given by

- is the structure comparison at scale j (j=1,…,M) given by

- is the component comparison (only computed at scale M) given by

where

x   	denotes the 8x8 window in the reference picture

y 	denotes the 8x8 window in the reconstructed picture

denotes the average sample value for pixels in x

denotes the average sample value for pixels in y

denotes the variance computed for pixel values in x

denotes the variance computed for pixel values in y;

denotes the covariance computed for pixel values in x and y;

and where

, , ,  and

are constants computed using

, , and  L = 2B−1

where B is the number of bits per sample in reference video.

# Reference

- Z. Wang, E. P. Simoncelli, and A. C. Bovik. "Multiscale structural similarity for image quality assessment." Signals, Systems and Computers, 2003. Conference Record of the Thirty-Seventh Asilomar Conference on. Vol. 2. IEEE, 2003.

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).